| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0012-v1 |
| --- | --- |

| Title: | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | M. Karczewicz Y. Ye L. Zhang B. Bross X. Li R. Chernyak K. Naser Y. Yu | Tel: Email: | martak@qti.qualcomm.com yan.ye@alibaba-inc.com lizhang.idm@bytedance.com benjamin.bross@hhi.fraunhofer.de xlxiangli@google.com chernyak@global.tencent.com karam.naser@interdigital.com yue.yu@oppo.com |
| Source: | AHG 12 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on enhanced compression beyond VVC capability (AHG12) between the 34th meeting, Rennes, FR, 17–24 April 2024, and the 35th meeting, Sapporo, Japan, 12–19 July 2024.

# Introduction

The mandates given to the AHG are:

- Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC.

- Discuss and propose refinements to the ECM13 algorithm description JVET-AH2025.

- Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools.

- Coordinate with AHG6 on ECM software development.

- Support AHG6 in generating anchors according to the test conditions in JVET-AF2017.

- Analyse the results of exploration experiments described in JVET-AH2024 in coordination with the EE coordinators.

- Coordinate with AHG11 to study the interaction with neural network-based coding tools.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A few emails were exchanged regarding EE2.

# Activities

The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-AH2024). The combined improvements of the ECM-13.0 over VTM-11.0ecm13.0 anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -13.18% | -13.89% | -24.88% | 980.4% | 467.2% |
| Class A2 | -20.15% | -22.13% | -26.46% | 984.0% | 487.8% |
| Class B | -13.54% | -20.05% | -18.04% | 920.7% | 475.4% |
| Class C | -13.58% | -10.35% | -11.48% | 937.0% | 437.6% |
| Class E | -17.30% | -20.19% | -17.25% | 898.4% | 497.5% |
| Overall | -15.22% | -17.24% | -18.99% | 940.6% | 470.9% |
| Class D | -11.47% | -7.48% | -7.91% | 920.9% | 477.3% |
| Class F | -29.12% | -32.42% | -32.73% | 571.1% | 498.7% |
| Class TGM | -42.06% | -47.48% | -46.92% | 459.9% | 468.9% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -26.04% | -21.59% | -33.88% | 946.3% | 892.5% |
| Class A2 | -29.16% | -31.81% | -36.71% | 887.1% | 1036.9% |
| Class B | -23.43% | -28.98% | -26.76% | 747.6% | 846.1% |
| Class C | -24.99% | -20.48% | -21.31% | 821.5% | 922.3% |
| Class E |  |  |  |  |  |
| Overall | -25.51% | -25.80% | -28.72% | 831.6% | 911.4% |
| Class D | -25.69% | -21.07% | -22.42% | 785.7% | 1008.7% |
| Class F | -31.65% | -34.02% | -34.69% | 660.1% | 557.6% |
| Class TGM | -40.63% | -45.97% | -46.06% | 634.7% | 487.9% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -20.42% | -32.78% | -30.25% | 739.1% | 718.3% |
| Class C | -22.71% | -23.31% | -24.26% | 755.1% | 802.1% |
| Class E | -20.11% | -22.85% | -22.09% | 694.2% | 500.6% |
| Overall | -21.11% | -27.14% | -26.21% | 732.8% | 680.9% |
| Class D | -23.81% | -25.09% | -24.57% | 745.0% | 892.1% |
| Class F | -28.69% | -36.15% | -36.26% | 627.5% | 550.3% |
| Class TGM | -38.57% | -47.78% | -48.13% | 584.2% | 493.6% |

The rate reduction for natural sequences over VTM in RA configuration for {Y, U, V} increased from ECM-12.0’s {-24.01%, -33.20%, -35.34%} to ECM-13.0’s {-25.51%, -25.80%, -28.72%}. For SCC sequences (class TGM) the rate reduction for RA configuration increased from ECM-21.0’s { -38.31%, -45.25%, -45.36%} to ECM-13.0’s { -40.63%, -45.97%, -46.06%}.

# Contributions

In addition to 39 EE2 contributions, 51 (comparing to 51 last meeting) EE2-related and AHG12-related contributions were received. The EE2-related and AHG12-related contributions can be subdivided as follows:

## Intra (21)

JVET-AI0076, "AHG12: The Modification of BV candidate List Construction", T. Lee, S. Kim, B. Park, D. Lee, J. Seo, D. Jun (DONG-A Univ.)

JVET-AI0079, "Non-EE2: Improved OBIC with PDP", J. Fu, Z. Li, J. Zhang, C. Jia, S. Ma (PKU), Y. Gao, C. Huang (ZTE)

JVET-AI0080, "Non-EE2: Disabling the combination of TIMD-ISP and TIMD-MRL", Z. Fan, T. Chujoh, T. Ikai (Sharp)

JVET-AI0085, "Non-EE2: Matrix-based position dependent intra prediction for TIMD", Y. Kidani, H. Kato, K. Kawamura (KDDI)

JVET-AI0106, "AHG12: Adaptive HoG for DIMD", S. Blasi, J. Lainema (Nokia)

JVET-AI0108, "Non-EE2: Unified reference area of IBC and IntraTMP", N. Zhang, K. Zhang, Y. Wang, Z. Deng, L. Zhao, W. Yin, L. Zhang (Bytedance)

JVET-AI0101, "Non-EE2: High-level control of intra prediction methods for screen contents", K. Kim, J.-H. Son, J.-S. Kwak (WILUS)

JVET-AI0102, "Non-EE2: Regression-based SGPM blending", X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)

JVET-AI0125, "Non-EE2: DIMD with filtered-template", Z. Lv, C. Zhou (vivo)

JVET-AI0132, "AHG12: SGPM improvements", K. Ding, H. Qin, J. Konieczny, A. Filippov, V. Rufitskiy (TCL)

JVET-AI0139, "Non-EE2: Modified L-shape template in IntraTMP", J. Lee, J. Kim, W. Lim, D. Kim, S.-C. Lim (ETRI)

JVET-AI0140, "Non-EE2: On DIMD edge operators", A. Filippov, V. Rufitskiy, J. Konieczny, H. Qin, T. Dong, Z. Xu (TCL)

JVET-AI0148, "Non-EE2: Non-adjacent DIMD mode derivation for TMRL intra mode candidate list construction", V. Rufitskiy, A. Filippov, J. Konieczny, H. Qin (TCL)

JVET-AI0149, "Non-EE2: Unified design of DIMD and non-adjacent DIMD", A. Filippov, V. Rufitskiy, J. Konieczny, H. Qin, K. Ding, T. Dong, Z. Xu (TCL)

JVET-AI0167, "Non-EE2: Improvements on EIP", L. Zhang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AI0168, "Non-EE2: Chroma TMRL", H. Huang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AI0201, "AHG12: neural network-based intra prediction", F. Urban, T. Dumas, F. Galpin, E. François (Interdigital)

JVET-AI0224, "EE2-related: Intra merge mode extension with BV improvement", J. Fu, Z. Li, J. Zhang, C. Jia, S. Ma (PKU), Y. Gao, C. Huang (ZTE)

JVET-AI0225, "AhG12: Neural network-based intra prediction with DIMD mode derivation", S. Eadie, P. Garus, T. Ryder, P. Nikitin, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AI0253, "EE2 related: BV Merge List Improvement", K. Naser, Y. Chen, M. Radosavljević, F. Le Léannec (InterDigital)

JVET-AI0262, "AHG 12: OBIC mode from reconstruction", R. G. Youvalari, M. Abdoli, A. Tissier (Xiaomi)

## Inter (14)

JVET-AI0051, "Template Matching Picture Boundary Padding for ECM", N. Neumann, M. Wien (RWTH Aachen Univ.)

JVET-AI0067, "AHG12: GPM mode extension", Y. Ahn, J. Nam, N. Park, J. Lim, S. Kim (LGE)

JVET-AI0109, "Non-EE2: A dual merge mode", R.-L. Liao, J. Chen, Y. Ye, X. Li (Alibaba)

JVET-AI0115, "Non-EE2: Regression-based GPM extension", K. Jia, J. Chen, X. Li, R.-L. Liao, Y. Ye (Alibaba)

JVET-AI0118, "AHG 12: Alternative affine motion model for affine DMVR", K. Andersson (Ericsson)

JVET-AI0121, "AHG 12: Fast hierarchical Subblock based DMVR", K. Andersson, C. Hollmann (Ericsson)

JVET-AI0126, "Non-EE2: Enhanced inter AMVP", Z. Deng, K. Zhang, N. Zhang, Y. Wang, W. Yin, L. Zhao, L. Zhang (Bytedance)

JVET-AI0154, "Non-EE2: OBMC extension with intra prediction", D. Kim, J. Lee, J. Kim, W. Lim, S.-C. Lim, J. S. Choi (ETRI)

JVET-AI0169, "Non-EE2: On sub-block merge mode", F. Wang, Y. Yu, H. Yu, D. Wang(OPPO)

JVET-AI0198, "Non-EE2: On POC conditions of BDOF", N. Yan, X. Xiu, W. Chen, X. Wang (Kwai)

JVET-AI0217, "EE2-related: on the sharp motion compensation filter for bi-prediction", H. Huang, R. Yu, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AI0218, "Non-EE2: On subblock inter mode improvement", H. Huang, Y. Zhang, J.-L Lin, Z. Zhang, P.-H Lin, C.-C Chen, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AI0226, "AHG 12: Longer motion compensation interpolation filter", R. Yu, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AI0229, "EE2-Related: LIC for Screen Content", T.M. Bae, S. Deshpande (Sharp)

## In Loop Filters (5)

JVET-AI0081, "AHG12: Adaptive Mantissa Precision for ALF Coefficients", V. Shchukin, P. Wennersten, J. Ström (Ericsson)

JVET-AI0135, "Non-EE2: Extended Usage of Fixed-Filters", W. Yin, K. Zhang, Y. Wang, Z. Deng, L. Zhao, N. Zhang, L. Zhang (Bytedance)

JVET-AI0152, "Non-EE2: Coding Information based Classification for ALF", W. Yin, K. Zhang, Y. Wang, Z. Deng, L. Zhao, N. Zhang, L. Zhang (Bytedance)

JVET-AI0200, "Non-EE2: Using Luma Reconstruction Signal for Chroma ALF", C. Ma, X. Xiu, X. Wang (Kwai)

JVET-AI0280, "No-EE2: Reuse of ALF control information", M. Karczewicz, H. Wang, N. Hu, V. Seregin (Qualcomm)

## Transform (2)

JVET-AI0223, "AHG12: Improved implicit MTS", C. Bonnineau, S. Puri, I. Marzuki, R. Utida, K. Naser, T. Poirier, F. Le Léannec (InterDigital)

JVET-AI0255, "Non-EE2: NSPT kernels for non-regular intra modes",	M. Karczewicz, G. Verba, B. Ray, P. Garus, M. Coban, V. Seregin (Qualcomm)

## Entropy Coding (3)

JVET-AI0145, "AHG12: CABAC slice-level model switch", F. Lo Bianco, F. Galpin (InterDigital)

JVET-AI0119, "Context modeling for tu_cb_coded_flag", M. Chen, L. Wang, W. Zhang, F. Yang, J. Huo (Xidian Univ.), Z. Yu, W. Li, B. Li, F. Xing (Hisense)

JVET-AI0281, "AHG12: Context retraining for ECM-13.0", P. Mahdavinia, P. Nikitin, V. Seregin, M. Karczewicz (Qualcomm)

## Inter and Intra (3)

JVET-AI0063, "AHG12: Improvements on GPM partitioning splits", K. Reuzé, Y. Chen, M. Le Pendu, P. Bordes (InterDigital)

JVET-AI0096, "AHG12: Fixes for Adaptive Clipping and Sign Prediction", J. Samuelsson-Allendes (Sharp)

JVET-AI0114, "Non-EE2: SATD-based reordering", Y. Wang, K. Zhang, W. Yin, Z. Deng, N. Zhang, L. Zhao, L. Zhang (Bytedance)

## Partitioning (2)

JVET-AI0261, "AHG 12: Rectangular Partitioning Restrictions	A. Tissier", M. Abdoli, R. G. Youvalari (Xiaomi)

JVET-AI0282, "AHG12: Corner and center subblocks for SBT", V. Seregin, M. Coban, B. Ray, H. Wang, N. Hu, M. Karczewicz (Qualcomm)

## CTC (1)

JVET-AI0097, "AHG12: A follow-up CTC modification for encoding runtime reduction under low-delay configuration", Z. Deng, K. Zhang, L. Zhao, W. Yin, Y. Wang, N. Zhang, L. Zhang (Bytedance), R. Utida, S. Puri, I. Marzuki, C. Bonnineau (InterDigital)

# Recommendations

The AHG recommends to:

- To review all the related contributions.