| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JT, 12–19 July 2024 | Document: JVET-AI0045-v1 |
| --- | --- |

| Title: | 4:4:4 MV-HEVC profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis M. Tourapis, Dimitri Podborski, Seethal Paluri Apple Inc. | Email: | {atourapis, dpodborski, s_paluri} @apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

Currently the HEVC specification does not define any multiview profile capable of handling 4:2:2 or 4:4:4 video formats. Such profiles can be quite useful not only for professional applications and content creators but also for consumer applications such as screen casting and video-based 3D gaming. This contribution proposes creating two new HEVC profiles, the Multiview 4:4:4 10 and Multiview 4:4:4 12 profiles, which will be able to support these chroma formats as well as allow support for up to 10 and 12 bits, respectively.

# Changes to the specification text of HEVC

# Subclause F.11.2

Replace Table F.3 with the following:

Table F.3 – Specification of CompatibleProfileList

| Profile to which the decoder conforms | Profiles that the decoder shall support CompatibleProfileList |
| --- | --- |
| Scalable Main | Scalable Main, Main, Main Still Picture |
| Scalable Main 10 | Scalable Main 10, Main, Main Still Picture, Main 10, Scalable Main |
| Scalable Monochrome | The compatible format range extensions profiles of the Monochrome profile, and the compatible scalable format range extensions profiles of the Scalable Monochrome profile |
| Scalable Monochrome 12 | The compatible format range extensions profiles of the Monochrome 12 profile, and the compatible scalable format range extensions profiles of the Scalable Monochrome 12 profile |
| Scalable Monochrome 16 | The compatible format range extensions profiles of the Monochrome 16 profile, and the compatible scalable format range extensions profiles of the Scalable Monochrome 16 profile |
| Scalable Main 4:4:4 | Scalable Main, Main, Main Still Picture, the compatible format range extensions profiles of the Main 4:4:4 profile, and the compatible scalable format range extensions profiles of the Scalable Main 4:4:4 profile |
| Multiview Main | Multiview Main, Main, Main Still Picture |
| Multiview Extended | Multiview Extended, Multiview Main, Main, Main Still Picture |
| Multiview Extended 10 | Multiview Extended 10, Main, Main Still Picture, Main 10, Multiview Main, Multiview Extended |
| Multiview Monochrome | Multiview Monochrome, Monochrome |
| Multiview Monochrome 10 | Multiview Monochrome 10, Multiview Monochrome, Monochrome 10, Monochrome |
| Multiview Monochrome 12 | Multiview Monochrome 12, Multiview Monochrome 10, Multiview Monochrome, Monochrome 12, Monochrome 10, Monochrome |
| Multiview Monochrome 16 | Multiview Monochrome 16, Multiview Monochrome 12, Multiview Monochrome 10, Multiview Monochrome, Monochrome 16, Monochrome 12, Monochrome 10, Monochrome |
| Multiview 4:4:4 10 | Multiview 4:4:4 10, Multiview Extended 10, Multiview Extended, Multiview Main, Multiview Monochrome 10, Multiview Monochrome, Main 4:4:4 10, Main 4:4:4, Main 10, Main, Monochrome 10, Monochrome |
| Multiview 4:4:4 12 | Multiview 4:4:4 12, Multiview 4:4:4 10, Multiview Extended 10, Multiview Extended, Multiview Main, Multiview Monochrome 12, Multiview Monochrome 10, Multiview Monochrome, Main 4:4:4 12, Main 4:4:4 10, Main 4:4:4, Main 12, Main 10, Main, Monochrome 10, Monochrome, |
| 3D Main | 3D Main, Multiview Main, Main, Main Still Picture |

Modify subclause G.11.1.2 as follows:

      - G.11.1.2 Multiview format range extensions profiles

The following profiles, collectively referred to as the multiview format range extensions profiles, are specified in this subclause:

- the Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16 profiles.

- the Multiview 4:4:4 10 and Multiview 4:4:4 12 profiles.

For a layer in an output operation point associated with an OLS in a bitstream, the layer conforming to the Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, or Multiview Monochrome 16, Multiview 4:4:4 10, or Multiview 4:4:4 12 profile, the following applies:

- Let olsIdx be the OLS index of the OLS, the sub-bitstream subBitstream is derived as specified in F.11.3.

The sub-bitstream subBitstream and the layer (referred to as the current layer below) shall obey the following constraints:

- All active VPSs shall have vps_num_rep_formats_minus1 in the range of 0 to 15, inclusive.

- All active SPSs for the current layer and its reference layers shall have chroma_format_idc equal to 0 only.

- All active SPSs for the current layer and its reference layers shall have separate_colour_plane_flag, cabac_bypass_alignment_enabled_flag, when present, equal to 0 only.

- CtbLog2SizeY derived from all active SPSs for the current layer and its reference layers shall be in the range of 4 to 6, inclusive.

- The constraints specified in Table G.1 shall apply for all active SPSs and PPSs for the current layer and its reference layers.

- For the current layer, referred to as layerA and having nuh_layer_id equal to i, and another layer layerB with nuh_layer_id equal to j, when layerB is a reference layer of layerA, the value of chroma_format_idc for layerA and the value of chroma_format_idc for layerB shall be identical, the value of bit_depth_luma_minus8 for layerA and the value of bit_depth_luma_minus8 for layerB shall be identical, the value of bit_depth_chroma_minus8 for layerA and the value of bit_depth_chroma_minus8 for layerB shall be identical, and AuxId[ i ] shall be equal to AuxId[ j ].

- The values of pic_width_in_luma_samples and pic_height_in_luma_samples in each active SPS for the layer shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, in each active SPS for all reference layers of that layer.

- All active SPSs for the current layer and its reference layers shall have the same value of chroma_format_idc.

- All active SPSs for the current layer and its reference layers shall have sps_scc_extension_flag equal to 0 only.

- All active PPSs for the current layer and its reference layers shall have pps_scc_extension_flag equal to 0 only.

- When an active PPS for the current layer or any of its reference layers has tiles_enabled_flag equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When an active PPS for the current layer or any of its reference layers has tiles_enabled_flag equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- For the current layer or any of its reference layers, the number of times read_bits( 1 ) is called in clauses 9.3.4.3.3 and 9.3.4.3.4 when parsing coding_tree_unit( ) data for any CTU shall be less than or equal to 5 * RawCtuBits / 3.

- For any active VPS, ViewOrderIdx[ i ] shall be greater than ViewOrderIdx[ j ] for any values of i and j among layerIdListTarget that was used to derive the sub-bitstream subBitstream such that one of these two layers is a reference layer of the other layer, AuxId[ i ] is equal to AuxId[ j ], and i is greater than j.

- The tier and level constraints specified for the Multiview Monochrome, Multiview Monochrome 10, and Multiview Monochrome 12, Multiview 4:4:4 10, or Multiview 4:4:4 12 profiles in clause G.11.2, as applicable, shall be fulfilled.

Table G.1 – Allowed values for syntax elements in the multiview format range extensions profiles

| Profile for which constraint is specified | chroma_format_idc | bit_depth_luma_minus8 and bit_depth_chroma_minus8 | transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, intra_smoothing_disabled_flag, persistent_rice_adaptation_enabled_flag, and log2_max_transform_skip_block_size_minus2 | extended_precision_processing_flag | chroma_qp_offset_list_enabled_flag |
| --- | --- | --- | --- | --- | --- |
| Multiview Monochrome | 0 | 0 | 0 | 0 | 0 |
| Multiview Monochrome 10 | 0 | 0..2 | 0 | 0 | 0 |
| Multiview Monochrome 12 | 0 | 0..4 | 0 | 0 | 0 |
| Multiview Monochrome 16 | 0 | 0..8 | 0 | 0 | 0 |
| Multiview 4:4:4 10 | - | 0..2 | - | 0 | - |
| Multiview 4:4:4 12 | - | 0..4 | - | 0 | - |

In the remainder of this clause and clause G.11.2.1, all syntax elements in the profile_tier_level( ) syntax structure refer to those in the profile_tier_level( ) syntax structure associated with the layer.

Conformance of a layer in an output operation point associated with an OLS in a bitstream for the multiview format range extensions profiles is indicated as follows:

- If OpTid of the output operation point is equal to vps_max_sub_layer_minus1, the conformance is indicated by general_profile_idc being equal to 10 or general_profile_compatibility_flag[ 10 ] being equal to 1, with the additional indications specified in Table G.2 for the general constraint flags.

- Otherwise (OpTid of the output operation point is less than vps_max_sub_layer_minus1), the conformance is indicated by sub_layer_profile_idc[ OpTid ] being equal to 10 or sub_layer_profile_‌compatibility_flag[ OpTid ][ 10 ] being equal to 1, with the additional indications specified in Table H.4 for the flags associated with the index OpTid.

All other combinations of general_max_14bit_constraint_flag, general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_8bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag, general_max_monochrome_constraint_flag, general_intra_constraint_flag, general_one_picture_only_constraint_flag, and general_lower_bit_rate_constraint_flag with general_profile_idc equal to 10 or general_profile_compatibility_flag[ 10 ] equal to 1 are reserved for future use by ITU-T | ISO/IEC. All other combinations of sub_layer_max_14bit_constraint_flag[ OpTid ], sub_layer_max_12bit_constraint_flag[ OpTid ], sub_layer_max_10bit_constraint_flag[ OpTid ], sub_layer_max_8bit_constraint_flag[ OpTid ], sub_layer_max_‌422chroma_constraint_flag[ OpTid ], sub_layer_max_420chroma_constraint_flag[ OpTid ], sub_layer_max_‌monochrome_constraint_flag[ OpTid ], sub_layer_intra_constraint_flag[ OpTid ], sub_layer_one_picture_only_‌constraint_flag[ OpTid ], and sub_layer_lower_bit_rate_constraint_flag[ OpTid ] with sub_layer_profile_idc[ OpTid ] equal to 10 or sub_layer_profile_compatibility_flag[ OpTid ][ 10 ] equal to 1 are reserved for future use by ITU-T | ISO/IEC. Such combinations shall not be present in bitstreams conforming to this document. However, decoders conforming to the multiview format range extensions profiles shall allow other combinations as specified below in this clause to occur in the bitstream.

Table G.2 –Bitstream indications for conformance to multiview range extensions profiles

| Profile for which the bitstream indicates conformance | general_max_14bit_constraint_flag or sub_layer_max_14bit_constraint_‌flag[ OpTid ] | general_max_12bit_constraint_flag or sub_layer_max_12bit_constraint_‌flag[ OpTid ] | general_max_10bit_constraint_flag or sub_layer_max_10bit_constraint_‌flag[ OpTid ] | general_max_8bit_constraint_flag or sub_layer_max_8bit_constraint_‌flag[ OpTid ] | general_max_422chroma_constraint_flag or sub_layer_max_422chroma_constraint_‌flag[ OpTid ] | general_max_420chroma_constraint_flag or sub_layer_max_420chroma_constraint_‌flag[ OpTid ] | general_max_monochrome_constraint_flag or sub_layer_max_monochrome_constraint_‌flag[ OpTid ] | general_intra_constraint_flag or sub_layer_intra_constraint_‌flag[ OpTid ] | general_one_picture_only_constraint_flag or sub_layer_one_picture_only_constraint_‌flag[ OpTid ] | general_lower_bit_rate_constraint_flag or sub_layer_lower_bit_rate_constraint_‌flag[ OpTid ] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Multiview Monochrome | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| Multiview Monochrome 10 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| Multiview Monochrome 12 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| Multiview Monochrome 16 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| Multiview 4:4:4 10 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Multiview 4:4:4 12 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

# Subclause G.11.2.1

Make the following changes:

# G.11.2.1 General tier and level limits

For purposes of comparison of tier capabilities, the tier with general_tier_flag or sub_layer_tier_flag[ i ] equal to 0 is considered to be a lower tier than the tier with general_tier_flag or sub_layer_tier_flag[ i ] equal to 1.

For purposes of comparison of level capabilities, a particular level of a specific tier is considered to be a lower level than some other level of the same tier when the value of the general_level_idc or sub_layer_level_idc[ i ] of the particular level is less than that of the other level.

The following is specified for expressing the constraints in this clause and clause G.11.2.2:

–	For the Multiview Main and Multiview Extended profiles, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor is the same as that specified in Table A.10 for the Main profile.

–	For the Multiview Extended 10 profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor is the same as that specified in Table A.10 for the Main 10 profile.

—	For the Multiview Monochrome profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Monochrome profile.

—	For the Multiview Monochrome 10 profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Monochrome 10 profile.

—	For the Multiview Monochrome 12 profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Monochrome 12 profile.

—	For the Multiview Monochrome 16 profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Monochrome 16 profile.

—	For the Multiview 4:4:4 10 profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Main 4:4:4 10 profile.

—	For the Multiview 4:4:4 12 profile, the value of each of the variables CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor is the same as that specified in Table A.10 for the Main 4:4:4 12 profile.

–	Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0 (i.e., the 0-th access unit).

–	Let the variable fR be set as follows:

–	If the bitstream or sub-layer representation is indicated to conform to the Main tier or is indicated to conform to a level that is lower than level 7.0, fR is set equal to 1  300.

–	Otherwise, fR is set equal to 1 ÷ 960.

–	Let the variable olsIdx be the index of the OLS.

–	For each layer with nuh_layer_id equal to currLayerId, let the variable layerSizeInSamplesY be derived as follows:

layerSizeInSamplesY = pic_width_vps_in_luma_samples * pic_height_vps_in_luma_samples	(G-2)

where pic_width_vps_in_luma_samples and pic_height_vps_in_luma_samples are found in the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS.

When the specified level is not level 8.5, each layer with nuh_layer_id equal to currLayerId conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in clause F.13, where "access unit" is used to denote the picture unit in the layer and the CPB is understood to be the BPB:

- The value of layerSizeInSamplesY shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.8 for the tier and level of the layer.

- The value of pic_width_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of pic_height_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of max_vps_dec_pic_buffering_minus1[ olsIdx ][ LayerIdxInVps[ currLayerId ] ][ HighestTid ] shall be less than or equal to MaxDpbSize as derived by Equation A-2, with PicSizeInSamplesY being replaced with layerSizeInSamplesY, for the tier and level of the layer.

- For level 5 and higher levels, the value of CtbSizeY for the layer shall be equal to 32 or 64.

- The value of NumPicTotalCurr for each picture in the layer shall be less than or equal to 8.

- When decoding each coded picture in the layer, the value of num_tile_columns_minus1 shall be less than MaxTileCols and num_tile_rows_minus1 shall be less than MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.8 for the tier and level of the layer.

- For the VCL HRD parameters of the layer, CpbSize[ i ] shall be less than or equal to CpbVclFactor * MaxCPB for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause F.13.1 and MaxCPB is specified in Table A.8 for the tier and level of the layer in units of CpbVclFactor bits.

- For the NAL HRD parameters of the layer, CpbSize[ i ] shall be less than or equal to CpbNalFactor * MaxCPB for at least one of the delivery schedules identified by bsp_sched_idx[ olsIdx ][ 0 ][ HighestTid ][ combIdx ][ LayerIdxInVps[ currLayerId ] ] for combIdx ranging from 0 to num_bsp_schedules_minus1[ olsIdx ][ 0 ][ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause F.13.1 and MaxCPB is specified in Table A.8 for the tier and level of the layer in units of CpbNalFactor bits.

- For each coded picture, the value of BinCountsInNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY ) ÷ 32.

Table A.8 specifies the limits for each level of each tier for levels other than level 8.5.

NOTE – Since there are no limits specified by Table A.8 for level 8.5, it is not possible in general for a practical decoder to be assured of being able to decode all bitstreams that conform to this level. The purpose of the definition of level 8.5 is to provide a suitable label for bitstreams that can exceed the limits of all other specified levels. When the bitstream is indicated to conform to level 8.5, a decoder is expected to examine the characteristics of the bitstream during its operation in order to determine whether it is capable of decoding the bitstream.

A tier and level to which a layer in an output operation point associated with an OLS in a bitstream conforms are indicated by the syntax elements general_tier_flag and general_level_idc if OpTid of the output layer set is equal to vps_max_sub_layer_minus1, and by the syntax elements sub_layer_tier_flag[ OpTid ] and sub_layer_level_idc[ OpTid ] otherwise, as follows:

–	If the specified level is not level 8.5, general_tier_flag or sub_layer_tier_flag[ OpTid ] equal to 0 indicates conformance to the Main tier, and general_tier_flag or sub_layer_tier_flag[ OpTid ] equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table A.8, and general_tier_flag and sub_layer_tier_flag[ OpTid ] shall be equal to 0 for levels below level 4 (corresponding to the entries in Table A.8 marked with "-"). Otherwise (the specified level is level 8.5), it is a requirement of bitstream conformance that general_tier_flag and sub_layer_tier_flag[ OpTid ] shall be equal to 1 and the value 0 for general_tier_flag and sub_layer_tier_flag[ OpTid ] is reserved for future use by ITU-T | ISO/IEC and decoders shall ignore the value of general_tier_flag and sub_layer_tier_flag[ OpTid ].

–	general_level_idc and sub_layer_level_idc[ OpTid ] shall be set equal to a value of 30 times the level number specified in Table A.8.

# Patent rights declaration(s)

Apple Inc,. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).