| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0047-v1 |
| --- | --- |

| Title: | AhG16: Higher-resolution Test Sequences and Test Results for Generative Face Video Compression |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bolin Chen, Shanzhi Yin, Yan Ye, Ru-Ling Liao, Jie Chen, Shiqi Wang | Email: | chenbolin.chenboli@alibaba-inc.com |
| Source: | Alibaba and City University of Hong Kong |  |  |

- Abstract

At the 35th meeting, GFVC software and test conditions were established as part of AHG16 activities. Besides, the GFV SEI message was also included into the draft 3 of TuC for future extensions of VSEI. However, the current GFVC software can only support low resolution (256×256) video and the GFVC test conditions described in JVET-AG2035 do not include higher-resolution test data.

In this contribution, we propose high-resolution testing data with suitable copyright license for GFVC experimentation. Besides, we retrain one of the GFVC models (i.e., CFTE) with high-resolution training data and provide results for further study in high-resolution compression. Experimental results show that the retrained CFTE model can also achieve promising RD performance in higher resolution for perceptual-level metrics in the GFVC test conditions.

Introduction

At the 34th JVET meeting, JVET-AH0113 proposed to extend GFVC to code higher resolution face video sequences, but this attempt was faced with some problems as follows:

- Lack of high-resolution training and testing data for face content, i.e., all data are sourced from the low-resolution via the traditional upsampling manner rather than having native higher resolution.

- Unoptimized algorithm design, i.e., base picture is at full resolution but dense motion field is at lower resolution and therefore needs to be upsampled for full-resolution face synthesis at the decoder.

To advance GFVC in higher resolution, in this contribution, we first propose higher-resolution test data (512×512) with suitable copyright license for GFVC experimentation. In particular, the new testing data span a considerable diversity of face-centric and head-and-shoulder video content. In addition, we retrain one of the GFVC models (i.e., CFTE) with higher-resolution training data and provide experimental results based on these proposed higher-resolution test sequences.

Experimental Settings

Training Settings

We retrain the CFTE algorithm included in current AhG16 software [3] with higher-resolution training dataset CelebV [4]. CelebV is a large-scale, high-quality, and diverse video dataset with diverse facial attribute annotations and head-and-shoulder content at the resolution of 512×512.

Testing Dataset

For the testing dataset, we propose to use 33 talking face video sequences selected and pre-processed from the CFVQA dataset [2]. A snapshot of the video set is shown in Figure 1.

- The copyright of CFVQA dataset [2] is pasted below and is therefore suitable for use by JVET. The CFVQA dataset is sourced from the Pexels website and we select 15 sequences with face-centric content as class A and 18 sequences with head-and-shoulder content as Class B.

“All photos and videos on Pexels can be downloaded and used for free (Please read the license terms here. https://www.pexels.com//license/).”

“What is allowed?

We tried to keep the Pexels License as simple as possible. Keep reading to find out more.

- All photos and videos on Pexels are free to use.

- Attribution is not required. Giving credit to the photographer or Pexels is not necessary but always appreciated.

- You can modify the photos and videos from Pexels. Be creative and edit them as you like.”

- These selected sequences span a considerable diversity of face video content, in terms of age, gender, ethnicity, appearance, expression, head position, head motion, camera motion, background, etc.

- Each of these testing sequences is cropped from the original sequence with the resolution of 512×512. The color format is RGB 444. The Class A sequences are 10-second long (i.e., 250 frames at 25 fps), and the Class B sequences are 5-second long (i.e.,125 frames at 25 fps).

- It should be mentioned that the proposed Class B test sequences are the same content as the Class B in GFVC test conditions specified in JVET-AG2035 [1] except for the resolution. In fact, the Class B sequences in JVET-AG2035 were downsampled from the proposed 512x512 sequences.

- The overall test dataset is available at testing_sequence.

  - Class A: Face-centric content

(b)  Class B: Head-and-shoulder content

Figure 1 - Test set of 33 face sequences selected and pre-processed from CFVQA [2].

Testing Conditions

Except for the higher-resolution test sequences, we follow other test conditions and software reference configurations for GFVC testing specified in JVET-AG2035 [1]. Test settings for VVC and for CFTE are as follows:

- For VVC, VTM 22.2 is used and the test is conducted under low-delay B (LDB) configuration with quantization parameters (QP) setting to 37, 42, 47 and 52.

- For CFTE, its base picture is compressed as an I picture using the VTM reference software 22.2, and other pictures are further represented with a series of facial parameters and compressed via a context-adaptive arithmetic coder. The QPs of the base picture are set to 22, 32, 42 and 52.

Experimental Results

RD Performance

Figure 2 shows the rate distortion performance (i.e., Rate-DISTS and Rate-LPIPS) of CFTE compared to that of VVC on two testing sequences. We can see that the CFTE algorithm show better performances on higher resolution of 512x512 compared to VVC in terms of perceptual-level quality measures DISTS and LPIPS.

(a) Overall – DISTS                                               (b) Overall – LPIPS

(c) Class A – DISTS                                               (d) Class A – LPIPS

(e) Class B – DISTS                                               (f) Class B – LPIPS

Figure 2 - Rate distortion performance of VVC and CFTE measured by two perceptual quality metrics DISTS and LPIPS in terms of overall dataset, Class A and Class B.

As shown in Table 1, CFTE can achieve 60.96% average bit-rate savings in terms of Rate-DISTS and 53.94% bit-rate savings in terms of Rate-LPIPS. Specifically, it can can achieve 64.75% average bit-rate savings in terms of Rate-DISTS and 55.79% bit-rate savings in terms of Rate-LPIPS on Class A and can achieve 57.18% average bit-rate savings in terms of Rate-DISTS and 52.09% bit-rate savings in terms of Rate-LPIPS on Class B. Notedly, #VALUE! And 0.00% can be attributed that the two curves do not overlap or intersect.

Table 1 - Average bit-rate savings of 33 face video sequences for CFTE algorithm compared with the traditional VVC codec evaluated in the RGB 444 and YUV 420 domain

| Sequences |  | Metrics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A (CFVQA_NEW) | Seq001 | -76.31% | -73.13% | 0.00% | -53.18% | 0.00% | 0.00% |
|  | Seq002 | -70.85% | -70.26% | -41.05% | -95.44% | 0.00% | -65.94% |
|  | Seq003 | -62.28% | -52.34% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq004 | -65.51% | -57.22% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq005 | -74.07% | -70.06% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq006 | -40.12% | -39.97% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq007 | -37.40% | 23.16% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq008 | -66.11% | -61.18% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq009 | -65.01% | -64.68% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq010 | -70.71% | -63.41% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq011 | -64.68% | -57.39% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq012 | -70.26% | -62.33% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq013 | -70.37% | -69.20% | 0.00% | -34.45% | 0.00% | 0.00% |
|  | Seq014 | -67.43% | -52.26% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq015 | -70.09% | -66.59% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B (CFVQA) | Seq001 | -53.75% | -35.67% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq002 | -62.12% | -49.56% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq003 | -63.17% | -55.09% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq004 | -64.01% | -57.31% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq005 | -56.38% | -53.95% | 0.00% | -7.59% | 0.00% | 3.91% |
|  | Seq006 | -58.48% | -43.69% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq007 | -63.15% | -59.60% | 0.00% | -33.43% | 0.00% | -20.46% |
|  | Seq008 | -47.25% | -44.55% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq009 | -47.86% | -43.11% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq010 | -52.33% | -48.33% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq011 | -57.55% | -60.00% | 0.00% | 0.00% | 0.00% | 8.16% |
|  | Seq012 | -51.20% | -53.12% | 0.00% | -60.77% | 0.00% | 0.00% |
|  | Seq013 | -65.27% | -64.57% | 0.00% | -54.04% | 0.00% | -40.18% |
|  | Seq014 | -59.53% | -59.83% | 0.00% | -36.46% | 0.00% | -24.70% |
|  | Seq015 | -55.64% | -52.94% | -2.46% | -29.86% | 95.06% | -29.77% |
|  | Seq016 | -58.22% | -47.46% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq017 | -14.07% | -0.18% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Seq018 | -62.20% | -41.49% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class A |  | -64.75% | -55.79% | -2.74% | -12.20% | 0.00% | -4.40% |
| Class B |  | -57.18% | -52.09% | -0.16% | -14.81% | 6.34% | -6.87% |
| Overall |  | -60.96% | -53.94% | -1.45% | -13.51% | 3.17% | -5.63% |

Subjective Quality Comparisons

Reconstructed face images are shown in Figure 3 to provide comparison of reconstruction quality using VVC and CFTE algorithm at similar bitrates on proposed higher-resolution testing sequences. It can be seen that at ultra-low bitrates, VVC reconstructed face images are seriously distorted or unrecognizable, whereas CFTE methods can still reconstruct vivid faces. Video demos using more test sequences will be shown during presentation at the meeting, and will emphasize very similar observations.

  - Class A: Face-centric content

  - Class B: Head-and-shoulder content

Figure 3 - Comparison of reconstruction quality using VVC vs. CFTE methods at similar coded bitrates on Class A and B.

Conclusion

This contribution proposes higher-resolution test data with suitable copyright license for GFVC experimentation. It is suggested to add these proposed test data to the AHG16 test condition. In addition, this contribution provides a CFTE model that can be used to code 512x512 video and achieve promising rate distortion performance and reconstruction quality. It is proposed to add the proposed CFTE model and the ability to code 512x512 video to the AHG16 software as the first step towards coding higher-resolution video with GFVC.

References

- McCarthy S, Chen B. Test conditions and evaluation procedures for generative face video coding, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG2035, January 2024.

- Li Y, Chen B, Chen B, et al. Perceptual Quality Assessment of Face Video Compression: A Benchmark and An Effective Method[J]. IEEE Transactions on Multimedia, 2024.

- Chen B, Ye Y, et al. AHG 16: Updated Common Software Tools for Generative Face Video Compression, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AH0114, April 2024.

- Zhu H, Wu W, Zhu W, et al. CelebV-HQ: A large-scale video facial attributes dataset[C]//European conference on computer vision. Cham: Springer Nature Switzerland, 2022: 650-667.

- McCarthy S, Chen J et al, Technologies under consideration for future extensions of VSEI (version 3), JVET-AG2032, online meeting, January 2024.

Patent rights declaration(s)

Alibaba and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).