| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0064-v3 |
| --- | --- |

| Title: | EE2-4.5: Multiple Transform Set Selection for Intra MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Charles Bonnineau Karam Naser Saurabh Puri Fabrice Le Léannec | Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 Test 4.5a and Test 4.5b on Multiple Transform Set Selection (MTSS) applied to explicit intra MTS, initially presented in the contribution JVET-AH0307. In this proposal, an alternative primary transform set from the explicit intra MTS lookup-table is considered in addition to the initial primary transform set for DIMD, TIMD, IntraTMP, MIP and EIP.

Two EE2 tests are presented. Test 4.5a corresponds to MTSS for intra MTS as initially proposed and Test 4.5b corresponds to a combination of 4.5a with 4.4a, which corresponds to the EE2 test about MTSS for LFNST/NSPT. The objective of 4.5b is to assess the orthogonality of the two methods.

The following results are obtained compared to ECM-13.0:

Test 4.5a

# AI: -0.04% 0.02% 0.02% 102.3% 100.2%

RA: -0.01% -0.07% -0.06% 100.9% 100.0%

Test 4.5b

# AI: -0.22% 0.10% 0.05% 110.9% 101.4%

RA: -0.12% -0.01% -0.06% 101.8% 99.8%

# Introduction

Multiple transform set selection (MTS) has been introduced in VVC, where DST7 and DCT8 transform pairs were tested in addition to the DCT2 primary transform. For intra directional mode, residual samples have larger absolute values if they are far away from the reference samples. Thus, the usage of a transform with an increasing basis function along the intra prediction direction might better catch the residual statistics.

To exploit this intra mode dependency in the ECM, it has been proposed to assign a transform set to each intra mode using a lookup table. Planar, DC and MIP intra modes have their own transform set in the lookup-table. Each transform set contains a total of six pairs of separable horizontal and vertical transforms. The number of candidates in each transform set is determined between 1, 4 or 6 by the absolute sum of coefficients in the block.

Some non-directional modes use a virtual intra prediction mode (VIPM) to obtain a transform set in this lookup-table. This VIPM is typically computed from the Histogram of Gradients (HoG), as in the directional intra mode derivation (DIMD) mode. However, some of these modes (e.g., IntraTMP, EIP) may not have a clear residual directionality compared to angular intra modes.

Recently, Multiple Transform Set Selection (MTSS) [2] offered the possibility to select an alternative transform set based on the HoG of the neighboring pixels of the current block for LFNST/NSPT.

It is asserted that for explicit intra MTS, primary transform pairs dedicated to planar and MIP modes can be adapted as alternative transform set to some special intra modes.

# Proposed method

In this contribution, it is proposed to use an alternative transform set for explicit intra MTS for DIMD, TIMD, IntraTMP, MIP and EIP modes. In the proposed method, 6 primary transform pair candidates are used in the alternative transform set for each of these modes. A flag is used in addition to the MTS transform index to signal the best transform set to the decoder after performing the rate-distortion optimization process.

The transform set associated with the MIP mode is used as alternative transform set for the EIP mode, while the transform set associated with PLANAR is used as alternative transform set for the other modes.

As some transform pairs are redundant from a primary transform set to another, some transform pair candidates from the alternative transform set can be already contained in the initial transform set. Therefore, we also use transform sets associated with adjacent TU size in the LUT to feed the alternative transform set with non-redundant primary transform pairs.

# Experimental results

The proposed method is implemented on top of ECM-13.0 [3]. The simulations were performed following the CTCs [4] for the all-intra (AI) and random access (RA) configurations. Test 4.5b corresponds to a combination of 4.5a with 4.4a, which corresponds to the EE2 test about MTSS for LFNST/NSPT. The objective of 4.5b is to assess the orthogonality of the two methods.

Table 1: Experimental results of Test 4.5a over ECM-13.0 (AI)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | 0.06% | 0.03% | 102.8% | 98.6% | 100.1% | 99.4% |
| Class A2 | -0.07% | -0.07% | 0.00% | 102.7% | 101.5% | 100.2% | 99.7% |
| Class B | -0.04% | 0.04% | -0.04% | 102.4% | 100.4% | 100.7% | 99.1% |
| Class C | -0.02% | -0.07% | 0.04% | 102.9% | 101.2% | 99.7% | 100.3% |
| Class E | -0.02% | 0.22% | 0.09% | 100.6% | 98.9% | 98.0% | 99.9% |
| Overall | -0.04% | 0.03% | 0.02% | 102.3% | 100.2% | 99.8% | 99.6% |
| Class D | -0.02% | 0.13% | -0.15% | 103.7% | 99.4% | 100.2% | 100.4% |
| Class F | -0.02% | 0.06% | 0.03% | 102.8% | 101.8% | 98.1% | 100.3% |
| Class TGM |  |  |  |  |  |  |  |

Table 2: Experimental results of Test 4.5a over ECM-13.0 (RA)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |
| Class A1 | -0.02% | -0.18% |  | -0.17% |  | 100.7% |  | 98.3% |  | 99.8% |  | 99.8% |
| Class A2 | -0.03% | 0.03% |  | 0.02% |  | 100.7% |  | 100.0% |  | 100.3% |  | 100.0% |
| Class B | -0.01% | -0.07% |  | -0.16% |  | 101.4% |  | 101.6% |  | 100.0% |  | 100.3% |
| Class C | -0.01% | -0.08% |  | 0.08% |  | 100.5% |  | 99.4% |  | 100.1% |  | 100.4% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.07% |  | -0.06% |  | 100.9% |  | 100.0% |  | 100.0% |  | 100.2% |
| Class D | 0.02% | -0.41% |  | -0.42% |  | 100.9% |  | 100.5% |  | 100.2% |  | 100.4% |
| Class F | 0.01% | 0.15% |  | 0.11% |  | 100.6% |  | 101.6% |  | 100.1% |  | 100.1% |
| Class TGM |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3: Experimental results of Test 4.5b over ECM-13.0 (AI)

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |
| Class A1 | -0.25% | 0.08% |  | 0.03% |  | 110.6% |  | 100.8% |  | 99.2% |  | 99.4% |
| Class A2 | -0.20% | 0.11% |  | 0.12% |  | 109.8% |  | 100.4% |  | 99.4% |  | 99.9% |
| Class B | -0.20% | 0.09% |  | -0.06% |  | 111.8% |  | 102.1% |  | 100.9% |  | 99.3% |
| Class C | -0.22% | 0.14% |  | 0.21% |  | 111.9% |  | 102.1% |  | 99.9% |  | 100.3% |
| Class E | -0.25% | 0.05% |  | -0.03% |  | 109.9% |  | 100.8% |  | 99.3% |  | 100.0% |
| Overall | -0.22% | 0.10% |  | 0.05% |  | 110.9% |  | 101.4% |  | 99.9% |  | 99.8% |
| Class D | -0.20% | -0.02% |  | -0.04% |  | 109.5% |  | 100.0% |  | 100.4% |  | 100.5% |
| Class F | -0.13% | 0.16% |  | 0.11% |  | 104.2% |  | 100.8% |  | 99.0% |  | 100.4% |
| Class TGM |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4: Experimental results of Test 4.5b over ECM-13.0 (RA)

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.19% | -0.22% | -0.24% | 102.6% | 100.6% | 100.0% | 100.2% |
| Class A2 | -0.08% | 0.00% | 0.09% | 100.7% | 98.8% | 100.0% | 100.1% |
| Class B | -0.13% | 0.02% | -0.18% | 102.3% | 100.9% | 100.2% | 100.4% |
| Class C | -0.08% | 0.11% | 0.11% | 101.2% | 98.5% | 100.6% | 100.5% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.12% | -0.01% | -0.06% | 101.8% | 99.8% | 100.2% | 100.3% |
| Class D | -0.07% | -0.23% | -0.05% | 101.5% | 100.2% | 100.3% | 100.5% |
| Class F | -0.08% | 0.20% | -0.13% | 101.9% | 101.8% | 100.3% | 100.4% |
| Class TGM |  |  |  |  |  |  |  |

# Conclusion

This contribution proposes to apply MTSS to intra MTS. Considering the coding efficiency improvement, we recommend integrating the tool in next version of the ECM.

# References

- C. Bonnineau, et. al., “AHG12: Multiple Transform Set Selection for Intra MTS” JVET-AH0307, April 2024

- F. Wang, et. al., “Non-EE2: Multiple Transform Set Selection for LFNST/NSPT” JVET-AG0062, January 2024

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AH2017, October 2023

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).