| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0077-v1 |
| --- | --- |

| Title: | AHG9: Design for content usage information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Tan Jangwon Lee Junghak Nam Jaehyun Lim Seung hwan Kim | Email: | chulkeun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

This contribution proposes signalling of information that may be used to guide the usage of decoded pictures. The recommendation may cover usage for AI-related processes (e.g., use for AI-training, AI-inference, etc) and video editing processes (e.g., video cropping, colorization, etc).

Two options are presented to implement such signalling, they are:

–	Option 1: specify a new SEI message.

–	Option 2: add the signalling in the TDI SEI message.

Introduction

At the 34th JVET meeting, contribution JVET-AH0112 brought the issue of the lack of clear guidelines or constraints on the use of video content in AI-driven processes. Two options were proposed and discussed to resolve this issue.

- Option 1: Include usage information in the Copyright Information SEI message.

- Option 2: Define a new SEI message to signal AI usage information.

Comments from the review of the contribution are summarized below, and further study of this issue was encouraged.

- Opposition to adding to the Copyright Information SEI: Some proponents opposed adding AI usage information to the Copyright Information SEI message and suggested that defining a new SEI message would be more appropriate. Note that during the meeting, it was agreed to cover the functionality of copyright information SEI message into the newly adopted text description information SEI message.

- Suggestion to use flags with specific purposes: Defining flags for specific purposes might be better than freeform strings, which would be difficult to interpret.

- Extensible mechanism is suggested: The set of flags to define may evolve over time, so an extensible mechanism is needed. The EOI SEI has a ue(v) with bit fields for the value, which is extensible.

- Scope of SEI message definition: It was suggested that the group should consider what scope of SEI messages should be worked on by our group vs. handled by other organizations.

The need for signalling content usage information

The ways in which content is used today are more varied and complex than ever before. With the advent of technologies such as artificial intelligence (AI) and advanced editing tools, the boundaries of content usage have expanded significantly. Additionally, the widespread ability for individuals to transmit and edit content has become commonplace, further increasing the complexity. This complexity necessitates clear guidelines to ensure proper and ethical use of content across various applications.

The need for AI usage information: Copyrights define legal regulations for content use and distribution, but with the advancement of AI technology, traditional copyright and license terms often cannot adequately address all possible use cases. The complexity of AI-driven content usage is heightened by the volume and variety of data, unclear fair use guidelines, and the rapid pace of technological advancements. Therefore, copyright holders should develop and announce specific restrictions and recommendations for AI usage of their content. This provides clear guidance to AI developers and users, promoting ethical and legal compliance and preventing unnecessary disputes.

The need for editing usage information: As the accessibility and ease of use of content editing tools have increased, modifying and repurposing digital media has become commonplace. This widespread practice raises issues related to transformative use, quality and integrity, proper attribution, and ethical considerations. By developing and announcing specific restrictions and recommendations for content editing, content owners can provide clear guidelines to editors and users. This helps promote legal and ethical compliance and prevents unnecessary disputes.

Proposal

Discussion

- Defining information for specific purposes using free-form strings

While defining flags for specific purposes is appropriate, there is an example for using free-form strings to define rules for specific purposes, such as using robots.txt for web crawling rules. It is possible to define information for specific purposes by separating fields within a free-form string.

- Copyright SEI no longer exists

During the last meeting, the TDI(Text Description Information) SEI was defined, and copyright-related information is now defined as one of the several purposes of the TDI SEI. The TDI SEI message is designed to provide text information for various purposes in a single SEI message, avoiding the need for multiple SEI messages. Considering its purpose, defining content usage information within TDI may be appropriate.

- Need for generic content usage information

The previous proposal focused solely on content usage information for AI. However, it may also be necessary to provide recommendations and restrictions on the usage of content for purposes other than AI.

- Extensible Mechanism for future needs

Content usage information can be derived through the bitmask operation with syntax defined by ue(v). Using this approach, new flags and constraints can be added in the future without requiring changes to the overall structure of the SEI message.

Based on this discussion, further study was conducted, resulting in the following updated proposals.

Proposal

Option 1: Define a new SEI message to signal content usage information

Syntax:

| content_usage_information( payloadSize ) { | Descriptor |
| --- | --- |
| cui_cancel_flag | u(1) |
| if( !cui_cancel_flag ) { |  |
| cui_persistence_flag | u(1) |
| cui_usage_category | ue(v) |
| if(cuiForAIFlag) |  |
| cui_usage_for_ai | ue(v) |
| if(cuiForEditingFlag) |  |
| cui_usage_for_editing | ue(v) |
| } |  |
| } |  |

Semantics:

cui_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous content usage information SEI message in output order that applies to the current layer. cui_cancel_flag equal to 0 indicates that content usage information follows.

cui_persistence_flag specifies the persistence of the content usage information SEI message for the current layer. cui_persistence_flag equal to 0 specifies that the persistence of the content usage information SEI message to the current decoded picture only. cui_persistence_flag equal to 1 specifies that the content usage information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a content usage information SEI message is output that follows the current picture in output order.

cui_usage_category indicates the usage category of the picture as specified in Table 1 where (cui_usage_category & bitMask) not equal to 0 indicates usage category with the bitMask value in Table 1 has been included. When cui_usage_category is equal to 0, usage category has been defined by the application. The value of cui_usage_category shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this version of this Specification. Values of 4 to 65 535, inclusive, for cui_usage_category are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of cui_usage_category is in the range of 4 to 65 535, inclusive, decoders conforming to this version of this Specification shall ignore cui_usage_category.

Table 1 – Definition of cui_usage_category

| bitMask | Interpretation |
| --- | --- |
| 0x01 | AI process usage. This SEI includes information on how associated pictures may be used with AI processes |
| 0x02 | Editing usage. This SEI includes information on how associated pictures may be used for editing or modification purposes |

The variables cuiForAIFlag and cuiForEditingFlag which specify whether cui_usage_category indicates the usage category to include AI process usage and editing usage, respectively, are derived as follows:

cuiForAIFlag = ( (cui_usage_category & 0x01 ) > 0 ) ? 1 : 0			(xx)
cuiForEditingFlag= ( ( cui_usage_category & 0x02 ) > 0 ) ? 1 : 0

cui_usage_for_ai when present, indicates usage information related to AI as specified in Table 2 where (cui_usage_for_ai & bitMask) not equal to 0 indicates usage information with the bitMask value in Table 2 has been applied. When cui_usage_for_ai is equal to 0, usage information has been defined by the application. The value of cui_usage_for_ai shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 65 535, inclusive, for cui_usage_for_ai are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of cui_usage_for_ai is in the range of 8 to 65 535, inclusive, decoders conforming to this version of this Specification shall ignore cui_usage_for_ai

Table 2– Definition of cui_usage_for_ai

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Usage information related to AI training; Indicates that pictures related to content usage information may be used for AI training |
| 0x02 | Usage information related to AI inference; Indicates that pictures related to content usage information may be used for AI inference |
| 0x04 | Usage information related to AI synthesis: Indicates that pictures related to content usage information may be used for AI synthesis |

The variables cuiAllowedForAITrainingFlag, cuiAllowedForAIInferencingFlag and cuiAllowedForAISynthesisFlag which specify whether the pictures associated with the SEI message may be used for AI training, inference, and synthesis, respectively, are derived as follows:

cuiAllowedForAITrainingFlag = ( (cui_usage_for_ai & 0x01 ) > 0 ) ? 1 : 0
cuiAllowedForAIInferenceFlag = ( ( cui_usage_for_ai & 0x02 ) > 0 ) ? 1 : 0			(xx)
cuiAllowedForAISynthesisFlag = ( ( cui_usage_for_ai & 0x04 ) > 0 ) ? 1 : 0

cui_usage_for_editing when present, indicates usage information related to editing as specified in Table 3 where (cui_usage_for_editing & bitMask) not equal to 0 indicates usage information with the bitMask value in Table 3 has been applied. When cui_usage_for_editing is equal to 0, usage information has been defined by the application. The value of cui_usage_for_editing shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 65 535, inclusive, for cui_usage_for_editing are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of cui_usage_for_editing is in the range of 8 to 65 535, inclusive, decoders conforming to this version of this Specification shall ignore cui_usage_for_editing.

Table 3 – Definition of cui_usage_for_editing

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Usage information related to cropping: Identifies that the picture may be cropped |
| 0x02 | Usage information related to colorization: Identifies that the picture may be colorized |
| 0x04 | Usage information related to masking: Identifies that the picture may be masked |

The variables cuiAllowedForCroppingflag, cuiAllowedForColorizationFlag, and cuiAllowedForMaskingFlag which specify whether the pictures associated with the SEI message may be used for cropping, colorization, and masking, respectively, are derived as follows:

cuiAllowedForCroppingflag = ( (cui_usage_for_editing & 0x01 ) > 0 ) ? 1 : 0
cuiAllowedForColorizationFlag = ( ( cui_usage_for_editing & 0x02 ) > 0 ) ? 1 : 0		(xx)
cuiAllowedForMaskingFlag = ( ( cui_usage_for_editing & 0x04 ) > 0 ) ? 1 : 0

Option 2: Defining content usage information within the TDI SEI.

To include content usage information in Text Description Information (TDI) SEI message, adding content usage information to txt_descr_purpose as shown in table 4.

This approach is similar to the one used in robots.txt, where specific rules for web crawlers are defined using a free-form string format. For instance, robots.txt uses lines such as "Disallow: /private" to specify that certain directories should not be accessed by crawlers. Similarly, usage information in the TDI SEI can use structured strings to convey detailed rules and constraints.

By detailing the constraints in this manner, the free-form string approach can effectively communicate specific usage information, providing clear guidelines and recommendations for various purposes, such as AI processes and editing permissions. For example, we may include the string such as “Content_usage_for_AI_training: allow” or “Content_usage_for_AI_training: disallow”.

Proposed changes to syntax and semantics on top of JVET-AH2006-v1 are marked with yellow for added text and with red strike-through for removed text.

Syntax:

| text_description( payloadSize ) { | Descriptor |
| --- | --- |
| txt_descr_id | u(14) |
| txt_cancel_flag | u(1) |
| if( !txt_cancel_flag ) { |  |
| txt_persistence_flag | u(1) |
| txt_descr_purpose | u(8) |
| txt_num_strings_minus1 | u(8) |
| for( i = 0; i <= txt_num_strings_minus1; i++ ) { |  |
| txt_descr_string_lang[ i ] | st(v) |
| txt_descr_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |

Semantics:

…

txt_descr_purpose indicates the purpose of the text description SEI as specified in Table 4. The value of text_descr_purpose shall be in the range of 0 to 56, inclusive. Values in the range of 67 to 255, inclusive, for text_descr_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of text_descr_purpose in the range of 0 to 255, inclusive.

Table 4 – Definition of txt_descr_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General comment information |
| 4 | Content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) |
| 5 | Tag URI for identifying the bitstream |
| 6 | Content usage information |
| 67..255 | Reserved |

…

When txt_descr_purpose is equal to 6, the txt_descr_string[ i ] specifies, when not a null string, content usage information that pertains to the picture(s) within the persistence scope of this SEI message.

NOTE: When txt_descr_purpose is equal to 6, the string can contain usage information of the pictures, such as constraints and recommendations may be related to either AI processes, editing permissions, or other relevant usage aspects.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).