| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0135-v0 |
| --- | --- |

| Title: | Non-EE2: Extended Usage of Fixed Filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Yang Wang1, Zhipin Deng1, Lei Zhao1, Na Zhang1, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to apply fixed filters defined in Adaptive Loop Filter (ALF) after the Deblocking Filter (DBF) to perform a refinement process at this stage. The output of the proposed method is further used as the input of the following tools, such as BF/SAO/CCSAO.

On top of ECM-13.0, the simulation results of the proposed method are reported as below:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

LB: %, %, %, %, %.

Introduction

In ECM-13.0, the fixed filters of ALF are well designed with classification and offline-trained coefficients. The fixed filters of ALF take reconstruction samples before DBF and reconstruction samples before ALF as input to produce the filtered results, which can be either directly used as the output of the ALF process or as the input of the ALF online filter.

Proposed Method

In this contribution, the fixed filters defined in ALF is also performed after DBF as shown in Figure 1.

Figure 1: The Proposed Method in Loop-Filter Progress.

The proposed fixed filters take the reconstruction samples before and after DBF as input to generate the filtered results at this stage. The classification and filtering logic of the fixed filter are directly reused as in ALF. The ALF process is kept unchanged as well, including both offline-filtering and online-filtering. The proposed method signals one slice level flag to achieve on/off control.

Simulation Results

The proposed method was implemented on top of the ECM-13.0 [1]. Simulations were performed following the common test conditions [2]. The performance of the proposed method compared to ECM-13.0 is shown in Table 1. More detailed results could be found in the attached excel sheets.

Table 1 Performance of the Proposed Method

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.07% | -0.06% | -0.09% |  |  |  |  |
| Class C | -0.08% | 0.00% | -0.02% |  |  |  |  |
| Class E | -0.14% | -0.09% | -0.08% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.01% | 0.01% | 0.04% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.05% | 0.09% | -0.04% |  |  |  |  |
| Class C | -0.08% | 0.13% | 0.16% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.09% | 0.08% | 0.10% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E | -0.10% | 0.40% | 0.44% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.24% | -0.61% | -0.10% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |

Conclusions

In this contribution, the fixed-filter of ALF is reused after DBF to refine the reconstruction samples at that stage. The classification and filtering logic of the fixed-filter are reused. The proposed method can achieve promising coding performance with negligible coding complexity. It is recommended to include the proposed method into the next round of EE2.

Reference

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AF2017, DE, October 2023.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).