| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0137-v1 |
| --- | --- |

| Title: | AHG16: Update on Pupil position SEI message for Generative Face Video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faming Ma, Anthony Trioux, Yusong Gao, Yayun Yao, Fuzheng Yang, Fang Xing, Zhikui Wang | Email: | fmma@stu.xidian.edu.cn anthony_trioux@xidian.edu.cn ysgao_1@stu.xidian.edu.cn yaoyayun@stu.xidian.edu.cn fzhyang@mail.xidian.edu.cn xingfang@hisense.com wangzhikui@hisense.com |
| Source: | Xidian University，Hisense Visual Tech. Co., Ltd. |  |  |

# Abstract

This is a follow-up contribution of JVET-AH0138[1], which proposed to convey the pupil position information as part of the (GFV) SEI messages, to generate more accurate eye positions in current Generative Face Video Compression (GFVC) architectures. Further study was encouraged during the last meeting, where a software implementation of the proposed solution was recommended. In this contribution, we implement the previous contribution on top of the existing Common Software Tools for GFVC and provide preliminary results.

# Introduction

1.1	 Overview of GFVC

Figure 1 shows the general block diagram of generative face video compression (GFVC). A base picture, also known as key frame, is first passed through a color format converter and then classically encoded for transmission through VVC. The following pictures, or subsequent pictures, are passed through an analysis model to extract facial representation (landmarks, keypoints, etc.), which are then encoded by arithmetic coding and transmitted. At the receiver-side, the base picture follows the reverse processing. Then, a generative model is used to regenerate the subsequent pictures, whose inputs include the decoded facial representation and the reconstructed base picture.

Figure 1 - General block diagram of GFVC.

1.2 Current limitation of GFVC without pupil redirection

As shown in Figure 2, we observed that most of the existing GFVC methods do not accurately represent eye pupil movement and position. However, as an important part of facial video communication, it is important to convey such information to improve the realism in GFVC architecture.

Figure 2  - Incorrect pupil position in current method

Therefore, we proposed in JVET-AH0138[1] a method to compute, encode and transmit the pupil center position as part of GFV SEI information. More specifically, as shown in Figure 3, we first define a coordinate system by taking the inner corner of the eye as the origin, the line connecting the inner and outer corners of the eye as the x-axis, and the perpendicular of the x-axis as the y-axis to establish a rectangular coordinate system. Such system is then used to compute the pupil center coordinates (x, y), which correspond to the distance between the pupil center and the x-axis and y-axis, respectively. The latter information is finally transmitted as part of GFV SEI information and used at the receiver side to refine the pupil position information.

Figure 3 - Coordinate system of pupil center position with respect to the corner of the eye.

In order to control the amount of transmitted bitrate while offering some scalability, several consideration have been taken into account:

- Left, right or both pupil center position(s) can be transmitted using this representation;

- The pupil center position information can be transmitted either as direct coordinate values (x, y) or as a predicted value with respect to the pupil center position in the base picture (dx, dy).

- The transmitted information can be quantized to further reduce the amount of data.

Corresponding detailed changes relative to JVET-AG2032[2] can be found in JVET-AH0138[1].

# Implementation of the GFVC with pupil redirection

The implementation is currently made on top of the existing common tools software for GFVC provided in the following link: https://vcgit.hhi.fraunhofer.de/jvet-ahg-gfvc/gfvc_v1/. Although there exists a software implementation of the GFV SEI message on top of VTM (see JVET-AH0239[3]), the latter was not officially released at the time of writing this contribution, it will be considered in future work.

Once the pupil position information has been received at the receiver side, one can use it to redirect the pupil in the generated video by injecting the corresponding information in GFVC algorithms. In a first approach, and as a proof-of-concept, we use the pupil position information in a post-processing module. As shown in Figure 4, we first extract the pupil position at the transmitter side through existing landmarks detection model, and then carry out encoding and transmission as described in JVET-AH0138[1]. In this proposal, only the residual approach has been considered. Note that the post-processing approach does not modify the base-picture (reference frame) as the pupil position is accurate enough, but we still extract its pupil center coordinate that serves as the reference to perform the residual coding. At the receiver side, the corresponding pupil position is extracted from the bitstream and used in a post-processing module to correct the video generated by GFVC. The post-processing module contains a network with a similar structure to FOMM[4].

Figure 4 - Illustration of GFVC with post-processing for pupil position

# Experiments

In this section, we provide preliminary results as well as performance analysis using the proposed FV2V, FOMM and CFTE algorithms. In this proposal, only class A (VoxCeleb) was used to perform the experiments as eye movements are almost inexistant in class B (CFVQA).

## Showcase demonstration of GFVC with pupil redirection

In this section, examples are provided to demonstrate the effectiveness of pupil redirection for GFVC. Figure 5 shows that the proposed method help to correct the pupil position of existing FV2V, FOMM and CFTE methods. Additional visual examples will be provided during the presentation.

Figure 5 - Corrected pupil position of FV2V, FOMM and CFTE methods

## Bitrate consideration

In terms of bitrate increase, we provide in Table 1 some results obtained considering the VoxCeleb sequence (Class A). As recalled in the beginning of the contribution, the additional bitrate corresponding to the transmission of pupil position as part of the GFV SEI message is scalable and controlled through the syntax element gfv_pupil_coordinate_precision_factor_minus1[1], which represents the number of bits used to encode the pupil position information, i.e., the quantization precision.

Table 1 - Bitrate with different precision factors

| Precision_factor | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |
| Bitrate increase (bps) | 210.45 | 230.72 | 265.92 | 322.51 | 410.56 |

One could possibly further reduce the bitrate by transmitting only one or none of the two pupil position, in an adaptive manner. The next step would be to evaluate the RD performance of the different approaches using the objective metrics defined in the CTC. It is however expected that objective metrics would not be able to accurately represents the perceived improvement as the approach only modify a small part of the reconstructed video (i.e., the eyes).

# Conclusion

This contribution provides a first step towards the implementation of the Pupil position SEI message for Generative Face Video as described in JVET-AH0138. Preliminary results with pupil redirection obtained on top of the common software tools are provided. Our examples demonstrates that the original GFVC algorithms (FV2V, FOMM, CFTE) are not accurate enough regarding eye animation and that transmitting the pupil position as part of the GFV SEI information can help to improve the realism of GFVC architectures. It is proposed to first consider the inclusion of the pupil position information as part of the GFV SEI message. Then, it is proposed that technical solutions including pupil redirection for GFVC be further studied in AHG16. Such solutions should be implemented considering the recent release of the official GFV SEI software (https://vcgit.hhi.fraunhofer.de/jvet-tuc/VVCSoftware_VTM/-/merge_requests/15).

# References

- A.Trioux, Y.Yao, F.Ma, F.Yang, F.Xing, Z.Wang. “AHG9/AHG16: Pupil position SEI message for Generative Face Video” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no.JVET-AH0138, April 2024.

- S. McCarthy, J. Chen, S. Deshpande, M. Hannuksela, Hendry. “Technologies under consideration for future extensions of VSEI (version 3)” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AG2032, January 2024.

- B. Chen, J. Chen, R.Zou, Y. Ye, R.-L. Liao, S. Wang. “AHG9/AHG16: Software Implementation of Generative Face Video SEI Message” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AH0239, April 2024.

- A. Siarohin, S. Lathuilière, S. Tulyakov, et al. First order motion model for image animation[J]. Advances in Neural Information Processing Systems, 2019, 32.

# Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Hisense Visual Tech. Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).