| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0158-v1 |
| --- | --- |

| Title: | [AHG9] Lens Optical Correction SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger Gilles Teniou | Email: | {swenger,teniou}[at]global[dot]tencent[dot]com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The Lens Optical Correction SEI message conveys parameters defining the level of radial distortion and transversal chromatic aberration due to the lens intrinsic characteristics. Those parameters, when present, allow for converting wide angle (and fisheye) footages into rectilinear mode and re-aligning the color fringes.

It is proposed to include the Optical Lens Correction SEI message into the next version of the TuC.

# Introduction

During JVET meeting #34 in Rennes, the Optical Lens Correction SEI message was first presented.

Clause 2 is reminder of the justification already presented in JVET-AH0204 with illustrations of the effects and artefacts to be corrected.

Clause 3 lists the comments raised during the previous meeting and addresses them.

Clause 4 contains an update of the consequent the proposed syntax and semantic.

# Problem statement

## Ultra-wide angle and fisheye cameras characteristics

The Fisheye lenses have an extremely wide field of view, often exceeding 180 degrees. They capture a spherical or hemispherical view, including a significant amount of distortion. Such a distortion results in either barrel distortion (outward curvature) or pincushion distortion (inward curvature) near the edges of the frame. Fisheye lenses also typically have very short focal lengths. The shorter the focal length, the greater potential depth of field.

Ultra-wide-angle lenses have also a wide field of view but do not typically reach the extreme degrees of coverage seen in fisheye lenses. They usually provide a field of view between 90 degrees and 120 degrees.

Although many action cameras have a linear mode for capturing a scene without distortion, it remains a post-processing step on the captured picture with a native wide-angle lens as shown in the figure below:

In content authoring, when post processing is needed (e.g., color and contrast calibration, transition insertion, video effects…) it is usually better to operate on the native video instead of a post-processed version.

Lens distortion parameters are then needed to be transmitted (when known) so that correction can be done on the last stage.

## Radial lens distortion

The radial distortion introduced by wide angle optics can be corrected with various correction models. Those models express the relation between the distance of a point (pixel) from the focal point (usually the center of the image) in the distorted image (noted rd) and in the undistorted image (noted ru) as illustrated bellow:

Among the most popular models some of them are:

- The Poly3 model is a 3rd order polynomial model, defined as:

rd = ru * ( 1 −  k1 + k1 * ru * ru )		(1-1)

With k1 being the correction term. k1 value is in the range [-1,1] with a default value of 0 (i.e., no distortion). When k1 > 0 the barrel distortion is observed whereas k1 < 0 	corresponds to the pincushion effect.

- The Poly5 model is the 5th order polynomial model defined as:

rd = ru * ( 1 −  k1 * ru * ru + k2 * ru * ru* ru* ru )		(1-2)

With k1 being the quadratic correction term, k2 being the double quadratic term. k1 and k2 	values range in [-1,1] with a default value of 0 (i.e., no distortion).

With the transmission of the above parameters, the video can be corrected with post filters such that the "lenscorrection" filter from ffmpeg [1].

The identification of the parameters usually requires a lens calibration but those from the most popular cameras can be found in the lensfun database [2].

NOTE: It is proposed in the SEI message syntax to use a signed integer using a multiplication factor of 1000, since by experience, parameters are floats with 1/1000th precision. Other approaches may be chosen if found more appropriate by the group.

## Chromatic aberration

Light is electromagnetic radiation depending on its wavelength. Colors light with a long wavelength appears red the shorter this wavelength gets it first becomes yellow then green then blue purple until we can't see it anymore. When going through the edge of the lens the rays are oriented to a different angle depending on their wavelength, that is called diffraction as shown below:

This effect is the cause of chromatic aberration, also known as color fringing, when the colors start to separate at the edges of a picture as illustrated below:

In this case transversal chromatic aberration (TCA) correction can compensate this effect.

There are several models, that all take the green component as the reference and try to align the red and blue with it.

- Linear TCA model is expressed as:

rd,R = ru,R * kR		(2-1)

rd,B = ru,B * kB		(2-2)

Where rd,R and rd,B are respectively the red and blue coordinates of a point in the distorted image, ru,R and ru,B respectively the red and blue coordinates in the undistorted corrected image, kr and kb the TCA parameters that default to 1.

- 3rd order polynomial TCA model is expressed as:

rd,R = ru,R * ( bR * ru,R * ru,R + cR * ru,R + vR )		(2-3)

rd,B = ru,B * ( bB * ru,B * ru,B + cB * ru,B + vB )		(2-4)

Where rd,R and rd,B are respectively the red and blue coordinates of a point in the distorted image, ru,R and ru,B respectively the red and blue coordinates in the undistorted corrected 	image, bR, cR, bB, vB, vR and vB the TCA parameters. bR, cR, bB, vB default to 0 and vR and vB are default to 1.

NOTE – It is proposed in the SEI message syntax to use a signed integer using a multiplication factor of 1000, since by experience, parameters are floats with 1/1000th precision. Other approaches may be chosen if found more appropriate by the group.

## Other effects

Other effects such as vignetting (the effect of light fall-off in the corners of images due to lens characteristics) are well known distortions that can be corrected with the transmission of a limited number of parameters. Such aspects are proposed to be for further study, in case relevant. Also other radial and TCA correction models such as the Adobe Camera Model [3] could be addressed and signaled, this contribution is limited to the most popular correction models.

# Comments from the JVET-AH0204

The following comments were raised:

- The contribution includes multiple formulas.

Correct, each formula corresponds to a correction model. They are now written in the style of specification.

- No software is currently available and would be helpful to confirm the design.

A software implementing the SEI message proposal and the application of those parameters has been developed and is proposed in JVET-AI-0160.

- The Ptlens format was indicated to be a Photoshop lens model, which would probably be difficult to reference in a specification. A formula was provided in the specification. It was suggested to avoid company-based names in our specification.

Ptlens is an add-on tool from the PanoramaTools (aka PanoTools) library. The library is under GNU Lesser General Public License. Ptlens formula may be seen as an extension of the 5th order polynomial formula that covers already many cases of the most popular action cameras. Moreover Ptlens is a shareware to Panotools. Even if it is used in the Lensfun database [2] and implemented in the Hugin software [4] it is proposed to not pursue its implementation for the time being.

- The proposed syntax and semantics do not follow our editorial practices.

The next clause describes the proposed syntax and semantic of the SEI message with carefully checking the editorial practices and Specification writing styles.

# Syntax and Semantics

## VSEI

| lens_optical_correction( payloadSize ) { | Descriptor |
| --- | --- |
| loc_cancel_flag | u(1) |
| if( !loc_cancel_flag ) { |  |
| loc_focal_parameters_present_flag |  |
| if( loc_focal_parameters_present_flag ) { |  |
| loc_focal_center_x | u(16) |
| loc_focal_center_y | u(16) |
| loc_focal_length | u(16) |
| } |  |
| loc_rd_parameters_present_flag | u(1) |
| if( loc_rd_parameters_present_flag ) { |  |
| loc_rd_model_id | u(8) |
| if( loc_rd_model_id = = 0 ) { |  |
| loc_rd_poly3_k1 | i(25) |
| } |  |
| if( loc_rd_model_id = = 1 ) { |  |
| loc_rd_poly5_k1 | i(25) |
| loc_rd_poly5_k2 | i(25) |
| } |  |
| } |  |
| loc_tca_parameters_present_flag | u(1) |
| if( loc_tca_parameters_present_flag ) { |  |
| loc_tca_model_id | u(8) |
| if( loc_tca_model_id = = 0 ) { |  |
| loc_tca_kr | u(25) |
| loc_tca_kb | u(25) |
| } |  |
| if( loc_tca_model_id = = 1 ) { |  |
| loc_tca_br | i(25) |
| loc_tca_cr | i(25) |
| loc_tca_vr | u(25) |
| loc_tca_bb | i(25) |
| loc_tca_cb | i(25) |
| loc_tca_vb | u(25) |
| } |  |
| } |  |
| loc _persistence_flag | u(1) |
| } |  |
| } |  |

## Lens optical correction semantics

This SEI message provides the decoder with a lens distortion model to enable image correction.

loc_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous lens optical correction SEI message in output order that applies to the current layer. loc_cancel_flag equal to 0 indicates that lens optical correction follows.

loc_focal_parameters_present_flag equal to 1 indicates that focal parameters of the lens are present in the lens optical correction SEI message syntax. loc_focal_parameters_present_flag equal to 0 indicates that no focal parameter is present.

loc_focal_center_x defines the horizontal location in the picture of the focal point. It is by default half of the width of the picture.

loc_focal_center_y defines the vertical location in the picture of the focal point. It is by default half of the height of the picture.

loc_focal_length defines the focal length of the lens, i.e., the distance from the lens in which rays of light that fall parallel to the optical axis into the lens are converged. It is expressed in units of 0.1 millimeters. A value of 0 indicates that the focal length is undefined. When not present, the value of loc_focal_length is inferred to be equal to 0.

loc_rd_parameters_present_flag equal to 1 indicates that radial distortion parameters are present in the lens optical correction SEI message syntax. loc_rd_parameters_present_flag equal to 0 indicates that no radial correction is expected.

loc_rd_model_id identifies the type of radial distortion model as specified in Table 1. The value of loc_rd_model_id shall be in the range of 0 to 1, inclusive. The values greater than 1 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore lens optical correction SEI messages with loc_rd_model_id greater than 1.

Table 1 – loc_rd_model_id

| Value | Description |
| --- | --- |
| 0 | Poly3 |
| 1 | Poly5 |

When loc_rd_model_id equals to 0 the polynomial 3rd order (Poly3) model is specified as

rd = ru * ( 1 −  k1 + k1 * ru * ru )		(1-1)

with rd referring to the distorted radius (normalized distance to image center), ru referring to the corrected undistorted radius, and k1 being the distortion coefficient.

When loc_rd_model_id equals to 1 the polynomial 5th (Poly5) order model is specified as

rd = ru * ( 1 −  k1 * ru * ru + k2 * ru * ru* ru* ru )		(1-2)

with rd referring to the distorted radius (normalized distance to image center), ru referring to the corrected undistorted radius, and k1 and k2 being the distortion coefficients.

loc_rd_poly3_k1 ÷224 indicates the value of the k1 parameter from the Poly3 radial distortion model from equation (1-1). When not present, the value of loc_rd_poly3_k1 is inferred to be equal to 0.

loc_rd_poly5_k1 ÷224 indicates the value of the k1 parameter from the Poly5 radial distortion model from equation (1-2). When not present, the value of loc_rd_poly5_k1 is inferred to be equal to 0.

loc_rd_poly5_k2 ÷224 indicates the value of the k2 parameter from the Poly5 radial distortion model from equation (1-2). When not present, the value of loc_rd_poly5_k2 is inferred to be equal to 0.

loc_tca_parameters_present_flag equal to 1 indicates that transversal chromatic aberration parameters are present in the lens optical correction SEI message syntax. loc_tca_parameters_parameters_present_flag equal to 0 indicates that no transversal chromatic aberration correction is expected.

loc_tca_model_id identifies the type of radial distortion model as specified in Table 2. The value of loc_tca_model_id shall be in the range of 0 to 1, inclusive. The values greater than 1 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore lens optical correction SEI messages with loc_tca_model_id greater than 1.

Table 2 – loc_tca_model_id

| Value | Description |
| --- | --- |
| 0 | Linear |
| 1 | Poly3 |

When loc_tca_model_id equals to 0 the linear model is specified as

rd,R = ru,R * kR		(2-1)

rd,B = ru,B * kB		(2-2)

with rd,R and rd,B referring to the distorted radius (normalized distance to image center) of the red and blue components respectively, ru,R and ru,B referring to the corrected undistorted radius of the red and blue components respectively, and kR and kB being the transversal chromatic aberration coefficients of the red and blue components respectively. kR and kB are equal to 1 by default.

NOTE – The  transversal chromatic aberration is corrected in the linear model on the red and blue components with the green component as the reference.

When loc_tca_model_id equals to 1 the polynomial 3rd order (Poly3) model is specified as

rd,R = ru,R * ( bR * ru,R * ru,R + cR * ru,R + vR )		(2-3)

rd,B = ru,B * ( bB * ru,B * ru,B + cB * ru,B + vB )		(2-4)

with rd,R and rd,B referring to the distorted radius (normalized distance to image center) of the red and blue components respectively, ru,R and ru,B referring to the corrected undistorted radius of the red and blue components respectively, and {br, cr, vr} and {bb, cb, vb} being the transversal chromatic aberration coefficients of the red and blue components respectively. kR and kB are equal to 1 by default. The br, cr, bb, cb coefficients are by default equal to 0 and the vr and vb coefficients are by default to 1.

NOTE - The transversal chromatic aberration is corrected in the Poly3 model on the red and blue components with the green component as the reference.

loc_tca_kr ÷224 indicates the value of the kr parameter from the linear transversal chromatic aberration model from equation (2-1). When not present, the value of loc_tca_kr is inferred to be equal to 224.

loc_tca_kb ÷224 indicates the value of the kb parameter from the linear transversal chromatic aberration model from equation (2-2). When not present, the value of loc_tca_kr is inferred to be equal to 224.

loc_tca_br ÷224 indicates the value of the br parameter from the Poly3 transversal chromatic aberration model from equation (2-3). When not present, the value of loc_tca_br is inferred to be equal to 0.

loc_tca_cr ÷224 indicates the value of the cr parameter from the Poly3 transversal chromatic aberration model from equation (2-3). When not present, the value of loc_tca_cr is inferred to be equal to 0.

loc_tca_vr ÷224 indicates the value of the vr parameter from the Poly3 transversal chromatic aberration model from equation (2-3). When not present, the value of loc_tca_vr is inferred to be equal to 224.

loc_tca_bb ÷224 indicates the value of the bb parameter from the Poly3 transversal chromatic aberration model from equation (2-4). When not present, the value of loc_tca_bb is inferred to be equal to 0.

loc_tca_cb ÷224 indicates the value of the cb parameter from the Poly3 transversal chromatic aberration model from equation (2-4). When not present, the value of loc_tca_cb is inferred to be equal to 0.

loc_tca_vb ÷224 indicates the value of the vb parameter from the Poly3 transversal chromatic aberration model from equation (2-4). When not present, the value of loc_tca_vb is inferred to be equal to 224.

loc_persistence_flag specifies the persistence of the lens optical correction SEI message for the current layer.

loc_persistence_flag equal to 0 specifies that the lens optical correction SEI message applies to the current decoded picture only.

loc_persistence_flag equal to 1 specifies that the lens optical correction SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a lens optical correction SEI message is output that follows the current picture in output order.

# Proposal

It is proposed to document the Clause 4 (Syntax and Semantics) into the next version of the TuC.

# References

[1] https://ffmpeg.org/ffmpeg-filters.html#lenscorrection

[2]	https://lensfun.github.io/lenslist/

[3]	http://download.macromedia.com/pub/labs/lensprofile_creator/lensprofile_creator_cameramodel.pdf.

[4]	https://hugin.sourceforge.io

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).