| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0159-v1 |
| --- | --- |

| Title: | EE2-2.9: derived MIP modes with fusion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2-2.9, which is about the derived MIP modes with fusion. A template-based method is utilized to derive the two best MIP modes and the transposed flags. Then those two derived MIP predictions are fused with weights. Besides, it is proposed to apply the DIMD process to the upsampled prediction samples to derive the LFNST/NSPT transform set. In EE2-2.9, three sub-tests are conducted. EE2-2.9a studies the performance of the derived MIP fusion, EE2-2.9b studies the up-sampled samples based DIMD process, and EE2-2.9c studies the combination of EE2-2.9a and EE2-2.9b. On top of ECM-13.0, the simulation results are reported as follows:

EE2-2.9a:

AI: -0.04% (Y),	-0.03% (U),	-0.04% (V), 100.5% (EncT), 99.8% (DecT)

EE2-2.9b:

AI: x.xx% (Y),	x.xx% (U),	x.xx% (V), xx.x% (EncT), xx.x% (DecT)

EE2-2.9c:

AI: -0.05% (Y),	0.03% (U),	-0.03% (V), 100.5% (EncT), 100.0% (DecT)

# Proposal

The proposed method [1] uses the derived MIP modes with fusion as one additional intra prediction tool, and it sends one CU-level flag in the bitstream to indicate its usage.

## Derived MIP modes with fusion

Similar to other template tools, the top and left neighboring areas are used as templates to derive the MIP modes and the transposed flags for the current block, where the size of templates is width×2 and 2×height, respectively. The proposed method first derives two best candidates {MIP modes, transposed flag} with the smallest SATD cost over the template areas, and then it fuses the two derived MIP predictions with weights to produce a prediction for the current block.

This fusion process is applied to the current coding unit only when the SATD cost of the best candidate is larger than half of the cost of the second-best candidate. In the case where the SATD cost of the best candidate is smaller than half of the cost of the second-best candidate, the prediction is produced using only the best candidate without fusion.

The fusion weights are calculated by their template costs as follows:

weightbest = SATDcostsecBest / (SATDcostbest+ SATDcostsecBest)

weightsecBest = 1 - weightbest

The proposed method can be applied to those blocks which are larger than 128.

## DIMD applied to upsampled predictors for non-square blocks

The DIMD process in MIP-LFNST transform set derivation is applied to the up-sampled prediction samples for non-square and sizeId {0, 1} blocks, which is shown in Fig.1 (b). For other blocks, the DIMD process is applied directly to the prediction samples as shown in Fig.1 (a).

Fig. 1 DIMD process in MIP-LFNST transform set derivation

# Simulation results

The proposed method has been implemented on top of ECM-13.0 [2]. Test results of the proposed method under the common AI/RA configurations are summarized in Tables 1, 2, and 3.

## EE2-2.9a

Table 1 EE2-2.9a over ECM-13.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.10% | -0.11% | 0.10% | 101.3% | 99.8% | 101.9% | 100.0% |
| Class A2 | -0.03% | 0.05% | 0.07% | 100.9% | 99.8% | 100.5% | 100.1% |
| Class B | -0.02% | -0.08% | -0.11% | 100.7% | 100.2% | 98.7% | 100.1% |
| Class C | -0.02% | 0.02% | -0.08% | 99.7% | 99.2% | 99.1% | 100.1% |
| Class E | -0.03% | 0.01% | -0.12% | 100.1% | 99.7% | 103.4% | 100.0% |
| Overall | -0.04% | -0.03% | -0.04% | 100.5% | 99.8% | 100.4% | 100.1% |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

## EE2-2.9b

Table 2 EE2-2.9b over ECM-13.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.11% | 0.05% | 100.0% | 99.3% | 99.9% | 100.0% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | -0.01% | -0.08% | 99.7% | 99.9% | 99.1% | 100.0% |
| Class C | 0.00% | -0.08% | -0.05% | 99.9% | 101.4% | 100.0% | 100.0% |
| Class E | 0.00% | 0.05% | -0.16% | 100.0% | 99.0% | 100.0% | 100.0% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

## EE2-2.9c

Table 3 EE2-2.9c over ECM-13.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.11% | 0.09% | -0.08% | 101.4% | 100.7% | 100.2% | 100.1% |
| Class A2 | -0.03% | 0.07% | 0.01% | 100.6% | 100.0% | 98.8% | 100.1% |
| Class B | -0.03% | 0.01% | -0.12% | 100.1% | 100.0% | 100.1% | 100.0% |
| Class C | -0.02% | 0.01% | 0.11% | 99.0% | 99.0% | 100.1% | 100.1% |
| Class E | -0.06% | 0.03% | -0.07% | 102.3% | 100.6% | 100.1% | 100.1% |
| Overall | -0.05% | 0.03% | -0.03% | 100.5% | 100.0% | 99.9% | 100.1% |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# Conclusions

In this contribution, a method of using the derive MIP modes with fusion is proposed, which aims to improve the coding performance of ECM reference software. Compared with ECM-13.0, the proposed method can provide 0.05% coding gain with negligible complexity. It is suggested to adopt EE2-2.9c into the next version of ECM.

# Reference

[1] Z. Xie, Y. Yu, H. Yu, D. Wang. “Non-EE2: derived MIP modes with fusion”, JVET-AH0058, April 17-24, Rennes, FR, 2024

[2] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-13.0

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).