| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JT, 12–19 July 2024 | Document: JVET-AI0161-v2 |
| --- | --- |

| Title: | EE2-3.2: Temporal BV for IBC merge/AMVP list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu, Yue Yu, Haoping Yu, Dong Wang (OPPO) Na Zhang, Kai Zhang, Li Zhang (Bytedance Inc.) | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-3.2, where temporal BV candidates are added to the IBC merge/AMVP list. The proposed method is implemented on top of the ECM-13.0 reference software. The simulation results are reported as follows:

RA: Class F -0.03%/0.02%/-0.07%; Class TGM -0.58%/-0.42%/-0.56%.

LB: Class F -0.24%/-0.32%/-0.69%; Class TGM -0.59%/-0.54%/-0.55%.

Introduction

In ECM, the IBC merge/AMVP list is constructed from adjacent spatial candidates, non-adjacent spatial candidates, HBVP candidates, ARBVP candidates, pairwise average candidates, and some predefined BV candidates. However, temporal BV candidates are not utilized. On the other hand, the temporal motion vector prediction (TMVP) is utilized for the inter merge/AMVP mode. In JVET-AD0095 [1], an EE test was conducted to investigate adding temporal BV candidates to the IBC merge/AMVP list. In this test, more temporal positions are added to improve the coding performance of the IBC merge/AMVP mode.

Proposed methods

This contribution reports the EE test established based on JVET-AH0059 [2]. In this contribution, temporal BV candidates are added to the IBC merge/AMVP list after the non-adjacent BV candidates. The temporal BVs are derived from collocated positions and shifted collocated positions in the collocated pictures. The IBC merge/AMVP list size, the number of processes applied for template matching and the number of full RDO processes are kept unchanged in the proposed method.

Collocated positions

There are five collocated positions defined in the proposed method. The temporal BVs can be obtained from the collocated positions in the collocated pictures.

Figure-1. Five collocated positions.

Shifted collocated positions

Offsets are used as guidance to change the collocated positions to form the shifted collocated positions. The temporal BVs can be obtained from the shifted collocated positions in the collocated pictures. The offsets can be obtained by:

- Fetching a MV pointing at a collocated picture from one of the five spatial neighboring positions in Figure-2 and using the MV as an offset of the collocated positions for the collocated picture;

- Fetching a MV pointing at a reference picture from one of the five spatial neighboring positions in Figure-2, scaling the MV to point at the collocated picture based on the POC distance between the reference picture and the current picture and the POC distance between the collocated picture and the current picture, and using the scaled MV as an offset of the collocated positions for the collocated picture;

- Fetching a MV pointing at a collocated picture from one of the five spatial neighboring positions in Figure-2 and calculating a mirrored MV pointing at the other collocated picture. Then using the mirrored MV as an offset of the collocated positions for the other collocated picture.

Figure-2. Five spatial neighboring positions.

Collocated pictures

Two collocated pictures are utilized which are the two reference pictures with the first two least POC distances relative to the current picture, which is the same as that of TMVP.

Experimental Results

The proposed method has been implemented on top of ECM-13.0 [2]. Testing results of the proposed method under the RA/LB configurations are summarized in Table 1 and Table 2.

Table 1: Coding performance of the proposed method (RA).

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.03% | 0.02% | -0.07% | 100.0% | 100.0% | 99.7% | 100.0% |
| Class TGM | -0.58% | -0.42% | -0.56% | 99.5% | 101.3% | 100.7% | 100.0% |

Table 2: Coding performance of the proposed method (LDB).

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F |  | -0.24% |  | -0.32% |  | -0.69% |  | 99.9% | 99.5% |  | 98.0% |  | 99.9% |  |
| Class TGM |  | -0.59% |  | -0.54% |  | -0.55% |  | 100.2% | 98.5% |  | 100.6% |  | 100.0% |  |

- Conclusions

In this contribution, the temporal BV candidates are proposed to be added to the IBC merge/AMVP list. The proposed method can bring some coding gain without coding complexity. It is recommended to adopt this method in the next ECM software.

References

[1] N. Zhang, K. Zhang, L. Zhang, “EE2-3.3: Temporal block vector prediction,” JVET-AD0095, Apr. 2023.

[2] L. Xu, Y. Yu, H. Yu, D. Wang, N. Zhang, K. Zhang, L. Zhang, “AHG12: Temporal BV for IBC merge list construction”, JVET-AH0059, Apr. 2024.

[3] ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-13.0

- Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).