| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JT, 12–19 July 2024 | Document: JVET-AI0166-v1 |
| --- | --- |

| Title: | EE2-5.3: CCALF with Chroma inputs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenyu Dai, Nan Song, Yue Yu, Haoping Yu, Dong Wang Nan Hu, Marta Karczewicz, Vadim Seregin 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | daizhenyu@oppo.com songnan@oppo.com nanh@qti.qualcomm.com martak@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | OPPO and Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this contribution, the results of EE2-5.3 are reported. In EE2-5.3, chroma signals at different stages of the chroma in-loop filtering process have been proposed as additional inputs to CCALF. These various chroma signals, in combination with the new filter taps, can be used individually or jointly in the new CCALF. There are 5 subtests in EE2-5.3, test 5.3a uses chroma SAO outputs, test 5.3b uses chroma fixed filter outputs and test 5.3c uses chroma ALF outputs. In test 5.3d, the combination of test 5.3a, test 5.3b, and test 5.3c is tested. In test 5.3e, the combination of test 5.3a and test 5.3c is tested.

On top of ECM-13.0, simulation results of the proposed methods are reported below:

EE2-5.3a:

AI: 0.02%, -0.77%, -1.05%, 100.2% EncT, 100.4% DecT.

RA: %, %, %, % EncT, % DecT.

LB: 0.05, -1.41%, -1.39%, 100.2% EncT, 99.6% DecT.

EE2-5.3b:

AI: 0.02%, -0.75%, -0.83%, 100.0% EncT, 101.0% DecT.

RA: 0.01%, -0.47%, -0.68%, 100.1% EncT, 100.0% DecT.

LB: -0.02%, -1.29%, -1.00%, 100.1% EncT, 100.3% DecT.

EE2-5.3c:

AI: 0.03%, -1.00%, -1.27%, 99.9% EncT, 98.9% DecT.

RA: 0.01%, -0.74%, -0.87%, 100.1% EncT, 100.0% DecT.

LB: 0.02%, -1.81%, -1.32%, 99.9% EncT, 99.4% DecT.

EE2-5.3d:

AI: 0.03%, -0.90%, -1.23%, 100.4% EncT, 101.7% DecT.

RA: %, %, %, % EncT, % DecT.

LB: 0.04%, -1.53%, -1.48%, 100.9% EncT, 100.6% DecT.

EE2-5.3e:

AI: 0.03%, -0.89%, -1.17%, 100.1% EncT, 99.2% DecT.

RA: 0.03%, -0.63%, -0.75%, 99.9% EncT, 100.0% DecT.

LB: 0.04%, -1.61%, -1.43%, 100.4% EncT, 99.4% DecT.

# Introduction

In the current ECM-13.0 design, an online-trained CCALF filter is constructed with 23 luma spatial reconstruction taps and 5 luma residual taps, as illustrated in Figure 1.

Fig. 1: The currently used filter shape of CCALF

# Proposed method

EE2-5.3 investigates the effect of using chroma signals at different stages of the chroma in-loop filtering process as additional inputs to CCALF, as illustrated in Figure 2.

Fig. 2: The filter shape of chroma signals at different stages

In test 5.3a, chroma SAO output samples applied to 4 taps in a 3x3 asymmetric cross shape are added for CCALF. In test 5.3b, chroma fixed filter output samples applied to 8 taps in a 5x5 symmetrical cross shape are added for CCALF. In test 5.3c, chroma ALF output samples applied to 4 taps in a 3x3 asymmetric cross shape are added for CCALF. In test 5.3d, a combination of test 5.3a, test 5.3b, and test 5.3c is tested. In test 5.3e, a combination of test 5.3a and test 5.3c is tested.

# Experiment results

The proposed method is implemented on top of ECM-13.0[1]. The simulations were carried out by following the ECM common test conditions[2]. Tables 1~ 5 report the simulation results of tests EE2-5.3a ~ EE2-5.3e, respectively.

Table 1. Results of test EE2-5.3a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.02% | -0.41% | -0.97% | 100.1% | 100.5% | 100.8% | 101.6% |
| Class A2 | 0.03% | -0.99% | -1.38% | 99.8% | 100.3% | 100.7% | 101.6% |
| Class B | 0.02% | -1.49% | -1.15% | 100.2% | 100.2% | 102.6% | 100.7% |
| Class C | 0.01% | -0.21% | -0.17% | 100.3% | 100.8% | 100.1% | 100.2% |
| Class E | 0.04% | -0.46% | -1.80% | 100.2% | 99.9% | 100.2% | 100.4% |
| Overall | 0.02% | -0.77% | -1.05% | 100.2% | 100.4% | 101.0% | 100.9% |
| Class D | 0.00% | -0.04% | -0.15% | 100.4% | 99.8% | 99.9% | 100.1% |
| Class F | 0.02% | -0.49% | -0.33% | 99.8% | 100.0% | 99.3% | 100.4% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.53% | -0.82% | 99.9% | 99.6% | 100.2% | 100.7% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.02% | -1.14% | -0.77% | 100.0% | 99.9% | 100.7% | 100.6% |
| Class C | -0.01% | -0.19% | -0.14% | 100.6% | 99.6% | 100.9% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.02% | -0.24% | -0.01% | 100.9% | 99.6% | 100.0% | 100.1% |
| Class F | 0.01% | -0.44% | -0.47% | 100.2% | 99.7% | 100.1% | 100.8% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.03% | -2.13% | -0.75% | 100.1% | 99.8% | 100.3% | 100.7% |
| Class C | 0.02% | -0.24% | -1.01% | 100.6% | 99.2% | 99.8% | 100.2% |
| Class E | 0.13% | -1.78% | -2.96% | 99.8% | 99.6% | 100.1% | 100.3% |
| Overall | 0.05% | -1.41% | -1.39% | 100.2% | 99.6% | 100.1% | 100.4% |
| Class D | -0.12% | 0.00% | -0.44% | 100.4% | 100.9% | 100.4% | 100.2% |
| Class F | -0.05% | -1.45% | -1.47% | 99.9% | 99.8% | 100.6% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |

Table 2. Results of test EE2-5.3b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.02% | -0.66% | -0.92% | 100.2% | 100.5% | 100.0% | 100.0% |
| Class A2 | 0.03% | -0.91% | -1.34% | 100.4% | 101.0% | 100.0% | 100.0% |
| Class B | 0.02% | -1.43% | -1.19% | 100.1% | 100.7% | 99.3% | 100.0% |
| Class C | 0.01% | -0.17% | -0.14% | 99.4% | 102.0% | 100.0% | 100.0% |
| Class E | 0.02% | -0.30% | -0.56% | 100.1% | 100.6% | 100.0% | 100.0% |
| Overall | 0.02% | -0.75% | -0.83% | 100.0% | 101.0% | 99.8% | 100.0% |
| Class D | 0.00% | -0.11% | -0.15% | 100.8% | 102.1% | 99.8% | 100.0% |
| Class F | 0.02% | -0.39% | -0.26% | 100.5% | 101.7% | 99.2% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.02% | -0.44% | -0.73% | 100.2% | 100.1% | 100.1% | 99.8% |
| Class A2 | 0.03% | -0.71% | -1.78% | 100.3% | 100.3% | 100.0% | 100.0% |
| Class B | 0.01% | -0.83% | -0.57% | 100.2% | 100.1% | 100.3% | 100.2% |
| Class C | -0.01% | 0.12% | 0.02% | 100.0% | 99.5% | 99.6% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.01% | -0.47% | -0.68% | 100.1% | 100.0% | 100.0% | 100.0% |
| Class D | -0.03% | -0.21% | 0.01% | 100.2% | 99.1% | 100.1% | 100.0% |
| Class F | 0.01% | -0.30% | -0.29% | 100.3% | 99.5% | 100.5% | 100.2% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.02% | -1.81% | -0.93% | 99.8% | 99.8% | 99.8% | 100.1% |
| Class C | -0.02% | -0.49% | -1.11% | 100.6% | 101.4% | 100.1% | 100.0% |
| Class E | -0.07% | -1.48% | -0.97% | 100.1% | 99.8% | 98.0% | 100.0% |
| Overall | -0.02% | -1.29% | -1.00% | 100.1% | 100.3% | 99.5% | 100.0% |
| Class D | -0.18% | -0.81% | -0.05% | 100.6% | 102.7% | 99.5% | 100.0% |
| Class F | 0.04% | -1.19% | -1.48% | 99.9% | 98.7% | 100.2% | 99.9% |
| Class TGM |  |  |  |  |  |  |  |

Table 3. Results of test EE2-5.3c

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.03% | -0.89% | -1.27% | 101.4% | 99.3% | 100.0% | 100.1% |
| Class A2 | 0.03% | -1.27% | -1.55% | 101.2% | 100.2% | 100.0% | 100.1% |
| Class B | 0.03% | -1.76% | -1.47% | 99.3% | 98.9% | 101.3% | 100.1% |
| Class C | 0.01% | -0.25% | -0.18% | 97.7% | 97.2% | 100.0% | 100.0% |
| Class E | 0.05% | -0.56% | -2.08% | 100.8% | 99.5% | 100.0% | 100.0% |
| Overall | 0.03% | -1.00% | -1.27% | 99.9% | 98.9% | 100.4% | 100.1% |
| Class D | 0.00% | -0.07% | -0.16% | 99.8% | 97.9% | 100.0% | 100.0% |
| Class F | 0.02% | -0.51% | -0.37% | 98.5% | 99.0% | 100.0% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.02% | -0.59% | -0.80% | 99.6% | 100.0% | 100.5% | 99.7% |
| Class A2 | 0.05% | -0.74% | -1.85% | 99.8% | 100.3% | 100.0% | 100.2% |
| Class B | 0.02% | -1.25% | -0.93% | 100.1% | 100.0% | 99.7% | 100.2% |
| Class C | -0.03% | -0.20% | -0.09% | 100.7% | 99.9% | 100.3% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.01% | -0.74% | -0.87% | 100.1% | 100.0% | 100.1% | 100.1% |
| Class D | -0.02% | -0.23% | 0.00% | 102.7% | 101.5% | 100.2% | 100.0% |
| Class F | 0.05% | -0.51% | -0.40% | 100.3% | 100.7% | 99.2% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | -2.46% | -1.27% | 100.1% | 99.8% | 100.0% | 99.9% |
| Class C | 0.00% | -0.57% | -0.95% | 99.9% | 100.1% | 100.6% | 100.0% |
| Class E | 0.08% | -2.36% | -1.90% | 99.6% | 97.6% | 99.0% | 100.1% |
| Overall | 0.02% | -1.81% | -1.32% | 99.9% | 99.4% | 100.0% | 100.0% |
| Class D | -0.06% | -0.47% | -0.06% | 100.0% | 100.6% | 99.3% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 4. Results of test EE2-5.3d

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.04% | -0.93% | -1.43% | 100.2% | 102.8% | 100.8% | 101.7% |
| Class A2 | 0.04% | -1.31% | -1.74% | 100.2% | 102.0% | 100.7% | 101.6% |
| Class B | 0.03% | -1.66% | -1.44% | 100.4% | 101.3% | 102.3% | 100.7% |
| Class C | 0.02% | -0.14% | -0.03% | 100.5% | 101.2% | 100.1% | 100.2% |
| Class E | 0.05% | -0.21% | -1.78% | 100.9% | 101.8% | 100.2% | 100.4% |
| Overall | 0.03% | -0.90% | -1.23% | 100.4% | 101.7% | 100.9% | 100.9% |
| Class D | -0.01% | 0.42% | 0.01% | 100.9% | 100.6% | 100.1% | 100.1% |
| Class F | 0.03% | -0.50% | -0.37% | 98.9% | 102.0% | 99.4% | 100.5% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.04% | -0.60% | -1.01% | 99.8% | 101.7% | 100.3% | 100.4% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.02% | -1.31% | -1.00% | 100.4% | 100.9% | 100.2% | 100.6% |
| Class C | 0.02% | 0.12% | 0.05% | 100.9% | 100.7% | 99.7% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.03% | 0.19% | 0.18% | 101.5% | 100.3% | 100.1% | 100.1% |
| Class F | 0.01% | -0.31% | -0.30% | 100.1% | 101.3% | 100.2% | 100.7% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.02% | -2.25% | -1.13% | 100.6% | 100.8% | 100.2% | 100.7% |
| Class C | -0.01% | -0.45% | -0.92% | 100.6% | 99.4% | 99.8% | 100.2% |
| Class E | 0.13% | -1.77% | -2.83% | 101.8% | 101.8% | 100.2% | 100.5% |
| Overall | 0.04% | -1.53% | -1.48% | 100.9% | 100.6% | 100.0% | 100.5% |
| Class D | -0.07% | 0.32% | -0.53% | 102.4% | 102.5% | 99.7% | 100.1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 5. Results of test EE2-5.3e

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.03% | -0.89% | -1.23% | 99.6% | 97.4% | 100.4% | 101.7% |
| Class A2 | 0.03% | -1.23% | -1.56% | 100.2% | 98.7% | 100.4% | 101.6% |
| Class B | 0.03% | -1.59% | -1.36% | 100.2% | 100.2% | 101.0% | 100.8% |
| Class C | 0.01% | -0.14% | -0.04% | 99.8% | 98.5% | 100.1% | 100.2% |
| Class E | 0.05% | -0.35% | -1.93% | 100.5% | 101.0% | 100.2% | 100.4% |
| Overall | 0.03% | -0.89% | -1.17% | 100.1% | 99.2% | 100.5% | 100.9% |
| Class D | 0.00% | 0.25% | 0.04% | 100.8% | 100.2% | 100.2% | 100.1% |
| Class F | 0.03% | -0.46% | -0.34% | 98.1% | 97.6% | 100.2% | 100.4% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.04% | -0.56% | -0.75% | 99.6% | 100.4% | 100.7% | 100.8% |
| Class A2 | 0.04% | -0.64% | -1.76% | 99.8% | 99.6% | 100.9% | 101.2% |
| Class B | 0.02% | -1.13% | -0.88% | 100.0% | 99.9% | 99.9% | 100.6% |
| Class C | 0.01% | -0.06% | 0.18% | 100.2% | 100.0% | 100.0% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.03% | -0.63% | -0.75% | 99.9% | 100.0% | 100.3% | 100.7% |
| Class D | -0.02% | 0.20% | 0.32% | 100.1% | 100.0% | 100.1% | 100.1% |
| Class F | -0.01% | -0.30% | -0.32% | 99.9% | 100.5% | 99.7% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.02% | -2.30% | -1.10% | 100.4% | 99.0% | 100.2% | 100.6% |
| Class C | -0.01% | -0.37% | -0.99% | 100.1% | 100.3% | 100.3% | 100.2% |
| Class E | 0.13% | -2.13% | -2.57% | 100.8% | 98.9% | 100.3% | 100.5% |
| Overall | 0.04% | -1.61% | -1.43% | 100.4% | 99.4% | 100.2% | 100.4% |
| Class D | 0.02% | -0.73% | -0.02% | 100.3% | 100.0% | 100.0% | 100.1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# References

- ECM13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0.

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October 2023.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).