| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0168-v1 |
| --- | --- |

| Title: | Non-EE2: Chroma TMRL |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hang Huang Yue Yu Haoping Yu Dong Wang | Tel: Email: | huanghang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution extends TMRL (template-based multiple reference line intra prediction) to chroma as a new chroma prediction mode. First, a list of combination candidates is constructed which consists of the information of the reference line and the chroma intra prediction mode. Then these candidate combinations of intra prediction mode and reference line are applied to the collocated luma block as well as a template of the current chroma block to generate corresponding predictions. Finally, the candidates are reordered based on a combined SATD cost. The first N combination candidates in the list are allowed to predict the current chroma block. On top of ECM-13.0, the performance of the proposed method is reported as follows:

AI {0.00% Y, -0.32% U, -0.30% V, 100.x% EncT, 100.x% DecT}

# Introduction

Template-based multiple reference line intra prediction (TMRL) mode combines reference line and prediction mode and uses a template matching method to construct a list of candidate combinations. An index to the candidate combination list is coded to indicate which combination of reference line and prediction mode is used in coding the current block.

In ECM-13.0, there are 7 non-CCP chroma intra prediction modes: chroma DBV mode, DM mode, chroma DIMD mode, and four default modes (i.e. planar mode, vertical mode, horizontal mode and DC mode).

For the chroma DBV mode, the first available BV found among the five positions {C, TL, TR, BL, BR} in the collocated luma block as shown in Figure 1 is scaled and used as the BV of the current chroma block.

For the DM mode, the intra prediction mode of the luma block covering the center position C in the collocated luma block as shown in Figure 1 is used to predict the current chroma block.

For the chroma DIMD mode, a histogram of gradients (HoG) is constructed based on the luma samples in the second and third lines in the collocated luma block and the chroma samples in the second neighboring row and column. Then the intra prediction mode with the largest histogram amplitude value in the HoG is used for performing chroma intra prediction of the current chroma block.

Figure 1 The collocated luma block and adjacent chroma block

# Proposed methods

In this contribution, a new non-CCP chroma intra prediction mode is proposed.

The proposed chroma TMRL mode includes the following aspects:

- the extended reference line candidate list and the intra-prediction-mode candidate list.

The extended reference line candidate list consists of {1, 3, 5}.

The size of the intra-prediction-mode candidate list is 8, and the construction process is described as follows:

- The non-CCP chroma intra prediction modes in the current ECM including DM, chroma DIMD, DC;

- The collocated luma modes: the intra prediction modes from luma blocks in the positions of TL, TR, BL and BR in the collocated luma block as shown in Figure 1;

- The adjacent chroma modes: the intra prediction modes from adjacent chroma blocks in the positions of L', T', BL', TR' and TL' as shown in Figure 1;

Some default intra modes will be considered if the intra-prediction-mode candidate list is not fully filled. The default intra modes are the first angular mode in the intra-prediction-mode candidate list shifted with {-1, +1, -2, +2, …}. In order to remove redundant intra prediction mode in the list, a pruning operation is applied. It is noted that the CCP mode, planar mode and DBV mode are excluded from the intra-prediction-mode candidate list.

- construction of the chroma TMRL combination list.

There are 3x8=24 combinations of the reference line and the allowed intra-prediction modes for a block.

Then the combination candidates in the list are used to predict the collocated luma block and the template of the current chroma block. The SATD cost between prediction and reconstructed pixel is then calculated. This template includes one top row and one left column of the current chroma block as shown in Figure 2. The candidates in the list are reordered based on a combined SATD cost, calculated as follows:

cost=8*costY+costCbTop+costCrTop≪logH+2+costCbLeft+costCrLeft≪logW+2

For the collocated luma block, the reference line {1, 3, 5} will be set as {2, 6, 10}. Only the first N combinations with the smallest cost can be allowed for the current chroma block, N is equal to 6 in this proposal.

Figure 2 Illustration of template area

- signaling of chroma TMRL

A chroma TMRL flag is signaled to indicate whether the chroma TMRL mode is applied or not. If the chroma TMRL mode is applied, an index is signaled to indicate which combination candidate in the list is used.

# Simulation results

The proposed method is implemented on top of the ECM-13.0 software [1] and tested based on the common test condition [2]. The test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.05% | -0.05% |  |  |  |  |
| Class A2 | -0.01% | -0.26% | -0.21% |  |  |  |  |
| Class B | 0.00% | -0.27% | -0.36% |  |  |  |  |
| Class C | 0.02% | -0.22% | -0.31% |  |  |  |  |
| Class E | -0.01% | -0.87% | -0.53% |  |  |  |  |
| Overall | 0.00% | -0.32% | -0.30% |  |  |  |  |
| Class D |  |  |  |  |  |  |  |

# Conclusions

This contribution proposes a chroma TMRL mode. The results show that the performance of non-CCP chroma intra prediction can be further improved with a minor complexity increase. It is recommended to investigate the proposed methods in the next round of EE2.

# References

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AF2017, October 2023

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).