| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0171-v3 |
| --- | --- |

| Title: | EE2-2.3: Combination of EE2-2.1/EE-2-2.5a and EE2-2.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohsen Abdoli, Ramin G. Youvalari, Alexandre Tissier (Xiaomi) Karam Naser, Ya Chen, Fabrice Le Léannec, Milos Radosavljević, Kevin Reuzé (InterDigital) Yao-Jen Chang, Vadim Seregin, Marta Karczewicz (Qualcomm) Jiaye Fu, Jiaqi Zhang, , Siwei Ma (PKU) C. Huang (ZTE) | Email: | mabdoli@xiaomi.com karam.naser@interdigital.com yjchang@qti.qualcomm.com jiaye.fu@ucdconnect.ie |
| Source: | Xiaomi Technologies, InterDigital, Qualcomm Incorporated, PKU, ZTE |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-2.3 exploration experiments. This test implements the joint performance of tests EE2-2.1/EE2-2.5 and EE2-2.2. In tests EE2-2.1/EE2-2.5, the TIMD-merge is proposed that computes a merge list of TIMD blocks in a neighbourhood of adjacent and non-adjacent neighboring CUs. Then, the list is sorted based on the template cost and finally, the best candidate is used as the TIMD-merge mode. TIMD-merge uses a CU-level flag that is signaled only when TIMD flag is true.

The second test, EE2-2.2, presents a TIMD with enhanced block vectors deployment. This test studies the use of block vectors (BVs) in TIMD mode with the extension of BV generation through the auto-relocated block vector prediction (ARBVP) process.

Here is a summary of the EE2-2.3 results:

AI { -0.12%, 0.02%, 0.00%, 101.0%, 101.1%},

RA* { -0.06%, -0.03%, 0.02%, 10x.x%, 10x.x%}

(*) Five missing points were copied from the anchor.

# Introduction

## Test EE2-2.1/EE2-2.5: Merge-based intra prediction

In tests EE2-2.1/EE2-2.5 [1], the TIMD-merge is proposed that computes a merge list of TIMD blocks in a neighbourhood of adjacent and non-adjacent neighboring CUs. Then, the list is sorted based on the template cost and finally, the best candidate is used as the TIMD-merge mode.

Usage of the TIMD-Merge mode is signaled with a CABAC coded PU level flag. The TIMD merge mode is used as a sub-mode of TIMD tool.

## Test EE2-2.2: TIMD with enhanced block vectors deployment

This EE test is based on the method proposed in JVET-AH0274. Specifically, the following is proposed:

- Allow TIMD with block vectors: When deriving the intra modes of TIMD (2 angular and 1 non-angular), the block vectors of the merge candidates of IntraTMP mode are also checked. If the template cost is smaller than any of the other modes, the mode is replaced by the block vector.

- Use ARBVP merge list in DIMD and TIMD: The additional block vector candidates generated by ARBVP process are included in the merge list of DIMD and TIMD, in the same manner as in IntraTMP.

# Results

Below are the results of Test EE2-2.3:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | 0.10% | 0.07% | 100.6% | 101.0% | 100.1% | 100.7% |
| Class A2 | -0.14% | -0.02% | -0.03% | 102.0% | 100.9% | 100.6% | 100.7% |
| Class B | -0.12% | -0.04% | -0.05% | 101.2% | 100.7% | 100.4% | 100.5% |
| Class C | -0.12% | 0.03% | -0.03% | 101.2% | 101.9% | 100.5% | 100.3% |
| Class E | -0.12% | 0.05% | 0.09% | 100.7% | 100.9% | 102.0% | 100.2% |
| Overall | -0.12% | 0.02% | 0.00% | 101.2% | 101.1% | 100.7% | 100.5% |
| Class D | -0.05% | 0.05% | 0.00% | 100.2% | 95.5% | 99.9% | 100.1% |
| Class F | -0.03% | 0.20% | 0.21% | 100.3% | 99.5% | 98.8% | 100.4% |
| Class TGM | 0.09% | 0.12% | 0.15% | 99.7% | 101.6% | 99.5% | 100.8% |

|  | Random Access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | -0.25% | -0.08% |  |  |  |  |
| Class A2 | -0.08% | -0.17% | -0.12% |  |  |  |  |
| Class B | -0.03% | 0.06% | -0.09% |  |  |  |  |
| Class C | -0.02% | 0.26% | 0.11% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.06% | -0.03% | 0.02% |  |  |  |  |
| Class D | 0.00% | -0.20% | 0.13% |  |  |  |  |
| Class F | 0.02% | 0.07% | 0.06% |  |  |  |  |
| Class TGM | 0.03% | 0.08% | 0.08% |  |  |  |  |

NOTE: Five missing points in RA were copied from the anchor

# References

[1] M. Abdoli et al. EE2-2.1/EE2-2.5: Merge-based intra prediction, JVET-AI0110, July 2024

[2] K. Naser et al. EE2-2.2: TIMD with enhanced block vectors deployment, JVET-AI0104, July 2024

[3] ECM-13.0 reference software, https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).