| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0179-v1 |
| --- | --- |

| Title: | AHG9: Persistence cancellation of SEI messages with IDs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela | Email: | {jill.boyce, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

When an SEI message contains an identifier syntax element, more than one SEI message of that type can be active at the same time if they have different ID values. A cancel flag in an SEI message can either be used to cancel persistence of the SEI messages of that type with a particular ID or can be used to cancel persistence of all SEI messages of that type, depending on the semantics of the particular SEI message. It is proposed to dedicate a particular ID value to indicate cancellation of the persistence of all SEI messages of that type. This provides the ability to signal either of the two types of persistence cancellation, while avoiding the need to signal two separate cancel flags.

It is proposed to adopt the proposed design to the text description SEI message of VSEI v4, in combination with the alternative persistence syntax proposed in JVET-AH0325, which was included in an annex of the VSEI TuC in JVET-AH2032.

# Description

For SEI messages that contain an identifier syntax element, it is proposed to have a dedicated ID value to indicate cancellation of persistence of all SEI messages of that type. When the ID value is set to the dedicated value it indicates cancellation of persistence of all SEI message of that type. When the ID value is set to a value other than the dedicated value, setting of the cancellation flag to 1 indicates cancellation of persistence of the SEI message of that type with the same ID value.

Having the ability to signal two types of persistence cancellation, both cancellation for a particular ID value and cancellation for all message of that type, may be useful for some use cases. Cancellation of SEI message persistence for a particular ID value is more bitrate efficient than cancelling the persistence for all ID values, because the latter would require re-signalling of messages that are intended to persist.

For splicing use cases, having the ability to cancel of all SEI messages of a particular type regardless of ID value is useful, because prior SEI messages may not be available, so the active SEI message ID values may be unknown.

Use of two separate cancel flags for the two purposes is an alternative but requires an additional syntax element and increases bitrate for signalling of two cancel flags instead of one for all non-cancelling SEI messages.

Syntax and semantics are provided for an example SEI message in section 2.1.

An alternative persistence syntax was proposed in JVET-AH0325 which uses a persistence indicator instead of separate persistence and cancel flags, and provides the ability to indicate that an SEI message persists until the end of the sequence. The alternative persistence design was included in an annex of the VSEI TuC.  Syntax and semantics are provided in section 2.2 for combining the current proposal with the alternative persistence syntax proposed in JVET-AH0325.

It is proposed to adopt the proposed design to the text description SEI message of VSEI v4. Proposed syntax and semantics are provided in section 3.1, and for the alternative persistence design of JVET-AH0325 in section 3.2.

For easy reference, section 4 provides the persistence and cancellation syntax and semantics of two existing SEI messages which use different types of persistence cancellation, the NNPFA SEI and the omnidirectional viewport SEI message. The NNPFA SEI has a cancel flag that cancels persistence of previous NNPFA SEI messages with the same value of nnpfa_target_id, while the omnidirectional viewport SEI message has a cancel flag that cancels persistence of all previous omnidirectional viewport SEI messages.

# Proposed Syntax and Semantics for Example SEI message

## Syntax and semantics for example SEI message

The proposed syntax applied to an example SEI message is shown below. In this example, the highest possible value of ex_id is the dedicated value used to indicate cancellation of all prior SEI messages of the same type.

| example( payloadSize ) { | Descriptor |
| --- | --- |
| ex_id | u(6) |
| if( ex_id != 63 ) { |  |
| ex_cancel_flag | u(1) |
| if( !ex_cancel_flag ) { |  |
| ex_persistence_flag | u(1) |
| /* additional syntax elements */ |  |
| } |  |
| } |  |
| } |  |

ex_id, in the range of 0 to 62, inclusive, specifies an identifier of the example SEI message. ex_id equal to 63 cancels persistence of any previous example SEI messages in output order that applies to the current layer.

ex_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous example SEI message with the same ex_id in output order that applies to the current layer. ex_cancel_flag equal to 0 indicates that additional syntax elements follow.

ex_persistence_flag specifies the persistence of the example SEI message for the current layer.

ex_persistence_flag equal to 1 specifies that the example SEI message applies to the current decoded picture only.

ex_persistence_flag equal to 0 specifies that the example SEI message persists until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an example SEI  message with the same ex_id as the current example SEI message is output that follows the current picture in output order.

## Proposed variant for use with alternate persistence syntax proposed in JVET-AH0325

An alternative persistence syntax was proposed in JVET-AH0325 and included in an annex of the TuC. This alternative syntax uses a persistence indicator instead of separate persistence and cancel flags, and provides the ability to indicate that an SEI message persists until the end of the sequence.

Proposed syntax and semantics for the example SEI message are provided to combine the proposal of this contribution with the alternative syntax of JVET-AH0325.

| example( payloadSize ) { | Descriptor |
| --- | --- |
| ex_id | u(6) |
| if( ex_id != 63 ) { |  |
| ex_persistence_idc | u(2) |
| if( ex_persistence_idc > 0 ) { |  |
| /* additional syntax elements */ |  |
| } |  |
| } |  |

ex_id, in the range of 0 to 62, inclusive, specifies an identifier of the example SEI message. ex_id equal to 63 cancels persistence of any previous example SEI messages in output order that applies to the current layer.

ex_persistence_idc specifies the persistence of the example SEI message for the current layer.

ex_persistence_idc equal to 0 indicates that the example SEI message cancels the persistence of any previous example SEI message in output order that applies to the current layer with the same ex_id of the current example SEI message.

ex_persistence_idc equal to 1 specifies that the example SEI message applies to the current decoded picture only.

ex_persistence_idc equal to 2 specifies that the example SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an example SEI message with the same value of ex_id is output that follows the current picture in output order.

ex_persistence_idc equal to 3 specifies that the example SEI message applies for the entire CLVS.

It is a requirement of bitstream conformance that when ex_persistence_idc is equal to 3 and this SEI message is not the first example SEI message in a CLVS, in decoding order, an example SEI message with the same value of ex_id shall be present in the first PU of the CLVS in decoding order.

All example SEI messages in a CLVS that have a particular ex_id value and ex_persistence_idc equal to 3 shall have identical SEI payload content.

When ex_persistence_idc is equal to 3 in the first PU in the CLVS in decoding order, all example SEI messages in the CLVS that have a particular ex_id value shall have identical SEI payload content.

# Text description SEI message

It is proposed to adopt the proposed design in the text description SEI message of VSEI v4. The current test description SEI message provides the capability to cancel the persistence of previous text description SEI message with the same value of txt_descr_id, but does not provide the capability to cancel all previous text descriptions SEI messages. With the proposed design, cancellation of all previous text description SEI messages is enabled by using the highest value of txt_descr_id.

Proposed syntax and semantics modifications are provided below.

## Proposed syntax and semantics

    - Text description information SEI message syntax

| text_description( payloadSize ) { | Descriptor |
| --- | --- |
| txt_descr_id | u(14) |
| if( txt_descr_id != 16383 ) { |  |
| txt_cancel_flag | u(1) |
| if( !txt_cancel_flag ) { |  |
| txt_persistence_flag | u(1) |
| txt_descr_purpose | u(8) |
| txt_num_strings_minus1 | u(8) |
| for( i = 0; i <= txt_num_strings_minus1; i++ ) { |  |
| txt_descr_string_lang[ i ] | st(v) |
| txt_descr_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |

    - Text description information SEI message semantics

The text description information SEI message provides text description about one or more pictures.

txt_descr_id in the range of 1 to 16382, inclusive, indicates the identifier value of this text description information SEI message. txt_descr_id equal to 16383 cancels persistence of any previous example SEI messages in output order that applies to the current layer. The value of txt_descr_id shall be in the range of 1 to 16383, inclusive. Value 0 is reserved.

txt_cancel_flag equal to 1 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_id in output order that applies to the current layer. txt_cancel_flag equal to 0 indicates that text description information follows.

txt_persistence_flag specifies the persistence of the text information description SEI message for the current layer.

txt_persistence_flag equal to 0 specifies that the text description information applies to the current decoded picture only.

txt_persistence_flag equal to 1 specifies that the text description information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a text description information SEI message with the same txt_descr_id is output that follows the current picture in output order.

txt_descr_purpose indicates the purpose of the text description SEI as specified in Table xx. The value of text_descr_purpose shall be in the range of 0 to 5, inclusive. Values in the range of 6 to 255, inclusive, for text_descr_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of text_descr_purpose in the range of 0 to 255, inclusive.

Table xx – Definition of txt_descr_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General comment information |
| 4 | Content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) |
| 5 | Tag URI for identifying the bitstream |
| 6..255 | Reserved |

txt_num_strings_minus1 plus 1 indicates the number of entries for txt_descr_string_lang[ i ] and txt_descr_string[ i ] that follow.

txt_descr_string_lang[ i ] specifies the language of the txt_descr_string[ i ]. The language of the txt_descr_string[ i ]  shall be given by a language tag as defined by IETF RFC 5646. The length of txt_descr_string_lang[ i ]  shall be in the range of 0 to 49, inclusive.

txt_descr_string[ i ] specifies i-th text description information string whose value is interpreted as specified by the txt_descr_purpose.

When txt_descr_purpose is equal to 0, the interpretation of what information is conveyed in the txt_descr_string is application-defined.

When txt_descr_purpose is equal to 1, the txt_descr_string[ i ] specifies copyright information that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 2, the txt_descr_string[ i ] specifies, when not a null string, AI marking information that pertains to the picture(s) within the persistence scope of this SEI message.

NOTE: When txt_descr_purpose is equal to 2 the string can contain information about the machine-learning-based processing, intended use of the decoded pictures, or other aspects relevant to the associated pictures.

When txt_descr_purpose is equal to 3, the txt_descr_string[ i ] specifies a general text label description that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 4, the txt_descr_string[ i ] specifies content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 5, txt_descr_string[ i ] contains a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the CLVS.

## Alternative proposed syntax and semantics

Syntax and semantics are provided for the alternative persistence signalling proposed in JVET-AH0325.

    - Text description information SEI message syntax

| text_description( payloadSize ) { | Descriptor |
| --- | --- |
| txt_descr_id | u(14) |
| if( txt_descr_id != 16383 ) { |  |
| txt_persistence_idc | u(1) |
| txt_cancel_flag | u(1) |
| if( !txt_cancel_flag ) { |  |
| txt_persistence_flag | u(1) |
| if( !txt_persistence_idc > 0 ) { |  |
| txt_descr_purpose | u(8) |
| txt_num_strings_minus1 | u(8) |
| for( i = 0; i <= txt_num_strings_minus1; i++ ) { |  |
| txt_descr_string_lang[ i ] | st(v) |
| txt_descr_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |

    - Text description information SEI message semantics

The text description information SEI message provides text description about one or more pictures.

txt_descr_id in the range of 1 to 16382, inclusive, indicates the identifier value of this text description information SEI message. txt_descr_id equal to 16383 cancels persistence of any previous text description SEI messages in output order that applies to the current layer. The value of txt_descr_id shall be in the range of 1 to 16383, inclusive. Value 0 is reserved.

txt_cancel_flag equal to 1 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_id in output order that applies to the current layer. txt_cancel_flag equal to 0 indicates that text description information follows.

txt_persistence_flagtxt_persistence_idc specifies the persistence of the text information description SEI message for the current layer.

txt_persistence_flag equal to 0 specifies that the text description information applies to the current decoded picture only.

txt_persistence_flag equal to 1 specifies that the text description information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a text description information SEI message with the same txt_descr_id is output that follows the current picture in output order.

txt_persistence_idc equal to 3 specifies that the example SEI message applies for the entire CLVS.

It is a requirement of bitstream conformance that when txt_persistence_idc is equal to 3 and this SEI message is not the first example SEI message in a CLVS, in decoding order, an example SEI message with the same value of txt_descr_id shall be present in the first PU of the CLVS in decoding order.

All example SEI messages in a CLVS that have a particular txt_descr_id value and txt_persistence_idc equal to 3 shall have identical SEI payload content.

When txt_persistence_idc is equal to 3 in the first PU in the CLVS in decoding order, all example SEI messages in the CLVS that have a particular txt_id value shall have identical SEI payload content.

txt_descr_purpose indicates the purpose of the text description SEI as specified in Table xx. The value of text_descr_purpose shall be in the range of 0 to 5, inclusive. Values in the range of 6 to 255, inclusive, for text_descr_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of text_descr_purpose in the range of 0 to 255, inclusive.

Table xx – Definition of txt_descr_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General comment information |
| 4 | Content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) |
| 5 | Tag URI for identifying the bitstream |
| 6..255 | Reserved |

txt_num_strings_minus1 plus 1 indicates the number of entries for txt_descr_string_lang[ i ] and txt_descr_string[ i ] that follow.

txt_descr_string_lang[ i ] specifies the language of the txt_descr_string[ i ]. The language of the txt_descr_string[ i ]  shall be given by a language tag as defined by IETF RFC 5646. The length of txt_descr_string_lang[ i ]  shall be in the range of 0 to 49, inclusive.

txt_descr_string[ i ] specifies i-th text description information string whose value is interpreted as specified by the txt_descr_purpose.

When txt_descr_purpose is equal to 0, the interpretation of what information is conveyed in the txt_descr_string is application-defined.

When txt_descr_purpose is equal to 1, the txt_descr_string[ i ] specifies copyright information that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 2, the txt_descr_string[ i ] specifies, when not a null string, AI marking information that pertains to the picture(s) within the persistence scope of this SEI message.

NOTE: When txt_descr_purpose is equal to 2 the string can contain information about the machine-learning-based processing, intended use of the decoded pictures, or other aspects relevant to the associated pictures.

When txt_descr_purpose is equal to 3, the txt_descr_string[ i ] specifies a general text label description that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 4, the txt_descr_string[ i ] specifies content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) that pertains to the picture(s) in the persistence scope defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 5, txt_descr_string[ i ] contains a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the CLVS.

# Background

For easy reference, the persistence and cancellation syntax and semantics of two existing SEI messages are shown which use different types of persistence cancellation.

## NNPFA SEI

The NNPFA SEI has a cancel flag that cancels persistence of previous NNPFA SEI messages with the same value of nnpfa_target_id.

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| /* Additional syntax elements */ |  |
| } |  |
| } |  |

nnpfa_target_id indicates the nnpfc_id of the target NNPF, which is specified by one or more NNPFC SEI messages that pertain to the current picture and have nnpfc_id equal to nnpfa_target_id. The value of nnpfa_target_id shall be in the range of 0 to 232 − 2, inclusive.

An NNPFA SEI message with a particular value of nnpfa_target_id shall not be present in a current PU unless one or both of the following conditions are true:

–	Within the current CLVS there is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_target_id present in a PU preceding the current PU in decoding order.

–	There is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_target_id in the current PU.

When a PU contains both an NNPFC SEI message with a particular value of nnpfc_id and an NNPFA SEI message with nnpfa_target_id equal to the particular value of nnpfc_id, the NNPFC SEI message shall precede the NNPFA SEI message in decoding order.

nnpfa_cancel_flag equal to 1 indicates that the persistence of the target NNPF established by any previous NNPFA SEI message with the same nnpfa_target_id as the current SEI message is cancelled, i.e., the target NNPF is no longer used unless it is activated by another NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 0. nnpfa_cancel_flag equal to 0 indicates that the nnpfa_persistence_flag, nnpfa_target_base_flag, nnpfa_no_prev_clvs_flag, nnpfa_no_foll_clvs_flag (when nnpfa_persistence_flag is equal to 1), and nnpfa_num_output_entries follow.

nnpfa_persistence_flag specifies the persistence of the target NNPF for the current layer.

nnpfa_persistence_flag equal to 0 specifies that the target NNPF may be used for post-processing filtering for the current picture only.

nnpfa_persistence_flag equal to 1 specifies that the target NNPF may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with an NNPFA SEI message with the same nnpfa_target_id as the current SEI message is output that follows the current picture in output order.

NOTE 2 – The target NNPF is not applied for this subsequent picture in the current layer associated with an NNPFA SEI message with the same nnpfa_target_id as the current SEI message.

Let the nnpfcTargetPictures be the set of pictures to which the NNPFC SEI message corresponding to the target NNPF pertains. Let nnpfaTargetPictures be the set of pictures for which the target NNPF is activated by the current NNPFA SEI message. It is a requirement of bitstream conformance that any picture included in nnpfaTargetPictures shall also be included in nnpfcTargetPictures.

## Omnidirectional viewport SEI message syntax

The omnidirectional viewport SEI message has a cancel flag that cancels persistence of all previous omnidirectional viewport SEI messages.

| omni_viewport( payloadSize ) { | Descriptor |
| --- | --- |
| omni_viewport_id | u(10) |
| omni_viewport_cancel_flag | u(1) |
| if( !omni_viewport_cancel_flag ) { |  |
| omni_viewport_persistence_flag | u(1) |
| /* Additional syntax elements */ |  |
| } |  |
| } |  |

The omnidirectional viewport SEI message specifies the coordinates of one or more regions of spherical-coordinate geometry, bounded by four great circles, corresponding to viewports recommended for display when the user does not have control of the viewing orientation or has released control of the viewing orientation.

When an effectively applicable frame packing arrangement SEI message, as specified in clause 8.15.2.2 or clause 8.15.3.2, that applies to the picture is present, the information indicated by the omnidirectional viewport SEI message applies to both views.

omni_viewport_id contains an identifying number that may be used to identify the purpose of the one or more recommended viewport regions.

omni_viewport_id equal to 0 indicates that the recommended viewports are per "director’s cut", i.e., a viewport suggested according to the creative intent of the content author or content provider. omni_viewport_id equal to 1 indicates that the recommended viewports are selected based on measurements of viewing statistics.

Values of omni_viewport_id from 2 to 511, inclusive, may be used as determined by the application. Values of omni_viewport_id from 512 to 1023 are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of omni_viewport_id in the range of 512 to 1023, inclusive, shall ignore it.

omni_viewport_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous omnidirectional viewport SEI message in output order. omni_viewport_cancel_flag equal to 0 indicates that omnidirectional viewport information follows.

omni_viewport_persistence_flag specifies the persistence of the omnidirectional viewport SEI message for the current layer.

omni_viewport_persistence_flag equal to 0 specifies that the omnidirectional viewport SEI message applies to the current decoded picture only.

omni_viewport_persistence_flag equal to 1 specifies that the omnidirectional viewport SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an omnidirectional viewport SEI message is output that follows the current picture in output order.

When no omnidirectional video projection is indicated to apply to a picture, e.g., by an equirectangular projection SEI message with erp_cancel_flag equal to 0 or a generalized cubemap projection SEI message with gcmp_cancel_flag equal to 0 being present in the CLVS that applies to the current picture, an omnidirectional viewport SEI message with omni_viewport_cancel_flag equal to 0 shall not be present in the CLVS that applies to the picture. Decoders shall ignore omnidirectional viewport SEI messages with omni_viewport_cancel_flag equal to 0 for pictures to which no omnidirectional video projection is indicated to apply.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).