| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, Japan, 12–19 July 2024 | Document: JVET-AI0190-v2 |
| --- | --- |

| Title: | AHG9: On GFV SEI persistence and picture presence |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Kaifa Yang2, Yue Li2, Yiling Xu2 1Bytedance Inc., US 2Shanghai Jiao Tong University, China | Email: | yekui.wang@bytedance.com sekiroyyy@sjtu.edu.cn yueli983@sjtu.edu.cn yl.xu@sjtu.edu.cn |
| Source: | Bytedance Inc., Shanghai Jiao Tong University |  |  |

_____________________________

# Abstract

This contribution proposes the following changes:

- Add asserted missing persistence scope specification for the GFV SEI message when gfv_base_pic_flag is equal to 1, as follows: When a GFV SEI message has gfv_base_pic_flag equal to 1, the GFV SEI message pertains to the current decoded picture only.

- Specify the persistence scope for the GFV SEI message when gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 as follows: When a GFV SEI message has gfv_base_pic_flag equal to 0 and gfv_drive_pic_fusion_flag equal to 0, the GFV SEI message pertains to the current decoded picture only.

- Specify the persistence scope for the GFV SEI message when gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 as follows: When a GFV SEI message with a particular gfv_id value has gfv_base_pic_flag equal to 0 and gfv_drive_pic_fusion_flag equal to 1, the GFV SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that is within the current CLVS, follows the current decoded picture in output order, and is associated with a GFV SEI message having that particular gfv_id value, whichever is earlier.

- Add a constraint to disallow pictures from being both a base picture and a "fusion picture": When a GFV SEI message gfvSeiA with a particular gfv_id value has gfv_cnt greater than 0 and a GFV SEI message with the same gfv_id value in the same picture unit has gfv_base_pic_flag equal to 1 (i.e., the current decoded picture is a base picture), the GFV SEI message gfvSeiA shall have gfv_drive_pic_fusion_flag equal to 0.

- Add a constraint to disallow the presence of both pictures associated with GFV SEI messages and pictures not associated with GFV SEI messages in a CLVS: Within a CLVS containing at least one GFV SEI message with a particular gfv_id value, each picture shall be associated with at least one GFV SEI message with the particular gfv_id value (i.e., the picture unit containing the picture contains a GFV SEI message with the particular gfv_id value, or the picture is within the persistence scope of a GFV SEI message with the particular gfv_id value).

If such a constraint is not imposed, then it is suggested to disallow decoding dependency of a picture associated with a GFV SEI message on a picture not associated with a GFV SEI message, to enable systems that do recognize and utilize GVF SEI messages to discard, thus avoid transmission and decoding, coded pictures not associated with GFV SEI messages.

# On the persistence scope of a GFV SEI message with gfv_base_pic_flag equal to 1

Currently, the persistence scope of a GFV SEI message is specified:

When a GFV SEI message that has a particular gfv_id value has gfv_base_pic_flag being equal to 0, this SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that follows the current decoded picture in output order within the current CLVS and is associated with a subsequent GFV SEI message, in decoding order, having gfv_base_pic_flag equal to 0 and that particular gfv_id value within the current CLVS, whichever is earlier.

As can be seen, the persistence scope of a GFV SEI message with gfv_base_pic_flag equal to 0 is specified. However, the persistence scope of a GFV SEI message with gfv_base_pic_flag equal to 1 is not specified.

Therefore, the following is proposed:

When a GFV SEI message has gfv_base_pic_flag equal to 1, the GFV SEI message pertains to the current decoded picture only.

# On the persistence scope of a GFV SEI message with gfv_base_pic_flag equal to 0 and gfv_drive_pic_fusion_flag equal to 0

Currently, a GFV SEI message with gfv_base_pic_flag equal to 0 and gfv_drive_pic_fusion_flag equal to 0 can pertain to more than one decoded picture, including the current decoded picture and one or more subsequent decoded pictures in output order.

However, when gfv_drive_pic_fusion_flag is equal to 0, a decoded picture within the persistence scope of the GFV SEI message is not included in the input to the generator network, therefore having the coded picture corresponding to any of such subsequent decoded pictures in output order in the CLVS is a pure waste of bits.

Therefore, the following is proposed:

When a GFV SEI message has gfv_base_pic_flag equal to 0 and gfv_drive_pic_fusion_flag equal to 0, the GFV SEI message pertains to the current decoded picture only.

# On the persistence scope of a GFV SEI message with gfv_base_pic_flag equal to 0 and gfv_drive_pic_fusion_flag equal to 1

Let's repeat the current persistence scope specification of a GFV SEI message below for convenience:

When a GFV SEI message that has a particular gfv_id value has gfv_base_pic_flag being equal to 0, this SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that follows the current decoded picture in output order within the current CLVS and is associated with a subsequent GFV SEI message, in decoding order, having gfv_base_pic_flag equal to 0 and that particular gfv_id value within the current CLVS, whichever is earlier.

The presence of "having gfv_base_pic_flag equal to 0" above creates an issue. For example, assuming that there are three pictures A, B, and C consecutive in increasing output order and also in increasing decoding order in a CLVS, each of them is associated with a GFV SEI message (i.e., each of the three picture units containing the three pictures contains a GFV SEI message), and all the GFV SEI messages have the same gfv_id value. The gfv_base_pic_flag values for the three GFV SEI messages for picture A, B, and C, are 0, 1, and 0, respectively. According to the above persistence scope specification, the GFV SEI message associated with the picture A would pertain to both picture A and picture B, while picture B is associated with its own GFV SEI message with the same gfv_id and gfv_base_pic_flag equal to 1. That is problematic, both semantically and on the usefulness of the GFV SEI message of picture B.

Therefore, the following is proposed to replace the current persistence scope specification (with "having gfv_base_pic_flag equal to 0" removed, among some other changes):

When a GFV SEI message with a particular gfv_id value has gfv_base_pic_flag equal to 0 and gfv_drive_pic_fusion_flag equal to 1, the GFV SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that is within the current CLVS, follows the current decoded picture in output order, and is associated with a GFV SEI message having that particular gfv_id value, whichever is earlier.

# On disallowing pictures from being both a base picture and a "fusion picture"

When a GFV SEI message gfvSeiA with a particular gfv_id value has gfv_cnt greater than 0 and a GFV SEI message with the same gfv_id value in the same picture unit has gfv_base_pic_flag equal to 1 (i.e., the current decoded picture is a base picture), it does not make sense for the GFV SEI message gfvSeiA to have gfv_drive_pic_fusion_flag equal to 1, because although the GFV SEI message gfvSeiA was included in the picture unit of the base picture, it was intended to be used for generation of a different picture than the base picture. Furthermore, the process for deriving the final output of the StoreOutputTensors( ) process gets messed up by this mixing of picture types, as per the process the output of a base picture is the cropped output picture while the output of a "fusion picture" is a generated picture with fusion.

Therefore, the following constraint is proposed to disallow pictures from being both a base picture and a "fusion picture":

When a GFV SEI message gfvSeiA with a particular gfv_id value has gfv_cnt greater than 0 and a GFV SEI message with the same gfv_id value in the same picture unit has gfv_base_pic_flag equal to 1 (i.e., the current decoded picture is a base picture), the GFV SEI message gfvSeiA shall have gfv_drive_pic_fusion_flag equal to 0.

# On disallowing the presence of both pictures associated with GFV SEI messages and pictures not associated with GFV SEI messages in a CLVS

When one picture in a CLVS is associated with at least one GFV SEI message (i.e., the picture unit containing the picture contains a GFV SEI message, or the picture is within the persistence scope of a GFV SEI message), it does not make sense to have another picture in the CLVS not associated with at least one GFV SEI message. This is because that for systems that do recognize and utilize GVF SEI messages, the bits representing those pictures not associated with any GFV SEI message would be a pure waste and allowing such pictures would forfeit the purpose of having GFV SEI messages for achieving very low bitrate at the first place.

Therefore, the following constraint is proposed to disallow the presence of both pictures associated with GFV SEI messages and pictures not associated with GFV SEI messages in a CLVS:

Within a CLVS containing at least one GFV SEI message with a particular gfv_id value, each picture shall be associated with at least one GFV SEI message with the particular gfv_id value (i.e., the picture unit containing the picture contains a GFV SEI message with the particular gfv_id value, or the picture is within the persistence scope of a GFV SEI message with the particular gfv_id value).

If such a constraint is not imposed, then it is suggested to disallow decoding dependency of a picture associated with a GFV SEI message on a picture not associated with a GFV SEI message, to enable systems that do recognize and utilize GVF SEI messages to discard, thus avoid transmission and decoding, coded pictures not associated with GFV SEI messages.

# Patent rights declaration(s)

Bytedance Inc. and Shanghai Jiao Tong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).