| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0196-v1 |
| --- | --- |

| Title: | AHG8: On combined pre- and post-processing and ROI-based adaptive QP for machine vision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shurun Wang Binzhe Li Jie Chen Yan Ye (Alibaba Group) Shiqi Wang (City University of Hong Kong) | Tel: Email: | {shurun.wsr, libinzhe.lbz, jiechen.cj, yan.ye}@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba Group, City University of Hong Kong |  |  |

_____________________________

- Abstract

This contribution reports the performance of enabling both pre- and post-processing and ROI-based adaptive QP for machine vision. According to experimental results, it is reported that when both pre- and post-processing and ROI-based adaptive QP in AGH8 software are enabled, the combined method achieves 36.74% (RA), 49.49% (LD) and 43.06% (AI) BD-rate savings for object detection task on the SFU-HW dataset under VCM CTC, which is higher than individually enabling pre- and post-processing or ROI-based adaptive QP for machine vision.

Description of both pre- and post-processing and ROI-based adaptive QP enabling

In previous meetings, JVET-AB0275 [1] proposed the ROI-based adaptive QP method (ROIAQP) for encoding optimization, and JVET-AH0157 [2] proposed the pre- and post-processing method (PREPOST), which consists of the pre-processing of foreground and background [3] and the enhancement post-filtering [4]. Based on the interesting coding performance introduced by these methods, they are included in the technical report (TR) [5] on optimization of encoders and receiving systems for machine analysis of coded video content, and the software for both methods are available in AHG8 software branch. In this contribution, the experiments of combining both methods are conducted and the results are reported.

The experiment flowchart is shown in Figure 1. The original video is processed with foreground and background pre-processing first. And then the pre-processed video is encoded with ROIAQP optimized VVC encoder. After decoding, the decoded video is further processed by the enhancement post-filter. And finally, the filtered video is fed to the machine consumption model.

Figure 1 The experiment flowchart on combined pre- and post-processing and ROI-based adaptive QP

Experimental results

The experiment is conducted under the VCM CTC specified in JVET-AH2031 [6], where the VTM version is 23.3. The overall BD rate performance for detection is shown in Table 1. Overall, better coding gain can be achieved from combined pre- and post-processing and ROI-based adaptive QP, noted as JOINT, compared with individually enabling pre- and post-processing or ROI-based adaptive QP.

Table 1 Overview of the averaged BD-rate performance in terms of mono-mAP

| Task | Dataset | Methods | RA | LD | AI |
| --- | --- | --- | --- | --- | --- |
| Detection | SFU-HW | PREPOST | -32.35% | -45.60% | -40.10% |
|  |  | ROIAQP | -10.81% | -25.69% | -8.55% |
|  |  | JOINT | -36.74% | -49.49% | -43.06% |

The per-class performance on SFU-HW dataset for object detection is shown in Table 2 and the per-sequence performance on TVD dataset for object tracking is given in Table 3. (Please note “/” in the tables indicates the performance cannot be calculated validly, due to the nonmonotonic machine task performance or the non-overlap between rate-accuracy curves). The results show that the combined pre- and post-processing and ROI-based adaptive QP achieves significant performance improvement under CTC.

Table 2 Performance of combined pre- and post-processing and ROI-based adaptive QP on SFU-HW dataset under RA, LD and AI configurations.

|  |  | End-to-End BD-rate (%) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Mono-mAP | mAP | End-to-End BD-mAP |
| SFU-HW | Class A | -18.70% | / | 2.72 |
|  | Class B | -37.63% | / | 7.79 |
|  | Class C | -40.16% | -41.03% | 5.95 |
|  | Class D | -36.95% | -38.22% | 4.19 |
|  | Average | -36.74% | / | 5.73 |
| LD | Object Detection | Mono-mAP | mAP | End-to-End BD-mAP |
| SFU-HW | Class A | -40.34% | / | 10.87 |
|  | Class B | -38.31% | / | 9.01 |
|  | Class C | -53.49% | -52.38% | 9.64 |
|  | Class D | -58.96% | -58.87% | 9.91 |
|  | Average | -49.49% | - | 9.62 |
| AI | Object Detection | Mono-mAP | mAP | End-to-End BD-mAP |
| SFU-HW | Class A | -45.67% | -45.14% | 15.04 |
|  | Class B | -50.40% | / | 13.14 |
|  | Class C | -46.27% | -46.64% | 9.14 |
|  | Class D | -31.87% | -31.87% | 5.18 |
|  | Average | -43.06% | / | 9.61 |

Proposal

In this contribution, the performance of combined pre- and post-processing and ROI-based adaptive QP is reported. Simulation results show that, jointly applying pre- and post-processing and ROI-based adaptive QP can achieve obvious coding performance improvement compared with pre- and post-processing or ROI-based adaptive QP only.

Thus, it is suggested to update the following text into section 5.1 of the next draft of technical report (TR) on optimization of encoders and receiving systems for machine analysis of coded video content, as shown in the attached TR draft file.

“It is noted that depending on specific use cases, the technologies outlined in this report can be implemented individually or in combination to optimize machine consumption performance within the constraints of the system capabilities. For example, a better performance can be achieved by applying a post-filtering being optimized towards object detection on the decoded video that was pre-processed with the method described in Annex A.2 and encoded with the ROI-based adaptive QP method described in Annex A.1. When employing multiple technologies simultaneously, it is important to consider that certain combinations can be impractical or infeasible due to inherent methodological constraints.”

Further, it is suggested to include the combined pre- and post-processing and ROI-based adaptive QP implementation into AHG8 software branch.

References

- C. Hollmann, J. Ström, P. Wennersten, L. Litwic, R. Sjöberg, M. Damghanian, “Information about Ericsson’s response to the CfP for Video Coding for Machines,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 input document JVET-AB0275, 28th Meeting, Mainz, DE, 20–28 October 2022 (available online at https://jvet-experts.org/)

- S. Wang, B. Li, J. Chen, Y. Ye, “AHG8: On combined pre-processing and post-processing for machine vision,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 input document JVET-AH0157, 34th Meeting, Rennes, FR, 17–27 April 2024 (available online at https://jvet-experts.org/).

- B. Li, S. Wang, J. Chen, Y. Ye, S. Wang, “AHG8: A preprocessing software implementation for OFM,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 input document JVET-AD0047, 30th Meeting, Antalya, TR, 21–28 April 2023 (available online at https://jvet-experts.org/).

- B. Li, S. Wang, J. Chen, Y. Ye, S. Wang, “AHG8: A post-processing algorithm for machine consumption,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 input document JVET-AG0212, 33rd Meeting, Online, 17–26 January 2024 (available online at https://jvet-experts.org/).

- S. Liu, J. Chen, C. Hollmann, “Optimization of encoders and receiving systems for machine analysis of coded video content (draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AH2030, 34th Meeting, Rennes, FR, 17–27 April 2024 (available online at https://jvet-experts.org/).

- S. Liu, C. Hollmann, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AH2031, 34th Meeting, Rennes, FR, 17–27 April 2024 (available online at https://jvet-experts.org/).

Patent rights declaration(s)

Alibaba group and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).