| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0199-v3 |
| --- | --- |

| Title: | AHG8: A new performance evaluation method for object detection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shurun Wang Binzhe Li Jie Chen Yan Ye (Alibaba Group) Shiqi Wang (City University of Hong Kong) | Tel: Email: | {shurun.wsr, libinzhe.lbz, jiechen.cj, yan.ye}@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba Group, City University of Hong Kong |  |  |

_____________________________

- Abstract

A new performance evaluation method for object detection is proposed in this contribution to improve the stability of evaluation results, i.e., mean average precision. In the proposed method, the results of object detection performed on the uncompressed sequences are used as the ground truth and the average precision is weighted averaged based on the number of instances in each class when the mean average precision is calculated. According to the experimental results, it is reported that the proposed method reduces the instability introduced by non-monotonicity of rate-accuracy curves.

In update v2, the slides have been uploaded.

In update v3, the slidesis updated.

Problem Statement

Current object detection performance evaluation is not stable. Specifically, with the coding bitrate increases, the object detection metric, mean average precision (mAP), may not always increase, which results in non-monotonic rate-accuracy curves. As shown in Fig. 1, the sequences in SFU-HW dataset are compressed with consecutive quantization parameters (QPs) from 27 to 47. There are obvious fluctuations of the mAP performance on various sequences under different coding configurations, showing the instability of mAP for object detection performance evaluation.

Fig. 1 The rate-accuracy curves for different sequences under random access (RA), low delay (LD) and all intra (AI) configurations. The codec is VTM-23.0 and the QP values are from 27 to 47, 21 rate-accuracy points in total. The object detection accuracy is measured by mAP with the original calculation method.

To tackle this problem, the object detection accuracy metric is investigated in the contribution and it is found that:

- The numbers of instances of each object class are unbalanced.

In current mAP calculation, the final object detection performance mAP is calculated by equally averaging the detection performance of each class, ignoring the unbalanced numbers of instances in each class. In Table 1, the numbers of instances in each object class are summarized. Obviously uneven distribution can be observed, indicating that applying equal averaging for mAP calculation may not be adequate.

Table 1 The numbers of instances in different classes in the human vision annotated ground truths for sequences in SFU-HW dataset.

| Cactus_1920x1080_50 |  | PartyScene_832x480_50 |  | BQSquare_416x240_60 |  | BasketballDrive_1920x1080_50 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class name | Instance number | Class name | Instance number | Class name | Instance number | Class name | Instance number |
| Common sunflower | 194 | Person | 148 | Person | 1210 | Person | 566 |
| Book | 97 | Coffee cup | 81 | Car | 46 | Football | 184 |
|  |  | Teddy Bear | 194 | Boat | 129 | Chair | 489 |
|  |  |  |  | Umbrella | 704 |  |  |
|  |  |  |  | Handbag | 15 |  |  |
|  |  |  |  | Chair | 1194 |  |  |

- Current evaluation ground truths are labeled by human vision, not machine vision.

To evaluate the impact of video coding distortion on the machine vision, i.e., the performance difference between a machine task performed on uncompressed videos and compressed videos, the ground truth should be machine analysis results on uncompressed sequences. And the machine analysis results on the compressed videos should be compared with this ground truth. However, in current evaluation framework, the ground truths are labeled by human vision, not machine vision, which means current evaluation of video coding for machines (VCM) mixes the impact of compression distortion with the accuracy of machine analysis model itself.

Proposal

In order to improve the stability of object detection performance evaluation, a new evaluation method is proposed in this contribution by jointly considering numbers of instances in each class and the ground truths labelled by machine analysis model on uncompressed sequences. Specifically, the object detection results on uncompressed sequences with the CTC model are denoted as {(bboxi,j,si,j)}, where bboxi,j and si,j are noted as the bounding box and confidence score of jth instance in object class i, respectively. Herein, 0≤si,j≤1, j=1,2,3,…,ni, ni is the instance number of object class i. For each object class i, the average precision can be achieved based on the bounding boxes {(bboxi,j)}, denoted as APi. Moreover, the proposed mAP is formulated as ij=1nisijNAPi, where N=ij=1nisij.

Evaluation

The proposed evaluation method is tested on SFU-HW dataset, under RA, LD and AI configurations, where the codec is VTM-23.0 and the QP values are from 27 to 47 to show the rate-accuracy monotonicity improvements. Herein, the monotonicity metric is defined based on first order gradient, noted as G. Specifically, two consecutive QP values are noted as a QP pair. For every QP pair, if the mAP value of the smaller QP is lower than that of the bigger QP (i.e., non-monotonical case), the first order gradient is calculated, formulated as gi=mAPi+1-mAPiri+1-ri, where mAPi is the object detection performance with bitrate ri, under the compression of ith QP value. If the mAP value of the smaller QP is greater than that of the bigger QP (i.e., monotonic case), gi=0. The monotonicity metric G is the sum of gi across all QP pairs. As such, G could be formulated as below, where n is the QP value number, ri+1 and ri are the bitrate with smaller QP and bigger QP in every QP pair, ri+1>ri.

G=i=1n-1gi=i=1n-1f(mAPi+1,mAPi,ri+1,ri)

fmAPi+1,mAPi,ri+1,ri=mAPi+1-mAPiri+1-ri,  if mAPi+1<mAPi0,                        if mAPi+1≥mAPi.

Based on this definition, G is a non-positive value. The closer G is to 0, the better monotonicity is.

Moreover, ablation studies are also conducted to investigate the independent effect of two aspects in the proposed method, which are the weighted average based on the number of instances in each class and the machine analysis ground truth labelled by object detection results on the uncompressed videos, respectively. As such, there are 4 tests conducted to evaluate the monotonicity of proposed method in Table 2.

Table 2 The experimental configurations of various tests. Test1 indicates original mAP calculation method and Test4 indicates proposed mAP calculation method.

| Test | Weighted average based on the number of instances in each class | Machine analysis ground truth labelled by object detection results on the uncompressed videos |
| --- | --- | --- |
| Test1 | False | False |
| Test2 | True | False |
| Test3 | False | True |
| Test4 | True | True |

The value of G calculated with various tests are summarized in Table 3. It’s obvious that the proposed method can improve the monotonicity significantly.

Table 3 The monotonicity metric G of 4 tests under RA, LD and AI configurations on SFU-HW dataset.

| G | RA |  |  |  | LD |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence name | Test1 | Test2 | Test3 | Test4 | Test1 | Test2 | Test3 | Test4 | Test1 | Test2 | Test3 | Test4 |
| Traffic_2560x1600_30 | -0.0116 | -0.0009 | -0.0906 | -0.0015 | -0.0258 | -0.0212 | -0.1010 | -0.0087 | -0.0006 | -0.0001 | -0.0156 | 0.0000 |
| ParkScene_1920x1080_24 | -0.1044 | -0.1084 | -0.1030 | -0.0311 | -0.5301 | -0.4067 | -0.1891 | -0.2319 | -0.0040 | -0.0013 | -0.0220 | 0.0000 |
| Cactus_1920x1080_50 | -0.5790 | -0.3646 | -0.0515 | -0.0561 | -0.6545 | -0.4210 | -0.0130 | -0.0307 | -0.0288 | -0.0185 | -0.0012 | 0.0000 |
| BasketballDrive_1920x1080_50 | -0.0059 | -0.0101 | -0.0254 | 0.0000 | -0.0102 | -0.0049 | -0.0516 | 0.0000 | -0.0008 | -0.0005 | -0.0062 | 0.0000 |
| BQTerrace_1920x1080_60 | -0.0101 | -0.0040 | -0.0025 | 0.0000 | -0.0044 | -0.0086 | -0.0023 | 0.0000 | -0.0001 | 0.0000 | 0.0000 | 0.0000 |
| BasketballDrill_832x480_50 | -0.0959 | -0.1071 | -0.2727 | -0.1541 | -0.0657 | -0.0869 | -0.3322 | -0.4564 | 0.0000 | 0.0000 | -0.0270 | 0.0000 |
| BQMall_832x480_60 | -0.0486 | -0.0281 | -0.2055 | 0.0000 | -0.0841 | -0.0361 | -0.2126 | -0.0348 | 0.0000 | 0.0000 | -0.0060 | 0.0000 |
| PartyScene_832x480_50 | -0.1433 | -0.1005 | -0.0630 | -0.0015 | -0.2737 | -0.2145 | -0.0535 | -0.0591 | 0.0000 | -0.0001 | -0.0010 | 0.0000 |
| RaceHorses_832x480_30 | -0.0544 | -0.0539 | -0.5550 | -0.0067 | -0.0636 | -0.0623 | -0.1857 | 0.0000 | -0.0016 | -0.0017 | -0.0653 | 0.0000 |
| BasketballPass_416x240_50 | -0.1175 | -0.0958 | -0.1824 | 0.0000 | -0.1549 | -0.2787 | -0.2197 | -0.0348 | -0.0027 | -0.0025 | -0.0589 | 0.0000 |
| BQSquare_416x240_60 | -0.6453 | -0.0893 | -0.0936 | -0.1483 | -1.7101 | -0.5135 | -0.0597 | -0.1336 | -0.0045 | -0.0002 | -0.0031 | 0.0000 |
| BlowingBubbles_416x240_50 | -0.9836 | -0.6587 | -0.1194 | -0.0924 | -0.8899 | -1.2954 | -0.0297 | -0.3220 | 0.0000 | 0.0000 | -0.0008 | 0.0000 |
| RaceHorses_416x240_30 | -0.1456 | -0.1500 | -0.6558 | -0.0955 | -0.0971 | -0.0977 | -0.5542 | -0.1256 | 0.0000 | 0.0000 | -0.1241 | -0.0014 |
| AVG | -0.2266 | -0.1363 | -0.1862 | -0.0452 | -0.3511 | -0.2652 | -0.1542 | -0.1106 | -0.0033 | -0.0019 | -0.0255 | -0.0001 |

The rate-accuracy curves of 4 tests under RA, LD and AI configurations are shown in Fig. 2. Based on monotonicity metric of Test2 and Test3, it’s found that weighted average based on the number of instances in each class and machine analysis ground truth labelled by object detection results on the uncompressed videos can both improve the monotonicity, compared with Test1, the original mAP calculation method. Moreover, it’s obvious that Test4, the proposed new object detection performance evaluation method, is more stable than other tests. It also removes the impact from machine analysis model from the evaluation of coding performance.

Fig. 2 The rate-accuracy curves of 4 tests under RA, LD and AI configurations

Conclusion

In this contribution, a new performance evaluation method for object detection is proposed in which the ground truth is the object detection results labelled by the machine analysis model on the uncompressed videos and the final mAP is calculated by weighted averaging AP of each class with the weights dependent on the number of instances of the corresponding class. Experimental results show the stability improvements of the proposed method. It is asserted that the propose method decouples machine analysis model performance from the coding performance. It’s suggested to replace the current object detection evaluation method with the proposed method.

Patent rights declaration(s)

Alibaba Group and City University of Hong Kong do not have any current or pending patent rights relating to the technology described in this contribution.