| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0218-v1 |
| --- | --- |

| Title: | Non-EE2: On subblock inter mode improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Yan Zhang, Jian-Liang Lin, Zhi Zhang, Po-Han Lin, Chun-Chi Chen, Vadim Seregin, Marta Karczewicz | Email: | hanhuang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes the modifications to the subblock inter modes including a control point motion vector search method for affine DMVR, 1/64-pel motion vector precision for the affine motion compensation, and additional combined candidates. It reports the following result {Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak} under CTC:

RA {-0.xx%, -0.xx%, -0.xx%, xxx.x%, xxx.x %, xxx.x %, xxx.x%}

LDB {-0.xx%, 0.xx%, 0.xx%, xxx.x %, xxx.x %, xxx.x%, xxx.x%}

# Introduction

In ECM-13.0, affine DMVR is applied to the affine merge mode when the DMVR enabling condition is satisfied. The affine DMVR process can be summarized as follows:

1)	performing integer-pel bilateral matching for subblocks and then accumulating the subblock bilateral matching cost to determine the best integer-pel MV offset;

2)	performing half-pel bilateral matching search using the best integer-pel MV offset as the initial offset and outputting the best MV offset that minimizes the bilateral matching cost for the same set of the subblocks of step 1;

3)	performing linear regression using the integer-pel sub-block MVs of step 1 as input and outputting a set of control-point motion vectors;

4)	selecting the set of control point motion vector with the lowest cost from steps 2 and 3.

In addition, the non-translation parameters of affine model are refined after the above steps. Each CPMV (control point motion vector) is fixed as base MV in turn, and an offset is added to the non-translation parameter of affine model by minimizing the bilateral matching cost, and then the other two CPMVs are calculated according to the fixed base MV and refine the non-translation parameters.

The affine CPMV’s have the same precision as regular motion vector, which is 1/16-pel. When performing affine motion compensation, per-pixel or subblock based motion vector is derived by the control point motion vectors and is quantized to the same 1/16-pel precision.

Subblock based temporal motion vector prediction is another subblock mode in ECM-13.0. Multiple SbTMVP candidates can be derived from different corresponding block shifted by different source MV and different reference pictures. The MV from the corresponding block is scaled according to the POC distances.

In JVET-AH meeting, chained motion vector prediction (CMVP) was adopted. CMVP candidates are derived as the accumulation of the recursively traced MVs based on the source MVs.

# Proposal

In this contribution, it is proposed the following modifications to the two subblock inter modes.

First, it is proposed to add a CPMV search process in the affine DMVR. For each control point, bilateral matching is independently performed for a block that centered by the control-points to derive the refined CPMVs. Then the refined CPMVs are used to derive the best set of CPMVs that minimize the bilateral matching cost of the current block.

Second, it is proposed to quantize the per-pixel or subblock based motion vector to 1/64-pel precision instead of 1/16-pel one for motion compensation. The 64-phase interpolation filter has the same frequency response as the current 12-tap filter in ECM. However, the subblock motion vectors that are stored for the motion vector prediction are kept in the same 1/16-pel precision, as in ECM.

Third, additional candidates are added by combing the source MV with either TMVP or CMVP. The source MV is the MV used to derive TMVP or CMVP, which is a uni-directional MV. For example, if the source MV comes from the reference picture list 0, the list 0 MV of the TMVP or CMVP is replaced by this MV and the result is used as a new combined candidate in both regular merge candidate list and subblock candidate list.

# Test results

The proposed method was implemented on top of ECM-13.0.  The simulation was conducted under CTC and the results are summarized below.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.14% | -0.05% | -0.11% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.19% | -0.03% | -0.01% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).