| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0254-v1 |
| --- | --- |

| Title: | AHG8: Pre-analysis based adaptive spatial resampling algorithm for machine vision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shurun Wang Binzhe Li Jie Chen Yan Ye | Tel: Email: | {shurun.wsr, libinzhe.lbz, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution proposes a pre-analysis based adaptive spatial resampling algorithm for machine vision. Following the common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content as defined in JVET-AH2031, the proposed method achieves 2.72% (RA), 2.68% (LD) and 2.40% (AI) BD-rate savings for object detection task on the SFU-HW dataset.

Algorithm description

Spatial resampling framework

Figure 1 shows the framework of pre-analysis based adaptive spatial resampling compression. Pre-analysis module determines the resampling ratio adaptively. Based on this, the input video I is rescaled with a spatial downsampling model at the encoder side and the output, downsampled video ID, is compressed with the VVC encoder. At the decoder side, the decompressed downsampled video ID' is decoded and reconstructed with the VVC decoder. The reconstructed video I' is the output of spatial upsampling, based on the resampling ratio transmitted along with the bitstream. Herein, the spatial resampling method is Bicubic filter.

Figure 1 The pre-analysis based adaptive spatial resampling compression framework.

Pre-analysis based adaptive algorithm

Motivated by the diversity of video content, a pre-analysis based adaptive spatial resampling algorithm is proposed in terms of the content complexity for machine vision. The machine vision-oriented content complexity is measured by the averaged object spatial area and the number of objects, based on detection results of the video sequence, noted as {(xi,yi,wi,hi,ci,si)}, i=1,2,3,…,n, where n is the total number of detected bounding boxes. Specifically, (xi,yi) are the coordinates of the bounding box center and wi and hi are the width and height of bounding box. Moreover, ci and si are the object class and the confidence score of this detected bounding box, where 0≤si≤1, indicating the possibility that the detected bounding box is correct.

The number of objects N is i=1n1×si, noted as N=i=1nsi. The averaged object spatial area A is defined as A=i=1nhiwisiN. Moreover, 1000 images are randomly selected from the training part of OpenImages dataset [1]. Based on the images, the averaged object spatial area and object number can be calculated, noted as {(Ai,Ni)}, i=1,2,3,…,1000. The mean values of {A} and {N} are noted as Am and Nm, respectively. Generally, spatial resampling tends to be effective for the content with large object spatial area and small object number. As such, the adaptive spatial resampling decision function f(At,Nt) is defined as below:

fAt,Nt=1, if Nt<NmAmAt  0, if Nt≥NmAmAt .

Herein, At and Nt are calculated based on the first frame of each test sequence. If fAt,Nt=1, spatial resampling is conducted. If fAt,Nt=0, no spatial resampling is conducted, as shown Figure 2. The resampling ratio is set as 0.75.

Figure 2 The visualization of the adaptive spatial resampling decsion function fAt,Nt.

Performance

We evaluate the performance of the proposed pre-analysis based adaptive spatial resampling algorithm following the common test conditions specified in JVET-AH2031 [2], with VTM-23.3. For SFU-HW dataset with object detection task, there are 2 sequences selected to be resampled, which are Cactus_1920x1080_50_val and RaceHorses_832x480_30_val. The performance of the 2 sequences is shown in Table 1. (Please note “/” in the tables indicates the performance cannot be calculated validly, due to the nonmonotonic machine task performance or the non-overlap between rate-accuracy curves). No spatial resampling is conducted for other sequences in SFU-HW dataset and the BD-rate savings are 0%. As such, the averaged BD-rate savings of SFU-HW dataset for object detection task are 2.72%, 2.68% and 2.40% under RA, LD and AI configurations, respectively.

Table 1 The performance of resampled sequences in SFU-HW dataset for object detection under RA, LD and AI configurations.

| Config | RA |  |  | LD |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence Name | Mono BD-rate | BD-rate | BD-mAP | Mono BD-rate | BD-rate | BD-mAP | Mono BD-rate | BD-rate | BD-mAP |
| Cactus_1920x1080_50_val | -19.13% | / | 6.71 | -26.35% | / | 10.13 | -15.31% | / | 5.12 |
| RaceHorses_832x480_30_val | -16.23% | -13.03% | 0.82 | -8.48% | -9.04% | 0.80 | -15.95% | -15.85% | 2.01 |

Conclusion

A pre-analysis based adaptive spatial resampling algorithm is described in this contribution to improve the coding efficiency of video coding for machines. Simulation shows that proposed algorithm is effective and performs superior to VVC under various coding configurations.

Thus, it is suggested to update the following text into section 7.4 of the next draft of technical report (TR) on optimization of encoders and receiving systems for machine analysis of coded video content, as shown in the attached TR draft file.

“If the analysis of a video shows that it contains primarily large objects, one way to increase the BD-rate performance can be to perform spatial subsampling on the input video. This will result in fewer pixels in subsampled frames to encode, and thus likely faster encoding and bit rate savings. The spatial subsampling can be applied adaptively, for example, based on the diversity characteristics of the visual content (such as the averaged object spatial area and the number of objects) and the target bit rate. For example, depending on if the input video is captured by regular camera as natural scenes, or is captured by infrared sensor as thermal images, different spatial subsampling methods can be applied.”

It’s also suggested to include the implementation of the pre-analysis based adaptive spatial resampling algorithm into AHG8 software branch.

References

- Kuznetsova A, Rom H, Alldrin N, et al. The open images dataset v4: Unified image classification, object detection, and visual relationship detection at scale[J]. International journal of computer vision, 2020, 128(7): 1956-1981.

- S. Liu, C. Hollmann, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AH2031, 34th Meeting, Rennes, FR, 17–27 April 2024 (available online at https://jvet-experts.org/).

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).