| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0261-v1 |
| --- | --- |

| Title: | AHG 12: Rectangular Partitioning Restrictions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexandre Tissier, Mohsen Abdoli, Ramin G. Youvalari | Email: | alexandret@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

This contribution proposes a modification of the partitioning process and more specifically its restrictions on the maximum BT and TT size parameters. The proposed method releases the constraint of maximum BT and TT sizes in I-Slices. However, on those new unconstrained sizes, a restriction based on the number of MTT split is proposed. The results of three tests are presented in this contribution which separates luma only modification, chroma only modification and a combination of both luma and chroma modifications. The impact on coding efficiency and runtimes of proposed method over ECM-13.0 are reportedly {for Y, U, V, EncT, DecT}:

Test 1: luma only modification

AI{ -0.08%, -0.06%, -0.06%, 101.6%, 99.9%}

Test 2: chroma only modification

AI{-0.06%, -0.94%, -0.90%, 102.7%, 100.2%}

Test 3: combination of both luma and chroma modifications

AI{ -0.14%, -1.00%, -0.96%, 104.0%, 98.9%}

# Introduction

The partitioning process performed in ECM-13.0 [1] supports the QT split plus the MTT partitioning structure which contains BT and TT that can be both applied horizontally and vertically. Those five possible splits lead to a certain complexity that is reduced with some restrictions. One of them is the maximum BT and TT sizes which defines the maximum blocks sizes where MTT is available. For I-Slices in luma, the maximum BT and TT sizes are set to 32. For I-Slices in chroma, the maximum BT and TT sizes are set to 64 and 32, respectively. Another restriction is the maximum MTT depth that does not allow MTT split when the current MTT depth of the block is higher than 3 for I-Slices.

# Proposed Method

In this contribution, it is proposed to partially relax the maximum BT and TT sizes constraints and enable them in a limited way for those newly relaxed blocks. The impacted blocks are referred to as MTT restricted blocks in this contribution.

Maximum BT and TT sizes are modified from 32 to 64 in luma I-Slices. In the chroma partitioning tree, maximum BT size is modified from 64 to 128 and maximum TT size is modified from 32 to 128. Figure 1 shows the current ECM configuration in black with only QT split allowed for MTT restricted blocks and the configuration proposed by the contribution that adds the MTT splits to the list of split available for MTT restricted blocks.

Figure 1: Enabling MTT splits for MTT restricted blocks

To limit the complexity increase, only one MTT split is applied on the MTT restricted block, i.e. the maximal MTT depth for those blocks are set to 1. Figure 2 depicts this restriction which allows only one MTT split on MTT restricted blocks.

Figure 2: Leaf restriction on MTT for MTT restricted blocks

Three tests are proposed for this contribution:

Test 1) Proposed method is only applied on luma channels in I-Slices,

Test 2) Proposed method is only applied on chroma channels in I-Slices,

Test 3) Proposed method is applied on both luma and chroma channels in I-Slices.

# Simulation Results

Tables below summarize the coding efficiency and runtime impact of the proposed method in ECM-13.0 [1] using the JVET common test conditions [2].

## Test 1: Only applied on luma channels in I-Slices

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.19% | -0.18% | -0.05% | 104.4% | 98.4% |  |  |
| Class A2 | -0.08% | -0.12% | -0.06% | 102.3% | 102.6% |  |  |
| Class B | -0.10% | -0.09% | -0.11% | 101.7% | 99.3% |  |  |
| Class C | 0.02% | 0.00% | -0.01% | 99.4% | 99.9% |  |  |
| Class E | -0.08% | 0.07% | -0.04% | 101.2% | 99.7% |  |  |
| Overall | -0.08% | -0.06% | -0.06% | 101.6% | 99.9% |  |  |
| Class D | 0.02% | 0.04% | -0.12% | 101.3% | 99.8% |  |  |
| Class F | -0.06% | 0.04% | 0.00% | 100.3% | 101.3% |  |  |
| Class TGM | -0.01% | -0.01% | -0.02% | 101.6% | 103.7% |  |  |

## Test 2: Only applied on chroma channels in I-Slices

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.11% | -1.18% | -1.15% | 102.1% | 99.5% |  |  |
| Class A2 | -0.09% | -1.30% | -1.05% | 104.4% | 102.9% |  |  |
| Class B | -0.05% | -1.14% | -1.15% | 101.9% | 99.6% |  |  |
| Class C | -0.03% | -0.27% | -0.42% | 103.2% | 99.9% |  |  |
| Class E | -0.04% | -0.91% | -0.70% | 101.9% | 99.6% |  |  |
| Overall | -0.06% | -0.94% | -0.90% | 102.7% | 100.2% |  |  |
| Class D | -0.02% | -0.26% | -0.29% | 104.9% | 99.6% |  |  |
| Class F | -0.09% | -0.67% | -0.53% | 101.5% | 101.2% |  |  |
| Class TGM | -0.16% | -0.50% | -0.51% | 103.4% | 103.5% |  |  |

## Test 3: Applied on both luma and chroma channels in I-Slices

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.28% | -1.39% | -1.37% | 106.2% | 97.2% |  |  |
| Class A2 | -0.15% | -1.23% | -1.15% | 105.0% | 99.0% |  |  |
| Class B | -0.14% | -1.21% | -1.15% | 103.6% | 98.8% |  |  |
| Class C | -0.03% | -0.40% | -0.41% | 102.5% | 100.1% |  |  |
| Class E | -0.12% | -0.85% | -0.78% | 103.3% | 99.1% |  |  |
| Overall | -0.14% | -1.00% | -0.96% | 104.0% | 98.9% |  |  |
| Class D | 0.01% | -0.26% | -0.34% | 105.9% | 99.8% |  |  |
| Class F | -0.14% | -0.97% | -0.73% | 101.2% | 99.5% |  |  |
| Class TGM | -0.18% | -0.55% | -0.53% | 101.0% | 101.0% |  |  |

# Conclusion

This contribution proposes to release the constraints on the maximum BT and TT size and to restrict the maximal MTT depth to 1 on those newly released blocks. The experimental results, in AI configuration, showed that this contribution provides 0.08% average luma BD-Rate gain and 0.06% average chroma BD-Rate gain when the method is applied to luma channel, 0.06% average luma BD-Rate gain, 0.92% average chroma BD-Rate gain when the proposed method is applied on chroma channels, and 0.14% average luma BD-Rate gain, 0.98% average chroma BD-Rate gain when the proposed method is applied on both luma and chroma channels. It is suggested to study the proposed method in the next EE2 round.

# References

- ECM-13.0 reference software, https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).