| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0279-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AI0141 (EE2-2.4c: Combination of EE2 tests 2.2 and 2.4a) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll, Jung-Kyung Lee. | Tel: Email: | druizcoll@ofinno.com |
| Source: | Ofinno |  |  |

_____________________________

# Abstract

This is the crosscheck of the EE2 Test 2.4c reported in JVET-AI0141, which combines all EE Tests 2.2 and 2.4a features. Additionally, a block vector-based TIMD predictor is allowed in the newly proposed TIMD mode in the same manner as it is deployed in TIMD. The crosscheck results match the BD rate reported by the proponents, with slight runtime differences.

# Experimental results

The software provided for the proponent [2] has been simulated following the EE2 description [3] and under JVET ECM common test conditions [4] in both AI and RA configurations. The partial results for Test 2.4c of the crosscheck are detailed in the accompanying spreadsheet and are further summarized in the subsequent tables. Note that tests for RA configuration are currently under simulation as of the edition of this report. Also, the test runtimes provided in this report may present some deviation due to the cluster's heterogeneity.

Table1. AI crosscheck results

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0-gcc-13.2 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.10% | -0.04% | 0.08% | 103.4% | 104.2% | 101.6% | 100.1% |
| Class A2 | -0.12% | -0.04% | -0.02% | 103.9% | 105.6% | 101.6% | 100.2% |
| Class B | -0.11% | -0.03% | -0.16% | 104.0% | 106.5% | 99.9% | 100.2% |
| Class C | -0.09% | 0.06% | 0.02% | 103.1% | 107.6% | 99.2% | 100.1% |
| Class E | -0.10% | -0.04% | -0.08% | 104.1% | 105.6% | 99.9% | 100.1% |
| Overall | -0.10% | -0.02% | -0.04% | 103.7% | 106.1% | 100.3% | 100.1% |
| Class D | -0.04% | 0.00% | -0.01% | 102.7% | 107.1% | 100.0% | 100.0% |
| Class F | 0.01% | 0.16% | 0.25% | 102.6% | 102.5% | 98.9% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Table2. RA crosscheck results

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0-gcc-13.2 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

# Conclusion

The crosschecker has confirmed that the "EE2-2.4c: Combination of EE2 tests 2.2 and 2.4a" proposed in JVET-AI0141 [1] is implemented on top of ECM-13.0, and the software implementation matches the technical description of the algorithms. The crosscheck's partial BD-rate results match those of the JVET-AI0141, and AI runtimes show some differences compared to those reported in [1].

# Patent rights declaration(s)

Ofinno does not have any current or pending patent rights relating to the technology described in this contribution.

# References

- Döne Bugdayci Sansli, Saverio Blasi, Jani Lainema, Karam Naser, Ya Chen, Fabrice Le Léannec, Milos Radosavljević, Kevin Reuzé, Jiaye Fu, Jiaqi Zhang, Siwei Ma, Ying Gao, Cheng Huang, "EE2-2.4c: Combination of EE2 tests 2.2 and 2.4", JVET-AI0141, JVET 35th Meeting, Sapporo, JP, 12–20 July 2024.

- EE2-3.2, https://vcgit.hhi.fraunhofer.de/ecm/jvet-ah-ee2/ECM/-/tree/TEST_24cd

- V. Seregin, et al., "Exploration experiment on enhanced compression beyond VVC capability (EE2)", JVET-AH2024, April 2024.

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AF2017, 32nd Meeting, Hannover, Oct. 2023.