| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0310-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AI0143 (EE2-2.4d: IntraTMP candidates with overlapping refinement window enhanced) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Waqas Ahmad | Email: | waqas.ahmad@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

The contribution reports the results of cross-check of the EE2 Test 2.4d proposed by Nokia.

IntraTMP creates a merge candidate list comprising candidates from adjacent and non-adjacent neighbouring PU.

In ECM-13.0, the TMP merge list is supplemented with the AR-BVP candidates derived from initial BV merge candidates, and additional sparse and refinement lists are also available for the TMP LIC mode.

Test 2.4d is the combination of Test 2.1 + Test 2.2 + Test 2.4a.

# Verification

The source code is used from ECM EE2 TEST_2.4 branch in which proposal is implemented on top of ECM 13.0.

The AI configuration results are as follows.,

Table 1 Coding performances of the EE TEST 2.4d on top of ECM-13.0 for AI configuration.

|  | All Intra Main 10 Over ECM-13.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.15% | -0.05% | -0.10% | 105.6% | 108.0% | 100.1% | 100.8% |
| Class C | -0.12% | -0.04% | -0.04% | 104.6% | 110.1% | 100.7% | 100.4% |
| Class E | -0.13% | 0.20% | -0.05% | 105.3% | 106.5% | 100.5% | 100.3% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.05% | 0.17% | 0.00% | 104.5% | 108.9% | 100.6% | 100.1% |

The RD performance is matched for the EE2 Test 2.4d with the provided partial results (AI Class B, C, E, and D, from proponent.

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.