| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 |  |  |  | Document: JVET-AI00339-v1 |
| --- | --- | --- | --- | --- |
| Title: | BoG on NNVC topics |  |  |  |
| Status: | input document to JVET |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | E. Alshina, F. Galpin | Email: | elena.alshina@huawei.com franck.galpin@interdigital.com |  |
| Source: | BoG |  |  |  |

_____________________________

# Abstract

This document summarizes discussion during BoG on NNVC conducted on July 16.  BoG was focusing on EE1 test discussion and software development guidelines. In total 40+?? experts attended BoG.

# On SADL software development

Document JVET-AI0128 was further discussed. An update of the document will be provided to illustrate some aspects of the new guidelines.

If proponent can run with ONNX then likely it is possible for most of AI accelerators. General agreement in the room:

- If model can be run with ONNX, without scripting, then it is static and acceptable as a model for the float version. If a feature is missed in SADL, it will be added.

- If inference using ONNX is not possible then it needs to be discussed.

Recent version of ONNX with the list of supported operators can be found here [3].

# EE1 rules

- Code base for the test should be NNVC10.0, anchor is default configuration of NNVC-10.0 (NN-Intra and LOP-3 filter enabled). NNVC common test conditions, results and complexity reporting template must be used.

For proposals in-loop filter, Inter and super-resolution categories, proponents are encouraged to use existing AhG11 training set 1. Proponents in NN-Inter category should specify details of training set if it is different from NNVC common test condition (DIV2K, BVI, TVD).  In all tests, training set must be clearly specified in the contribution.

- For tests competing with technologies in NNVC-10 it is recommended to configure the proposed solution to have a complexity close to related NNVC(LOP/HOP), but not exceeding:

- kMAC/pxl of EE1 test  kMAC/pxl NNVC (must),

- Number of Parameters EE1 test   Number of Parameters NNVC (if possible).

Encoding and decoding run time must be reported. Excel table with architecture and kMAC/pxl computation [2].

NN architecture provided in this test description should not be changed, beside minor adjustment for parameters (such as channels number) in order to meet the recommendations above. Exact parameters settings can be specified by 2nd AhG11/14 teleconference (see timeline in last section).

Inference cross-check is required for all EE1 tests. Candidates for adoption to NNVC required to undergo training cross-check, implementation must be compatible with SADL. The layers, not supported by majority of AI accelerators, must be explained in the contribution with details of implementation (including quantization).

# List of EE1 tests

This round of EE1 tests will include:

- EE1-1: LOP and VLOP in-loop filter

  - EE1-1.1 – Partial Convolution and Over-Parameterization

    - JVET-AI0068, proponents: A. Li, C. Zhu, L. Luo (UESTC), Y. Huo, Y. Liu (Transsion)

  - EE1-1.2 – Simplified residual groups with attention

    - JVET-AI0221, proponents: T. Ryder, S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)

  - EE-1.3 - NN in-loop filters using early cropping

    - JVET-AI0095, proponents: J. Ström, M. Damghanian, D. Liu, P. Wennersten (Ericsson)

  - EE-1.4 – Reduced complexity input feature extraction for LOP & VLOP

    - JVET-AI0204, proponents: D. Rusanovskyy, M. Karczewicz (Qualcomm)

  - EE-1.5 - LOP filter with multiscale blocks

    - JVET-AI0134, proponents:  R. Yang, F. Cricri, M. Santamaria, H. Zhang, J. Lainema, M.M. Hannuksela

- EE1-2: HOP in-loop filter

  - EE1-2.1 – Attention block with transformers

    - JVET-AI0176, proponents: Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)

  - EE1-2.2 – Block-size invariant implementation of HOP

    - JVET-AI0175, proponents: Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)

  - EE1-2.3 – Block based QP information in NN loop filters

    - JVET-AI0188, proponents: F. Galpin, T. Poirier, T. Dumas, P. Bordes (InterDigital)

- EE1-3: NN-inter prediction

  - EE1-3.1 – RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding

    - JVET-AI0089, proponents: Q. Qin, C. Jung (Xidian Univ.)

- EE1-4: NN-super-resolution

  - EE1-4.1 – Wavelet transform for super-resolution loss function

    - JVET-AI0105, proponents:  J. Ye, K. Wu, Q. Liu(HUST)

# EE1 tests description

## EE1-1: LOP and VLOP in-loop filter

### EE1-1.1 – Partial Convolution and Over-Parameterization

    - JVET-AI0068, proponents: A. Li, C. Zhu, L. Luo (UESTC), Y. Huo, Y. Liu (Transsion)

This test will include three subtests to assess the effects of partial convolution and over-parameterization, which were studied in previous EE1 round. Only package will be tested in this EE1

As shown in Figure EE1-1.1.1, we evaluate the effectiveness of the over-parameterization block (OP Block), which expands the model during the training stage while compressing the model during the inference stage.

Training Stage:

As shown in Figure EE1-1.1.1, for a given input feature X∈RC1×h×w , where C1 is the number of channels, h is the feature height, and w is the feature width, the sizes of the two convolution kernels are W1∈RC2×C1×3×3 and W2∈RC1×C2×1×1, respectively. The convolution process can be expressed as:

Xout=(X*W1)*W2,

where ∗ represents the convolution process, and  Xout∈RC1×h×w.

Inference Stage:

By the property of convolution, the above process can be re-expressed as:

Xout=X*W1*W2=X*Wnew,

where Wnew∈RC1×C1×3×3 indicates the fused weight. Through the above transformation, it is possible to use a new convolution kernel to replace the two previous kernels, thereby completing the over-parameterization process.

Figure EE1-1.1.1: LOP3 with Over-Parameterization Blocks.

As shown in Figure EE1-1.1.2, we evaluate the effectiveness of the Partial Convolution Block (PC Block), which splits the input features into two branches and performs the convolution on only one of them.

Figure EE1-1.1.2: LOP3 with Partial Convolution Blocks.

As shown in EE1-1.1.3, we combine the Over-Parameterization Block (OP Block) and Partial Convolution Block (PC Block) to evaluate the overall performance."

Figure EE1-1.1.3: LOP3 with OP Blocks and PC Blocks.

Training will follow the LOP3 training strategy, using the Stage 3 LOP dataset. The training process may be modified to reflect changes in the architecture and improve performance. Parameter settings may be adjusted to meet the requirements of the LOP structure.

Training: Use the LOP3 training strategy and the Stage 3 LOP dataset.

Inference: Employ NNVC 10.0 software or later versions, and NNVC CTC, following the LOP3 anchor settings.

Proponent: UESTC and Transsion

Crosscheck: TBD.

### EE1-1.2 – Simplified residual groups with attention

    - JVET-AI0221, proponents: T. Ryder, S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)

Training was modified compared to LOP3, it was requested to apply same training strategy to LOP3 and show results to evaluate impact of training. Proponents were asked to align architecture with LOP3 .

EE1-1.2.1 proposed training but for LOP3

EE1-1.2.2 proposed training for proposed architecture

EE1-1.2.3 proposed training and architecture with relevant LOP3 elements

### EE-1.3 - NN in-loop filters using early cropping for LOP and VLOP

    - JVET-AI0095, proponents: J. Ström, M. Damghanian, D. Liu, P. Wennersten (Ericsson)

In this test, the neural network patch size is made smaller by using padding=(0,0) (i.e., no padding) in the first two backbone blocks. This has the effect of lowering the complexity in terms of kMAC/pixeland the saved complexity is then spent by widening and deepening the LOP3 models from N Y=14 to N Y=15 backbone blocks and from C1Y=144 to C1Y=176 channels.

Figure: The proposed architectural changes to LOP3.

Training will follow LOP training strategy and stage 3 LOP dataset to be used. The training process may be modified to reflect change in the architecture and improve performance.

Training: Following LOP/VLOP training strategy, Stage 3 LOP dataset.

Inference: NNVC 10.0 software or latter, NNVC CTC, following LOP/VLOP anchor settings.

Proponent: Ericsson

Crosscheck: TBD.

Tests to be carried out:

EE1-1.3.1 LOP

EE1-1.3.2 VLOP

If training is modified – one more sub-test is required.

### EE-1.4 – Reduced complexity input feature extraction for LOP & VLOP

    - JVET-AI0204, proponents: D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)

EE1-1.4.1 no DCT for BS, IPB and QP-map - LOP

EE1-1.4.2 no DCT for BS, IPB and QP-map, merging two Q-maps into one – LOP

EE1-1.4.3 no DCT for BS, IPB and QP-map, merging two Q-maps into one - VLOP

### EE-1.5 - LOP filter with multiscale blocks

    - JVET-AI0134, proponents:  R. Yang, F. Cricri, M. Santamaria, H. Zhang, J. Lainema, M.M. Hannuksela

EE1-1.5.1 match complexity to LOP

EE1-1.5.2 match complexity to VLOP

## EE1-2: HOP in-loop filter

### EE1-2.1 – Attention block with transformers

    - JVET-AI0176, proponents: Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)

EE1-2.1.1 Transformers w/o normalization in all BBBs

Training strategy might be changed to solve convergence problem.

EE1-2.1.2 Transformers with normalization in all BBBs

Integerization and implementation for SADL

### EE1-2.2 – Block-size invariant implementation of HOP

    - JVET-AI0175, proponents: Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)

EE1-2.2 only inference for HOP5, find proper implementation

### EE1-2.3 – Block based QP information in NN loop filters

    - JVET-AI0188, proponents: F. Galpin, T. Poirier, T. Dumas, P. Bordes (InterDigital)

Test conditions with QP change, activate perceptual optimization as in VTM

EE1-2.3.1 only inference change – use an average QP as input

EE1-2.3.2 retrain HOP (only stage 3), on updated data set (generated with  perceptual optimization as in VTM)

## EE1-3: NN-inter prediction

### EE1-3.1 – RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding

    - JVET-AI0089, proponents: Q. Qin, C. Jung (Xidian Univ.)

EE1-3.1.1 trained on Vimeo-90K triplet

EE1-3.1.2 trained on BVI and TVD

If possible, port code into NNVC-10.0 SW.

## EE1-4: NN-super-resolution

### EE1-4.1 – Wavelet transform for super-resolution loss function

    - JVET-AI0105, proponents:  J. Ye, K. Wu, Q. Liu(HUST)

EE1-4.1.1 train NNSR from NNVC-10.0 w/o wavelets in loss (regenerated data set)

EE1-4.1.2 train NNSR from NNVC-10.0 with wavelets in loss (regenerated data set)

# Expected EE1 Timeline

T1 – 2.5 weeks after JVET-AI meeting (06 -August-2024): 1st teleconference. NN-Intra training cross-check is over. Results discussed during teleconference.

T2 – 3 weeks after JVET-AI meeting (09 -August-2024) EE description (JVET-AH2023) finalized and uploaded. NNVC-10.0 software is available, including anchor performance.

T3 – 7 weeks after JVET-AI meeting (30 -September-2024) 2nd teleconference. Final setting for parameters to be announced, partial results discussed, combinational tests (if any) decided.

T4 – 2 weeks before T6 (16-October -2024): EE1 software is frozen (both inference and training), write access closed, proponents are recommended to notify crosschecked about commit to be used.

T5 – 3 days before T6 (25-October-2024): Cross-checkers report status to EE1 coordinators (sending e-mail).

T6 –30-October-2024: EE1 summary is uploaded as input contribution.

# References

1 JVET-AF2016 ”Common test conditions and evaluation procedures for neural network-based video coding technology”	E. Alshina, R.-L. Liao, S. Liu, A. Segall

2 JVET-AD0380, “BoG report on NN-filter design unification”, E. Alshina, F. Galpin

[3] ONNX operqtors https://onnx.ai/onnx/operators/