| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0007-v2 |
| --- | --- |

| Title: | AHG report: ECM tool assessment (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiang Li (chair) Lien-Fei Chen Zhipin Deng Jonathan Gan Edouard François Hong-Jheng Jhu Xinwei Li Hongtao Wang (vice chairs) | Tel: Email: | xlxiangli@google.com lienfeichen@global.tencent.com zhipin.deng@bytedance.com v-jonathan.gan@oppo.com Edouard.Francois@interdigital.com jhuhong-jheng@kwai.com sid.lxw@alibaba-inc.com hongtaow@qti.qualcomm.com |
| Source: | AhG7 on ECM tool assessment |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG7 on ECM tool assessment that has taken place between the 35th and 36th JVET meetings.

# Mandates

The mandates given to the AHG are:

- Investigate methodology of tool assessment.

- Coordinate with AHG6 on resolving tool-off test related software issues (missing tool controls and software bugs).

- Prepare configuration files and generate bitstreams and results of tool-on/tool-off testing.

- Prepare reporting of tool assessment results.

- Collect simulation results on non-CTC sequences (e.g., those used in previous verification tests), and identify a set of non-CTC sequences that would be appropriate for additional testing.

- Investigate the possibility of conducting subjective tests on subsets of tools in coordination with AHG4 and AG 5

- Develop methodology of more reliable runtime measurement

# Group off tests

## Test settings and crosschecking

The same four groups were used in this meeting cycle.

- Group 1: Inter template matching tools

- Group 2: Coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process

- Group 3: Intra and IBC template matching (with search) related tools

- Group 4: Tools that require more processing on the neighboring reconstructed samples than VVC

Five group-off tests were performed and crosschecked on top of ECM-14. The two anchors are ECM 14.0 and VTM-11ECM14.0. The cfg files used are included in the ECM software package.

The testers and crosscheckers are summarized in the table below.

| Tests | Testers | Crosscheckers |
| --- | --- | --- |
| Group 1 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Charles Salmon-Legagneur (charles.salmon-legagneur@interdigital.com) |
| Group 2 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Xinwei Li (sid.lxw@alibaba-inc.com) |
| Group 3 off | Xiang Li (xlxiangli@google.com) | Zhipin Deng (zhipin.deng@bytedance.com) |
| Group 4 off | Xiang Li (xlxiangli@google.com) | Hong-Jheng Jhu (jhuhong-jheng@kwai.com) |
| Group 1-4 off | Lien-Fei Chen (lienfei.chen@global.tencent.com), Xiang Li (xlxiangli@google.com) | Hongtao Wang (hongtaow@qti.qualcomm.com) |

ECM-14.0 has encoding crash when TM tools are off (software issue #74). The issue was fixed in commit 844dd563c after ECM-14.0 was released. Therefore, different software versions were used in the AHG7 tool off tests, as summarized in the table below.

| Tests | Testers | Crosscheckers |
| --- | --- | --- |
| Group 1 off | ECM-14.0-844dd563c | ECM-14.0-844dd563c |
| Group 2 off | ECM-14.0 | ECM-14.0 |
| Group 3 off | ECM-14.0-844dd563c | ECM-14.0 |
| Group 4 off | ECM-14.0-844dd563c | ECM-14.0 |
| Group 1-4 off | ECM-14.0-844dd563c | ECM-14.0844dd563c |

It was further observed that different compilers may generate different bitstreams under ECM-14.0 CTC. The mismatch appears to be limited to RA configuration. In some test points, small differences in bitrates may be observed. PSNRs are still matched. The issue seems fixed in commit 981cee013. However, no further tool off tests were performed with this or later version.

## Group 1 off

Group 1 includes inter template matching tools. The offgroup1.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM14.0 |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100.3% | 99.8% | 100% | -13.23% | -14.28% | -25.57% | 1024.9% | 499.6% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100.1% | 100.2% | 100% | -20.23% | -22.93% | -27.49% | 1011.0% | 530.1% |
| Class B | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 102% | -13.68% | -21.41% | -19.25% | 945.3% | 543.2% |
| Class C | -0.01% | -0.01% | -0.01% | 99.7% | 100.2% | 98% | -13.74% | -10.65% | -11.73% | 969.1% | 509.4% |
| Class E | -0.02% | -0.02% | -0.02% | 101.5% | 102.1% | 99% | -17.63% | -20.84% | -19.16% | 927.3% | 589.2% |
| Overall | -0.01% | -0.01% | -0.01% | 100.3% | 100.4% | 100% | -15.37% | -17.99% | -19.99% | 971.1% | 533.1% |
| Class D | -0.04% | -0.04% | -0.04% | 99.4% | 98.1% | 100% | -11.62% | -7.71% | -8.21% | 927.4% | 547.1% |
| Class F | -0.02% | -0.02% | -0.02% | 100.2% | 99.2% | 100% | -29.29% | -32.85% | -33.10% | 651.7% | 549.7% |
| Class TGM | 0.00% | 0.00% | 0.00% | 99.5% | 99.7% | 100% | -42.24% | -47.92% | -47.18% | 544.7% | 616.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM14.0 |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 4.88% | 4.24% | 4.48% | 87.4% | 72.3% | 100% | -22.72% | -19.46% | -31.79% | 959.3% | 805.4% |
| Class A2 | 5.71% | 5.58% | 5.61% | 87.5% | 65.1% | 100% | -25.42% | -29.05% | -34.67% | 917.8% | 869.3% |
| Class B | 4.47% | 4.33% | 4.40% | 83.1% | 67.8% | 100% | -20.41% | -27.81% | -24.94% | 696.1% | 807.1% |
| Class C | 4.72% | 4.74% | 5.29% | 79.2% | 62.5% | 100% | -22.05% | -17.26% | -17.82% | 673.9% | 734.3% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 4.87% | 4.67% | 4.90% | 83.7% | 66.6% | 100% | -22.31% | -23.58% | -26.36% | 777.6% | 798.5% |
| Class D | 3.80% | 3.62% | 4.04% | 77.5% | 61.3% | 100% | -23.61% | -18.93% | -19.58% | 690.7% | 911.5% |
| Class F | 3.60% | 3.56% | 3.61% | 89.3% | 75.1% | 100% | -29.55% | -31.73% | -32.60% | 650.0% | 465.4% |
| Class TGM | 4.24% | 4.11% | 4.05% | 89.7% | 80.7% | 101% | -39.08% | -44.94% | -44.70% | 612.5% | 518.2% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM14.0 |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 5.51% | 5.85% | 6.15% | 72.9% | 59.6% | 100% | -16.60% | -31.22% | -27.06% | 596.7% | 473.5% |
| Class C | 5.46% | 5.66% | 6.12% | 70.5% | 51.5% | 99% | -19.08% | -20.08% | -20.73% | 536.7% | 433.5% |
| Class E | 5.79% | 6.82% | 4.97% | 78.3% | 68.7% | 100% | -15.91% | -20.66% | -19.87% | 547.4% | 355.2% |
| Overall (Ref) | 5.57% | 6.03% | 5.84% | 73.4% | 58.8% | 99% | -17.25% | -24.87% | -23.15% | 563.7% | 427.9% |
| Class D | 4.77% | 4.81% | 5.05% | 70.5% | 51.0% | 100% | -20.92% | -22.38% | -21.80% | 539.2% | 501.9% |
| Class F | 5.39% | 4.83% | 4.87% | 84.6% | 67.5% | 100% | -25.44% | -34.09% | -34.45% | 595.1% | 355.8% |
| Class TGM | 6.02% | 5.67% | 5.75% | 87.6% | 73.0% | 101% | -36.23% | -46.78% | -46.55% | 569.8% | 362.9% |

## Group 2 off

Group 2 includes coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process. The offgroup2.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM14.0 |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 0.25% | 0.38% | 0.55% | 94.2% | 97.3% | 102% | -13.00% | -13.96% | -25.12% | 962.5% | 486.7% |
| Class A2 | 0.33% | 0.45% | 0.52% | 93.3% | 97.0% | 102% | -19.96% | -22.59% | -27.12% | 942.6% | 513.0% |
| Class B | 0.26% | 0.51% | 0.65% | 92.1% | 95.4% | 104% | -13.45% | -21.01% | -18.71% | 870.5% | 518.4% |
| Class C | 0.21% | 0.28% | 0.29% | 91.4% | 96.2% | 99% | -13.55% | -10.40% | -11.45% | 888.3% | 488.9% |
| Class E | 0.39% | 0.63% | 0.73% | 93.5% | 97.8% | 104% | -17.28% | -20.33% | -18.56% | 854.0% | 564.1% |
| Overall | 0.28% | 0.45% | 0.55% | 92.7% | 96.5% | 102% | -15.12% | -17.63% | -19.54% | 898.2% | 512.6% |
| Class D | 0.16% | 0.18% | 0.32% | 90.7% | 94.4% | 100% | -11.44% | -7.50% | -7.87% | 845.6% | 526.3% |
| Class F | 0.67% | 0.71% | 0.74% | 91.2% | 97.4% | 101% | -28.82% | -32.35% | -32.60% | 593.0% | 539.5% |
| Class TGM | 0.39% | 0.39% | 0.40% | 91.7% | 98.6% | 99% | -42.01% | -47.72% | -46.97% | 502.2% | 610.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM14.0 |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 2.53% | 0.98% | 1.77% | 88.1% | 98.0% | 100% | -24.48% | -21.74% | -33.74% | 967.0% | 1091.3% |
| Class A2 | 1.50% | 1.24% | 1.76% | 86.4% | 99.3% | 100% | -28.38% | -31.90% | -37.03% | 906.4% | 1326.8% |
| Class B | 1.54% | 1.08% | 1.43% | 86.9% | 97.1% | 100% | -22.66% | -30.06% | -27.04% | 728.0% | 1156.5% |
| Class C | 1.13% | 0.94% | 1.15% | 85.9% | 94.9% | 100% | -24.77% | -20.31% | -21.07% | 730.7% | 1115.1% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.62% | 1.05% | 1.49% | 86.8% | 97.1% | 100% | -24.73% | -26.17% | -28.79% | 805.8% | 1163.6% |
| Class D | 0.63% | 0.08% | 0.28% | 83.5% | 94.9% | 100% | -25.99% | -21.72% | -22.56% | 743.5% | 1412.5% |
| Class F | 2.12% | 2.14% | 2.13% | 88.4% | 96.3% | 100% | -30.65% | -32.78% | -33.63% | 643.4% | 596.6% |
| Class TGM | 3.32% | 5.33% | 5.39% | 89.9% | 99.6% | 100% | -39.72% | -44.59% | -44.32% | 614.1% | 639.3% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM14.0 |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.92% | 3.60% | 3.94% | 78.9% | 94.2% | 100% | -19.47% | -32.69% | -28.53% | 645.9% | 749.3% |
| Class C | 1.65% | 3.21% | 3.52% | 79.1% | 90.8% | 99% | -22.05% | -21.97% | -22.70% | 602.5% | 763.8% |
| Class E | 1.21% | 2.73% | 1.43% | 81.9% | 94.3% | 100% | -19.55% | -23.62% | -22.44% | 572.3% | 488.1% |
| Overall (Ref) | 1.65% | 3.25% | 3.17% | 79.7% | 93.1% | 100% | -20.35% | -26.85% | -25.06% | 612.3% | 677.5% |
| Class D | 0.96% | 3.73% | 4.12% | 76.1% | 93.1% | 99% | -23.78% | -23.29% | -22.48% | 582.0% | 916.1% |
| Class F | 2.48% | 2.63% | 3.71% | 85.6% | 94.7% | 99% | -27.60% | -35.60% | -35.34% | 601.5% | 499.1% |
| Class TGM | 2.45% | 7.53% | 8.60% | 88.1% | 91.6% | 100% | -38.54% | -46.36% | -45.97% | 573.0% | 454.9% |

## Group 3 off

Group 3 includes intra and IBC template matching (with search) related tools. The offgroup3.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14-844dd563c |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.09% | 3.75% | 6.08% | 76.2% | 81.7% | 100% | -12.27% | -11.28% | -21.48% | 651.6% | 299.6% |
| Class A2 | 2.37% | 6.03% | 6.01% | 82.3% | 74.9% | 99% | -18.28% | -18.61% | -23.56% | 677.4% | 284.3% |
| Class B | 2.31% | 4.77% | 5.14% | 74.6% | 71.9% | 101% | -11.71% | -18.01% | -15.32% | 685.1% | 359.6% |
| Class C | 2.00% | 2.61% | 2.82% | 72.7% | 76.4% | 101% | -12.03% | -8.38% | -9.29% | 680.6% | 355.0% |
| Class E | 3.43% | 4.62% | 4.33% | 69.9% | 64.0% | 100% | -14.78% | -17.30% | -15.68% | 664.3% | 384.6% |
| Overall | 2.23% | 4.31% | 4.79% | 74.8% | 73.5% | 100% | -13.48% | -14.73% | -16.44% | 673.7% | 338.2% |
| Class D | 1.50% | 2.17% | 2.19% | 72.6% | 67.2% | 100% | -10.25% | -5.70% | -6.17% | 687.0% | 304.8% |
| Class F | 6.18% | 7.05% | 7.24% | 72.1% | 59.2% | 101% | -25.09% | -28.50% | -28.57% | 438.7% | 322.5% |
| Class TGM | 11.61% | 13.54% | 13.90% | 75.5% | 47.2% | 100% | -35.70% | -41.05% | -40.05% | 397.0% | 301.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-14-844dd563c |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.08% | 2.37% | 6.26% | 92.9% | 96.4% | 100% | -25.57% | -20.81% | -31.62% | 740.2% | 653.8% |
| Class A2 | 1.43% | 3.64% | 3.48% | 93.8% | 96.1% | 100% | -28.44% | -30.37% | -36.04% | 713.0% | 807.1% |
| Class B | 1.23% | 4.39% | 4.32% | 91.4% | 97.3% | 100% | -22.92% | -27.86% | -25.09% | 749.8% | 913.8% |
| Class C | 0.85% | 1.34% | 1.95% | 91.3% | 94.3% | 99% | -24.97% | -19.96% | -20.40% | 795.0% | 979.8% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.14% | 3.02% | 3.91% | 92.1% | 96.1% | 100% | -25.10% | -24.84% | -27.33% | 752.0% | 849.3% |
| Class D | 0.81% | 1.64% | 1.97% | 90.0% | 89.8% | 100% | -25.89% | -20.48% | -21.18% | 772.8% | 1043.5% |
| Class F | 4.16% | 4.41% | 4.89% | 87.5% | 90.9% | 100% | -29.47% | -31.67% | -32.24% | 637.1% | 568.8% |
| Class TGM | 7.41% | 9.18% | 9.64% | 92.1% | 91.5% | 100% | -37.25% | -42.35% | -41.83% | 610.6% | 529.1% |

## Group 4 off

Group 4 includes tools that require more processing on the neighboring reconstructed samples than VVC.

The offgroup4.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14-844dd563c |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.76% | 6.28% | 8.84% | 80.3% | 85.2% | 100% | -11.70% | -9.30% | -19.46% | 687.1% | 312.2% |
| Class A2 | 2.63% | 6.39% | 7.60% | 78.5% | 88.0% | 100% | -18.24% | -18.35% | -22.59% | 646.3% | 334.3% |
| Class B | 1.15% | 6.72% | 5.59% | 84.4% | 91.2% | 99% | -12.70% | -16.77% | -15.01% | 775.5% | 456.1% |
| Class C | 1.06% | 2.82% | 2.74% | 80.3% | 87.9% | 99% | -12.84% | -8.22% | -9.36% | 752.0% | 408.4% |
| Class E | 1.39% | 6.24% | 4.98% | 82.2% | 81.5% | 99% | -16.48% | -16.26% | -15.22% | 781.7% | 490.0% |
| Overall | 1.52% | 5.65% | 5.73% | 81.5% | 87.2% | 99% | -14.12% | -13.80% | -15.80% | 733.2% | 401.5% |
| Class D | 0.90% | 2.44% | 2.34% | 78.9% | 85.9% | 100% | -10.80% | -5.45% | -6.03% | 746.2% | 389.2% |
| Class F | 1.30% | 4.07% | 3.96% | 88.1% | 89.6% | 99% | -28.37% | -30.33% | -30.54% | 535.5% | 488.1% |
| Class TGM | 2.17% | 3.87% | 3.98% | 88.9% | 91.1% | 100% | -41.05% | -46.00% | -45.18% | 467.2% | 582.3% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-14-844dd563c |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.69% | 3.42% | 5.74% | 89.9% | 98.1% | 100% | -25.12% | -20.06% | -31.67% | 716.8% | 665.9% |
| Class A2 | 1.23% | 3.39% | 3.57% | 91.9% | 98.1% | 100% | -28.58% | -30.52% | -36.04% | 698.4% | 823.8% |
| Class B | 1.03% | 5.69% | 5.28% | 90.7% | 101.1% | 100% | -23.07% | -27.10% | -24.54% | 744.5% | 949.4% |
| Class C | 1.07% | 1.76% | 1.95% | 90.6% | 96.1% | 100% | -24.82% | -19.65% | -20.43% | 789.5% | 998.5% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.21% | 3.73% | 4.14% | 90.8% | 98.5% | 100% | -25.05% | -24.39% | -27.17% | 741.0% | 871.3% |
| Class D | 0.92% | 1.81% | 1.46% | 91.2% | 93.3% | 99% | -25.78% | -20.40% | -21.70% | 782.9% | 1084.2% |
| Class F | 1.31% | 2.97% | 3.35% | 92.8% | 95.7% | 100% | -31.27% | -32.48% | -33.04% | 675.6% | 598.8% |
| Class TGM | 0.87% | 1.76% | 1.64% | 91.3% | 97.0% | 100% | -41.08% | -46.24% | -46.06% | 605.4% | 560.9% |

## Group 1-4 off

In this test, all the tools in the group 1-4 are switched off. The offgroup1-4.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14-844dd563c |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 3.39% | 10.26% | 15.66% | 55.0% | 65.0% | 102% | -10.27% | -6.11% | -14.96% | 470.7% | 238.2% |
| Class A2 | 5.83% | 13.56% | 14.11% | 57.4% | 57.4% | 100% | -15.60% | -13.17% | -18.17% | 472.8% | 217.8% |
| Class B | 4.26% | 12.43% | 11.36% | 54.1% | 59.1% | 102% | -10.04% | -12.61% | -10.56% | 496.8% | 295.8% |
| Class C | 3.80% | 6.29% | 6.60% | 50.5% | 57.4% | 101% | -10.50% | -5.19% | -6.01% | 473.0% | 267.0% |
| Class E | 6.50% | 12.03% | 10.66% | 51.7% | 55.0% | 100% | -12.26% | -11.76% | -10.62% | 491.3% | 330.5% |
| Overall | 4.65% | 10.83% | 11.36% | 53.6% | 58.7% | 101% | -11.48% | -9.83% | -11.56% | 482.1% | 270.0% |
| Class D | 2.99% | 5.34% | 5.29% | 51.0% | 53.3% | 100% | -8.95% | -2.81% | -3.34% | 482.1% | 241.7% |
| Class F | 12.57% | 16.53% | 17.14% | 58.5% | 51.3% | 100% | -20.83% | -22.85% | -22.58% | 355.6% | 279.6% |
| Class TGM | 17.86% | 21.78% | 22.80% | 59.7% | 44.0% | 100% | -32.23% | -36.88% | -35.50% | 313.7% | 280.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-14-844dd563c |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 10.79% | 11.63% | 20.44% | 61.6% | 68.7% | 100% | -18.41% | -13.82% | -22.96% | 491.2% | 466.4% |
| Class A2 | 10.94% | 15.26% | 16.09% | 61.9% | 62.0% | 100% | -21.72% | -22.73% | -28.32% | 470.7% | 520.6% |
| Class B | 8.87% | 16.33% | 15.53% | 58.6% | 67.1% | 100% | -17.04% | -19.75% | -17.13% | 481.3% | 630.4% |
| Class C | 8.37% | 9.89% | 11.28% | 55.3% | 57.5% | 99% | -19.33% | -13.21% | -13.14% | 481.8% | 597.2% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 9.53% | 13.46% | 15.49% | 58.9% | 63.7% | 100% | -18.86% | -17.42% | -19.47% | 481.2% | 563.1% |
| Class D | 6.46% | 8.78% | 9.44% | 55.3% | 57.7% | 99% | -21.62% | -14.87% | -15.35% | 474.2% | 670.7% |
| Class F | 14.18% | 16.40% | 17.18% | 63.4% | 67.1% | 99% | -22.78% | -24.13% | -24.54% | 461.8% | 419.8% |
| Class TGM | 18.69% | 23.64% | 24.38% | 66.8% | 68.4% | 99% | -30.78% | -34.94% | -34.31% | 443.0% | 395.5% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-14-844dd563c |  |  |  |  |  | Over VTM-11ecm14 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 9.15% | 12.87% | 13.06% | 49.3% | 52.1% | 99% | -13.72% | -26.90% | -22.57% | 400.2% | 403.6% |
| Class C | 9.08% | 10.89% | 11.66% | 48.5% | 49.1% | 99% | -16.34% | -16.20% | -16.57% | 371.2% | 396.5% |
| Class E | 9.18% | 17.27% | 13.14% | 58.7% | 64.9% | 98% | -13.17% | -13.22% | -13.55% | 430.2% | 366.9% |
| Overall (Ref) | 9.13% | 13.31% | 12.61% | 51.2% | 53.9% | 99% | -14.46% | -19.91% | -18.31% | 397.4% | 391.8% |
| Class D | 7.40% | 10.81% | 11.88% | 48.8% | 44.2% | 99% | -18.89% | -18.13% | -16.82% | 362.3% | 418.6% |
| Class F | 13.67% | 15.65% | 16.90% | 60.9% | 56.8% | 99% | -19.78% | -27.81% | -27.64% | 409.6% | 325.7% |
| Class TGM | 17.32% | 25.05% | 27.02% | 68.7% | 59.0% | 100% | -29.52% | -37.55% | -36.88% | 438.8% | 307.4% |

## Summary

The tool-off results on top of the recent ECM versions are summarized below for BD-PSNR-Y. Note that SCC results are not included.

# Issues

## Resolved issues

- #74, ECM master tip (6e5deca1, 8/13/2024) encoding crash with AhG7 offgroup1-4.cfg

## Open issues

- The group of tool PDP

  - It was commented that PDP should be in group 4 (Tools that require more processing on the neighboring reconstructed samples than VVC) due to the complexity it introduced

  - The proponent claimed that PDP should be in group 5 (others)

  - It was agreed to discuss this in plenary

- #78, Encoder crash in RA with --LMChroma=0

- #77, Encoder crash in RA with --DepQuant=0 --NumSignPred=0

- #71, Encode/decode mismatch when using single tree

- #66, Difference in encoding results of ECM-12.0 in AhG7 group 2 tool off test

- #65, Different encoding results of ECM12 in AHG7 group1-4 off tests

- #64, Encode/decode mismatch and decoder crash when inter-CCCM is disable

- #53, Decoding mismatch was observed when AMVR is off

# Input contributions

| JVET-AJ0145 | m69685 | 2024-10-23 11:45:02 |  |  | AHG7: Inter tool assessments for each ECM version | R. Ishimoto, Z. Fan, T. Chujoh, T. Ikai (Sharp) |
| --- | --- | --- | --- | --- | --- | --- |

# Recommendations

- Continue and improve tool assessment

- Resolve identified software issues related to the tool assessment

- Request tool-off test (sub-part of CTC with less number of frames) as a sanity check before an ECM MR is merged