| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0024-v2 |
| --- | --- |

| Title: | EE2: Summary Report on Enhanced Compression beyond VVC capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Roman Chernyak Karam Naser Jacob Ström Fan Wang Martin Winken Xiaoyu Xiu Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com chernyak@tencent.com Karam.Naser@InterDigital.com jacob.strom@ericsson.com wangfan6@oppo.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as intra prediction, inter prediction, transform and coefficient coding, in-loop filtering, and entropy coding.

The software basis for this EE is ECM-14.0, released at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-14.0. ECM-14.0 is used as an anchor in the tests.

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-ai-ee2/ECM/-/branches.

Test results can be found in input JVET contributions, cross-check results are uploaded to https://vcgit.hhi.fraunhofer.de/ecm/jvet-ai-ee2/simulation-results if cross-check reports are not submitted as they are optional for EE tests.

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Partitioning |  |  |  |
| 1.1a | Rectangular partitioning restrictions with normative solution | A. Tissier (Xiaomi) | W. Ahmad (Ericsson) |
| 1.1b | Rectangular partitioning restrictions with non-normative solution | A. Tissier (Xiaomi) | M. He (Nokia) |
| 2 Intra prediction |  |  |  |
| 2.1 | Matrix-based position dependent intra prediction for TIMD | Y. Kidani (KDDI) | Z. Sun (OPPO) |
| 2.2a | TIMD merge mode extension with BV improvement | J. Fu (Peking Univ.) Y.-J. Chang (Qualcomm) M. Abdoli (Xiaomi) | X. Li (Alibaba) |
| 2.2b | Test 2.2a + DIMD merge mode (Intra merge mode extension with BV improvement) | J. Fu (Peking Univ.) Y.-J. Chang (Qualcomm) M. Abdoli (Xiaomi) | X. Li (Alibaba) |
| 2.3 | TIMD with enhanced block vectors deployment | K. Naser (InterDigital) J. Fu (Peking Univ.) | D. Ruiz Coll (Ofinno) |
| 2.4 | Test 2.3 + BV merge list improvement | K. Naser (InterDigital) | D. Ruiz Coll (Ofinno) |
| 2.5 | TIMD merge mode | M. Abdoli (Xiaomi) Y.-J. Chang (Qualcomm) J. Fu (PKU) | V. Rufitskiy (TCL) |
| 2.6 | Additional TIMD mode with different cost metric | D. Bugdayci Sansli (Nokia) | W. Ahmad (Ericsson) |
| 2.7a | Disabling TIMD related combinations in normative way | Z. Fan (Sharp) | Z. Lv (vivo) F. Wang (OPPO) |
| 2.7b | Disabling TIMD related combinations in non-normative way | Z. Fan (Sharp) | F. Wang (OPPO) |
| 2.7c | Test 2.7a + additional RDOs | Z. Fan (Sharp) | Z. Lv (vivo) F. Wang (OPPO) |
| 2.7d | Test 2.7b + additional RDOs | Z. Fan (Sharp) | F. Wang (OPPO) |
| 2.8 | Combination of TIMD and intra merge related tests |  |  |
| 2.8a | Combination of 2.5 and 2.7a | M. Abdoli (Xiaomi) Y.-J. Chang (Qualcomm) Z. Fan (Sharp) J. Fu (PKU) | X. Li (Alibaba) |
| 2.8b | Combination of 2.5 and 2.7c | M. Abdoli (Xiaomi) Y.-J. Chang (Qualcomm) Z. Fan (Sharp) J. Fu (PKU) | X. Li (Alibaba) |
| 2.8c | Combination of 2.6 and 2.7a | D. Bugdayci Sansli (Nokia) |  |
| 2.8d | Combination of 2.6 and 2.7c | D. Bugdayci Sansli (Nokia) | J. Choi (LGE) |
| 2.8e | Combination of 2.6 and 2.7b | D. Bugdayci Sansli (Nokia) |  |
| 2.8f | Combination of 2.6 and 2.7d | D. Bugdayci Sansli (Nokia) |  |
| 2.8i | Combination of 2.5 and 2.1 | M. Abdoli (Xiaomi) Y.-J. Chang (Qualcomm) J. Fu (PKU) Y. Kidani (KDDI) | X. Li (Alibaba) |
| 2.9 | DIMD with filtered-template |  | Z. Deng (Bytedance) |
| 2.10a | Adaptive HoG for DIMD | S. Blasi (Nokia) | S. Puri (InterDigital) |
| 2.10b | Adaptive HoG without increased predictors | S. Blasi (Nokia) | S. Puri (InterDigital) |
| 2.10c | Adaptive HoG without scaling and increased predictors | S. Blasi (Nokia) | S. Puri (InterDigital) |
| 2.11 | DIMD with 2x2 edge operator applied to small blocks | A. Filippov (TCL) | R. Youvalari (Xiaomi) |
| 2.12 | Non-adjacent DIMD mode derivation for TMRL intra mode candidate list construction | V. Rufitskiy (TCL) | Z. Fan (Sharp) |
| 2.13a | Test 2.11 + Test 2.10 | A. Filippov (TCL) S. Blasi (Nokia) | R. Ishimoto (Sharp) |
| 2.13b | Test 2.11 + Test 2.9 | V. Rufitskiy (TCL) Z. Lv (vivo) | Z. Deng (Bytedance) |
| 2.13c | Test 2.11 + Test 2.12 | V. Rufitskiy (TCL) | R. Ishimoto (Sharp) |
| 2.14 | High-level control of intra prediction methods | K.-Y. Kim (WILUS) | J. Kim (ETRI) |
| 2.15a | Regression-based SGPM blending | X. Li (Alibaba) | Z. Xie (OPPO) |
| 2.15b | SGPM with PDP replacement | X. Li (Alibaba) | Z. Xie (OPPO) |
| 2.15c | Test 2.15a + Test 2.15b | X. Li (Alibaba) | Y. Kidani (KDDI) |
| 2.15c* | Test 2,15c with encoder optimization | X. Li (Alibaba) | Y. Kidani (KDDI) |
| 2.16 | Unified reference area of IBC and IntraTMP | N. Zhang (Bytedance) | X. Li (Alibaba) |
| 2.17 | Modified L-shape template in IntraTMP | J. Lee (ETRI) | J.-K. Lee (Ofinno) |
| 2.18 | Chroma TMRL | H. Huang (OPPO) | X. Li (Alibaba) |
| 2.19a | Multi-model EIP | L. Zhang (OPPO) | X. Li (Alibaba) |
| 2.19b | Block vector guided EIP | Z. Xie (OPPO) | X. Li (Alibaba) |
| 2.19c | Test 2.19a + Test 2.19b | L. Zhang (OPPO) | X.Li (Alibaba) |
| 3.10a | SATD-based reordering for intra coding | Y. Wang (Bytedance) | L. Zhang (OPPO) |
| 2.22 | Test 2.19c + Test 3.10a | Y. Wang (Bytedance) L. Zhang (OPPO) | C. Ma (Kwai) |
| 2.20 | Neural network-based intra prediction with DIMD mode derivation | S. Eadie (Qualcomm) |  |
| 2.21 | Neural network-based intra prediction | T. Dumas (InterDigital) |  |
| 3 Inter prediction |  |  |  |
| 3.1 | Dual merge mode | R.-L. Liao (Alibaba) | F. Pu (Dolby) |
| 3.2a | Affine inter AMVP list with template matching | Z. Deng (Bytedance) | J.-L. Lin (Qualcomm) |
| 3.2b | Test 3.2a + Regular inter AMVP list with more candidates | Z. Deng (Bytedance) | J.-L. Lin (Qualcomm) |
| 3.3a | OBMC extension with intra prediction | D. Kim (ETRI) | Y. Ahn (LGE) |
| 3.3b | Test 3.3a + DIMD with 2x2 edge operator | D. Kim (ETRI) | Y. Ahn (LGE) |
| 3.3c | Test 3.3a + using intra prediction mode of neighbouring block | D. Kim (ETRI) | J.-K. Lee (Ofinno) |
| 3.3d | Test 3.3a + more neighbouring blocks are checked for available motion information to generate inter subblock | D. Kim (ETRI) | J.-K. Lee (Ofinno) |
| 3.4a | GPM mode extension (64 to 112 modes) | Y. Ahn (LGE) | K. Jia (Alibaba) |
| 3.4b | GPM mode extension with block shape dependency (64 to 80 modes) | K. Reuzé (InterDigital) |  |
| 3.4c | GPM mode extension with additional distances for hor/ver angles only (64 to 70 modes) | Y. Ahn (LGE) |  |
| 3.4d | Test 3.4b not applied on IBC-GPM nor SGPM | K. Reuzé (InterDigital) |  |
| 3.4e | GPM mode extension with block size adaptation | Y. Ahn (LGE), K. Reuzé (InterDigital) | K. Jia (Alibaba) |
| 3.5 | On POC conditions of BDOF | N. Yan (Kwai) | M. Salehifar (Bytedance) |
| 3.6 | GPM-affine with TM | Y. Wang (Bytedance) | K. Jia (Alibaba) |
| 3.7a | Sub-block merge mode extension with normative conditioning | F. Wang (OPPO) | J. Chen (Alibaba) |
| 3.7b | Sub-block merge mode extension with non-normative conditioning | F. Wang (OPPO) | J. Chen (Alibaba) |
| 3.8 | Sharp motion compensation filter extension | H. Huang (Qualcomm) | withdrawn |
| 3.9a | Affine CPMV search for affine DMVR | J.-L. Lin (Qualcomm) | Z. Deng (Bytedance) |
| 3.9b | High precision MV for affine motion compensation | J.-L. Lin (Qualcomm) | Z. Deng (Bytedance) |
| 3.9c | Additional combined merge candidates | J.-L. Lin (Qualcomm) | Z. Deng (Bytedance) |
| 3.9d | Test 3.9a + Test 3.9b +Test 3.9c | J.-L. Lin (Qualcomm) | Z. Deng (Bytedance) |
| 3.10b | SATD-based reordering for inter coding | Y. Wang (Bytedance) | L. Zhang (OPPO) |
| 3.11a | Regression-based GPM with TM | K. Jia (Alibaba) | Y. Ahn (LGE) F. Wang (OPPO) |
| 3.11b | Regression-based GPM with TM and MMVD | K. Jia (Alibaba) | Y. Ahn (LGE) F. Wang (OPPO) |
| 3.12a | Test 3.6 + Test 3.11a | Y. Wang (Bytedance) K. Jia (Alibaba) | H. Wang (Qualcomm) C. Ma (Kwai) |
| 3.12a* | Test 3.6 + Test 3.11a + encoder optimization | Y. Wang (Bytedance) K. Jia (Alibaba) | H. Wang (Qualcomm) C. Ma (Kwai) |
| 3.12b | Test 3.6 + Test 3.11b | Y. Wang (Bytedance) K. Jia (Alibaba) | H. Wang (Qualcomm) C. Ma (Kwai) |
| 4 Transform and coefficients coding |  |  |  |
| 4.1 | NSPT kernels for non-regular intra modes | G. Verba (Qualcomm) | Fabrice Le Léannec (InterDigital.com) |
| 4.2a | Improved implicit MTS applied to ISP blocks (CTC) | C. Bonnineau (InterDigital) | S. Blasi (Nokia) |
| 4.2b | Improved implicit MTS applied to intra blocks (non-CTC) | C. Bonnineau (InterDigital) | D. Ruiz Coll (Ofinno) |
| 4.2c | 4.2b with default implicit MTS method applied to ISP (non-CTC) | C.Bonnineau (InterDigital) | D. Ruiz Coll (Ofinno) |
| 4.3a | Corner subblocks for SBT | V. Seregin (Qualcomm) | L. Zhao (Bytedance) |
| 4.3b | Corner and center subblocks for SBT | V. Seregin (Qualcomm) | L. Zhao (Bytedance) |
| 5 In-loop filtering |  |  |  |
| 5.1a | Restriction of ALF coefficient values to form m*2^b + Huffman-based coding | V. Shchukin (Ericsson) | N. Hu (Qualcomm) |
| 5.1b | Restriction of ALF coefficient values to form m*2^b + modified Golomb-Rice coding | V. Shchukin (Ericsson) | N. Hu (Qualcomm) |
| 5.1c | Simulated annealing to derive ALF coefficients (encoder-only) | V. Shchukin (Ericsson) | N. Hu (Qualcomm) |
| 5.1d | Test 5.1c + m*2^b coefficient values restriction (encoder-only) | V. Shchukin (Ericsson) | G. Boisson (InterDigital) |
| 5.1e | Test 5.1a + Test 5.1c | V. Shchukin (Ericsson) | G. Boisson (InterDigital) |
| 5.2 | Using luma reconstruction signal for chroma ALF | C. Ma (Kwai) | J. Chen (Alibaba) |
| 5.3 | Extended usage of fixed-filters | W. Yin (Bytedance) | Z. Lv (vivo) |
| 5.4a | Coding information based classification for online-filter of ALF | W. Yin (Bytedance) | N. Hu (Qualcomm) I. Jumakulyyev (Nokia) |
| 5.4b | Coding information based classification for offline-filter of ALF | W. Yin (Bytedance) | N. Hu (Qualcomm) I. Jumakulyyev (Nokia) |
| 5.4c | Test 5.4a + Test 5.4b | W. Yin (Bytedance) | N. Hu (Qualcomm) I. Jumakulyyev (Nokia) |
| 5.5a | ALF joint optimization (encoder only) | H. Wang (Qualcomm) | W. Yin (Bytedance) |
| 5.5b | ALF joint optimization with reuse of ALF control information | H. Wang (Qualcomm) | W. Yin (Bytedance) |
| 6 Entropy coding |  |  |  |
| 6.1a | Context initialization retraining | P. Nikitin (Qualcomm) F. Galpin (InterDigital) |  |
| 6.1b | Context bins coding is redistributed between context and EP coding | P. Nikitin (Qualcomm) F. Galpin (InterDigital) |  |
| 6.1c | Bins coding is redistributed between context and EP coding | P. Nikitin (Qualcomm) F. Galpin (InterDigital) | J. Lainema (Nokia) |
| 6.2a | Bins redistribution between context, NEP and EP with no constraint on the number of arithmetic coded bins. | F. Galpin (InterDigital) P. Nikitin (Qualcomm) | J. Lainema (Nokia) |
| 6.2b | Bins redistribution between context, NEP and EP, with a constraint on the number of arithmetic coded bins (alpha=0.5) | F. Galpin (InterDigital) P. Nikitin (Qualcomm) | M. Abdoli (Xiaomi) |
| 6.2c | Bins redistribution between context, NEP and EP, with a constraint on the number of arithmetic coded bins (alpha=1.0) | F. Galpin (InterDigital) P. Nikitin (Qualcomm) | M. Abdoli (Xiaomi) |
| 6.2d | Context bin coding is redistributed between context and NEP | P. Nikitin (Qualcomm) F. Galpin (InterDigital) |  |

# Description of tests

## Partitioning

### Test 1.1: Rectangular partitioning restrictions (JVET-AJ0179)

In ECM, for I-slices (in AI configuration) in luma, the maximum BT and TT sizes are set to 32, while in chroma, the maximum BT and TT sizes are set to 64 and 32, respectively. Maximum MTT depth is set equal to 3 for I-slices in AI configuration.

In the test, maximum BT and TT sizes constraints are modified for I-slices as follows:

- It allows MTT split in some blocks, where anchor disallows MTT via max BT and TT size values.

- It allows “only one” MTT split on those blocks.

Next figure shows the current ECM configuration in black with only QT split allowed for MTT restricted blocks and the configuration of the test that adds the MTT splits to the list of split available for MTT restricted blocks.

To limit the complexity increase, only one MTT split is applied for those blocks, i.e. the maximum MTT depth is set to 1 shown in the next figure.

Two tests were performed:

Test 1.1a: Rectangular partitioning restrictions with normative solution

Test 1.1b: Rectangular partitioning restrictions with non-normative solution.

## Intra prediction

### Test 2.1: Matrix-based position dependent intra prediction for TIMD (JVET-AJ0077)

In the test, PDP which replaces part of the conventional intra prediction modes, is extended to TIMD, where PDP is also employed in template cost calculation for TIMD. It is also applied to the inter prediction tools using TIMD, such as CIIP and GPM intra.

Since the number of modes in TIMD is doubled comparing to regular intra modes, PDP is only used for modes 0, 1, and (2+4*k) for coding blocks with width and height up to 16 samples; otherwise, PDP is only used for modes 0, 1, and (2+8*k).

PDPC is not applied to the PDP samples in TIMD, just as PDPC is not applied to the PDP samples in regular intra prediction.

### Test 2.2: Intra merge mode extension with BV development (JVET-AJ0149)

In the test, intra merge mode is the method constructing an intra candidate list by the intra modes that are used in intra-coded neighbouring blocks to predict a current coding block, and BV based prediction can be also included in the prediction according to the template cost analysis.

In Test 2.2a, an intra candidate list is constructed by each CU coded with TIMD mode in both adjacent and non-adjacent area. The BV candidate list is also constructed considering adjacent and non-adjacent area. Template cost is calculated using TIMD and BV candidates and the one with smallest cost is selected to predict the current block. A flag is signalled to indicate the mode usage.

In Test 2.2b, DIMD mode candidates are also included into the intra merge list.

### Tests 2.3-2.4: TIMD with enhanced block vectors deployment and BV merge list improvement (JVET-AJ0191)

In the tests, BV predictor is introduced to TIMD mode replacing one of the modes in the fusion process. BV candidates can be derived from IBC, IntraTMP and other modes containing a BV, such as SGPM or DIMD.

Test 2.3 comprises of the following aspects:

- Allow TIMD with block vectors. When deriving the intra modes of TIMD (2 angular and 1 non-angular), the block vectors candidates are also tested. If the template cost is smaller than any of the other modes, that mode is replaced by the block vector.

- Use ARBVP candidates in TIMD. The additional block vector candidates generated by ARBVP process are considered for TIMD.

- Include IntraTMP-LIC best-by-cost candidate when constructing ARBVP candidates and merge candidates.

- Use adjacent candidates as in OBIC.

- Include BVs from DIMD, TIMD, and SGPM when constructing merge candidates.

- Include BVs from DIMD, TIMD, and SGPM when constructing ARBVP candidates.

- Include BVs from SGPM blocks in SGPM candidates.

- Refinement of ARBVP candidates in IntraTMP uses 5x5 window size while merge candidates use 11x11 window size.

In addition, following aspects are used to limit complexity:

- Up to 30 local and non-local merge candidates are used in SGPM and in DIMD.

- IntraTMP candidates are sorted based on the position relative to the IntraTMP search region and 30 candidates are kept. Candidates outside the IntraTMP search region are prioritized.

- In ARBVP process for TIMD, the number of ARBVP candidates added to the list is dynamically adjusted according the TIMD merge list size under construction.

- In ARBVP process for IntraTMP, the number of ARBVP candidates added to the list is dynamically adjusted according the IntraTMP merge list under construction.

- In IntraTMP, if overlap is detected, clustering is not done if merge candidate's TM cost is too high.

Test 2.4 adds additional elements on top of Test 2.3, which include clustering and refinement of DIMD and TIMD BVs. A refinement window is defined around each BV from the merge list, and for all coinciding BVs, the same refinement window is used, i.e. multiple BVs are coinciding in one refinement area. All overlapping BVs are fused together, and a single refinement region is used.

### Test 2.5: TIMD merge mode (JVET-AJ0061)

In ECM, TIMD prediction process is based on a template cost analysis in which several intra modes from a candidates list comprising MPMs, wide-angular, DC, horizontal, vertical, and extended modes are tested. Up to three intra modes (IPM1, IPM2, IPM3) providing the smallest SATD costs are selected and blended in a fusion process according to their respective template cost.

In the test, TIMD candidate list is constructed from adjacent and non-adjacent spatial neighboring blocks coded in TIMD or TIMD merge mode. TIMD information (prediction modes, fusion flag, fusion weights and wide-angle conditions of TIMD modes, MTS transform types) are added to a TIMD merge list. Up to five TIMD candidates are added into the initial list.

The merge candidates are sorted based on the SAD cost over the template of the current block, the template size is set equal to 1, e.g. the first neighbour line. For the cost calculation, the SAD cost of the primary IPM of each TIMD candidate is used. If two primary modes are the same, the one with the lower index is chosen first in the sorting.

The candidate with the smallest cost, i.e. the first in the list, is used to derive the prediction.

The TIMD merge mode is applied to only luma blocks and is disabled for 4x4 block sizes, the mode usage is indicated by a flag.

### Test 2.6: Additional TIMD mode with different cost metric (JVET-AJ0146)

In this test, an additional TIMD based intra mode is introduced where the cost metric is changed from SATD to MR SAD, the number of template lines/columns around the current block are doubled to 4/8, and the list of TIMD candidate modes is modified to avoid redundancy with respect to the conventional TIMD mode.

The candidate list is derived based on the sorted modes determined by the conventional TIMD process. An additional refinement to the candidate list is applied based on the DIMD modes.

The mode usage is indicated by a flag, it is disabled for 4xN and Nx4 blocks.

### Test 2.7: Disabling the combination of TIMD-ISP and TIMD-MRL (JVET-AJ0079)

In this test, a combination of TIMD and ISP, and a combination of TIMD and MRL are disabled with the corresponding flags signalling. After disabling them, the number of RDO is reduced, so the saved RDOs are applied to other intra prediction modes, potentially leading to performance improvement.

Test 2.7a: Disabling TIMD with ISP and MRL in normative way.

Test 2.7b: Disabling TIMD with ISP and MRL in non-normative way.

Test 2.7c: Test 2.7a with additional RDOs.

Test 2.7d: Test 2.7b with additional RDOs.

### Test 2.8: Combination of TIMD related tests

Test 2.8a: Test 2.5 + Test 2.7a (JVET-AJ0180).

Test 2.8b: Test 2.5 + Test 2.7c (JVET-AJ0180).

Test 2.8c: Test 2.6 + Test 2.7a (JVET-AJ0328).

Test 2.8d: Test 2.6 + Test 2.7c (JVET-AJ0328).

Test 2.8e: Test 2.6 + Test 2.7b (JVET-AJ0328).

Test 2.8f: Test 2.6 + Test 2.7d (JVET-AJ0328).

Test 2.8i: Test 2.1 + Test 2.5 (JVET-AJ0341).

### Test 2.9: DIMD with filtered-template (JVET-AJ0100)

In the test, DIMD modes with filtered templates are introduced. Compared to the current DIMD, one additional row towards above side, and one additional column towards left side are extended if available, up to (W×2+5)×5 above reconstructed samples and 5×(H×2) left reconstructed samples of current block W×H are used as template if available. When there are un-available reconstructed samples, the unfiltered samples in the adjacent row and column are used for gradient computation.

3×3 filter is applied on the template.

F=-1-1-1-19-1-1-1-1

The mode usage is indicated by a flag and filtered-template DIMD is only applied to block whose area is larger than or equal to 256, and smaller than or equal 1024. And it is applied to I-slice only.

### Test 2.10: Adaptive HoG for DIMD (JVET-AJ0267)

Instead of using the fixed template comprising three lines above and on the left of the current block, the DIMD process is modified so that an adaptive number of samples is considered, with the goal of filling the HoG until the total cumulative amplitude is greater or equal than a given threshold.

After a given sample is visited in the DIMD template, its direction and amplitude are computed using the Sobel filter, and the HoG is updated accordingly. Then, the cumulative sum of all amplitudes in the HoG is computed. If this sum is smaller than a given threshold then another sample is considered in the DIMD template for computing the HoG, else, the computation of the HoG early terminates. The DIMD templates are extended to include more lines above and on the left of the current block. This process is performed independently for the templates above, left, and above-left the current block. The thresholds used to fill the HoG are determined depending on the block size.

When computing a given gradient for a reference sample, its amplitude is scaled with a weight, which depends on the distance of the reference sample from the current block and on the direction of the neighbouring template sample.

Also, the DIMD process is modified so that more intra modes are extracted from the DIMD histogram for larger blocks (with area larger or equal than 128). Up to 8 DIMD predictors are blended in blocks with an area larger or equal than 128 samples, whereas the number of predictors is kept to 6 as in conventional DIMD for smaller blocks. The extracted modes are then blended as in conventional DIMD.

The method is only used on blocks with an area larger or equal than 128 samples. In smaller blocks, conventional HoG calculation is used instead.

In Test 2.10a, the method is used in full.

In Test 2.10b, the number of DIMD predictors is kept to 6 (as in conventional DIMD) for all block sizes.

In Test 2.10c, the number of DIMD predictors is kept to 6 for all block sizes, and no scaling is used for the amplitude in the HoG calculation.

### Test 2.11: DIMD with 2x2 edge operator applied to small blocks (JVET-AJ0203)

In the test, a 2x2 kernel instead of 3x3 Sobel filter is applied to 4x4, 4x8 and 8x4 blocks when calculating Histogram of Gradients in DIMD process.

Horizontal and vertical filters of the 2x2 operator are defined as follows:

Fx=-1-111

Fy=1-11-1

### Test 2.12: Non-adjacent DIMD mode derivation for TMRL intra mode candidate list construction (JVET-AJ0212)

In the test, non-adjacent DIMD modes for TMRL intra mode candidate list construction are introduced, where HoG is computed using 2x2 operators as in Test 2.11, histogram is sorted and first 5 modes are selected. This process is repeated for each reference line independently. The resulting candidate list is further reordered in such a way that similar modes are placed at the bottom of the candidate list.

The candidate list is sorted in ascending order by the value of template matching cost (as specified by the TMRL process) is iterated starting from the top of the list (i.e. TMRL candidates with the minimum template cost). On each iteration, a similarity check is performed for a pair of candidate predictors to find which of them have a difference lower than a predetermined threshold.

The similarity check for a pair of candidate predictors is performed by calculating a metric value for the difference of the predictors obtained using intra prediction parameters defined for these candidates.

### Test 2.13: Combination of DIMD related tests

Test 2.13a: Test 2.10a + Test 2.11 (JVET-AJ0268).

Test 2.13b: Test 2.9 + Test 2.11 (JVET-AJ0221).

Test 2.13c: Test 2.11 + Test 2.12 (JVET-AJ0220).

### Test 2.14: High-level control of intra prediction methods (JVET-AJ0057)

In the test, SPS flags are introduced for reference sample filtering, PDPC (and gradient PDPC), and intra prediction fusion. Each SPS flag is set based on the hash block hit percentage at the encoder, a metric already available in ECM and used in other tools. If the percentage exceeds a certain threshold, the video sequence is interpreted as screen content, and the specified methods are not applied to that sequence.

### Test 2.15: Regression-based SGPM blending (JVET-AJ0112)

In Test 2.15a, the SGPM candidate list is further extended with regression-based SGPM candidates, the regression-based method is already utilized for GPM in ECM.

In the regression-based GPM blending method, two integer blending matrices (W0 and W1) are derived from the template (1 line) which can be modelled as an affine linear function of the sample positions (x,y) in the current CU as:

W0x,y=32-W1(x,y)

W1x,y=a*x+b*y+c

The parameters (a, b, c) are derived from the template using the same solver as the one used for CCCM. A list of candidate pairs is built from the regular GPM candidates and reordered with the template cost. If a pair of candidates is selected, the two candidates are used to predict the current block and blended with the derived blending matrices as:

predx,y=W0x,y*pred0x,y+W1x,y*pred1x,y+16≫5

Four regression-based SGPM candidates are added to the front of the existing SGPM candidate list. Each regression-based candidate contains two intra prediction modes which are selected from the primary MPM list and up to 6 block vectors of neighbouring blocks.

Two integer blending matrices are derived from the template (1 line) for each regression-based SGPM candidate and used to blending the predictors of the two intra prediction modes. If the regression model for a regression SGPM candidate is not available, the model will fall back to a 1:1 weighted average of the two predictors. Moreover, the DIMD method is used to derive an intra mode to select the transform kernel if a regression-based SGPM candidate is selected.

In Test 2.15b, the matrix-based intra prediction replacement method (PDP method) is extended to SGPM. Each of the two conventional intra prediction modes in SGPM prediction will be replaced by a matrix-based intra prediction mode if the replacement conditions are satisfied. The same replacement conditions as in ECM-14 are used.

In Test 2.15c, the combination of Test 2.15a and Test 2.15b is tested.

In Test 2.15c*, Test 2.15c with encoder optimization is tested.

### Test 2.16: Unified reference area of IBC and IntraTMP (JVET-AJ0172)

In ECM, the reference area of IBC is extended to two CTU rows above when CTU size is 128 and the reference area of IBC is limited to one CTU row above when CTU size is 256 as shown in next figures.

On the other hand, a BV candidate of IntraTMP is determined to be valid as long as the corresponding reference block is already reconstructed inside the current picture.

In the test, the reference area of IBC and IntraTMP is unified such that the valid IBC reference area is extended to all the reconstructed samples in the picture.

### Test 2.17: Modified L-shape template in IntraTMP (JVET-AJ0143)

In IntraTMP, there are three template types: L-shape, top, and left. In the L-shape template, top, left, and top-left samples of a current block are used to calculate SAD and MRSAD, and to derive FLM parameters.

In the test, top-left samples are excluded from L-shape template as illustrated in next figure.

It is asserted that excluding those samples provides better template matching and thus prediction for small blocks, and complexity reduction.

### Test 2.18: Chroma TMRL (JVET-AJ0081)

In the test, chroma TMRL mode is introduced with the following elements.

Reference lines 1, 3, 5 and 8 intra prediction mode candidates are used, there are total 3x8=24 combinations, the intra mode candidates are derived as follows:

- The non-CCP chroma intra prediction modes in the current ECM including DM, chroma DIMD, DC;

- The collocated luma modes: the intra prediction modes from luma blocks in the positions of TL, TR, BL and BR in the collocated luma block as shown in the next figure;

- The adjacent chroma modes: the intra prediction modes from adjacent chroma blocks in the positions of L', T', BL', TR', and TL' as shown in the next figure

Combination candidates from the list are used to predict the collocated luma block and the template of the current chroma block. This template includes one top row and one left column of the current chroma block as shown in next figure. The SATD cost between the prediction and the reconstructed samples is then calculated for collocated luma block and chroma templates. The candidates in the list are reordered based on a combined SATD cost, calculated as follows:

cost=8*costY+costCbTop+costCrTop≪logH+2+costCbLeft+costCrLeft≪logW+2

First 6 combination candidates with the smallest cost are used.

A chroma TMRL flag is signalled to indicate whether the chroma TMRL mode is applied or not. If the chroma TMRL mode is applied, an index is signalled to indicate which combination candidate in the list is used.

### Test 2.19: Multi-model and BV guided EIP

In Test 2.19a (JVET-AJ0082), multi-model EIP is introduced, where samples are classified into two groups by a threshold, and the samples in both groups are used as training sets to derive two separate extrapolation filters, respectively. The threshold is set as the average value of all the samples in the neighbouring reconstructed area. An EIP training sample with reference <= threshold is classified into Group1; while an EIP training sample with reference > threshold is classified into Group2.

In addition, the regularization parameters λ is adaptively selected according to the number of training samples when deriving filter coefficients. The same classification rule is applied when predicting the current block.

The prediction sample is calculated according to the reference value shown in the figure below of the current inputs and the threshold.

The multi-model EIP mode is signalled by a flag when the current block’s EIP mode flag is true and EIP-merge mode flag is false. The number of derived EIP filter candidates are modified so that the total number of derived EIP filter candidates (including both EIP and MM-EIP) doesn’t exceed the previous number. The maximum template size of EIP filter derivation process is set to 8.

In Test 2.19b (JVET-AJ0083), a block-vector guided EIP uses a block vector to determine the reference area for calculating the EIP filter parameters instead of directly using the adjacent spatial reference area shown in the next figure. Then the current coding unit utilizes the calculated EIP filter parameters and neighboring reconstructed samples to create the prediction.

A BV used in this method is derived from the rough searching process of the IntraTMP in the current ECM.

Additionally, the square-shaped EIP model calculated from the reference area is added into the initial EIP merge list, where the reference area used for this model calculation is guided by the BV. The EIP model candidates reordering process and the final sizeof the EIP merge list are kept unchanged.

Test 2.19c (JVET-AJ0084): Test 2.19a + Test 2.19b.

Test 2.22 (JVET-AJ0277): Test 2.19c + Test 3.10a.

### Tests 2.20-2.21: Neural network-based intra prediction with DIMD mode derivation (JVET-AJ0249)

In the test, NN-based intra prediction is introduced, which has the following elements:

- SADL for inference

- 16-bit integer for model weights and intermediate results, using 32-bit integer for internal computation

- 6 models depending on the block size

- sparse weights enforced at training time for complexity reduction.

DIMD process is used to derive the intra prediction direction for transform kernel selection, LFNST transform is applied to this mode. An alternative IPM is used for LFNST/NSPT only for luma CBs of size larger than 64. In addition to the 1st DIMD IPM, PLANAR mode is used as an additional IPM candidate. A CU-level flag indicating the IPM index is signalled only for luma CBs of size larger than 64.

Model complexity is summarized in the next table.

| Model | Parameter count (k) | MACs/pixel | Memory Footprint (MB) |
| --- | --- | --- | --- |
| 4x4 | 50.823 | 3176 | 0.195 |
| 8x4 | 61.886 | 1934 | 0.224 |
| 16x4 | 123.420 | 1928 | 0.450 |
| 8x8 | 132.260 | 2066 | 0.472 |
| 16x8 | 178.754 | 1396 | 0.590 |
| 16x16 | 215.376 | 841 | 0.596 |

Number of parameters for the whole intra prediction mode is 762519.

## Inter prediction

### Test 3.1: Dual merge mode (JVET-AJ0110)

In the dual merge mode, two motion information (inter prediction direction, motion vectors, reference picture indices, BCW index, LIC parameters and MHP information) from previous coded blocks are inherited, each motion information can be either uni- or bi-predicted motion information, and dual merge list is constructed as follows:

- derive dual merge candidates from spatial, temporal and non-adjacent spatial neighbouring blocks if the neighbouring blocks are coded in the dual merge mode;

- if the dual merge list is not full, construct a regular merge list using a process similar to that of the regular merge mode, and candidates in the regular merge list are paired and added to the dual merge list;

- perform ARMC to dual merge candidates.

An index is signalled to indicate which one of dual merge candidates is selected. The two selected motion information are used to generate two prediction blocks using the same motion compensation process as normal inter modes. Then, the two prediction blocks are averaged to generate the final prediction for the CU. MHP, DMVR, BDOF, LIC and OBMC are not applied to the dual merge mode.

To store the two motion information for the dual merge coded blocks, the size of motion storage buffer is doubled. When deriving merge candidates from a dual merge coded block for other merge modes, the two motion information are treated as two separate merge candidates. In addition, both motion for two GPM partitions are stored for the blending area of GPM mode.

### Test 3.2: Enhanced inter AMVP (JVET-AJ0126)

In Test 3.2a, affine AMVP list construction is modified. Template matching is used to refine and reorder the affine AMVP candidates. An initial list with up to 10 candidates is built.

Besides the existing candidates in the current ECM, additional non-adjacent affine candidates in the target reference picture are checked if the current list is not full. The positions of the non-adjacent candidates in the reference picture are the same as the positions of the temporal candidates in the affine merge mode.

The first 2 candidates with the lowest template costs are selected to construct the final affine AMVP list, and the index of the selected affine MVP candidate is signalled in the bitstream.

On top of Test 3.2a, Test 3.2b further checks additional non-adjacent spatial candidates for the regular inter AMVP candidate list construction and this change is applied to non-low-delay pictures only. An initial list with up to 10 candidates is built. Same as the current ECM, the candidate with the smallest template cost is selected as the MVP.

### Test 3.3: OBMC extension with intra prediction (JVET-AJ0161)

In OBMC of ECM, top and left boundary samples of the current block are only blended with inter prediction block generated using motion information of neighbouring block. However, top and left boundary samples adjacent to intra block remain unrefined due to the absence of motion information of neighbouring intra block.

In Test 3.3a, in addition to the existing OBMC process, the top and left boundary samples adjacent to intra block are blended with intra prediction subblock generated using the intra prediction mode derived by applying DIMD on the neighbouring reconstructed samples as shown in the next figure.

The blending of top and left boundary samples adjacent to intra blocks is performed only when the intra prediction mode derived from DIMD is in the range of [34, 66] and [2, 34] for the top and left boundary samples, respectively, as shown in the next figure. This condition limits the usage of padded reference samples for intra prediction.

Test 3.3b: Test 3.3a where 3x3 Sobel operator is replaced with 2x2 from Test 2.11.

Test 3.3c: Test 3.3a where neighbouring intra mode is used instead of being derived by DIMD.

Test 3.3d: Test 3.3a where more neighbouring blocks are checked to derive motion information. For subblocks on the top boundary, in addition to the above block, the blocks to its left and right are also checked. For subblocks on the left boundary, in addition to the left block, the blocks above and below it are also checked.

### Test 3.4: GPM mode extension (JVET-AJ0107)

In ECM, GPM supports 64 partitioning modes from the combination between 20 angles and 4 distances (64 GPM modes = 8 angles × 4 distances + 8 angles × 3 distances + 4 angles × 2 distances).

In the test, additional angles and distances are added. The total number of GPM angles are extended from 20 to 32. In Tests 3.4a/e 12 additional angles with equal spacing are used, and in Test 3.4b/d 12 additional angles with un-equal spacing.

In Test 3.4a, the total number of GPM partitioning modes is extended to 112 modes. The angle index near vertical angle direction (horizontal split directions) are removed since most nature objects or motions are vertically. The additional angles can be combined with 4 distances same as the conventional angles, and total of 42 GPM modes are newly added. (42 GPM modes = 6 angles × 4 distances + 6 angles × 3 distances) Additionally, 6 GPM modes are added for 6 horizontal and vertical distances. In the current GPM, six distance indices for horizontal and vertical angles (0, 8, 16, 24) are removed because these six partitioning modes are likely to overplay with binary tree and ternary tree split. However, blending process can be applied to the boundary samples of GPM partitions.

In Test 3.4b, partitions used by GPM, SGPM and IBC-GPM are dependent on the block shape, similar to wide angle intra prediction. It is asserted that diagonal splits are favored for a given block shape, therefore diagonal and antidiagonal splits are included for each block shape, as well as splits with angles close to those, the range of angles available GPM, SGPM and IBC-GPM is increased from 20 to 32.

For each width/height ratio from width/height=1/8 to width/height=8 a list of GPM partitions is derived so that each block shape uses the most relevant partitions. The partitions are ordered so that the first 8 partitions constitute the IBC-GPM first set, the next 40 are the ones to be used for IBC-GPM second set, and the first 26 are the ones to be used for SGPM.

The number of modes for regular GPM is not increased to reduce the encoding runtime. The changes are applied to GPM, SGPM and IBC-GPM.

In Test 3.4d, the changes are only applied to GPM.

In Test 3.4e, the total number of GPM partitioning modes is extended to 112, 84, 70, or 64 modes according to block size and shapes as follows:

- Small blocks (cuWidth*cuHeight <= 64): 64 GPM modes (same as 64 GPM modes in VVC/ECM)

- 2NxN and Nx2N blocks: 84 GPM modes

- 4NxN and Nx2N blocks: 70 GPM modes

- Large 2Nx2N blocks: 112 GPM modes

### Test 3.5: On POC conditions of BDOF (JVET-AJ0097)

In the test, BDOF is additionally applied to the bi-prediction CUs having two different reference pictures (i.e., different POCs) that are either before or after the current picture in the display order, aside of that the the existing BDOF design in ECM-14.0 is kept unchanged.

### Test 3.6: GPM-affine with TM (JVET-AJ0095)

In Test 3.6, GPM-affine is harmonized with GPM-TM, a GPM affine candidate can be refined by template matching using the same method as in affine mode. A GPM sub-partition can be predicted by GPM-affine-TM or GPM-TM.

### Test 3.7: Sub-block merge mode extension (JVET-AJ0085)

In the test, the CU size restriction for SbTMVP mode “cbWidth >= 8 && cbHeight >= 8” is changed to “cbWidth * cbHeigh >= 32” so it can be applied to 4xN and Nx4 blocks.

In the second aspect, neighbouring blocks’ coding information is used to determine whether sub-block merge mode is allowed for blocks whose block size is in the grey area shown in the next figure. Seven adjacent neighbouring blocks and five non-adjacent neighbouring blocks are checked. If at least one of the checked neighbouring blocks is coded with sub-block merge mode or inter affine mode or GPM-affine mode, sub-block merge mode is allowed for current block. Otherwise, sub-block merge mode is forbidden for the current block. Separate context models are used for the sub-block merge flag for blocks whose block size is in the grey area.

In Test 3.7a, the sub-block merge flag is not signalled if sub-block merge mode is forbidden by the condition check.

In Test 3.7b, the sub-block merge flag is signalled regardless of the condition check.

### Test 3.9: Sub-block inter mode improvement (JVET-AJ0158)

This test investigates the following modifications to the subblock inter modes:

First, it is proposed to add a CPMV search process in the affine DMVR. For each control point, bilateral matching is independently performed for a block that centered by the control-points to derive the refined CPMVs. Then the refined CPMVs are used to derive the best set of CPMVs that minimize the bilateral matching cost of the current block.

Second, it is proposed to quantize the per-pixel or subblock based motion vector to 1/64-pel precision instead of 1/16-pel precision for motion compensation. The 64-phase interpolation filter has the same frequency response as the current 12-tap filter in ECM. However, the subblock motion vectors that are stored for the motion vector prediction are kept in the same 1/16-pel precision, as in ECM.

Third, additional candidates are added by combining the source MV with either TMVP or CMVP. The source MV is the MV used to derive TMVP or CMVP, which is a uni-directional MV. For example, if the source MV comes from the reference picture list 0, the list 0 MV of the TMVP or CMVP is replaced by this MV and the result is used as a new combined candidate in subblock candidate list.

Test 3.9a: Affine CPMV search for affine DMVR

Test 3.9b: High precision MV for affine motion compensation

Test 3.9c: Additional combined merge candidates

Test 3.9d: Test 3.9a + Test 3.9b + Test 3.9c

### Test 3.10: SATD-based reordering (JVET-AJ0096)

In ECM, reordering method is used for coding tools including ARMC, template-based multiple reference line intra prediction (TMRL), extrapolation filter-based intra prediction (EIP), template matching based BCW index derivation for merge mode, etc. Sum of absolute difference (SAD) is calculated as the template cost in these reordering methods.

In Test 3.10a, SATD-based reordering is used for intra and IBC coding tools, such as reordering in TMRL and EIP, and adaptive reordering of merge candidates (ARMC) for IBC.

In Test 3.10b, SATD-based reordering is used for inter coding tools, such as ARMC for affine and template matching (TM) based reordering for MMVD.

### Test 3.11: Regression-based GPM extension (JVET-AJ0115)

In ECM, TM and MMVD is only applied to regular GPM, but not applied to regression-based GPM.

In Test 3.11a, only TM extension is applied. When regression-based GPM is used for the current block, a TM flag is signalled to indicate whether the motion vector is to be refined by TM. If the TM flag is true, the motion vectors of both partitions are refined. The template used in the TM process consists of above and left reconstructed samples. The TM search process is the same as that of regular GPM-TM.

In Test 3.11b, both TM and MMVD extensions are applied. The TM extension is same as Test 3.11a. When TM flag is true, the motion vectors of both partitions are refined by TM. When TM flag is false, an MMVD flag is signalled to indicate whether the motion vectors of partitions are refined by the MVD. If the MMVD flag is true, an MVD index is signalled to indicate the MVD. In order to reduce the encoder complexity of MMVD extension, the number of MVD candidates is reduced to 40.

### Test 3.12: Combination of GPM related tests

Test 3.12: Test 3.6 + Test 3.11 (JVET-AJ0274)

## Transform and coefficient coding

### Test 4.1: NSPT kernels for non-regular intra modes (JVET-AJ0175)

In the test, the existed NSPT kernels set is replaced with the three new ones:

- The first set is applied for conventional intra predicted blocks.

- The second set is applied for blocks using TIMD, DIMD, EIP, MIP or SGPM.

- The third set is applied for blocks using IntraTMP and inter CUs.

The usage of the additional set depends on the prediction mode and no additional signalling is needed.

The proposed kernels require 2.2 MB of additional storage.

### Test 4.2: Improved implicit MTS (JVET-AJ0257)

In this test, the primary transform pair is directly inferred from the intra prediction mode and the TU size of the current block (sizeIdx) using a lookup-table (LUT). The LUT contains transform pairs based on the separable primary transforms of ECM-14.0. No additional primary transforms are added.

For ISP blocks, the sizeIdx used as input to the LUT is based on the position of the current ISP sub-partition within the ISP block.

In test 4.2a, the proposed implicit MTS method is only applied to ISP blocks following CTC configuration settings.

In Test 4.2b and test 4.2c, the proposed implicit MTS method is applied to angular intra modes. The default implicit MTS method (i.e., based on the shape of the TU) is kept for TIMD and DIMD modes. DCT2 is used for MIP, EIP, SGPM and IntraTMP modes. Anchor in this test is ECM-14.0 with MTS=0 and MTSImplicit=1. In Test 4.2c, the default implicit MTS method from ECM-14.0 is used for ISP blocks.

### Test 4.3: Corner and center subblocks for SBT (JVET-AJ0260)

In this test, four corners and center subblocks are introduced to SBT. The size of the subblock can be width/2 x height/2 or width/4 x height/4, the latter can be applied for transform blocks 16x16 and larger.

Transform kernels are implicitly derived, DCT-8 and DST-7 combination is used for corner subblocks, and DST-1 is used for the center.

Similarly to the existing SBT, the split mode and the location of the non-zero subblock is signalled. The signalling is done if SBT mode is enabled and placed after the existing SBT modes.

LFNST transform is also applied, and choice of the transform is signalled for those subblocks following the adoption of LFNST transform usage for SBT in JVET-AI meeting.

Two tests are performed:

Test 4.3a: Corner subblocks are added for SBT

Test 4.3b: Corner and center subblocks are added for SBT

## In-loop filtering

### Test 5.1: Adaptive mantissa precision for ALF coefficients (JVET-AJ0138)

In the test, ALF coefficients are limited to power of 2, e.g. m*2b, where m and b are integers in the ranges of m∈[-2BM+1, 2BM-1] and b∈[0, BALF-1], the range of coefficients is [-2BALF-1, 2BALF-1], where BALF is the precision factor with the corresponding mantissa precision BM.

For the chroma components, mantissa precision BM=1 is utilized, i.e., only pure powers of two are allowed, but for the luma component, the mantissa precision is configured as BM∈{1,2}. For luma, the encoder signals one bit in the APS to indicate which mantissa precision is used, BM=1 or BM=2. The choice of BM value for luma is based on RD-cost.

This restriction on coefficient values is applied in Tests 5.1a-b, d-e.

Coefficients are signaled using Huffman coding and 18 Huffman codes (16 for luma and 2 for chroma) are utilized. The frequencies for Huffman code are collected by compression of BVI-DVC sequences using ECM-based encoder.

Huffman-based signaling is used in Tests 5.1a and 5.1e, while in Tests 5.1c and 5.1d the signaling method of ECM-14.0 (based in Golomb-Rice coding) is preserved.

Modified Golomb-Rice coding is used in Test 5.1b. In this test, the signaling method utilizes the knowledge of tuple (luma/chroma, BALF, BM, coefficients group) as follows: the absolute value of a coefficient is replaced by the corresponding index in the alphabet subarray of positive elements in increasing order. For example, for (luma, 6, 1, 0) coefficient values -32,-16,-8,-4,-2,-1,0,1,2,4,8,16,32 are mapped to -6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6, correspondingly. Afterwards, Golomb-Rice coding method from ECM-14.0 is applied to the mapped counterparts.

Simulated annealing is used at encoder to find the ALF coefficients. The simulated annealing process is launched several times with different initial solutions. First, the existing coordinate descent optimization method from ECM-14.0 is run, and the first simulated annealing run is using that as a starting point. Then simulated annealing is run 39 more times, where each run takes the output of the previous run as its starting point.

A random number generator is extensively utilized in the simulated annealing approach. There is a variety of pseudo-random generators that can be used, for example, Xorshift. To provide an encoder reproducibility, we use the same hardcoded pseudo-random number generator with a fixed seed for each simulated annealing launch.

Simulated annealing is applied in tests 5.1c-e.

Test 5.1a: ALF m*2^b coefficient values with Huffman-based coding.

Test 5.1b: ALF m*2^b coefficient values with modified Golomb-Rice coding.

Test 5.1c: Simulated annealing to derive ALF coefficients (encoder-only).

Test 5.1d: Test 5.1c + m*2^b restriction (encoder-only).

Test 5.1e: Test 5.1a + Test 5.1c.

### Test 5.2: Using luma reconstruction signal for chroma ALF (JVET-AJ0144)

In the test, luma reconstruction signal is utilized as additional inputs to the chroma ALF. The luma reconstruction signal is firstly down-sampled to have the same resolution with the chroma signal, where the down-sampling filter is [1:2:1; 1:2:1]/8. Then, a 5x5 cross filter shape is applied to the down-sampled luma reconstruction signal and added to the chroma ALF filter as shown in the next figure as the last term on the right.

### Test 5.3: Extended usage of fixed filters (JVET-AJ0186)

In the test, the fixed filters defined in ALF are reused after the DBF process as shown in next figure.

The method reuses the existed fixed filters of ALF and takes the input samples before DBF and the output samples after DBF as input to produce the filtered results at this stage. The classification and filtering logic of the fixed filters are directly reused. The ALF process is kept unchanged and a flag is signalled in the slice header to control the method.

### Test 5.4: Coding information-based classification for ALF (JVET-AJ0188)

In the test, a method of coding information-based classification for ALF is introduced, towards both online filters and offline filters.

For the online filters, each 2×2 unit is classified into 2 noise levels according to the partitioning information. For a 2×2 unit, it is classified into noise level-1 if it is located at a CU or TU boundary, and into noise level-0 if it is not. The class number of existing classifiers is reduced from 25 to 12. These 12 classes are further combined with the proposed 2 noise levels to generate final 12×2 = 24 classes in total. The number of classifiers for online filters is kept as 3 and no additional encoder selection is introduced.

For the offline filters, each 2×2 unit is classified into 2 noise levels in the same way as the classification of online filters. Besides, each 2×2 unit is further classified into 2 residual levels based on a predefined threshold also in the same way as the classifier of online filters. Furthermore, the generated offset of offline filters is adjusted based on the boundary level and residual level accordingly, where a stronger offset is applied on the positions at boundaries or with high residuals.

In Test 5.4a, the partitioning information is introduced to associate the classification of online filters of ALF.

In Test 5.4b, the partitioning and residual information is introduced into the classification of offline filters of ALF.

Test 5.4c: Test 5.4a + Test 5.4b.

### Test 5.5: Reuse of ALF control information (JVET-AJ0231)

In the test, ALF control information, including on/off decision and ALF APS usage information, from a previously encoded picture is reused. To indicate this mode, a flag is signalled if ALF is enabled for a picture, ALF control information is reused and not signalled.

At encoder side, ALF filter is derived for a group of pictures jointly which share the ALF on/off decision. The method is only applied to non-reference pictures, bitstreams between the first and last pictures are not written out until the joint ALF optimization of the group is done.

In Test 5.5a, encoder-only part is tested. In this test, ALF coefficients and control information are derived for a group of pictures and signalled for each frame. The signalling is the same as ECM-14.0.

In Test 5.5b, a flag is signalled if ALF is enabled for a picture and ALF control information from a previously coded picture is reused and not signalled for the current picture.

## Entropy coding

### Tests 6.1-6.2: Entropy coding extensions (JVET-AJ0211)

In ECM, slice data can be coded either using CABAC context or in bypass (equal probability) mode.

In the test, a mode is added where the probability depends on the syntax element to encode (non-equal probable - NEP), but there is no context update in the CABAC engine.

Training on CTC bitstreams were performed to identify bins to be placed into the new non-equal probable mode.

Test 6.1a: Retrained CABAC contexts.

Test 6.1b: Context bins coding is redistributed between context and EP coding.

Test 6.1c: Bins coding is redistributed between context and EP coding.

Test 6.2a: Bins redistribution between context, NEP, and EP with no constraint on the number of arithmetic coded bins.

ECM-14.0 has 5096 context bin + 1360 EP bins (without counting bins related to residual coding).

| Model | Number of contexts |
| --- | --- |
| EP | 84 |
| NEP | 606 |
| CABAC | 5766 |

Test 6.2b: Bins redistribution between context, NEP, and EP with a constraint on the number of arithmetic coded bins (alpha=0.5). Alpha parameter controls the amount of NEP bins as follows:

( number_contexts + alpha * number_nep )  ≤ number_ECM14_contexts

| Model | Number of contexts |
| --- | --- |
| EP | 829 |
| NEP | 1062 |
| CABAC | 4565 |

Test 6.2c: Bins redistribution between context, NEP, and EP with a constraint on the number of arithmetic coded bins (alpha=1.0).

| Model | Number of contexts |
| --- | --- |
| EP | 1360 |
| NEP | 242 |
| CABAC | 4854 |

Test 6.2d: Context bin coding is redistributed between context and NEP.

| Model | Number of contexts |
| --- | --- |
| EP | 0 |
| NEP | 95 |
| CABAC | 5001 |

Number of coded bins is summarized in the next table.

| Model | % coded bins |  |  |
| --- | --- | --- | --- |
|  | Ecm14 | Test 6.1c | Test 6.2a |
| EP | 12.6 | 9.5 | 9.5 |
| NEP | 0 | 0 | 0.4 |
| Context | 87.4 | 90.5 | 90.1 |

# Test results

See the accompanied Excel table.

Asterisk (*) with cursive font is used to indicate estimated results, where typically incomplete rate points are replaced with anchor.