| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0042-v1 |
| --- | --- |

| Title: | AHG10: Teleconference on encoder optimization for multi-layer coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | P. de Lagrange | Email: | philippe.delagrange@interdigital.com |
| Source: | AHG10 |  |  |

_____________________________

# Abstract

This document summarizes discussions during teleconference on encoder optimization for multi-layer coding conducted by JVET AHG10 between the 35th meeting (Sapporo, JP, 12–19 July 2024) and the 36th meeting (Kemer, TR, 1–8 November 2024).

# Agenda

This teleconference is a follow-up of the one that took place during the previous meeting cycle; this section is copied from the JVET-AI0042 report.

The agenda is related to the AHG mandate related to multi-layer coding:

- Study optimized encoding for reference picture resampling and scalability modes in VTM, and coordinate with AHG4 on improving encoders and test settings for multi-layer verification testing.

The work is related to an issue found during visual tests of spatial scalability in Geneva (31st meeting), where the VTM encoder could create an enhancement layer that degrades visual quality (see JVET-AE0288), introducing coding artefacts that outweigh the benefits of detail enhancement. The situation was improved at the following meeting (see JVET-AF0186 and JVET-AF0311) but still not satisfying.

The goal of the teleconference is to organize encodings and preliminary checks (both objective metrics and informal visual reports) with different configuration and/or new ideas and spread the load, so that meaningful visual tests can be conducted with AHG4/AG5.

The focus is on the first two test categories of the JVET-AH2021 test plan:

- “Spatial scalability”: how does dual-layer compare with adaptive resolution & single-layer (with enhancement layer typically 10% of total bitrate)

- “Quality ladder”: how much can be saved with dual-layer compared to simulcast, when enhancement layer brings significant quality improvement over base layer (typically 50% of total bitrate)

# Discussion

The first teleconference was held on August 20th, 2024, at 15:30h UTC.

## Description of visual quality issues under study

The dual-layer encoding performance issues to be investigated were described in JVET-AI0042: in a nutshell, sometimes, the enhancement layer has lower visual quality than the inter-layer prediction because that prediction is not selected.

Also the R/D performance of the enhancement layer may be significantly lower (visually) than the base layer: for the same total bitrate, the bitrate taken by the enhancement layer means a higher QP for the base layer than a standalone reduced-resolution encoding, with stronger visual artefacts that are not corrected by the enhancement layer (loss of detail in low-contrast areas, pumping effect, etc), resulting in a lower visual quality rating despite of effective detail improvement.

A typical example is shown in Figure 1, where details are better on high-contrast areas (e.g. cars), but low-contrast areas suffer (e.g. trees, road surface in the shade).

Figure 1 – Comparison of QP=26 upscaled with QP=28+35 dual-layer on TallBuildings2 (similar bitrate). Enhancement layer is effective on high-contrast details, but the higher base QP washes out low-contrast detail.

## Comments

Comments raised in the previous teleconference were related to QP adaptation, LMCS, pumping effect, scaling ratio, comparison with SHVC, quantization matrices, MCTF, and deblocking strength.

One participant commented that for adaptive QP, we should not go for large local changes (contrary to real-life encoder), to stay close to the flat QP configuration we are used to.

## Test sequences and rate points

For the time being, the focus can be on the sequences and rate points listed in Table 1, especially the two highlighted ones. Other sequences listed in JVET-AH2021 and the attached spreadsheet can be optionally tested.

Depending on experiments, it can be useful to test EL QPs ranging from BL QP+6 to BL QP+10, in addition to the QPs indicated in Table 1 or in JVET-AH2021. For rate alignment purpose, it may also be useful to generate reduced resolution encodings with QP -1 or -2 compared to the base layer of dual-layer encodings (as done for the expert viewing reported in JVET-AE0288).

Table 1 – Selected test sequences and QP settings for viewing reported in JVET-AF0311

| Sequence | QP SL | QP UP | QP BL QP EL |
| --- | --- | --- | --- |
| CorsairJoy | 33 | 27 | 27 34 |
| CorsairJoy | 40 | 34 | 34 41 |
| TallBuildings2 | 34 | 28 | 28 35 |
| TallBuildings2 | 42 | 36 | 36 43 |
| DrivingPOV3Logo | 32 | 27 | 27 34 |
| DrivingPOV3Logo | 38 | 33 | 33 40 |
| DrivingPOV3Logo | 42 | 37 | 37 44 |
| BoxeLogo | 34 | 30 | 30 37 |
| BoxeLogo | 38 | 32 | 32 39 |
| BoxeLogo | 42 | 37 | 37 44 |

The location and md5sum of source videos are found in the attached spreadsheet.

## Encoding configuration, tools

Details about running the VTM in scalable configuration are given in the annex of JVET-AH2021 (some of which are repeated here). It is suggested to focus on random-access configuration and 2x scaling ratio for the time being.

The VTM version to be used for new experiments is VTM-23.4 + merge request 2747. Once MR !2747 is merged, the commit to be used will be indicated here.

The first step is to downscale source videos for the base layer using TAppDownConvert (see details in JVET-AH2021, section 4.1.1). An example is given below:

TAppDownConvertStatic 3840 2160 TallBuildings2_3840x2160_30fps_10bit_420pf.yuv \
  1920 1080 TallBuildings2_1920x1080_30fps_10bit_420pf.yuv 10 10

Then dual-layer encodings can be run using configuration files found in MR !2747 (two_layers_scalable.cfg and layer1_gop_randomaccess.cfg in cfg/multi-layer folder), and sequence-specific configuration files as usual (to specify e.g. source video resolution and frame rate); a “resHD_30.cfg” example is given below:

InputBitDepth      : 10         # Input bitdepth

InputChromaFormat  : 420        # Ratio of luminance to chrominance samples

FrameRate          : 30         # Frame Rate per second

FrameSkip          : 0          # Number of frames to be skipped in input

SourceWidth        : 1920       # Input  frame width

SourceHeight       : 1080       # Input  frame height

FramesToBeEncoded  : 300        # Number of frames to be coded

Level              : 4.1

A UHD configuration file would adjust width and height, and set Level to 5.1.

Example dual-layer VTM encoder command line:

EncoderApp -c encoder_randomaccess_vtm.cfg -c two_layers_scalable.cfg \
  -l0 -q 28 -l0 -c resHD_30.cfg -l0 -i TallBuildings2_1920x1080_30fps_10bit_420pf.yuv \
  -l1 -q 34 -l1 -c resUHD_30.cfg -l1 -i TallBuildings2_3840x2160_30fps_10bit_420pf.yuv \
  -l1 -c layer1_gop_randomaccess.cfg -l1 --Level=6.1 \
  -ip 32 -fs 0 -b TallBuildings2_3840x2160_30fps_10bit_420pf_2834.bin \
  --InternalBitDepth=10 --OutputBitDepth=10 --SEIDecodedPictureHash=1 \
  --DMVREncMvSelect=1 --Verbosity=6 --PrintHexPSNR=1 --PrintMSSSIM=1 -o /dev/null \
  >> TallBuildings2_3840x2160_30fps_10bit_420pf_2834.log

Decoding is performed as usual, but adding “-p 0” to get base output, or “-p 1” to get enhanced output. See notes about MR !2686 in section 2.4.4 below to upscale the base layer on the fly.

### GOP-based parameters of enhancement layer (e.g. reference picture list)

Previously, the patch from JVET-AF0186 was applied to put inter-layer reference pictures in the first position in the reference picture lists.

The expected method is now to use explicit configuration as discussed in JVET-AI0248 and provided by MR !2747. This MR includes example configuration files, like cfg/layer1_gop_randomaccess.cfg.

Beyond controlling inter-layer references, it is also possible to configure specific QP offsets and deblocking strength within the GOP for the enhancement layer (see the FrameX parameters, and section 3.1 “GOP structure table” in the VTM software manual).

### Option: internal downscale, to avoid generating downscaled source videos

It is possible to perform the downscale on the fly for the base layer instead of providing a externally downscaled source, using the --SourceScalingRatioHor and --SourceScalingRatioVer parameters.

Note: currently, this is not compatible with MCTF on the base layer (a fix is expected soon).

### Option: “DebugBitstream” to avoid recoding the base layer

The --DebugBitstream option is working in recent VTM versions and is very helpful to test different enhancement layer configurations using the same base layer:

- A first encoding is performed, either in dual-layer, or just the base layer in single-layer configuration (leaving room for future EL ALF APS ids using --MaxNumALFAPS=4)

- Then, dual-layer encodings can use -l0 --DebugBitstream=<base-layer.bin> to avoid re-encoding the base layer. This is much faster since the base layer usually takes most of the encoding time.

Example command line:

EncoderApp -c encoder_randomaccess_vtm.cfg -c two_layers_scalable.cfg \
  -l0 --DebugBitstream=TallBuildings2_3840x2160_30fps_10bit_420pf_2834.bin
  -l0 -q 28 -l0 -c resHD_30.cfg -l0 -i TallBuildings2_1920x1080_30fps_10bit_420pf.yuv \
  -l1 -q 35 -l1 -c resUHD_30.cfg -l1 -i TallBuildings2_3840x2160_30fps_10bit_420pf.yuv \
  -l1 -c layer1_gop_randomaccess.cfg -l1 --Level=6.1 \
  -ip 32 -fs 0 -b TallBuildings2_3840x2160_30fps_10bit_420pf_2835.bin \
  --InternalBitDepth=10 --OutputBitDepth=10 --SEIDecodedPictureHash=1 \
  --DMVREncMvSelect=1 --Verbosity=6 --PrintHexPSNR=1 --PrintMSSSIM=1 -o /dev/null \
  >> TallBuildings2_3840x2160_30fps_10bit_420pf_2835.log

### Expected tool improvements

More improvements are expected on a few practical issues:

- There is no tool to compute the MS-SSIM metric from YUV files, so this metric is not available to compare upscaled anchor to video source. A workaround is to use the ssim/ms-ssim metric from the VMAF tool (for all cases). Extracting the MS-SSIM computation from the VTM to make it available independently of the encoder would be helpful.

- Similarly,

  - Upscale from YUV is not officially available as a separate tool, but a patch exists.

  - Upscale within the decoder is available by merging the changes from MR !2686 and using --UpscaledOutput=2 --outputWidth xxx --outputHeight xxx --UpscaleFilterForDisplay=0 (to use the normative filter coefficients)

  - Downscale of the source (to encode base layer) is performed using the TAppDownConvert tool from the SHM.

These features could be grouped into one rescale tool, made available in the VTM.

# Experiments

## List of experiments

| Test | Volunteer |
| --- | --- |
| Adaptive QP - Both layers - Enhancement layer only (optional) | F. Bossen to try the functionality available in the VTM and report if it brings visual benefits |
| LMCS - Disabled, on both anchor and test - Both layers | O. Chubach |
| Pumping analysis - Check per-picture rate and distortion for anomalies (compared to base layer or single layer) | P. de Lagrange |
| RDO related to inter-layer prediction - Optimize to avoid degrading base layer quality | Work in progress at InterDigital |
| BL/EL QP offset - Consider 10% EL rate instead of +7QP | The EL ratio can be computed from data available, e.g. adding a computed column in the spreadsheet. |
| MCTF - Disable on base layer | O. Chubach |
| Deblocking - Increase strength on enhancement layer |  |
| DMVR - Disable, either on both layers or just enhancement layer. - Objective metrics and visual outcome compared to --DMVREncMvSelect=1 |  |

A spreadsheet is attached to this document and is to be used to report tests performed and objective metrics. It contains details about test configuration and test sequences (including md5sums for source, downscales and upscales), and example reporting tabs.

Results of experiments will be collected in subsequent versions of this document (or attachments).

## Results

### Local QP adaptation

F. Bossen verbally reported that the --qpa parameter works in dual-layer configuration and seems to help visually. The legacy method has not been tested yet.

### LMCS off

Bitstreams are available on the FTP site but were not analyzed yet.

### EL bitrate ratio

A column (BitRatio1) has been added to the reporting spreadsheet, computing the EL/(BL+EL) ratio.

# Closing notes

A second teleconference is planned on September 5th 15:30 UTC.

End of meeting: 16:40h UTC.

# Participants

- Olena Chubach (MediaTek - US)

- Lorenzo Ciccarelli (V-Nova - UK)

- Philippe de Lagrange (InterDigital - FR)

- Jens-Rainer Ohm (RWTH - DE)

- Fabrice Urban (InterDigital - FR)

- Mathias Wien (RWTH - DE)

- Ping Wu (ZTE - UK)