| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0046-v2 |
| --- | --- |

| Title: | AHG9: Handling of processing chains - constraints and special cases |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Jizheng Xu1, Li Zhang1, Yue Li2, Kaifa Yang2, Yiling Xu2, Miska M. Hannuksela3 1Bytedance Inc., US 2Shanghai Jiao Tong University, China 3Nokia, Finland | Tel: Email: | yekui.wang@bytedance.com xujizheng@bytedance.com lizhang.idm@bytedance.com yueli983@sjtu.edu.cn sekiroyyy@sjtu.edu.cn yl.xu@sjtu.edu.cn miska.hannuksela@nokia.com |
| Source: | Bytedance Inc., Shanghai Jiao Tong University, Nokia |  |  |

_____________________________

# Abstract

This contribution proposes the following changes related to handling of processing chains based on the proposal in JVET-AJ0045-v2:

- Specify, for the special cases in VSEI v3 and VVC v3, the values of the variables and lists for the SPO-persistence-level processing stages and the picture-level list of associated SEI messages, such that the special cases are also supported by both the processes of the breadth-first and depth-first handling in a backward compatible manner.

- Add a NOTE to express that pictures in the final list of output pictures of a processing chain are in increasing output order and within the final list of output pictures there are no two pictures having the same output order.

- Add a constraint to disallow generating output pictures between any particular pair of consecutive input pictures more than once within a processing stage.

The detailed proposed text changes, marked relative to the proposed text in JVET-AJ0045-v2, are included in an attachment to this contribution. The attached spec text document also includes changes for JVET-AJ0129 items 1 and 2, which make changes to the same places as item 1 of this contribution.

# Problem statement

The current design for handling of a processing chain, including both breadth-first handling and depth-first handling, has the following problems:

- The special NNPF cascading case is not fully supported, making the design not backward compatible to VSEI v3 and VVC v3.

- The case of a single SEI message in its own processing chain is not full supported, making the design not backward compatible to VSEI v3 and VVC v3.

- There lacks a constraint to disallow two pictures within the final list of output pictures of a processing chain to have the same output order.

- There lacks a constraint requiring pictures in the final list of output pictures of a processing chain to be in increasing output order.

- There lacks a constraint to disallow generating output pictures between any particular pair of consecutive input pictures more than once within a processing stage.

# Proposal

The proposal for resolving the above problems is summarized in the abstract above, and the detailed proposed text changes, marked relative to the proposed text in JVET-AJ0045-v2, are included in an attachment to this contribution.

# Patent rights declaration(s)

Bytedance Inc., Shanghai Jiao Tong University, and Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).