| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0054-v3 |
| --- | --- |

| Title: | EE1-1.3: Neural network-based in-loop filters using early cropping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström, Mitra Damghanian, Du Liu and Per Wennersten | Email: | {jacob.strom, mitra.damghanian, du.liu, per.wennersten}@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In JVET-AI0095, it was proposed to reduce the patch size of the LOP3 neural network-based in-loop filter early in the network. This has the effect of lowering complexity in terms of kMAC/pixel, and this saved complexity is instead spent on widening and deepening the network. This contribution reports the result for EE1-1.3, where this technique is tested for LOP and VLOP. For LOP, the freed-up complexity is spent by increasing from N Y=14 to N Y=15 backbone blocks and from C1Y=144 to C1Y=176 channels. The result is a model with a claimed complexity of 16.83 kMAC/pixel, which is similar to (but smaller than) that of LOP3 (16.90 kMAC/pixel). It is asserted that the deepening and widening of the network increases the parameter count from 0.20M (LOP3) to 0.25M (proposed). The reported int16 results over NNVC-10.0 with NNIntra and LOP3 enabled are

RA-BDR 	-0.27% (Y)	-1.22% (U)	-0.95% (V)
AI-BDR	-0.28% (Y)	-1.15% (U)	-0.92% (V)
LD-BDR	-0.26% (Y)	-0.76% (U)	-0.79%. (V)

For VLOP, the freed-up complexity is spent by widening the network from C1Y=96 to C1Y=112, resulting in a claimed complexity of 5.06 kMAC/pixel, which is similar to, but smaller than that of VLOP2 (5.16 kMAC/pixel). It is asserted that the widening of the network increases the parameter count from 0.061M (VLOP2) to 0.067M (proposed). The reported int16 results over NNVC-10.0 with NNIntra and VLOP2 enabled are

RA-BDR 	-0.03% (Y)	 0.15% (U)	-0.10% (V)
AI-BDR	-0.08% (Y)	-0.01% (U)	-0.06% (V)
LD-BDR	 0.02% (Y)	 0.31% (U)	-0.96%. (V)

It is asserted that the same inference software can be used for both LOP and VLOP. It is proposed to adopt the method into the next version of NNVC.

# Introduction

The current neural network-based in-loop filters (HOP, LOP and VLOP) operate as illustrated in Figure 1. To improve a 128x128 patch of the picture, a larger patch of 144x144 samples (8 samples on each side) is inputted to the network.

Figure 1. The current operation of the neural network-based in-loop filters in JVET. A patch of 144x144 samples is processed and results in an NN output of 144x144 samples, which is then cropped before adding with the input to produce the final output.

An output of the same size (144x144) is produced, and this is then cropped to 128x128 to modify the block. This process is wasteful since the neural network calculates a lot of values that are never used: Only values in the middle 128x128 area (marked with the dashed rectangle) of the NN are used for final output, the ones outside are simply discarded in the subsequent cropping step. In JVET-AH0195 it was suggested to not perform the unnecessary calculations needed to obtain the margin samples that are thrown away. That approach would lower the complexity in terms of kMAC/pixel by about 0.8% and produce identical results with LOP3. In JVET-AI0095, it was proposed to instead reduce the patch size of the LOP3 neural network-based in-loop filter earlier in the network. This has the effect of lowering complexity in terms of kMAC/pixel further down the network, and this saved complexity is spent on widening and deepening the network. It was decided to study this technology for LOP and VLOP in EE1-1.3.

# Proposed method LOP3

The current architecture of the LOP3 filter is shown in Figure 2. A patch of size 144x144 (a) is inputted to the network. After DCT it has size 72x72 (b), and after convolution layers with stride 2 it has size 36x36 (e). This size is now kept throughout the backbone layers (f through j) and the tail (k) until the PixelShuffle layer, which increases the resolution to 72x72 (l). After IDCT the patch size is 144x144 and now cropping is performed to obtain a size of 128x128 that is added to the input which was also cropped to this size (o and p).

Figure 2. The architecture of the LOP3 filter, including patch sizes.

The proposed architecture for LOP is shown in Figure 3. It is the same up to the point (f), which is the input to the first backbone layer, and where the size is 36x36. Instead of a regular backbone block (which we will refer to as backbone block B), a modified backbone block “backbone block A” is used. This uses pad=(0,0) in the sCONV layers (both the 1x3 and 3x1) which has the effect of shrinking the size by two samples in both dimensions. Hence the output of the first backboneA block at (g) is 34x34 samples. This is followed by another backbone block of type A, shrinking the size to 32x32 (h). We have now reached the correct size, and no further shrinking is necessary. Therefore, the remaining backbone blocks are of the regular type B, which does not change the size of the input. After pixelshuffle the size is 64x64 (l), and after IDCT the size is 128x128 (m), which means that the last crop layer between (m) and (n) is no longer necessary.

Figure 3. The proposed architecture for LOP3. Changes highlighted in blue.

Figure 4. The proposed architecture for VLOP. Changes highlighted in blue.

The proposed architecture for VLOP is shown in Figure 4. It is the same as VLOP2 up to and including point (g), hence the first backbone block is of type B (i.e., without cropping). Then two type-A backbone blocks follow between (g) to (i).  The last three backbone blocks are of the regular type B. For the chroma path, the (single) backbone block is not changed. Instead, the tail blocks marked with (o), (p) and (q) produce cropped output using pad=(0,0).

To evaluate the performance of this method at (near to) equal complexity to LOP3, we increase the depth (the number of backbone blocks) as well as the width (the CY1 parameter) of the network. In summary we go from

NY=14 (LOP3)  NY=15 (current proposal), and

CY1=144 (LOP3)  CY1=176 (current proposal).

For VLOP, the complexity saved is not sufficient to both broaden and widen the network. We have tested both approaches and found that broadening the network provides the best gain. We go from

CY1=96 (VLOP2) to CY1=112 (current proposal).

# Training and inference information

Since the network architecture has changed, it is necessary to retrain the network. This is done the same way as for LOP3/VLOP2.

The inference software is a modified version of NNVC10.0rc1, where the output of the neural network is assumed to be the correct size (e.g., 128x128) and it is hence not cropped. The current SADL implementation supports cropping using slicing. As an example, cropping the tensor a of size [:,:,36,36] to [:,:,34,34] can be performed using b = a[:, :, 1:-1, 1:-1]. However, pad=(0,0) is not supported. Therefore, we have emulated pad=(0,0) with pad=(1,1) followed by cropping using slicing. Therefore, SADL will not count kMAC/pixel correctly. Instead we have calculated kMAC/pixel manually using the attached excel file, as well as checking using the thop library on a python model where pad=(0,0) has not been emulated with slicing.

| Table 1. Network Information for the proposed models in training, LOP and VLOP. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU: NVIDIA A100-PCIE-40GB |
|  | Framework: | PyTorch v2.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | Stage3: 60 |
|  | Batch size: | Stage3: 64 |
|  | Loss function: | L1 and MSE |
|  | Training time: | LOP: Stage3: 4.9 days VLOP: Stage3: 4.3 days |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | The LOP stage 3 dataset |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 (input), 128x128 (output) |
|  | Learning rate: | LOP: Stage3: 2e-4, milestones [41, 51, 55] VLOP: Stage3: 4e-4 milestones [41, 51, 55] |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

| Table 2. Network Information for the LOP model in inference. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 248037 |
|  | Parameter Precision (Bits) | fp32, int16 |
|  | Memory Parameter (MB) | fp32: 0.95MB, int16: 0.47MB |
|  | Multiplay Accumulate (MAC) | 16.83 kMAC/pixel |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

| Table 2. Network Information for the VLOP model in inference. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 67283 |
|  | Parameter Precision (Bits) | fp32, int16 |
|  | Memory Parameter (MB) | fp32: 0.26MB, int16: 0.13MB |
|  | Multiplay Accumulate (MAC) | 5.06 kMAC/pixel |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Results

## LOP3-based model

Results for the proposed int16 model based on LOP3 over NNVC10.0 with NNIntra and LOP3. Encoder runtimes are not reliable.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC10.0 + NNIntra + LOP3 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -0.12% | -1.75% | -1.05% | 0.21% | -2.45% | -1.49% | 133% | 110% |
| Class A2 | -0.26% | -1.89% | -0.79% | -0.16% | -1.92% | -0.93% | 122% | 111% |
| Class B | -0.30% | -1.29% | -1.19% | -0.21% | -1.74% | -1.59% | 108% | 111% |
| Class C | -0.37% | -0.24% | -0.70% | -0.09% | -0.29% | -1.07% | 103% | 104% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.27% | -1.22% | -0.95% | -0.08% | -1.53% | -1.30% | 114% | 109% |
| Class D | -0.57% | -0.44% | -0.85% | 0.02% | -1.13% | -0.40% | 103% | 106% |
| Class F | -0.18% | -0.20% | 0.12% | -0.01% | -0.61% | -0.91% | 116% | 107% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC10.0 + NNIntra + LOP3 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -0.31% | -1.32% | 0.11% | -0.15% | -1.75% | -1.11% | 103% | 114% |
| Class A2 | -0.23% | -1.42% | -0.74% | -0.19% | -1.70% | -1.16% | 97% | 112% |
| Class B | -0.23% | -1.22% | -1.21% | -0.22% | -1.72% | -1.57% | 97% | 111% |
| Class C | -0.28% | -0.77% | -0.75% | -0.17% | -1.56% | -1.92% | 98% | 109% |
| Class E | -0.39% | -1.10% | -1.87% | -0.30% | -0.95% | -2.13% | 94% | 112% |
| Overall | -0.28% | -1.15% | -0.92% | -0.20% | -1.56% | -1.60% | 97% | 111% |
| Class D | -0.39% | -0.49% | -0.38% | -0.28% | -1.56% | -1.55% | 100% | 109% |
| Class F | -0.21% | -0.14% | -0.07% | -0.14% | -0.93% | -1.05% | 92% | 110% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC10.0 + NNIntra + LOP3 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class B | -0.17% | -1.69% | -1.70% | 0.07% | -1.59% | -0.58% | 126% | 114% |
| Class C | -0.37% | 0.39% | 0.36% | 0.00% | 0.14% | 2.84% | 136% | 103% |
| Class E | -0.27% | -0.61% | -0.31% | 0.16% | -0.08% | 1.80% | 99% | 92% |
| Overall | -0.26% | -0.73% | -0.67% | 0.07% | -0.63% | 1.16% | 122% | 104% |
| Class D | -0.52% | -0.36% | 1.23% | 0.26% | -2.10% | 5.90% | 91% | 101% |
| Class F | -0.28% | 0.99% | -2.03% | 0.06% | -0.55% | -0.95% | 137% | 105% |

## VLOP2-based model

Results for the proposed int16 model based on VLOP2 over NNVC10.0 with NNIntra and VLOP2. Encoder runtimes are not reliable.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 + NNIntra + LOP3 int16 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.10% | -0.15% | -0.28% | 0.15% | -0.05% | -0.71% | 105% | 106% |
| Class A2 | -0.05% | 0.17% | 0.21% | 0.02% | 0.31% | 0.16% | 96% | 110% |
| Class B | -0.04% | 0.31% | -0.09% | 0.01% | 0.24% | -0.21% | 105% | 107% |
| Class C | -0.10% | 0.17% | -0.19% | -0.06% | 0.24% | -0.25% | 107% | 105% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.03% | 0.15% | -0.10% | 0.02% | 0.20% | -0.25% | 104% | 107% |
| Class D | -0.09% | -0.31% | -0.05% | -0.15% | -0.58% | -0.04% | 104% | 105% |
| Class F | -0.15% | 0.02% | -0.77% | -0.12% | 0.14% | -0.82% | 99% | 108% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 + NNIntra + LOP3 int16 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.06% | -0.14% | 0.23% | -0.01% | 0.13% | 0.10% | 94% | 105% |
| Class A2 | -0.11% | -0.04% | 0.04% | -0.02% | -0.01% | 0.00% | 105% | 109% |
| Class B | -0.07% | 0.01% | -0.14% | -0.02% | -0.14% | -0.26% | 115% | 104% |
| Class C | -0.13% | 0.04% | -0.19% | -0.03% | 0.06% | -0.57% | 107% | 105% |
| Class E | -0.11% | 0.01% | -0.15% | -0.06% | 0.12% | -0.12% | 101% | 104% |
| Overall | -0.08% | -0.01% | -0.06% | -0.03% | 0.01% | -0.20% | 105% | 105% |
| Class D | -0.17% | -0.43% | 0.06% | -0.07% | -0.75% | -0.18% | 99% | 105% |
| Class F | -0.22% | 0.11% | -0.29% | -0.14% | 0.65% | -0.11% | 120% | 104% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-10.0 + NNIntra + LOP3 int16 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class B | 0.09% | 0.75% | -0.88% | 0.13% | 0.56% | -0.60% | 99% | 106% |
| Class C | -0.04% | 0.43% | -0.15% | 0.07% | 0.37% | 1.57% | 94% | 103% |
| Class E | -0.02% | -0.59% | -2.17% | -0.19% | 0.51% | 1.98% | 127% | 106% |
| Overall | 0.02% | 0.31% | -0.96% | 0.03% | 0.48% | 0.77% | 104% | 105% |
| Class D | -0.09% | 0.00% | -0.94% | -0.13% | -2.21% | 0.35% | 106% | 110% |
| Class F | -0.09% | -0.25% | -0.43% | -0.10% | 0.15% | -0.30% | 110% | 113% |

# Conclusion

In this contribution we have presented results for a method where the extra margin in the loopfilter is removed earlier. The complexity that is saved in this manner is spent on widening and/or deepening the network. This provides more gain for both the LOP and VLOP models. The output size of both models are the same (128x128 samples) which means that the same inference software can be used to perform inference on both models. It is recommended to adopt both the LOP-based model and the VLOP-based model into the next version of NNVC.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).