| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0059-v1 |
| --- | --- |

| Title: | AHG9: On NNPFC SEI message with multiple instances of temporal extrapolated pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Claire-Hélène Demarty, Didier Doyen, Philippe de Lagrange | Tel: Email: | 1stname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution is a follow-up of contributions JVET-AI0296 and JVET-AI0322 on the possibility to generate multiple versions of a same temporal extrapolated picture for a same time instant. The contribution aims at explaining the latency issue in interactive applications such as cloud gaming or virtual reality (VR), and the way it may be solved thanks to producing, in advance of the rendering, several versions of a same extrapolated picture and selecting the proper version at the time of the rendering. The contribution proposes adaptations of the NNPFC SEI message design to support this feature when the purpose of the SEI message is temporal extrapolation.

# Introduction

This contribution is a follow-up of contributions JVET-AI0296 and JVET-AI0322 on the possibility to generate multiple versions of a same temporal extrapolated picture for a same time instant.

Temporal extrapolation may be used in interactive applications such as cloud gaming or virtual reality, that are challenged by the issue of end-to-end latency, also known as motion-to-photon latency [Lawlor2016, Dixit2023]. This latency issue is described in the next section, and solutions to address this problem based on the NNPFC SEI message with temporal extrapolation purpose are proposed.

# Usage of multiple temporal extrapolation hypotheses in interactive applications

An example of interactive application where latency is key is cloud gaming. Cloud gaming allows for partly offloading the game rendering process to some game servers located in the cloud. Figure 1 illustrates its basic principle. The 3D graphics rendering, which requires costly and power consuming devices, is performed by game servers in the cloud. The rendered frames are encoded with a regular video encoder and the video stream is decoded on client side with a regular decoder. An additional lightweight module is in charge on managing the gamer interaction commands and frame synchronization.

Figure 1. Cloud gaming principle.

One key factor for user comfort in gaming applications is the motion-to-photon latency [Dixit2023] corresponding to the latency between a user action (motion) and the rendering of the results of this action on the display device (photon).

Figure 2 illustrates a typical motion-to-photon pipeline in a cloud gaming application. At each stage of this pipeline, a processing latency is introduced:

- The hardware latency of the input device, for example a joypad, called input lag, is typically ~10ms.

- The transmission latency (from client to server and server to client), that, depending on the connection quality, can range from few ms to few 100ms.

- The game engine rendering stage depends on the Frame-per-Second which is typically 60. It means that rendering takes less than 17ms.

- The encoder is typically used in low-delay configuration. Real-time encoder usually encodes a frame in few ms.

- The decoder latency can be considered as a few ms.

- The latency of high-end monitor is usually ~20ms.

- Additional hardware latency might add few 10ms.

In total, the final motion-to-photon may reach a few 100ms. Typically, user discomfort starts when latency is above 200ms. For VR using headset visualization, an even lower latency is needed for a proper user comfort (down to less than 20ms, [Lawlor2016, Dixit2023, Yang2019]).

Figure 2. Cloud gaming motion-to-photon path.

In order to reduce the latency, given a current state or rendered picture, one common solution is to perform an approximate rendering of the next rendered picture, [Liu2022]. In a cloud gaming pipeline, this solution would consist in temporally extrapolating the future picture(s) to render, based on the latest decoded picture(s). The concept is illustrated in Figure 3 and Figure 4. Figure 3 depicts the latest decoded picture. After decoding this picture, in this example, three extrapolated pictures may be generated, corresponding to three possible actions of the user: “does not move” (picture of Figure 3), “turns to the left” (left picture of Figure 4), “turns to the right” (right picture of Figure 4). Another example is provided in Figure 5, where three alternate views are extrapolated given current “League of Legends” game state.

Figure 3. illustration of negative latency; latest decoded picture (source https://pinheadsinteractive.com/top-15-best-360-panorama-viewers-2020/)

Figure 4. illustration of negative latency; extrapolated left and right views

Figure 5. illustration of negative latency; three possible alternate views (bottom) deduced from the current League of Legends game state (top).

A typical scenario example is as follows. At a given time instant t, the client produces in advance different extrapolated picture versions from the latest decoded pictures dec(t–K)…dec(t). For example, N extrapolated picture versions pict1(t+dt)…pictN(t+dt) are generated for time t+dt from the available decoded pictures dec(t–K)…dec(t). At time t+dt, considering that the decoded picture dec(t+dt) is not yet available due to the system latency, the rendered picture is picked among the N extrapolated picture versions pict1(t+dt)…pictN(t+dt) generated at time t. The displayed version may be chosen among the N versions depending on the real game state, the actual user action, etc.

Note that in this example, extrapolated pictures are generated for one future time instant t+dt. The process may also generate extrapolated pictures for several future time instants {pict1(t+1)…pictN(t+1)} …{pict1(t+M)…pictN(t+M)}, and the extrapolated picture finally rendered is selected based on the actual latency dt (in 1..M) among the set {pict1(t+dt)…pictN(t+dt)}, and on the user action or game state.

This is illustrated in Figure 6, showing a simple case with a latency dt:

- Some time after t and before t+dt

  - The picture at instant t is received and decoded.

  - The client anticipates latency issues by performing an extrapolation from the latest available decoded picture(s) to generate several possible extrapolated versions {pict1(t+dt)…pictN(t+dt)} for a future time instant (t+dt) (here for simplicity, we consider only one future time instant (t+dt)).

- Time t+dt

  - The decoded picture (at time t+dt) is not yet received or decoded in time (due to latency) and cannot be displayed at time t+dt.

  - The client selects among the various extrapolated pictures {pict1(t+dt)…pictN(t+dt)} the version pictk(t+dt) corresponding to the real user action / game state

  - The client sends this version for rendering at time instant (t+dt).

- The process continues for the following pictures …

Figure 6. usage of extrapolated pictures for reducing latency.

In this scenario, the latency is removed thanks to the fact that the extrapolated pictures are generated in advance to the rendering instant. This requires generating in advance several versions of a same extrapolated picture, the correct version being selected a little before the rendering based on the game state or user action (for instance, the head motion in case of a headset).

# Proposal

Adaptations of the NNPFC SEI message design are proposed for enabling generating multiple versions of a same extrapolated picture for a given time instant.

## Aspect1: define subpictures in the output sensor corresponding to different versions of extrapolated picture

It is proposed to allow an NNPF to return multiple temporal extrapolation versions for a given time instant, by generating as output tensor of the NNPF inference a single large output picture that embeds multiple versions of a temporally extrapolated picture. Each version may be considered as a subpicture of the output 2D tensor. The different versions of the temporally extrapolated picture are packed into the output 2D tensor, as a packed 2D array of subpictures. Each extrapolated picture version is a subpicture that can be identified by a subpicture index. This is illustrated in Figure 7, where 11 subpictures are considered.

Figure 7. illustration of the splitting of the output tensor into subpictures corresponding to different versions of an extrapolated picture.

The identification of the subpicture should be made by external means.

The following syntax changes, highlighted in yellow, are proposed (in 8.28.2.1).

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| … | u(1) |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| … |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) { |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| nnpfc_extrapolated_subpics_minus1 | u(4) |
| nnpfc_extrapolated_subpics_per_row_minus1 | u(4) |
| nnpfc_extrapolated_subpics_prompt_present_flag | u(1) |
| } |  |
| … |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

The following semantics changes are proposed in 8.28.2.2.

After the definition of nnpfc_extrapolated_pics_minus1:

nnpfc_extrapolated_subpics_minus1 plus one indicates the number of alternative temporal extrapolation versions generated in the output tensor outputTensor.

nnpfc_extrapolated_subpics_per_row_minus1 plus one indicates the number of alternative temporal extrapolation versions per row in the output tensor outputTensor. When not present, nnpfc_extrapolated_subpics_per_row_minus1 is inferred to be equal to 0.

The variables subpicPerRow and subpicPerCol, representing respectively the number of subpictures per row and per column present in the output tensor of the NNPF, are derived as follows:

subpicPerRow = (nnpfc_extrapolated_subpics_per_row_minus1 + 1)

subpicPerCol = (nnpfc_extrapolated_subpics_minus1 + 1 + subpicPerRow – 1) / subpicPerRow

When nnpfc_extrapolated_subpics_per_row_minus1 is not present, subpicPerRow and subpicPerCol are set equal to 1.

Complete equations (77) in semantics of pfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 as follows.

…

The variable nnpfcOutputPicWidth, representing the width of the luma sample arrays of the NNPF-generated pictures, is derived as follows:

nnpfcOutputPicWidth = Ceil( CroppedWidth *		(1)
		( nnpfc_pic_width_num_minus1 + 1 ) ÷ ( nnpfc_pic_width_denom_minus1 + 1 ) )

When SpatialExtrapolation is equal to 1, nnpfcOutputPicWidth is updated as follows:

nnpfcOutputPicWidth += outSubWidthC * ( nnpfc_spatial_extrapolation_left_offset +	(2)
		nnpfc_spatial_extrapolation_right_offset )

When TemporalExtrapolation is equal to 1, the following applies:

- The parameter subpicWidth is set equal to nnpfcOutputPicWidth

- nnpfcOutputPicWidth is updated as follows:

nnpfcOutputPicWidth *= subpicPerRow			(3)

Complete equations (77) in semantics of pfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 as follows.

…

The variable nnpfcOutputPicHeight, representing the height of the luma sample arrays of the NNPF-generated pictures, is derived as follows:

nnpfcOutputPicHeight = Ceil( CroppedHeight *		(4)
		( nnpfc_pic_height_num_minus1 + 1 ) ÷ ( nnpfc_pic_height_denom_minus1 + 1 ) )

When SpatialExtrapolation is equal to 1, nnpfcOutputPicHeight is updated as follows:

nnpfcOutputPicHeight += outSubHeightC * ( nnpfc_spatial_extrapolation_top_offset +	(5)
		nnpfc_spatial_extrapolation_bottom_offset )

When TemporalExtrapolation is equal to 1, the following applies:

- The parameter subpicHeight is set equal to nnpfcOutputPicHeight

- nnpfcOutputPicHeight is updated as follows:

nnpfcOutputPicHeight *= subpicPerCol			(6)

Change equations (88) and (89) in semantics of nnpfc_extended_patch_height_cd_delta_minus1 as follows.

outPatchWidth = ( ( nnpfcOutputPicWidth / subpicPerRow ) * inpPatchWidth ) / CroppedWidth	(7)

outPatchHeight = ( ( nnpfcOutputPicHeight / subpicPerCol ) * inpPatchHeight ) / CroppedHeight	(8)

…

It is a requirement of bitstream conformance that outPatchWidth * CroppedWidth shall be equal to ( nnpfcOutputPicWidth / subpicPerRow ) * inpPatchWidth and outPatchHeight * CroppedHeight shall be equal to ( nnpfcOutputPicHeight / subpicPerCol) * inpPatchHeight.

Add the following text after the definition of StoreOutputTensors( ):

When TemporalExtrapolationFlag is true, the process AccessOutputVersion( ), for deriving sample values in the sample arrays FilteredYSubpic, FilteredCbSubpic, and FilteredCrSubpic, for a subpicture of index subpicIdx in the range 0..nnpfc_extrapolated_subpics_minus1, from the sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, is specified as follows:

- idxX is set equal to subpicIdx / subpicPerRow

- idxY is set equal to subpicIdx % subpicPerRow

for( i = 0; i < numPicsInOutputTensor; i++ ) {
	if( nnpfc_out_order_idc  = =  0 | | nnpfc_out_order_idc  = =  2 | | nnpfc_out_order_idc  = =  3 )
		for( y = 0; y < subpicHeight; y++)
			for( x = 0; x < subpicWidth; x++ ) {
				yY = idxY * subpicHeight + y
				xY = idxX * subpicWidth + x	
				FilteredYSubpic[ i ][ x ][y ] = FilteredYPic[ i ][ xY ][yY ]
			}
	else if( nnpfc_out_order_idc  = =  1 | | nnpfc_out_order_idc  = =  2 | | nnpfc_out_order_idc  = =  3  )
		for( y = 0; y < subpicHeight / outSubHeightC; y++)
			for( x = 0; x < subpicWidth / outSubHeightC; x++ ) {
				yY = idxY * (subpicHeight / outSubHeightC)   + y
				xY = idxX * (subpicWidth / outSubWidthC) + x	
				FilteredCbSubpic[ i ][ x ][y ] = FilteredCbPic[ i ][ xY ][yY ]
				FilteredCrSubpic[ i ][ x ][y ] = FilteredCrPic[ i ][ xY ][yY ]
			}
}

Note: the parameter subpicIdx is provided by external means not specified in this document.

## Aspect2: signal informative text prompt for each subpicture

It is further optionally proposed to insert text prompts giving information about each subpicture. This data is informative. It is not used as input of the NNPF.

The following syntax changes, highlighted in green, are proposed (in 8.28.2.1).

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| … | u(1) |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| … |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) { |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| nnpfc_extrapolated_subpics_minus1 | u(4) |
| nnpfc_extrapolated_subpics_per_row_minus1 | u(4) |
| nnpfc_extrapolated_subpics_prompt_present_flag | u(1) |
| if (nnpfc_extrapolated_subpics_prompt_present_flag) { |  |
| for (i=0; i<= nnpfc_extrapolated_subpics_minus1; i++) |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_extrapolated_subpics_alignment_zero_bit_c | u(1) |
| nnpfc_extrapolated_subpics_prompt[ i ] | st(v) |
| } |  |
| } |  |
| … |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

The following semantics changes are proposed in 8.28.2.2.

After the definition of nnpfc_extrapolated_pics_minus1:

nnpfc_extrapolated_subpics_minus1 plus one indicates the number of alternative temporal extrapolation versions generated in the output tensor outputTensor.

nnpfc_extrapolated_subpics_per_row_minus1 plus one indicates the number of alternative temporal extrapolation versions per row in the output tensor outputTensor. When not present, nnpfc_extrapolated_subpics_per_row_minus1 is inferred to be equal to 0.

The variables subpicPerRow and subpicPerCol, representing respectively the number of subpictures per row and per column present in the output tensor of the NNPF, are derived as follows:

subpicPerRow = (nnpfc_extrapolated_subpics_per_row_minus1 + 1)

subpicPerCol = (nnpfc_extrapolated_subpics_minus1 + 1 + subpicPerRow – 1) / subpicPerRow

nnpfc_extrapolated_subpics_prompt_present_flag equal to 1 specifies that nnpfc_extrapolated_subpics_prompt[ i ] syntax elements are present and nnpfc_extrapolated_subpics_alignment_zero_bit_c syntax element may be present. nnpfc_extrapolated_subpics_prompt_present_flag equal to 0 specifies that nnpfc_extrapolated_subpics_prompt[ i ] syntax elements and nnpfc_extrapolated_subpics_alignment_zero_bit_c syntax element are not present.

nnpfc_extrapolated_subpics_alignment_zero_bit_c shall be equal to 0.

nnpfc_extrapolated_subpics_prompt[ i ] specifies the text string prompt assigned to the ith version of the temporal extrapolation image. When nnpfc_extrapolated_subpics_prompt_present_flag is equal to 1, nnpfc_extrapolated_subpics_prompt[ i ] shall not be a null string.

# References

[Lawlor2016] Pat Lawlor, Keeping the virtual world stable in VR,  2016, https://www.qualcomm.com/news/onq/2016/06/keeping-virtual-world-stable-vr

[Dixit2023] A. Dixit and S. R. Sarangi, Minimizing the Motion-to-Photon-delay (MPD) in Virtual Reality Systems, 2023, https://arxiv.org/pdf/2301.10408

[Yang2019] Minxia Yang, Jiaqi Zhang, and Lu Yu, Perceptual Tolerance to Motion-To-Photon Latency with Head Movement in Virtual Reality, PCS, Nov. 2019, Ningbo, China, https://ieeexplore.ieee.org/abstract/document/8954518

[Liu2022] S. Liu, X. Xu, and M. Claypool. A survey and taxonomy of latency compensation techniques for network computer games. ACM Computing Surveys, Sept. 2022. http://web.cs.wpi.edu/~claypool/papers/lag-taxonomy/paper.pdf

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).