| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0066-v2 |
| --- | --- |

| Title: | EE1-1.4: Reduced complexity input feature extraction for LOP & VLOP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy, Yun Li, Marta Karczewicz | Tel: Email: | dmytror@qti.qualcomm.com yli30@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In JVET-AI0150, a new VLOP2 filter based on LOP3 architecture was adopted to NNVC. In a related contribution JVET-AI0204, it was proposed to modify VLOP2 architecture by removal of the DCT 2x2 transform over the sparse input data and input data volume and futher reduce number of sparse data inputs by combining them. In this document simulation results for EE1-1.4.1 are reported with only DCT transform is being removed from sparse input data which results in 25% input data volume reduction between VTM and NN model.

Proposed architecture was trained with NNVC training guidelines for VLOP and tested with the NNVC10.0 inference, modified to accommodate introduce changes in input data.

The performance of resulting VLOP models with NNIntra enabled is compared to the respective VLOP anchors. Summary results for EE1-1.4.1 VLOP are shown below:

RA  	-0.1% (Y), 1.3% (U), 1.0% (V), 
AI 		-0.1% (Y), 1.4% (U), 1.1% (V), 
LDB	-0.1% (Y), 0.4% (U), 0.3% (V),

It is proposed to adopt method EE1-1.4.1 for NNVC and continue study on EE1-1.4.2 in the next round of EE1.

- Proposed model for VLOP

In EE1-1.4 complexity reduction of the input feature extraction of VLOP2 filter is tested. It is proposed to remove application of the DCT transform on the sparse input data types, such as IPB and BS, as well as reduce number of BS input planes from 6 to 3 and IPB planes from 4 to 1. This method is tested in EE1-1.4.2 and achieves around 25% input data volume reduction comparing to VLOP2 method.

Additionally, input data volume is further reduced by combing IBP and BS data into a single input as well as combining SliceQP and BaseQP. This method is tested in EE1-1.4. and achieves around 40% of input volume reduction, comparing to the VLOP2 filter.

Block diagram of the proposed filter is shown in Figure 2, with blocks affected in this proposal marked in red.

Prepr

Fig. 2. Tested filter architecture in EE1-1.4.1 and EE1-1.4.2.

- Training and inference information

Training of the model is performed over LOP2 Stage 3 dataset and follows the NNVC VLOP3 training hyperparameters.

| Table 1. Network Information for the model in EE1-1.4.1 in training. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU: NVIDIA A100-PCIE-40GB |
|  | Framework: | PyTorch v2.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | Stage3: 60 |
|  | Batch size: | Stage3: 64 |
|  | Loss function: | L1 and MSE |
|  | Training time: | Stage3: 12 days |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | The LOP2 stage 3 dataset |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | Stage3: 2e-4, milestones [41, 51, 55] |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

| Table 2. Network Information for the model in inference. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 59625 |
|  | Parameter Precision (Bits) | int16 |
|  | Memory Parameter (MB) | fp32: 0.25MB, int16: 0.13MB |
|  | Multiplay Accumulate (MAC) | 5.16 kMAC/pixel |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

- Results

The proposed method was integrated to the NNVC-10.0 and tested under NNVC CTC with SADL int16. The summary of results in comparison to the respective VLOP anchor is shown in Table 1.

Table 1. BD-rate performance of the EE1-1.4.1 VLOP, comparing to VLOP anchor.

|  | AI |  |  | RA |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.10% | 1.51% | 1.11% | -0.14% | 1.18% | 0.54% |  |  |  |
| Class A2 | -0.06% | 1.52% | 0.89% | -0.06% | 1.59% | 0.92% |  |  |  |
| Class B | -0.08% | 1.24% | 1.08% | -0.15% | 1.47% | 1.14% | -0.12% | 1.78% | 0.55% |
| Class C | -0.10% | 0.85% | 1.12% | -0.11% | 0.93% | 1.08% | -0.02% | 0.51% | 1.00% |
| Class E | -0.08% | 1.91% | 1.05% |  |  |  | -0.04% | -1.89% | -1.09% |
| Overall | -0.08% | 1.36% | 1.06% | -0.12% | 1.29% | 0.96% | -0.07% | 0.44% | 0.29% |
| Class D | -0.10% | 0.67% | 1.34% | -0.12% | 0.73% | 1.46% | -0.13% | 0.71% | 1.28% |
| Class F | -0.17% | 0.67% | 0.37% | -0.19% | 0.42% | 0.00% | -0.15% | -1.17% | -1.15% |

The summary of results in comparison to the respective LOP anchor is shown in Table 2 for float and Table 3 for int16 implementation.

Table 2. BD-rate performance of the EE1-1.4.1 LOP (float), comparing to VLOP anchor.

|  | AI |  |  | RA |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |
| Class C | -0.09% | 0.15% | 0.18% | 0.01% | 0.77% | -0.29% | 0.01% | 1.61% | -1.18% |
| Class E | -0.08% | 0.17% | -0.30% |  |  |  | -0.21% | 1.03% | -4.26% |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | -0.17% | -0.47% | -0.34% | -0.07% | -0.12% | -0.34% | -0.07% | 0.67% | -0.01% |
| Class F |  |  |  |  |  |  |  |  |  |

Table 3. BD-rate performance of the EE1-1.4.1 LOP (int16), comparing to VLOP anchor.

|  | AI |  |  | RA |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0.01% | 1.02% | 0.93% | 0.09% | 1.19% | 0.81% |  |  |  |
| Class A2 | 0.09% | 0.30% | 0.38% | 0.08% | 0.39% | 0.21% |  |  |  |
| Class B | -0.01% | 0.41% | 0.41% | 0.02% | 0.90% | 0.30% | 0.01% | -0.49% | 1.48% |
| Class C | 0.09% | 0.69% | 0.69% | -0.05% | 0.52% | 0.49% | -0.01% | 0.22% | 1.37% |
| Class E | 0.05% | 0.60% | 0.55% |  |  |  | 0.07% | 1.76% | 0.50% |
| Overall | 0.04% | 0.59% | 0.58% | 0.03% | 0.75% | 0.43% | 0.02% | 0.31% | 1.20% |
| Class D | 0.04% | 0.95% | 1.09% | -0.08% | 0.45% | 0.79% | 0.04% | -0.04% | 6.56% |
| Class F | 0.04% | 0.93% | 1.15% | 0.05% | 0.42% | 0.64% | 0.14% | 0.71% | -0.68% |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).