| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0075-v1 |
| --- | --- |

| Title: | AHG9: On the constituent rectangles SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang 8910 University Center Ln San Diego, CA92122, USA | Email: | xujizheng@bytedance.com yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes for the constituent rectangles SEI message:

- Signal cr_rect_id_len_minus1 instead of cr_rect_id_len.

- Add a value range for each of the ue(v)-coded syntax elements cr_num_cols_minus1 and cr_num_rows_minus1.

- Add a constraint to require that rows times cols shall be equal to the total number of constituent rectangles.

- Enable signalling ID for EMPTY constituent rectangles.

- Define primary rectangle.

- Disable an EMPTY rectangle being associated.

- Add sensible constraints on crRectWidth and crRectHeight.

- Various editorial improvements

The detailed proposed text changes, marked relative to JVET-AI2032-v3, are included in an attachment to this contribution.

# Explanation and proposed text changes

The following description and text changes are based on the document JVET-AI2032-v3.docx. For the text changes, relevant parts that have been added or modified are highlighted in turquoise, and some of the deleted parts are highlighted in yellow strikethrough in red fonts.

Item 1)

cr_rect_id_len specified length of the cr_rect_id[ i ],  cr_associated_rect_id [ i ], and cr_rect_group_id[ i ] syntax elements. syntax elements. Its value is always greater than 0. Thus, signalling the value minus 1 makes more sense.

The proposed text changes for item 1) are as below:

cr_rect_id_len_minus1 plus 1 specifies the length, in bits, of the cr_rect_id[ i ],  cr_associated_rect_id[ i ], and cr_rect_group_id[ i ] syntax elements. syntax elements.

Item 2)

The syntax elements cr_num_cols_minus1 and cr_num_rows_minus1 are coded in ue(v). A range is missing.

Item 3)

The values of cr_num_cols_minus1 and cr_num_rows_minus1 may be contradict with the value of cr_num_rects_minus1. A constraint should be added to avoid such a confliction.

The proposed text changes for items 2) and 3) are as below:

cr_num_cols_minus1 plus 1 and cr_num_rows_minus1 plus 1 specify the number of columns and rows, respectively, of the constituent rectangle grid when cr_rect_same_size_flag equal to 1. The values of cr_num_cols_minus1 and cr_num_rows_minus1 shall be in the range of 0 to 4095, inclusive.

The variable crNumCols is set equal to cr_num_cols_minus1 + 1.

The variable crNumRows is set equal to cr_num_rows_minus1 + 1.

It is a requirement of bitstream conformance that when cr_rect_same_size_flag is present and equal to 1, crNumCols * crNumRows shall be equal to cr_num_rects_minus1 + 1.

Item 4)

In the current text, for an EMPTY constituent rectangle, signalling ID is disabled. However, an EMPTY constituent rectangle has an ID, which is inferred. It is proposed that ID should be allowed to signal regardless of the rectangle type.

The proposed text changes for item 4) are as below:

| constituent_rectangles( payloadSize ) { |  |
| --- | --- |
| ... |  |
| if ( cr_rect type_idc[ i ] != 255 ) { |  |
| if ( cr_rect id_enabled_flag ) { |  |
| cr_rect_type_present_flag[ i ] | u(1) |
| if ( cr_rect id_present_flag ) |  |
| cr_rect_id[ i ] | u(v) |
| } |  |
| if ( cr_rect type_idc[ i ] != 255 ) { |  |
| if ( cr_associated_rect id_enabled_flag ) { |  |
| cr_associated_rect_id_present_flag[ i ] | u(1) |
| if ( cr_associated_rect_id_present_flag[ i ] ) |  |
| cr_associated_rect_id[ i ] | u(v) |
| } |  |
| if ( cr_group_id_present_flag ) { |  |
| cr_rect_group_id[ i ] | u(v) |
| if ( cr_rect type_description_enabled_flag ) |  |
| cr_rect_type_description_present_flag[ i ] | u(1) |
| } |  |
| ... |  |

Item 5)

In the current text, cr_associated_rect_id can be signalled to indicate the ID of an associated primary rectangle. However, there is no definition of primary rectangle.

Item 6)

In the current text, the associated primary rectangle may be an EMPTY constituent rectangle, which does not make sense.

The proposed text changes for items 5) and 6) are as below:

cr_associated_rect_id[ i ] indicates the ID of an associated primary rectangle of the i-th rectangle. The length of the syntax element is cr_rect_id_len bits.

The value of cr_rect_type_idc of a primary rectangle shall be equal to 0.

It is a requirement of bitstream conformance that when present, the value of cr_associated_rect_id[ i ] shall be equal to cr_rect_id[ j ] for some at least one value of j in the range of 0 .. to cr_num_rects_minus1, inclusive, and cr_rect_type_idc[ j ] shall not be equal to 255.

Item 7)

In the current text, certain crRectWidth and crRectHeight values may make the constituent rectangle out of the picture range. It is proposed to add sensible constraints.

The proposed text changes for item 7) are as below:

It is a requirement of bitstream conformance that crRectWidth[ i ] and crRectHeight[ i ] shall both be greater than 0.

It is a requirement of bitstream conformance that crRectTopLeftX[ i ] + crRectWidth[ i ] shall be less than or equal to MaxPicWidth, and crRectTopLeftY[ i ] + crRectHeight[ i ] shall be less than or equal to MaxPicHeight.

Item 8)

Some editorial improvements, e.g. typos fixed, are also proposed. The enclosed attachment document includes all changes for items 1) to 8).

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).