| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0086-v2 |
| --- | --- |

| Title: | Non-EE2: LFNST/NSPT set derivation for CCP coded block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hang Huang, Yue Yu, Haoping Yu, Dong Wang | Tel: Email: | huanghang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes a new method to derive the transform set for intra chroma blocks coded with LFNST/NSPT and CCP mode. First, as aspect 1, it is proposed to use DIMD to derive a traditional intra prediction mode based on the prediction samples, and  the LFNST/NSPT transform set is then determined according to the derived intra mode. To further improve the performance, it is proposed, as aspect 2, that one of the two alternative LFNST/NSPT transform sets is signaled in the bit-stream to select the best transform set.

It is reported that the coding performance on top of ECM 14.0 is as below:

Aspect 1: AI(Y/U/V): -0.01%/-0.29%/-0.30%, EncT 100.5%, DecT 99.9%

Aspect 2: AI(Y/U/V): 0.01%/-0.58%/-0.58%, EncT 102.8%, DecT 100.6%

Introduction

For chroma coding, a flag is first signalled to indicate whether CCP prediction or non-CCP prediction is used. Cross-component prediction (CCP), including cross-component linear model (CCLM), convolutional cross-component model (CCCM), gradient linear model (GLM), and their variants, is adopted in ECM to exploit the cross-component correlation.

LFNST (low frequency non-separable transform) is a transform coding tool in VVC. In ECM, LFNST is further enhanced with more transform sets for a finer granularity of directions, i.e., from 4 to 35, and more transform kernels in each set, i.e., from 2 to 3. The transform set idx lfnstTrSetIdx is defined according to the predModeIntra list in Table 1.

Table 1 LFNST transform set selection

Non-separable primary transform (NSPT) is introduced to replace the separable DCT-II plus LFNST transform combinations for the block shapes of 4x4, 4x8/8x4, 4x16/16x4, 8x8, 8x16/16x8, 4x32/32x4, and 8x32/32x8 in ECM. The kernel mapping method for intra prediction mode is the same as LFNST.

The LFNST/NSPT transform set is implicitly decided by an intra prediction mode (IPM). For a block coded with PLANAR, DC, or directional modes, the IPM is set as the intra prediction mode itself. For a chroma block coded with CCP, the IPM is set as the intra prediction mode of the collocated luma block.

However, the prediction strategies of CCP are different from normal intra prediction modes like PLANAR, DC, and directional modes. The prediction residual may illustrate different features from the derived IPM.

Proposed methods

In this contribution, a new method to derive the transform set for intra chroma blocks coded with LFNST/NSPT and CCP mode is proposed to replace the original method.

Aspect 1: it is first proposed to use the DIMD to derive the intra prediction mode of the current block based on the CCP predicted samples. Specifically, a horizontal gradient and a vertical gradient are calculated for each predicted sample to build an HoG, as shown in Figure 1. Then the intra prediction mode with the largest histogram amplitude values is used to determine the LFNST/NSPT transform set.

Figure 1 CCP prediction samples to build HOG

Aspect 2: it is further proposed to use DIMD to derive 3 different intra prediction modes for the current block based on the CCP predicted samples and to use the DM to derive 1 intra prediction mode of the current block based on the collocated luma block. Two candidate IPMs are selected from those 4 intra prediction modes to ensure that the resulting transform sets are different. One bin is employed to indicate whether the 1st or 2nd candidate IPM is used to derive the LFNST/NSPT transform set.

Simulation results

The proposed method is implemented on top of the ECM-14.0 software [1] and tested based on the common test condition [2]. The test results are summarized below.

Test 1: Aspect 1 -- using one derived mode to decide the transform set

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.26% | -0.35% | 100.7% | 99.7% | 99.0% | 100.0% |
| Class A2 | 0.00% | -0.35% | -0.42% | 101.0% | 100.2% | 99.9% | 100.0% |
| Class B | -0.01% | -0.38% | -0.33% | 100.9% | 100.1% | 100.3% | 99.9% |
| Class C | 0.00% | -0.32% | -0.28% | 99.6% | 99.3% | 101.3% | 100.0% |
| Class E | -0.02% | -0.09% | -0.09% | 100.2% | 100.1% | 101.1% | 99.9% |
| Overall | -0.01% | -0.29% | -0.30% | 100.5% | 99.9% | 100.4% | 100.0% |
| Class D | 0.00% | 0.01% | -0.29% | 99.8% | 97.2% | 100.2% | 100.0% |
| Class F | -0.03% | -0.20% | -0.19% | 99.7% | 100.7% | 99.2% | 100.0% |

Test 2: Aspect 2 -- using one of the two candidate modes to decide the transform set

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.94% | -0.69% | 103.1% | 100.5% | 100.4% | 100.0% |
| Class A2 | 0.06% | -0.76% | -0.84% | 104.1% | 100.2% | 100.1% | 100.0% |
| Class B | 0.00% | -0.57% | -0.63% | 102.3% | 100.5% | 101.2% | 99.9% |
| Class C | 0.02% | -0.39% | -0.49% | 102.9% | 101.3% | 100.3% | 100.0% |
| Class E | -0.01% | -0.31% | -0.24% | 101.9% | 100.2% | 100.8% | 99.9% |
| Overall | 0.01% | -0.58% | -0.58% | 102.8% | 100.6% | 100.6% | 100.0% |
| Class D | 0.00% | -0.30% | -0.41% | 103.6% | 100.2% | 100.0% | 100.0% |
| Class F | -0.03% | -0.44% | -0.28% | 101.6% | 99.6% | 99.2% | 100.0% |

Conclusions

This contribution proposes a new method to derive the transform set for intra chroma blocks coded with LFNST/NSPT and CCP mode. The proposed first aspect brings around -0.30% chroma coding gain and the second aspect brings around -0.58% chroma coding gain under the CTC AI configuration. It is recommended to investigate the proposed method in EE2.

References

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AI2017, July, 2024.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).