| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0098-v3 |
| --- | --- |

| Title: | Non-EE2: OBIC with improved sampling strategy and scaling weights |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yutian Liu, Yongkai Huo(Transsion) Gang Li, Junjie Chen, Lei Luo, Hongwei Guo, Ce Zhu(UESTC) | Tel: Email: | yutian.liu@transsion.com yongkai.huo@transsion.com 202222011930@std.uestc.edu.cn 202221010107@std.uestc.edu.cn luolei1010@uestc.edu.cn hwguo@uestc.edu.cn eczhu@uestc.edu.cn |
| Source: | Transsion Holdings, University of Electronic Science and Technology of China |  |  |

_____________________________

- Abstract

This contribution proposes an improved sampling strategy for the occurrence-based intra coding (OBIC) mode. In OBIC mode, the intra prediction mode of a block is determined according to the occurrence of the intra modes in the block’s spatial neighborhood. The proposed method utilizes a sampling strategy to expand the number of adjacent spatial neighborhood blocks and non-adjacent spatial neighborhood blocks. Furthermore, the area-related amplitudes are scaled with different weights before the cumulative sum of occurrence values in the Histogram of Occurrence.

The performance of the proposed method over ECM-14.0 is reported as follows {for Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}:

AI { -0.02%, 0.03%, 0.04%, 102.8%, 101.8%, 99.6%, 100.0%}

RA { xxx%, xxx%, xxx%, xxx%, xxx%, xxx%, xxx%}

Introduction

In ECM 14.0, when using OBIC, the spatial neighboring blocks of the current block are identified using 13 adjacent and 18 non-adjacent sampling points. Area-related amplitudes are then calculated based on these neighboring blocks. Specifically, if a fusion method is not involved in the prediction process of a spatial neighboring block a single area-related amplitude is generated. Otherwise, a fusion method is used for spatial neighboring blocks such as DIMD, TIMD, SPGM, or OBIC block, it may produce multiple area-related amplitudes corresponding to different angular modes. Finally, an OBIC histogram is constructed from these area-related amplitudes, and up to five angular modes with the highest occurrence, along with the plane mode or block vector-based mode, are selected and used for the final prediction of the current block.

Proposal

Aspect #1. Sampling strategy of spatial neighborhood blocks

In this aspect, we propose to improve the current sampling strategy of spatial neighborhood blocks  (i.e. adjacent spatial neighborhood blocks and non-adjacent spatial neighborhood blocks) in OBIC. Specifically, for adjacent sampling points, we extend the number from 13 to 21, which are selected from the top area, left area, and top-left area of the current block for locating the adjacent spatial neighborhood blocks. These 21 sampling points are exemplified in Figure 1. The size of top area, left area and top-left area is derived based on the size of the current block. Furthermore, similar to [2], we extend the number from 18 to 48 for non-adjacent sampling points as shown in Figure 2.

Figure 1. Adjacent sampling strategy

Figure 2. Non-adjacent sampling strategy

Aspect #2. Weights for area-related amplitudes

In ECM14.0, area-related amplitudes are accumulated directly to obtain Histogram of Occurrences. This aspect proposes two weighting methods for scaling amplitudes. In the first method, weights are applied on all the original area-related amplitudes. The weights are selected from {4,2,1,0}, depending on the current block size and the distance of the sampling points from the current block. In the second method, the weights are applied on area-related amplitudes derived from DIMD, TIMD, SGPM, or OBIC blocks by individually counting their IPM mode number and divding accordingly. The details of these weights are shown in Table 1.

Table 1. weights for DIMD, TIMD, SGPM, or OBIC blocks

| blocks | minimum value of weight |
| --- | --- |
| DIMD | 1/6 |
| TIMD | 1/3 |
| SGPM | 1/2 |
| OBIC | 1/6 |

Experimental results

The proposed method is implemented on top of the ECM-14.0 software [3] and tested with JVET ECM common test conditions [4] in AI configurations. The following results are obtained:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.04% | -0.09% | 103.4% | 102.2% | 99.9% | 100.0% |
| Class A2 | -0.01% | -0.06% | 0.02% | 103.0% | 102.2% | 100.0% | 100.0% |
| Class B | -0.02% | 0.06% | 0.12% | 102.6% | 101.4% | 97.9% | 100.0% |
| Class C | -0.01% | 0.05% | -0.10% | 102.4% | 102.1% | 99.6% | 100.0% |
| Class E | -0.03% | 0.12% | 0.27% | 103.1% | 101.6% | 102.1% | 100.0% |
| Overall | -0.02% | 0.03% | 0.04% | 102.8% | 101.8% | 99.6% | 100.0% |
| Class D | -0.01% | -0.01% | -0.10% | 102.7% | 101.9% | 99.9% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

Conclusion

This contribution proposes to expand the sampling strategy of spatial neighborhood blocks. The proposed method brings some coding gains. It is proposed to study this method in EE2.

- References

[1] R. G. Youvalari, M. Abdoli, A. Tissie, “EE2-2.2: Occurrence-based intra coding (OBIC)”, JVET-AH0076, April, 2024.

[2] J. Fu, Z. Li, A. J. Zhang et. al., “Non-EE2: Improved OBIC with PDP”, JVET-AI0079, July, 2024.

- [3] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0?ref_type=tags

- [4] M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing,", JVET-AF2017, 32nd Meeting, Hannover, Oct. 2023.

Patent rights declaration(s)

Shenzhen Transsion Holdings Co., Ltd. (Transsion) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

University of Electronic Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).