| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0109-v1 |
| --- | --- |

| Title: | AHG9: On AI usage restrictions SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Tan Junghak Nam Jangwon Lee Jaehyun Lim Seung hwan Kim | Email: | chulkeun.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes the following modifications that are asserted to improve the signaling and semantics of the AI usage restrictions SEI.

- Clarify semantics when no context is present.

- Defining additional restrictions in aur_restriction

- Defining constraints to avoid redundancy in aur_restriction signaling

Introduction

The current Technologies under Consideration for future extensions of VSEI (JVET-AI2032) includes AUR (AI usage restrictions) SEI message that signals restriction and optional context information for usage by AI applications. This contribution proposes several modifications to improve some aspects of the AUR SEI message. The parts of the syntax and semantics that are relevant for this contribution are shown below.

| ai_usage_restrictions ( payloadSize ) { | Descriptor |
| --- | --- |
| aur_cancel_flag | u(1) |
| if( !aur_cancel_flag ) { |  |
| aur_persistence_flag | u(1) |
| aur_num_restrictions_minus1 | ue(v) |
| for( i = 0; i <= aur_num_restrictions_minus1; i++ ) { |  |
| aur_restriction[ i ] | ue(v) |
| aur_context_present_flag[ i ] | u(1) |
| if( aur_context_present_flag[ i ] ) |  |
| aur_context[ i ] | ue(v) |
| } |  |
| } |  |

The artificial intelligence (AI) usage restrictions SEI message signals restriction and optional context information for usage by AI applications.

aur_cancel_flag equal to 1 specifies that the SEI message cancels the persistence of any previous AI usage restrictions SEI message in output order. aur_cancel_flag equal to 0 specifies that AI usage restriction information follows.

aur_persistence_flag specifies the persistence of the AI usage restrictions SEI message for the current layer.

aur_persistence_flag equal to 0 specifies that the AI usage restrictions SEI message applies to the current decoded picture only.

aur_persistence_flag equal to 1 specifies that the AI usage restrictions SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an AI usage restrictions SEI message is output that follows the current picture in output order.

aur_num_restrictions_minus1 plus one specifies the number of restriction entries that follow.

aur_restriction[ i ] specifies the i-th restriction as specified in Table xx. The value of aur_restriction[ i ] shall be in the range of 0 to 2, inclusive. Values greater than 2 for aur_restriction[ i ] are reserved for future use by ITU-T | ISO/IEC and shall be ignored in bitstreams conforming to this version of this Specification.

Table xx – Definition of aur_restriction [ i ]

| Value | Interpretation |
| --- | --- |
| 0 | Do not use for AI training |
| 1 | Do not use for generative (modification or creation) AI |
| 2 | Do not use in any AI application |

aur_context_present_flag [ i ] indicates the presence of context information for the value in aur_restriction[ i ].

aur_context[ i ], when present, indicates the context for the i-th aur_restriction[ i ] as specified in Table xy, where (aur_context & bitMask ) not equal to zero indicates that the i-th restriction applies to the context associated with the bitMask value in Table xy. When aur_context[ i ] is greater than 0 and ( aur_context[ i ] & bitMask ) is equal to zero, the application of the i-th restriction is undefined in the context associated with the bitMask value in Table xy. The value of aur_context[ i ] shall be in the range of 1 to 15, inclusive, in bitstream conforming to this version of this Specification. Values greater than 15 are reserved for future use by ITU-T | ISO/IEC and shall be ignored in bitstreams conforming to this version of this Specification. When the value of aur_context[ i ] is greater than 15 decoders conforming to this version of this Specification shall ignore aur_context[ i ]. aur_context[ i ] shall not be equal to 0.

Table xy – Definition of aur_context[ i ]

| Bitmask | Interpretation |
| --- | --- |
| 0x0001 | commercial use |
| 0x0002 | non-commercial use |
| 0x0004 | official government use |
| 0x0008 | research and academic use |

Item 1: Clarify semantics when no context is present

Problem statement

In the current semantics of aur_context_present_flag, it only defines its purpose, as shown below, and does not clearly specify the behavior when the value is 0 or 1. Additionally, it is not clear what it really means when context is not present for a restriction.

aur_context_present_flag [ i ] indicates the presence of context information for the value in aur_restriction[ i ].

Proposed modification

We propose to update the semantics of aur_context_present_flag[ i ] to be clearer about what it means when its value is equal to 0 or 1. Furthermore, we also propose to clarify that when no context is present for a particular restriction, it means the restriction applies to all, regardless of any context.

The following modification to aur_context_present_flag is proposed. Text that are added are indicated by yellow highlight. Text that are removed are indicated by red, strickenthrough font.

aur_context_present_flag [ i ] indicates the presence of context information for the value in aur_restriction[ i ].

aur_context_present_flag[ i ] equal to 1 specifies that the context information for aur_restriction[ i ] is present. aur_context_present_flag[ i ] equal to 0 specifies that no context information is present for aur_restriction[ i ]. When aur_context_present_flag[ i ] is equal to 0, aur_restriction [ i ] applies regardless of the context.

Item 2: Defining additional restrictions in aur_restriction

Problem statement

Currently, aur_restriction defines three restrictions, as shown in Table xx. Value 0 represents "Do not use for AI training," value 1 represents "Do not use for generative (modification or creation) AI," and value 2 represents "Do not use in any AI application." Value 2 is a broad restriction. Excluding this, there are only two specific restrictions defined, which are asserted to be insufficient to fully reflect the diverse AI use cases. As AI technology evolves rapidly and its use cases become increasingly diverse, the current restrictions may not adequately cover more specific scenarios.

Table xx – Definition of aur_restriction [ i ]

| Value | Interpretation |
| --- | --- |
| 0 | Do not use for AI training |
| 1 | Do not use for generative (modification or creation) AI |
| 2 | Do not use in any AI application |

Proposed modification

As AI advances rapidly, defining all restrictions in detail is challenging. However, for common use cases, more specific restrictions can be defined. This approach remains effective despite the fast pace of AI development. Using this approach, AI's use of pictures can be categorized into the following phases:

The use of pictures in AI can be broadly divided into two stages: development and deployment.

AI model development typically consists of two steps: training, where AI learns from data, and validation, where the model’s accuracy is assessed.

AI deployment can be categorized into Discriminative AI and Generative AI. Discriminative AI focuses on tasks like classification and prediction, while Generative AI modifies or creates new content. The outputs of Generative AI typically involve modification, extraction, or content generation.

Figure 1 illustrates the categorized AI's use of pictures and the corresponding restrictions. The table below presents the proposed definition of aur_restriction[ i ], reflecting this categorization. Text that are added are indicated by yellow highlight.

Figure 1. Categorized AI's use of pictures and the corresponding restriction

Table xx – Definition of aur_restriction [ i ]

| Value | Interpretation |
| --- | --- |
| 0 | Do not use in any AI application |
| 1 | Do not use the picture as training data for AI models. |
| 2 | Do not use the picture for validating AI models (e.g., performance evaluation, testing). The picture must not be used to assess or test the accuracy of AI models. |
| 3 | Do not use the picture for AI inference (e.g., object detection, object tracking). Analysis for real-world applications is restricted. |
| 4 | Do not modify the picture. Modification refers to altering existing elements without adding new element. (e.g., object removal, rotation, flipping). Quality enhancements are not considered structural changes |
| 5 | Do not extract parts of the picture. Extraction refers to separating or isolating parts of the picture or video without altering the remaining content. (e.g., frame-level extraction, object cropping, sub-clip creation). |
| 6 | Do not generate entire or partial new picture (e.g., merging multiple sources, AI-generated synthetic media like deepfakes). |

Item 3: Defining constraints to avoid redundancy in aur_restriction signaling

Problem statement

In the current table of restrictions defined by AUR SEI message, some restrictions may be superset of others, meaning that if a broad restriction like 'Do not use in any AI application' is applied, more specific restrictions, such as those for AI training or generative AI, are automatically covered. This makes signaling the subset restrictions unnecessary and potentially inefficient, as the broader restriction already includes them.

Proposed modification

We propose to prevent signaling redundancy in the following two cases through the definition of the following constraints:

- If the restriction "do not use in any AI application" is applied regardless of context, signaling the application of other restrictions in any context would be redundant.

- If the restriction "do not use in any AI application" is applied to a specific context, signaling the application of other restrictions for that context would be redundant.

The proposed restrictions are as follows:

When aur_restriction[i] is equal to 2 (i.e., Do not use in any AI application) and aur_context_present_flag[ i ] is equal to 0, then i shall be equal to 0 and aur_num_restrictions_minus1 shall be equal to 0.

When aur_restriction[ i ] is equal to 2, aur_context_present_fag[ i ] is equal to 1, and (aur_context[ i ] & bitMaskX ) is not equal to 0 for any bitMaskX is included in the bitMask table xy , the following applies:

- There shall be no j where aur_restriction[ j ] is not equal to 2 and j is not equal to i, and (aur_context[ j ] & bitMaskX ) is not equal to 0.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).