| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0113-v2 |
| --- | --- |

| Title: | Non-EE2: Intra OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Ru-Ling Liao Jie Chen Yan Ye | Tel: Email: | {sid.lxw, ruling.lrl, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In ECM-14.0, overlapped block motion compensation (OBMC) is employed to mitigate boundary discontinuities between two adjacent inter coded blocks. In EE2 Test 3.3, OBMC is applied when the current block is an inter coded block and the neighboring block is an intra coded block. This contribution proposes further extending OBMC to two adjacent non-inter coded blocks as well. It is reported that on top of ECM-14.0, the overall coding performance for {Y, U, V, EncT, DecT} is {-0.06%, -0.04%, -0.04%, 101.1%, 102.3%} for AI and {*0.00%, -0.17%, 0.02%, 100.4%, 100.0%} for RA.

Introduction

OBMC [1] is performed on the 4x4 sub-block level for samples on top and left boundaries between adjacent inter coded blocks. For each sub-block on the boundaries, the motion information of adjacent inter block is used to refine the samples in the sub-block. However, OBMC is not applied to the boundaries of non-inter coded blocks.

In EE2 Test 3.3 [2], OBMC is proposed to be applied when the current block is an inter coded block and the neighboring block is an intra coded block. Specifically, boundary samples of the current inter coded block which are adjacent to intra coded blocks are blended with an intra predictor generated by an intra prediction mode derived by applying DIMD method on the neighbouring reconstructed samples.

In this contribution, an intra OBMC scheme is proposed to further extend OBMC to the boundaries between adjacent non-inter coded blocks.

Proposed Method

In order to addresses the discontinues between two non-inter coded blocks, the intra OBMC scheme is proposed.

The intra OBMC is applied to top and left boundaries of the current block as follows:

- For luma component:

  - Both the current block and the neighboring block are coded by BV modes;

  - The current block is coded by a BV mode and the neighboring block is coded by a non-BV intra mode;

- For chroma components:

  - The current block is coded by a CCP mode and the neighboring block is coded by a non-CCP and non-BV mode;

The BV mode includes IBC mode and intra TMP mode for luma and DBV mode for chroma.

For 4:2:0 color format, the intra OBMC is performed on 4x4 sub-block level for luma and 2x2 sub-block level for chroma. If intra OBMC is applied, each sub-block on the top and left boundaries within the current block is blended with a predictor generated by the information of the neighboring block. If the neighboring block is coded by a BV mode, the BV information (includes BV, fusion parameters, LIC parameters etc.) is used to generate the predictor. If the neighboring block is coded by a non-BV intra mode, an intra prediction mode derived by applying DIMD method on the adjacent reconstructed samples is used to generate the predictor.

The blending weights for the proposed intra OBMC are identical to those used in inter OBMC when template matching-based OBMC is not applied.

The proposed intra OBMC is only applied to camera-captured sequences.

Simulation results

The proposed methods are implemented on top of ECM-14.0 and are tested under CTC for enhanced compression tool testing [3].

Table 1. Simulation results of Test 2.15a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.01% | 0.00% | 100.8% | 102.2% | 100.4% | 100.8% |
| Class A2 | -0.09% | -0.07% | -0.06% | 101.0% | 102.2% | 100.8% | 100.7% |
| Class B | -0.04% | 0.00% | -0.02% | 101.3% | 102.2% | 101.9% | 100.7% |
| Class C | -0.05% | -0.13% | -0.06% | 101.0% | 102.8% | 101.4% | 100.4% |
| Class E | -0.10% | 0.00% | -0.03% | 101.0% | 102.0% | 103.5% | 100.3% |
| Overall | -0.06% | -0.04% | -0.04% | 101.1% | 102.3% | 101.6% | 100.6% |
| Class D | -0.02% | 0.00% | -0.30% | 101.9% | 102.4% | 100.7% | 100.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.26% | -0.13% | 100.7% | 100.1% | 100.4% | 100.8% |
| Class A2 | 0.00% | -0.18% | -0.09% | 100.2% | 100.2% | 100.3% | 100.2% |
| Class B | -0.01% | -0.17% | -0.14% | 100.6% | 100.0% | 100.3% | 100.6% |
| Class C | 0.01% | -0.10% | 0.40% | 99.9% | 99.7% | 100.2% | 100.6% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.17% | 0.02% | 100.4% | 100.0% | 100.3% | 100.5% |
| Class D | 0.02% | -0.34% | -0.13% | 99.7% | 100.0% | 100.5% | 100.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

| All Intra Main 10. Over ECM-14.0. Y; U; V; EncT; DecT; EncVmPeak; DecVmPeak. Class A1; -0.02%; 0.01%; 0.00%; 100.8%; 102.2%; 100.4%; 100.8%. Class A2; -0.09%; -0.07%; -0.06%; 101.0%; 102.2%; 100.8%; 100.7%. Class B; -0.04%; 0.00%; -0.02%; 101.3%; 102.2%; 101.9%; 100.7%. Class C; -0.05%; -0.13%; -0.06%; 101.0%; 102.8%; 101.4%; 100.4%. Class E; -0.10%; 0.00%; -0.03%; 101.0%; 102.0%; 103.5%; 100.3%. Overall; -0.06%; -0.04%; -0.04%; 101.1%; 102.3%; 101.6%; 100.6%. Class D; -0.02%; 0.00%; -0.30%; 101.9%; 102.4%; 100.7%; 100.2%. Class F; #VALUE!; #VALUE!; #VALUE!; #DIV/0!; #DIV/0!; #DIV/0!; #DIV/0!. Class TGM; #VALUE!; #VALUE!; #VALUE!; #DIV/0!; #DIV/0!; #DIV/0!; #DIV/0!. Random Access Main 10. Over ECM-14.0. Y; U; V; EncT; DecT; EncVmPeak; DecVmPeak. Class A1; 0.00%; -0.26%; -0.13%; 100.7%; 100.1%; 100.4%; 100.8%. Class A2; 0.00%; -0.18%; -0.09%; 100.2%; 100.2%; 100.3%; 100.2%. Class B; -0.01%; -0.17%; -0.14%; 100.6%; 100.0%; 100.3%; 100.6%. Class C; 0.01%; -0.10%; 0.40%; 99.9%; 99.7%; 100.2%; 100.6%. Class E. Overall; 0.00%; -0.17%; 0.02%; 100.4%; 100.0%; 100.3%; 100.5%. Class D; 0.02%; -0.34%; -0.13%; 99.7%; 100.0%; 100.5%; 100.2%. Class F; #VALUE!; #VALUE!; #VALUE!; #NUM!; #NUM!; #NUM!; #NUM!. Class TGM; #VALUE!; #VALUE!; #VALUE!; #NUM!; #NUM!; #NUM!; #NUM!. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Conclusion

The contribution proposed an intra OBMC scheme which extends OBMC to non-inter coded blocks. The experimental results show that the proposed method results in the average luma BD-rates changes of -0.06% for AI and 0.00% for RA, respectively. It is suggested to study the proposed method in the next round of EE.

References

- M. Coban, R.-L. Liao, K. Naser, J. Ström and L. Zhang, “Algorithm description of Enhanced Compression Model 14 (ECM 14)”, JVET-AI2025, Oct. 2024.

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu and K. Zhang, “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AI2024, Aug. 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, Aug. 2024.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).