| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0114-v1 |
| --- | --- |

| Title: | AHG9: On signalling input pictures shift in NNPFA SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Tan Jangwon Lee Junghak Nam Chul Keun Kim Jaehyun Lim Seung hwan Kim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the 35th JVET meeting, contribution JVET-AI0072 proposed modification to the signalling design of input pictures for a neural-network post-filter (NNPF) at the activation. The proposed signalling allows decoder to do input picture shift for each original input picture of the target NNPF.

In this contribution, it is asserted that the presented use-cases would be good if they can be supported. However, the solution proposed by JVET-AI0072 has unintended consequence such that it allows gap in between pictures that are used as input for the NNPF. This creates problem with the original design of NNPFC SEI message in the aspect of handling interpolated pictures that are outputted by the filter. The gap created by this solution will mess up with the signalling in NNPFC SEI message that specifies where the newly generated pictures should be located.

To overcome the asserted problem, a simplified version of the solution is proposed by allowing shift for the whole set of input pictures, not for individual input picture. The detail of the proposed solution is as follows:

- Signal the following information in the extension of NNPFA SEI message:

  - A syntax element that specifies the number of input picture shift. This syntax element may be called nnpfa_num_input_pic_shift.

  - The value of nnpfa_num_input_pic_shift may be in the range 0 from 63, inclusive.

  - When nnpfa_num_input_pic_shift is equal to 0, it means that the input pictures specified by the target NNPFC SEI message are used.

- The input pictures for an NNPF are selected in two phases

  - First, the candidate input pictures are selected the same way as input pictures for an NNPF are selected in VVC v3.

  - Second, the input pictures are selected from the candidate input pictures based on the above proposed signalling for controlled selection of input pictures.

# Introduction

Contribution JVET-AI0072 asserted that the current design of neural-network post-filter characteristic (NNPFC) and neural-network post-filter activation (NNPFA) SEI messages cannot support use-case when the picture associated with an NNPFA SEI message is not the input picture for the invocation of the target NNPF. The following use-cases are copied from JVET-AI0072.

------------------------ Use-cases from JVET-AI0072 start------------------------

When a processing chain includes picture rate upsampling, it is asserted that it is unclear which pictures the subsequent processing stages in the same processing chain apply. Use cases where the subsequent processing stages do not apply to both the current picture and all the pictures that were generated through picture rate upsampling include the following:

- An interpolated picture is enhanced since the picture rate upsampling NNPF (also called the interpolator NNPF) has low complexity and produces relatively poor picture quality. Thus, a visual enhancement filter is to be applied only on the interpolated pictures, but not the cropped decoded pictures. See the following figure.

Additionally, it is asserted that the current design does not allow an encoder to make a decision between the base NNPF or an updated NNPF for interpolated pictures, but rather the choice between the base NNPF or an updated NNPF that is indicated for a cropped decoded picture also applies to the interpolated pictures. An example:

- An encoder finetunes a quality-enhancement NNPF based on interpolated pictures of a random access segment and sends an NNPF update through an NNPFC SEI message. Ideally, an encoder would indicate the use of the base quality-enhancement NNPF for the cropped decoded pictures and the updated NNPF for the interpolated pictures, but this is not possible with the current design. See the following figure.

Furthermore, it is asserted that when a processing chain includes a picture rate upsampling NNPF, the current design does not allow an encoder to indicate to which pictures in a list of generated pictures resulting from a single NNPF activation the subsequent processing stages in the same processing chain apply. An example:

- An interpolator NNPF generates three pictures based on two input pictures, e.g., generates pictures 1, 2, 3 based on pictures 0 and 4. It is likely that the inner-most generated picture, e.g., generated pictures 2, is of lower quality than the other generated pictures in the list of generated pictures, e.g., than generated pictures 1 and 3. In order to enhance only the inner-most generated picture, e.g., generated picture 2, an encoder would need to indicate the use of a quality-enhancement NNPF for that inner-most generated picture only. See the following figure.

------------------------ Use-cases from JVET-AI0072 end-------------------------

To support the above use-cases, the following solution was proposed in JVET-AI0072.

------------------------ Solution proposed in JVET-AI0072 starts------------------------

The proposal assumes that a candidate input picture list for a processing stage is derived like described in VVC subclause D.11.3 (Handling of a processing chain) included in the VSEI TuC (JVET-AH2032).

The proposal includes the following aspects:

- Two options are proposed for NNPF activation with controlled selection of input pictures:

  - Extension of the NNPFA SEI message by adding new syntax elements for controlled selection of input pictures at the end of the syntax structure.

    - In order to maintain compatibility with v3 decoders, the extended syntax is allowed only when the NNPFA SEI message is included in a PON SEI message.

  - NNPF extended activation (NNPFEA) SEI message, which includes syntax elements that correspond to those in the NNPFA SEI message and the syntax elements for controlled selection of input pictures.

    - It is disallowed to use both the NNPFA and NNPFEA SEI messages to activate the same NNPF for the same picture.

- The input pictures for an NNPF are selected in two phases:

First, the candidate input pictures are selected the same way as input pictures for an NNPF are selected in VVC v3.

Second, the input pictures are selected from the candidate input pictures based on the NNPFA (option 1.a) or NNPFEA (option 1.b) syntax elements for controlled selection of input pictures.

------------------------ Solution proposed in JVET-AI0072 ends------------------------

While the proposed solution has two options, the difference between the two options is basically just where the additional signalling is present. The main idea of the solution is to have additional signalling in NNPFA SEI message to allow decoder to do input picture shift for each original input picture of the target NNPF. This would allow decoder to take different pictures as the input from preceding pictures in output order.

# Problem statement

In our assertion, the presented use-cases would be good if they can be supported. However, the solution proposed by JVET-AI0072 has unintended consequence such that it allows gap in between pictures that are used as input for the NNPF. This creates problem with the original design of NNPFC SEI message in the aspect of handling interpolated pictures that are outputted by the filter. The gap created by this solution will mess up with the signalling in NNPFC SEI message that specifies where the newly generated pictures should be located.

# Proposal

We propose to simply the solution proposed by JVET-AI0072 by allowing shift for the whole set of input pictures, not for individual input picture. The detail of the proposed solution is as follows:

- Signal the following information in the extension of NNPFA SEI message:

  - A syntax element that specifies the number of input picture shift. This syntax element may be called nnpfa_num_input_pic_shift.

  - The value of nnpfa_num_input_pic_shift may be in the range 0 from 63, inclusive.

  - When nnpfa_num_input_pic_shift is equal to 0, it means that the input pictures specified by the target NNPFC SEI message are used.

- The input pictures for an NNPF are selected in two phases

  - First, the candidate input pictures are selected the same way as input pictures for an NNPF are selected in VVC v3.

  - Second, the input pictures are selected from the candidate input pictures based on the above proposed signalling for controlled selection of input pictures.

# Text implementation

…

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| nnpfa_target_base_flag | u(1) |
| nnpfa_no_prev_clvs_flag | u(1) |
| if( nnpfa_persistence_flag ) |  |
| nnpfa_no_foll_clvs_flag | u(1) |
| nnpfa_num_output_entries | ue(v) |
| for( i = 0; i < nnpfa_num_output_entries; i++ ) |  |
| nnpfa_output_flag[ i ] | u(1) |
| if( more_data_in_payload( ) ) |  |
| nnpfa_num_input_pic_shift | ue(v) |
| } |  |
| } |  |

…

nnpfa_num_input_pic_shift specifies the number of input pictures shift in the list of candidate input pictures to get the final input pictures for the target NNPF. When not present, the value of nnpfa_num_input_pic_shift is inferred to be equal to 0. The value of nnpfa_num_input_pic_shift shall be in the range from 0 to 63, inclusive.

…

## Selection of input pictures in two phases

The highlighted changes are proposed to VVC D.12.11 (Use of the neural network post-filter characteristics SEI message and the neural network post-filter activation SEI message)

The variable numCandInputPics, which indicates the number of candidate input pictures to the NNPF, is derived as follows:

numCandInputPics = nnpfc_num_input_pics_minus1 + 1 + nnpfa_num_input_pic_shift	(xx)

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

–	The arrays inputPic[ i ] candInputPic[ i ] and inputCandPresentFlag[ i ] for i in the range of 0 to numCandInputPics − 1, inclusive, representing all the candidate input pictures and the presence of candidate input pictures, respectively, are specified as follows:

–	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, inputPic[ k ] candInputPic[ k ] is set to be currPic and inputCandPresentFlag[ k ] is set equal to 0.

–	The j-th candidate input picture, inputPic[ j ] candInputPic[ j ], is set to be currPic and inputCandPresentFlag[ j ] is set equal to 1.

–	When numCandInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numCandInputPics − 1, inclusive, in increasing order of i:

–	If both of the following conditions are true, inputPic[ i ] candInputPic[ i ] is set to be prevPic and inputCandPresentFlag[ i ] is set equal to 1:

–	Either of the following conditions is true:

–	pictureRateUpsamplingFlag is equal to 1 and currPic is associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId, precede inputPic[ i − 1 ] candInputPic[ i − 1 ] in output order, and are associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

–	pictureRateUpsamplingFlag is equal to 0 or currPic is not associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] candInputPic[ i − 1 ] in output order.

–	nnpfa_no_prev_clvs_flag is equal to 0 or the coded picture corresponding to prevPic and the current picture are present in the same CLVS.

–	Otherwise, the following applies:

–	inputPic[ i ] candInputPic[ i ] is set to be the same picture as inputPic[ i − 1 ] candInputPic[ i − 1 ] and inputCandPresentFlag[ i ] is set equal to 0.

–	It is a requirement of bitstream conformance that, when pictureRateUpsamplingFlag is equal to 1, nnpfc_interpolated_pics[ i − 1 ] shall be equal to 0.

–	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures, respectively, are specified as follows:

for( i = 0, candIdx = nnpfa_num_input_pic_shift; i <= nnpfc_num_input_pics_minus1; i++, candIdx++ ) {
	inputPic[ i ] = candInputPic[ candIdx ]		(xx)
	inputPresentFlag[ i ] = inputCandPresentFlag[ candIdx ]
}

–	For each value of i in the range 0 to numInputPics − 1, inclusive, it It is a requirement of bitstream conformance that when inputPresentFlag[ i ] is equal to 0 and nnpfc_input_pic_output_flag[ i ] is equal to 1, the value of nnpfa_output_flag[ idx ] shall be equal to 0 for the value of idx such that InpIdx[ idx ] is equal to i.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).