| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0116-v2 |
| --- | --- |

| Title: | Non-EE2: On interpolation filter selection for TIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zixiang Zhang Jie Chen Xinwei Li Ru-Ling Liao Yan Ye | Tel: Email: | {zhangzixiang.zzx, jiechen.cj, sid.lxw, ruling.lrl, yan.ye}@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution proposes to use TM cost to select interpolation filter for TIMD. In the proposed method, the 6-tap cubic and 6/4-tap gauss interpolation filter are applied on the template area and the one producing the smaller SATD cost is used for TIMD prediction. It is reported that on top of ECM-14.0, the overall coding performance for {Y, U, V, EncT, DecT} is {-0.04%, -0.01%, -0.02%, 100.4%, 100.3%} for AI.

Introduction

In ECM-14.0, the interpolation filter[1] used in the angular intra prediction is determined by the mode dependent intra reference sample smoothing (MDIS) condition[2]. The MDIS condition for fractional angular modes of luma can be described as follows:

- If one of the following three conditions is satisfied, a 6-tap cubic interpolation filter is used.

- the reference line index equals 0 (no MRL);

- the TU size is no more than 32;

- the block is ISP mode.

- otherwise,

- Set minDistVerHor equal to Min( abs( predModeIntra − VER_IDX ), abs( predModeIntra − HOR_IDX ) )

- Set intraHorVerDistThres[ nTbS ] as specified below, where nTbS equal to ( Log2 (W) + Log2 (H) ) >> 1

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 24 | 14 | 2 | 0 | 0 | 0 |

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ], Gaussian filter (6-tap for blocks with width>=32 and height>=32 and 4-tap for other blocks) is used for the interpolation.

- Otherwise, a 6-tap cubic filter is used for the interpolation

The magnitude response of these interpolation filters for 16/32 phase is illustrated in the Figure.1

Fig. 1 The magnitude response of the different interpolation filters for 16/32 phase

Proposed Method

Instead of using a fixed condition to select intra interpolation filter for all the modes and all the blocks, it is proposed to use TM cost of the current coding block to select the intra interpolation filter for TIMD.

In ECM-14.0, up to three intra prediction modes will be derived in TIMD. After that, it is proposed to apply these intra prediction modes on the template area with two different interpolation filters, 6-tap cubic interpolation filter and 4/6-tap Gaussian interpolation filter (6-tap for blocks with height/width>32 and 4-tap for other blocks) respectively. Then the TM cost is calculated as the SATD between the predictor and the reconstructed template. The filter which produces the smaller TM cost is selected as the luma interpolation filter for all of the three intra prediction modes in TIMD prediction. For simplicity, the intra fusion[3] and location-dependent blending[4] which are used in TIMD are disabled in the filter selection. The flowchart of the proposed method is shown in Figure. 2.

Fig. 2 The proposed TM cost based interpolation filter selection for TIMD

Simulation results

The proposed methods are implemented on top of ECM-14.0 and are tested under CTC for enhanced compression tool testing[5].

Table 1. Simulation results of the proposed method

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.05% | -0.04% | 100.6% | 100.2% | 99.4% | 100.0% |
| Class A2 | -0.04% | -0.05% | -0.08% | 100.4% | 100.1% | 100.0% | 100.1% |
| Class B | -0.02% | -0.04% | 0.11% | 100.5% | 100.3% | 102.2% | 100.0% |
| Class C | -0.04% | 0.00% | -0.10% | 100.0% | 100.2% | 98.6% | 99.9% |
| Class E | -0.06% | 0.11% | -0.05% | 100.3% | 100.7% | 103.0% | 99.8% |
| Overall | -0.04% | -0.01% | -0.02% | 100.4% | 100.3% | 100.7% | 100.0% |
| Class D | -0.03% | 0.00% | -0.14% | 100.3% | 99.9% | 100.2% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0.02% | 0.05% | 0.10% | 99.8% | 100.1% | 100.0% | 99.9% |
| Class C | -0.01% | -0.12% | -0.02% | 100.4% | 100.4% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | 0.00% | -0.04% | 0.14% | 99.6% | 101.0% | 99.7% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Conclusion

The contribution proposed an adaptive interpolation filter selection method based on TM cost for TIMD. It is reported that -0.04% BD-rate reduction can be achieved under AI configuration with negligible run time increase. It is recommended to further study this proposal in the next round of EE2.

References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 13 (ECM 13)”, JVET-AH2025, Apr. 2024.

- J. Chen, Y. Ye, S. H. Kim, “Algorithm description for Versatile Video Coding and Test Model 11 (VTM 11)”, JVET-T2002, Oct. 2020.

- L. Xu, Y. Yu, H, Yu, D. Wang, H. Wang, V. Seregin, B. Ray, M. Karczewicz, “EE2-1.12: Combination of EE2-1.10 and EE2-1.11”, JVET-AB0157, Oct. 2022.

- P. Andrivon, M. Blestel, “EE2-1.20: TIMD fusion with non-angular predictor”, JVET-AG0092, Jan. 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, Aug. 2024.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).