| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0118-v2 |
| --- | --- |

| Title: | Non-EE2: Virtual intra prediction mode (VIPM) for chroma intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi Myungoh Hong Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung-Hwan Kim 10680 Treena St, San Diego, CA 92131, USA | Tel: Email: | +82-2-6912-6477 jangwon84.choi@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to use virtual intra prediction mode (VIPM) in chroma intra mode coding. The proposed method inserts the VIPM of co-located luma block into the list of non-CCP chroma intra mode candidates. Additionally, the VIPM of CCP predictors is used to select the NSPT/LFNST transform set for the CCP chroma intra mode coding. Experimental results for the proposed method, implemented on top of the ECM-14.0, are reported as follows:

For AI configuration: -0.03%/-0.22%/-0.21%, 100.1%(EncT), 102.3%(DecT).

For RA configuration: -0.01%/-0.20%/-0.06%, 100.3%(EncT), 99.8%(DecT).

Introduction

In the current ECM, the chroma intra modes that are not a CCP mode is selected from the pre-defined mode set. This mode set is obtained from the current chroma block and its co-located luma block, as well as the corresponding neighboring blocks. When the luma and chroma blocks are not aligned or have different sizes, the accuracy of the existing method can be reduced. As an alternative, virtual intra prediction mode (VIPM) which was employed to derive the LFNST transform set of MIP[1] and the NSPT/LFNST transform set of inter coding[2] can be employed for the chroma mode derivation.

Proposed methods

In the proposed method, prioritizes the VIPM of the co-located luma block by inserting it as the first candidate in the list of non-CCP chroma intra mode. The other candidates are selected same as the existing method: {DM mode, chroma DIMD mode, three default mode, intra modes of co-located luma blocks, intra modes of neighbor chroma blocks}, with the same list size of previous non-CCP chroma intra prediction. The VIPM of co-located luma block is calculated with the reconstructed pixels of co-located luma block.

Figure 1. chroma block (grey area) and co-located luma block

The VIPM of CCP predictors are also used to select the NSPT/LFNST transform set in the CCP chroma intra coding. When the current chroma block is applied one of the CCP prediction mode, the VIPM is obtained as follows:

- If the CCP mode of current chroma block is CCLM(LT, L, T, MM_LT, MM_L, MM_T) or CCCM(LT, L, T, MM_LT, MM_L, MM_T), the VIPM of chroma Cb predictor is calculated.

- Else (the CCP mode of current chroma block is not included in (1) case), the VIPM of co-located luma block is calculated.

After this process, the NSPT/LFNST transform set in the current intra chroma block is selected based on the derived VIPM above.

Experimental results

The proposed method is tested on top of ECM-14.0 [3] under the common test conditions [4].

Table 1 Experimental results of proposed method

|  |  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over ECM-14.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | U |  | V |  | EncT |  |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 |  | -0.04% | -0.27% |  | -0.22% |  | 100.2% |  |  | 102.3% |  | 100.1% |  | 100.0% |  |
| Class A2 |  | -0.03% | -0.27% |  | -0.32% |  | 100.1% |  |  | 102.2% |  | 99.5% |  | 100.1% |  |
| Class B |  | -0.02% | -0.24% |  | -0.18% |  | 100.2% |  |  | 102.4% |  | 97.7% |  | 100.0% |  |
| Class C |  | -0.02% | -0.24% |  | -0.29% |  | 100.0% |  |  | 102.2% |  | 100.5% |  | 99.9% |  |
| Class E |  | -0.04% | -0.07% |  | -0.02% |  | 99.9% |  |  | 102.6% |  | 103.4% |  | 100.1% |  |
| Overall |  | -0.03% | -0.22% |  | -0.21% |  | 100.1% |  |  | 102.3% |  | 100.0% |  | 100.0% |  |
| Class D |  | -0.03% | -0.13% |  | -0.35% |  | 100.1% |  |  | 102.0% |  | 100.0% |  | 100.0% |  |
| Class F |  | -0.07% | -0.12% |  | -0.11% |  | 100.0% |  |  | 101.9% |  | 100.7% |  | 100.1% |  |
| Class TGM |  | -0.09% | -0.19% |  | -0.28% |  | 99.9% |  |  | 101.4% |  | 100.0% |  | 99.9% |  |
|  |  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Over ECM-14.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  | U |  | V |  | EncT | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 |  | -0.01% |  |  | -0.43% |  | 0.00% |  | 100.2% | 100.3% |  | 100.1% |  | 99.8% |  |
| Class A2 |  | 0.01% |  |  | -0.11% |  | -0.14% |  | 100.2% | 99.6% |  | 100.6% |  | 100.0% |  |
| Class B |  | -0.02% |  |  | -0.16% |  | -0.17% |  | 100.3% | 99.8% |  | 99.9% |  | 99.9% |  |
| Class C |  | 0.01% |  |  | -0.16% |  | 0.09% |  | 100.3% | 99.4% |  | 100.0% |  | 100.0% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -0.01% |  |  | -0.20% |  | -0.06% |  | 100.3% | 99.8% |  | 100.1% |  | 99.9% |  |
| Class D |  | -0.04% |  |  | -0.38% |  | -0.53% |  | 99.9% | 99.9% |  | 99.9% |  | 100.0% |  |
| Class F |  | 0.03% |  |  | -0.33% |  | -0.12% |  | 100.2% | 101.1% |  | 99.9% |  | 99.7% |  |
| Class TGM |  | -0.05% |  |  | 0.02% |  | 0.01% |  | 100.1% | 101.1% |  | 99.9% |  | 99.9% |  |

Conclusion

In this contribution, the VIPM of co-located luma block and CCP predictors are introduced to the chroma intra mode coding, with the promising coding performance. It is recommended to include the proposed method into the next round of EE2.

References

- J. Huo, “EE2-4.1: Modification of LFNST for MIP coded block”, document JVET-AB0067, Oct. 2022.

- F. Wang, “EE2-3.3: Utilizing LFNST/NSPT for inter coding”, document JVET-AG0061, Jan. 2024.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, document JVET-AI2017, Jul. 2024.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).