| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0119-v1 |
| --- | --- |

| Title: | AHG9: On signalling identifier in encoder optimization information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Hendry Tan Chul Keun Kim Jangwon Lee Jaehyun Lim Seung hwan Kim | Email: | junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

The current design of encoder optimization information (EOI) SEI message allows only one EOI SEI message to be active at a time. Consequently, all optimization types described by an EOI SEI message must have the same persistence in a CLVS. Any change, however, small or big it is to one of the optimization type included in an EOI SEI message would cause the whole information need to be resend. It is asserted that it is desirable to be able to control persistence of a specific optimization type rather than cancelling or signaling whole persistence of the EOI SEI message. This contribution proposes to add a syntax element, i.e. eoi_id, to specify an identifier for an EOI SEI message so that it is possible to have more than one EOI SEI message to be active at a time. This would enable different types of optimizations to have different persistency.

Introduction

The current design of the encoder optimization information (EOI) SEI message includes signaling information on encoder optimization methods for various types that are applied prior to or during the encoding process. Additionally, only one EOI SEI message can be active at a time, meaning that all optimizations must have the same persistence. However, the persistence for the different optimization types may be different. As shown in Figure 1, it may be desirable to apply spatial resampling like upsampling to all source pictures and to apply spatial quality enhancement and privacy protection optimization to some pictures.

Figure1. Example of different persistence for optimization types

It is asserted that activating multiple EOI SEI messages with an access unit (AU) may be useful for expressing optimization types with different persistence scopes.

Proposal

We propose the following modifications to the design of EOI SEI message:

- Add signalling of identifier for EOI SEI message so that it is possible to have more than one EOI SEI messages to be active. The proposed syntax element can be named eoi_id.

- When the eoi_cancel_flag is equal to 1, any previous EOI SEI message with the same eoi_id in this EOI SEI message is cancelled.

- When the eoi_persistence_flag is equal to 1, the optimization information applies for the current picture and all subsequent pictures of the current layer in output order until a picture with an EOI SEI message with the same value of eoi_id is output that follows the current picture in output order.

Text that are added are indicated by yellow highlight. Text that are removed are indicated by red, strickenthrough font.

…

8.31.1 Encoder optimization information SEI message syntax

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_id | u(6) |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| } |  |
| if(EoiSpatialResamplingFlag) { |  |
| eoi_orig_pic_dimensions_flag | u(1) |
| if( eoi_orig_pic_dimensions_flag ) { |  |
| eoi_orig_pic_width | u(16) |
| eoi_orig_pic_height | u(16) |
| } |  |
| else |  |
| eoi_spatial_resampling_type_flag | u(1) |
| } |  |
| if( EoiPrivacyProtectionFlag ) { |  |
| eoi_privacy_protection_method_idc | u(4) |
| eoi_privacy_info_type | u(8) |
| } |  |
| } |  |
| } |  |

8.32.2 Encoder optimization information SEI message semantics

The encoder optimization information SEI message is used to indicate if the video has been optimized for human viewing or machine analysis and which types of optimization have been applied in pre-processing or encoding.

eoi_id specifies an identifier of the EOI SEI message. The value of eoi_id shall be in the range of 0 to 63, inclusive.

eoi_cancel_flag equal to 1 specifies that the persistence of any previous the encoder optimization information SEI message with the same eoi_id included in any previous PU in output order is cancelled. eoi_cancel_flag equal to 0 indicates that information on optimization that has been applied in pre-processing or encoding follows.

eoi_persistence_flag specifies the persistence of the optimization information provided in this SEI message. eoi_persistence_flag equal to 0 specifies that the optimization information applies for the current picture only. eoi_persistence_flag equal to 1 specifies that the optimization information applies for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with an encoder optimization information SEI message with the same value of eoi_id is output that follows the current picture in output order.

…

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).