| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0120-v1 |
| --- | --- |

| Title: | AHG9: Miscellaneous modifications for SEI messages in the VSEI draft |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Hendry Tan Chul Keun Kim Jangwon Lee Jaehyun Lim Seung hwan Kim | Email: | junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes miscellaneous modifications that are asserted to provide improvement to some aspects of SEI messages in VSEI v4 draft. The proposed modifications are as follows,

(Related to SPO/PON SEI messages)

1. Update conditions on repetition of SPO SEI message

2. Simplify constraints on presences of PON-nested SEI message

(Related to OMI SEI message)

3. Move position of byte-alignment for OMI SEI message

4. Add a constraint on free-form string of OMI SEI message

(Related to TDI SEI message)

5. Add a syntax to indicate importance level of TDI SEI message

6. Add a constraint on free-form string of TDI SEI message

Update conditions on repetition of SPO SEI message

Problem statement

Current design of SEI processing order (SPO) SEI message allows repetition of the SPO SEI message with the same po_id in a CVS. In the current semantics of SPO SEI message, there is text that specifies the conditions to determine whether two SPO SEI messages are the same SPO SEI message. However, some syntax elements are missing at the conditions. It is asserted that it may cause wrong determination of the same SPO SEI messages and lead to incorrect decoder operation.

Proposed modification

SPO SEI messages with the same po_id within a CVS should have the same values of all syntax elements. Text that are added are indicated by yellow highlight. Text that are removed are indicated by red, strickenthrough font.

8.30.1.2 SEI processing order SEI message semantics

…

When more than one SPO SEI message with a particular value of po_id is present in a CVS, the values of po_for_human_viewing_idc, po_for_machine_analysis_idc, po_num_sei_messages_minus2, po_breadth_first_flag and, for each value of i and j, the values of po_sei_wrapping_flag[ i ], po_sei_importance_flag[ i ], po_sei_processing_degree_flag[ i ], po_sei_payload_type[ i ], po_sei_prefix_flag[ i ], po_sei_importance_flag[ i ], po_sei_payload_type[ i ], po_sei_processing_order[ i ], po_num_bits_in_prefix_indication_minus1[ i ] (when present), po_sei_prefix_data_bit[ i ][ j ] (when present) shall be the same as in the other SPO SEI messages in the CVS with the same value of po_id.

Simplify constraints on presences of PON-nested SEI message

Problem statement

An SEI message that is part of a processing chain specified by an SEI processing order (SPO) SEI message can be present either inside or outside of a processing order nested (PON) SEI message. The current design of PON SEI message has some specific constraints on each SEI message that are included in the PON SEI message. In authors’ opinion, maintaining constraint for each of those SEI messages would be hard since more SEI messages may later be allowed to be present in PON SEI message. Furthermore, it is also not easy for implementation since there will be too many individual checking needed. It would be desirable if one or more general form of constraints can be defined to cover those constraints for those individual SEI messages.

Proposed modification

This contribution specifies general constraints for the presence of PON-nested SEI messages in PON SEI message based on the presence of identifier or persistence. Text that are added are indicated by yellow highlight. Text that are removed are indicated by red, strickenthrough font.

…

It is a requirement of bitstream conformance that the semantics and effect of an SEI message that is not a PON-nested SEI message shall not depend on any PON-nested SEI message. Consequences of this constraint include the following specific constraints, in which an associated SEI message is considered to be an SEI message that affects the semantics or effect of a particular SEI message:

–	When a neural-network post-filter characteristics SEI message is present with a particular value of nnpfc_id that is a PON-nested SEI message, any associated neural-network post-filter activation SEI messages with nnpfa_target_id equal to that particular value of nnpfc_id shall also be PON-nested SEI messages.

It is a requirement of bitstream conformance that when an SEI message seiA of a particular payload type that is not a PON-nested SEI message, has an particular identifier or target identifier, and has persistence flag that is equal to 1, if the next picture in output order in the same CLVS that has an SEI message seiB where seiA and seiB have the same payload type and have the same identifier or target identifier, then seiB shall not be a PON-nested SEI message.

–	When a neural-network post-filter activation (NNPFA) SEI message is present with nnpfa_persistence_flag equal to 1 and a particular value of nnpfa_target_id that is not a PON-nested SEI message, the next picture in output order in the same CLVS that has an NNPFA SEI message with the same value of nnpfa_target_id (if any) shall not have an associated NNPFA SEI message that is a PON-nested SEI message.

–	When a frame packing arrangement SEI message is present with fp_arrangement_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated frame packing arrangement SEI message in the same CLVS with fp_arrangement_cancel_flag equal to 1 or the same value of fp_arrangement_id that is a PON-nested SEI message.

–	When a colour transform indication SEI message is present with colour_transform_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated colour transform indication SEI message in the same CLVS with colour_transform_cancel_flag equal to 1 or the same value of colour_transform_id that is a PON-nested SEI message.

It is a requirement of bitstream conformance that when an SEI message that is not a PON-nested SEI message, has no identifier or target identifier, and has persistence flag that is equal to 1, there shall not be an associated SEI message of the same payload type is present in the same CLVS that is a PON-nested SEI message.

–	When a film grain characteristics SEI message is present with fg_characteristics_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated film grain characteristics SEI message in the same CLVS that is a PON-nested SEI message.

–	When a content colour volume SEI message is present with ccv_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated frame packing arrangement SEI message in the same CLVS that is a PON-nested SEI message.

–	When an equirectangular projection SEI message is present with erp_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated equirectangular projection SEI message in the same CLVS that is a PON-nested SEI message.

–	When a sphere rotation SEI message is present with sphere_rotation_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated sphere rotation SEI message in the same CLVS that is a PON-nested SEI message.

–	When a region-wise packing SEI message is present with rwp_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated region-wise packing SEI message in the same CLVS that is a PON-nested SEI message.

–	When an omnidirectional viewport SEI message is present with omni_viewport_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated omnidirectional viewport SEI message in the same CLVS that is a PON-nested SEI message.

–	When a sample aspect ratio SEI message is present with sari_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated sample aspect ratio SEI message in the same CLVS that is a PON-nested SEI message.

It is a requirement of bitstream conformance that when an SEI message that is not a PON-nested SEI message, has no identifier or target identifier, and no persistence flag, there shall not be an associated SEI message of the same payload type is present in the same CLVS that is a PON-nested SEI message.

–	When an annotated regions SEI message is present that is not a PON-nested SEI message, there shall not be an associated annotated regions SEI message in the same CLVS that is a PON-nested SEI message.

–	When an alpha channel information SEI message is present that is not a PON-nested SEI message, there shall not be an associated alpha channel information SEI message in the same CLVS that is a PON-nested SEI message.

–	When a display orientation SEI message is present that is not a PON-nested SEI message, there shall not be an associated display orientation SEI message in the same CLVS that is a PON-nested SEI message.

…

Move position of byte-alignment for OMI SEI message

Problem statement

The current design of object mask information (OMI) SEI message can specify the contents of label associated with an object mask. Since the label information in OMI SEI message is coded with st(v) syntax descriptor, there is byte-alignment to ensure that the syntax element is present at a byte-aligned position in the bitstream. However, byte alignment in the current semantics of OMI SEI message is always present regardless of whether the label information is present or not. It is asserted that byte alignment signalling is needed only when the label information is present.

Proposed modification

For signaling label information for an object mask in object mask information (OMI) SEI message, we propose to move the byte-alignment signalling to just before the label information at the current OMI SEI message. Text that are added are indicated by yellow highlight. Text that are removed are indicated by red, strickenthrough font.

8.33.1 Object mask information SEI message syntax

| object_mask_info( payloadSize ) { | Descriptor |
| --- | --- |
| omi_cancel_flag | u(1) |
| if( !omi_cancel_flag ) { |  |
| omi_persistence_flag | u(1) |
| omi_num_aux_pic_layer_minus1 | ue(v) |
| omi_mask_id_length_minus1 | ue(v) |
| omi_mask_sample_value_length_minus8 | ue(v) |
| omi_mask_confidence_info_present_flag | u(1) |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence_length_minus1 | u(4) |
| omi_mask_depth_info_present_flag | u(1) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth_length_minus1 | u(4) |
| omi_mask_label_info_present_flag | u(1) |
| if( omi_mask_label_info_present_flag ) { |  |
| omi_mask_label_language_present_flag | u(1) |
| if( omi_mask_label_language_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| omi_mask_label_language | st(v) |
| } |  |
| } |  |
| for( i = 0; i < omi_num_aux_pic_layer; i++ ) { |  |
| omi_mask_pic_update_flag[ i ] | f(1) |
| if( omi_mask_pic_update_flag[ i ] ) { |  |
| omi_num_mask_in_pic_update[ i ] | ue(v) |
| for( j = 0; j < omi_num_mask_in_pic_update[ i ]; j++ ) { |  |
| omi_mask_id[ i ][ j ] | u(v) |
| omi_mask_cancel[ i ][ j ] | u(1) |
| if( !omi_mask_cancel[ i ][ j ] ) { |  |
| omi_aux_sample_value[ i ][ j ] | u(v) |
| omi_mask_bounding_box_present_flag[ i ][ j ] | u(1) |
| if( omi_mask_bounding_box_present_flag[ i ][ j ] ) { |  |
| omi_mask_top[ i ][ j ] | u(16) |
| omi_mask_left[ i ][ j ] | u(16) |
| omi_mask_width[ i ][ j ] | u(16) |
| omi_mask_height[ i ][ j ] | u(16) |
| } |  |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence[ i ][ j ] | u(v) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth[ i ][ j ] | u(v) |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| if( omi_mask_label_info_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| omi_mask_label[ i ][ j ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Add a constraint on free-form string of OMI SEI message

Problem statement

The current semantics of omi_mask_label[ i ][ j ] in object mask information (OMI) SEI message are as follows:

omi_mask_label[ i ][ j ] specifies the contents of the label associated with the j-th object mask of the object mask picture in the i-th auxiliary layer associated with the current primary layer. The length of the omi_mask_label[ i ][ j ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

This syntax element is defined with st(v) descriptor, which is null-terminated string encoded as universal coded character set (UCS) transmission format-8 (UTF-8) characters, as specified in ISO/IEC 10646. This string can be composed in free-form with UTF-8 characters, since there are no constraints or regulations on it. Therefore, it may also be a null string that has no information. It is asserted that it causes unnecessary use of bits.

Proposed modification

This contribution proposes to add a constraint that the omi_mask_label[ i ][ j ] shall not be a null string when syntax element omi_mask_label_info_present_flag is equal to 1 in an OMI SEI message. Text that are added are indicated by yellow highlight. Text that are removed are indicated by red, strickenthrough font.

…

omi_mask_label[ i ][ j ] specifies the contents of the label associated with the j-th object mask of the object mask picture in the i-th auxiliary layer associated with the current primary layer. The length of the omi_mask_label[ i ][ j ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte. When omi_mask_label_info_present_flag equal to 1, omi_mask_label[ i ][ j ] shall not be a null string.

…

Add a syntax to indicate importance level of TDI SEI message

Problem statement

When a decoder receives a text description information (TDI) SEI message with purpose such as copyright information, AI marking information, or content advisory rating information, it may have idea what to do or what it is really about. On the other hand, a TDI SEI message with purpose of application defined or general comment information may be a bit vague and decoder may not know what to do with it. Sometime one may even simply ignore it. Hence, it may be desirable to have further information such as category (e.g., importance level) that may be help decoder to decide if it may need to do something about it.

Proposed modification

When txt_descr_purpose is equal to a certain value which may includes application defined or general comment information, additional syntax element that specifies the importance level (e.g. txt_descr_importance) of the text information is present. Text that are added are indicated by yellow highlight. Text that are removed are indicated by red, strickenthrough font.

8.35.1 Text description information SEI message syntax

| text_description( payloadSize ) { | Descriptor |
| --- | --- |
| txt_descr_purpose | u(8) |
| txt_cancel_flag | u(1) |
| if( !txt_cancel_flag ) { |  |
| txt_descr_id | u(13) |
| txt_id_cancel_flag | u(1) |
| if( !txt_id_cancel_flag ) { |  |
| txt_persistence_flag | u(1) |
| if( txt_descr_purpose = = 0 ¦ ¦ txt_descr_purpose = = 3 ) |  |
| txt_descr_importance | u(8) |
| txt_num_strings_minus1 | u(8) |
| for( i = 0; i <= txt_num_strings_minus1; i++ ) { |  |
| txt_descr_string_lang[ i ] | st(v) |
| txt_descr_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |

8.35.2 Text description information SEI message semantics

…

txt_descr_importance specifies the level of importance of the text description in the SEI. When txt_descr_importance is greater than 0, the higher the value of txt_descr_importance the more important the text description is. When txt_descr_importance is present and equal to 0, the importance of the text description is unknown. The value of txt_descr_importance shall be in the range from 0 to 255, inclusive.

…

Add a constraint on free-form string of TDI SEI message

Problem statement

The current semantics of txt_descr_string[ i ] in text description information (TDI) SEI message is as follows:

txt_descr_string[ i ] specifies i-th text description information string whose value is interpreted as specified by the txt_descr_purpose.

When txt_descr_purpose is equal to 0, the interpretation of what information is conveyed in the txt_descr_string is application-defined.

When txt_descr_purpose is equal to 1, the txt_descr_string[ i ] specifies copyright information that pertains to the picture(s) in the persistence scope of this SEI message.

When txt_descr_purpose is equal to 2, the txt_descr_string[ i ] specifies, when not a null string, AI marking information that pertains to the picture(s) within the persistence scope of this SEI message.

NOTE: When txt_descr_purpose is equal to 2 the string can contain information about machine-learning-based processing, intended use of the decoded pictures, or other aspects relevant to the associated pictures.

When txt_descr_purpose is equal to 3, the txt_descr_string[ i ] specifies a general text label description that pertains to the picture(s) in the persistence scope of this SEI message.

When txt_descr_purpose is equal to 4, the txt_descr_string[ i ] shall specify content advisory rating information conforming to US and Canadian Rating Region Tables (RRT) as defined by CTA-766-D that pertains to the picture(s) in the persistence scope of this SEI message.

When txt_descr_purpose is equal to 5, txt_descr_string[ i ] shall contain a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the CLVS.

This syntax element is defined by st(v) descriptor, which is null-terminated string encoded as universal coded character set (UCS) transmission format-8 (UTF-8) characters, as specified in ISO/IEC 10646. This string can be composed in free-form with UTF-8 characters, since there are no constraints or regulations on it. Therefore, it may also be a null string that has no information. It is asserted that it causes unnecessary use of bits.

Proposed modification

This contribution proposes to add a constraint that the txt_descr_string[ i ] shall not be a null string when syntax element txt_descr_string[ i ] is present in a TDI SEI message. The text in this embodiment is based on the VSEI and VVC specifications. Text that are added are indicated by yellow highlight. Text that are removed are indicated by red, strickenthrough font.

…

txt_descr_string[ i ] specifies i-th text description information string whose value is interpreted as specified by the txt_descr_purpose.

When txt_descr_purpose is equal to 0, the interpretation of what information is conveyed in the txt_descr_string is application-defined.

When txt_descr_purpose is equal to 1, the txt_descr_string[ i ] specifies copyright information that pertains to the picture(s) in the persistence scope of this SEI message.

When txt_descr_purpose is equal to 2, the txt_descr_string[ i ] specifies, when not a null string, AI marking information that pertains to the picture(s) within the persistence scope of this SEI message.

NOTE: When txt_descr_purpose is equal to 2 the string can contain information about machine-learning-based processing, intended use of the decoded pictures, or other aspects relevant to the associated pictures.

When txt_descr_purpose is equal to 3, the txt_descr_string[ i ] specifies a general text label description that pertains to the picture(s) in the persistence scope of this SEI message.

When txt_descr_purpose is equal to 4, the txt_descr_string[ i ] shall specify content advisory rating information conforming to US and Canadian Rating Region Tables (RRT) as defined by CTA-766-D that pertains to the picture(s) in the persistence scope of this SEI message.

When txt_descr_purpose is equal to 5, txt_descr_string[ i ] shall contain a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the CLVS.

txt_descr_string[ i ] shall not be a null string.

…

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).