| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0124 |
| --- | --- |

| Title: | EE1-2.3: Block based QP information in NN loop filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Franck Galpin, Tangi Poirier Thierry Dumas Philippe Bordes | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This document reports the results of EE1-2.3. It shows the impact of using block based QP information in NNVC loop filter. In particular, the impact of using the Adaptive Quantization (AQ) mode in NNVC is reported for both the current trained model and new models trained on non flat QP. On top of NNVC-10 using HOP3 model, the following results are reported compared to using the QP slice as an input:

- {xxx} for YUV in RA configuration.

On top of NNVC-10 using HOP5 model, the following results are reported compared to using the QP slice as an input:

- {xxx} for YUV in RA configuration,

- {-1.62%	0.44%	-0.31%} for YUV in AI configuration.

# Introduction

In NNVC, several loop filters and post-filters use the QP slice information. However, in real encoding, adaptive QP is often used to control the quality inside the slice and allow an increase of the subjective quality. We report the results of using the QP of the block on a model which is trained using flat QP (legacy model) and AQ (new model).

# Simulation

## Training description

A new dataset was created following exactly the last step of HOP stage 3 creation:

- The CTC datasets are used (DIK2K, BVI, TVD)

- Use the official HOP stage II model

- Follow the encoding steps for the dataset, except that default adaptive QP of VTM is activated using “--AdaptiveQP -aqr 12”

The models are then trained exactly as before for HOP3 and HOP5, except that the QP slice is replaced by the QP block of the luma component.

The inference is performed on top of NNVC where the following options are used “--AdaptiveQP -aqr 12 --MaxCuDQPSubdiv=2”.

## HOP3 results

### NNVC-10.0 HOP3 vs NNVC-10.0 HOP3-average QP [AQ mode]

In his test, the QP slice input is replaced by the average QP of the patch.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 1.13% | 2.13% | 2.77% | 1.23% | 2.26% | 2.86% | 104% | 112% |
| Class A2 | 0.83% | 1.74% | 1.57% | 1.12% | 2.56% | 1.69% | 102% | #NUM! |
| Class B | 0.88% | 2.40% | 3.06% | 1.25% | 2.41% | 3.16% | 100% | 117% |
| Class C | 0.68% | 2.09% | 2.38% | 0.99% | 2.46% | 2.23% | 101% | 103% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.87% | 2.13% | 2.52% | 1.15% | 2.42% | 2.56% | 102% | #NUM! |
| Class D | 2.97% | 7.41% | 8.45% | 3.39% | 7.65% | 8.35% | 103% | #NUM! |
| Class F | 0.55% | 2.53% | 2.79% | 0.91% | 3.32% | 2.99% | 98% | 114% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- Note: early version with AQ12 subdiv=0, to be updated

### NNVC-10.0 HOP3 vs NNVC-10.0 HOP3-AQ [AQ mode]

In his test, the QP slice input is replaced by the QP of the block using the model trained with AQ mode.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.63% | -1.98% | -2.17% | -1.00% | -3.07% | -2.20% | 101% | 104% |
| Class A2 | -0.77% | -1.70% | -1.39% | -0.99% | -2.35% | -0.94% | 101% | 101% |
| Class B | -0.59% | -0.67% | -1.77% | -1.02% | -1.33% | -2.18% | 103% | 112% |
| Class C | -0.39% | 0.49% | 0.15% | -1.03% | 0.64% | 0.00% | 103% | 104% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.58% | -0.83% | -1.26% | -1.01% | -1.36% | -1.36% | 102% | 106% |
| Class D | -0.30% | 0.29% | 0.84% | -1.00% | -0.72% | 0.12% | 92% | 92% |
| Class F | -0.16% | 0.70% | 0.72% | -0.69% | 0.08% | -0.04% | 99% | 106% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- Note: early version with AQ12 subdiv=0, to be updated

## HOP5 results

### NNVC-10.0 HOP5 vs NNVC-10.0 HOP5-average QP [AQ mode]

In his test, the QP slice input is replaced by the average QP of the patch.

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  | DecT CPU |  |
| Class A1 | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #DIV/0! |  |
| Class A2 | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class B | 0.56% | 1.34% |  | 1.69% |  | 0.80% |  | 1.44% |  | 1.89% |  | 108% |  | #NUM! |  |
| Class C | 0.38% | 0.50% |  | 0.74% |  | 0.29% |  | 0.56% |  | 0.68% |  | 107% |  | 113% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class D | 0.19% | 0.16% |  | 0.30% |  | 0.26% |  | 0.16% |  | 0.93% |  | 107% |  | 114% |  |
| Class F | 0.44% | 0.77% |  | 1.04% |  | 0.88% |  | 1.06% |  | 1.66% |  | 110% |  | 121% |  |
| Class H | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  | DecT CPU |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class C | 0.32% | 0.16% |  | -0.19% |  | 0.12% |  | 0.15% |  | -0.42% |  | 108% |  | 112% |  |
| Class E | 0.32% | 0.73% |  | 0.44% |  | 0.53% |  | 0.48% |  | -0.90% |  | 114% |  | 112% |  |
| Overall | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class D | 0.08% | -0.77% |  | 0.55% |  | -0.11% |  | -1.48% |  | 0.25% |  | 108% |  | 112% |  |
| Class F | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM | EncT | DecT CPU |  |
| Class A1 | 0.78% |  | 1.90% |  | 2.73% |  | 1.67% |  | 2.17% |  | 2.93% | 50% | 50% |  |
| Class A2 | 0.53% |  | 1.67% |  | 1.69% |  | 1.26% |  | 1.73% |  | 1.59% | 51% | 52% |  |
| Class B | 0.62% |  | 1.90% |  | 2.76% |  | 1.02% |  | 1.83% |  | 3.10% | 102% | 110% |  |
| Class C | 0.58% |  | 1.41% |  | 1.97% |  | 0.84% |  | 1.48% |  | 2.60% | 100% | 112% |  |
| Class E | 1.33% |  | 3.64% |  | 4.14% |  | 2.22% |  | 2.60% |  | 3.30% | 104% | 110% |  |
| Overall | 0.74% |  | 2.04% |  | 2.63% |  | 1.33% |  | 1.92% |  | 2.74% | 81% | 86% |  |
| Class D | 0.37% |  | 1.09% |  | 1.95% |  | 0.45% |  | 1.07% |  | 2.90% | 100% | 112% |  |
| Class F | 0.73% |  | 2.18% |  | 2.02% |  | 1.30% |  | 2.38% |  | 3.42% | 104% | 114% |  |

### NNVC-10.0 HOP5 vs NNVC-10.0 HOP5-AQ [AQ mode]

In his test, the QP slice input is replaced by the QP of the block using the model trained with AQ mode.

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  | DecT CPU |  |
| Class A1 | -1.33% | -0.01% |  | 0.05% |  | -1.53% |  | 0.66% |  | -0.93% |  | 108% |  | #DIV/0! |  |
| Class A2 | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class B | -0.95% | 1.13% |  | 0.16% |  | -1.37% |  | 1.28% |  | 0.08% |  | 108% |  | 112% |  |
| Class C | -0.64% | 0.61% |  | 0.90% |  | -1.55% |  | 1.30% |  | 1.20% |  | 107% |  | 113% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class D | -0.42% | 0.48% |  | -0.29% |  | -1.30% |  | -0.17% |  | -1.04% |  | 106% |  | 112% |  |
| Class F | -0.46% | 0.30% |  | -0.20% |  | -0.65% |  | 2.85% |  | 0.15% |  | 110% |  | 117% |  |
| Class H | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  | DecT CPU |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class C | -0.70% | -2.76% |  | -1.34% |  | -1.24% |  | -3.81% |  | -2.43% |  | 108% |  | 115% |  |
| Class E | -1.93% | -0.66% |  | -1.21% |  | -1.07% |  | -1.81% |  | -1.99% |  | 115% |  | 122% |  |
| Overall | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class D | -0.26% | -0.77% |  | -0.57% |  | -1.04% |  | -4.68% |  | -3.08% |  | 109% |  | 114% |  |
| Class F | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM | EncT | DecT CPU |  |
| Class A1 | -1.58% |  | -0.85% |  | -1.77% |  | -1.48% |  | -0.28% |  | -2.15% | 104% | 112% |  |
| Class A2 | -1.65% |  | -1.67% |  | -1.88% |  | -1.27% |  | -0.69% |  | -1.26% | 103% | #NUM! |  |
| Class B | -1.50% |  | 2.14% |  | 0.86% |  | -1.24% |  | 2.08% |  | 0.55% | 101% | 111% |  |
| Class C | -1.32% |  | 1.64% |  | 0.89% |  | -1.33% |  | 1.65% |  | 1.36% | 102% | 113% |  |
| Class E | -2.21% |  | -0.59% |  | -0.81% |  | -1.10% |  | -0.30% |  | 0.07% | 103% | 113% |  |
| Overall | -1.62% |  | 0.44% |  | -0.31% |  | -1.28% |  | 0.73% |  | -0.10% | 102% | #NUM! |  |
| Class D | -1.31% |  | -0.01% |  | -0.25% |  | -1.33% |  | 0.04% |  | -0.41% | 100% | 111% |  |
| Class F | -0.75% |  | 1.52% |  | -0.67% |  | -0.64% |  | 3.83% |  | 0.20% | 105% | 113% |  |

# Conclusion

It is proposed to replace the QP slice of the models by the QP of the block. For current CTC, the results are not changed since the QP is flat in the slice and the results are bit exact with current models. However, for further exploration, when Adaptive QP would be used for training and inference, it would increase the performance of the loop filters.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).