| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0135-v1 |
| --- | --- |

| Title: | AHG9/AHG16: Refined Methodology for Pupil Position SEI Message for Generative Face Video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faming Ma, Anthony Trioux, Fuzheng Yang, Bin Li, Fang Xing, Zhikui Wang | Email: | mfm@stu.xidian.edu.cn anthony_trioux@xidian.edu.cn fzhyang@mail.xidian.edu.cn libin19@hisense.com xingfang@hisense.com wangzhikui@hisense.com |
| Source: | Xidian University，Hisense Visual Tech. Co., Ltd. |  |  |

# Abstract

Building on our previous proposal, JVET-AH0138[1], this contribution aims to enhance the methodology for pupil center position computation and transmission. In light of recent advancements and feedbacks, we have identified several key areas for improvement to enhance the effectiveness and efficiency of the existing framework. Our enhancements focus on the following key aspects:

Aspect 1: Modification of the original coordinate system to support accurate prediction for both the left and right eyes.

Aspect 2: Integration of relevant syntax elements into GFVE instead of GFV, in line with the recommendations from the previous meeting.

Aspect 3: Enhancement of the original syntax elements by incorporating a mechanism for predicting the left eye based on the right eye, along with modifications to the quantization parameter transmission and corresponding improvements to the encoding method.

Furthermore, we propose an example of usage of such SEI and its implementation relying on the current software (gfvc_v1-AhG16_GFVC_Software_v3).

- Introduction

Generative Face Video Coding (GFVC) architectures combine a compact representation of facial priors and the powerful inference capabilities of deep generative models to achieve high-quality face video communication in ultra-low bandwidth scenarios by transmitting both a source image and facial features. As shown in JVET-AI0137[2], most of the existing GFVC methods do not accurately represent eye pupil position and movements. However, as an important part of facial video communication, it is crucial to convey such information to improve realism in GFVC.

In JVET-AH0138[1], we proposed a method to compute, encode, and transmit the pupil center position as part of the GFV SEI information. JVET-AI0137 later provided experimental results for this scheme; better reconstructed quality was achieved for the eyes at the cost of an additional bitrate increase. To address this, we optimize the approach by adding an option to predict the right eye using data from the left eye. Additionally, we refer to the quantization parameter skip mechanism from JVET-AI0191[3] to further reduce the bitrate, introducing a flag, gfve_pupil_update_parameter_flag, to signal whether the quantization parameters need to be transmitted or not. Lastly, following suggestions from the last meeting, we integrated our syntax elements into GFVE instead of GFV.

We conducted experiments based on the gfvc_v1-AhG16_GFVC_Software_v3 (https://vcgit.hhi.fraunhofer.de/jvet-ahg-gfvc/gfvc_v1).

- New pupil position representation

Similar to JVET-AH0138, we continue to establish a coordinate system based on the local image of the eyes. However, in this new contribution, we adopt a more simplified coordinate representation that retains the correlation between the left and right eyes.

Specifically, as shown in Figure 1, we take the midpoint of the line connecting the inner and outer corners of both eyes as the center point. The side length of the coordinate system is set to 1.2 times the horizontal distance L between the inner and outer corners of the eyes, establishing a rectangular area. The midpoint serves as the origin of the coordinate system, with the horizontal axis directed to the right as the positive x-axis and the vertical axis directed downward as the positive y-axis. All coordinates are normalized to the range of [-1, 1].

Figure 1 - Coordinate system of pupil center position with respect to the corner of the eye.

- Proposal

Following the suggestions from the last meeting notes, we have incorporated our syntax elements into GFVE. Specially, we propose the corresponding changes relative to JVET-AG2032[4].

| generative_face_video_enhancement ( payloadSize ) { | Descriptor |
| --- | --- |
| gfve_id | ue(v) |
| gfve_gfv_id | ue(v) |
| gfve_gfv_cnt | ue(v) |
| gfve_nn_present_flag | u(1) |
| if( gfve_nn_present_flag ) { |  |
| gfve_nn_base_flag | u(1) |
| gfve_nn_mode_idc | ue(v) |
| if( gfve_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfve_nn_alignment_zero_bit_a | u(1) |
| gfve_nn_tag_uri | st(v) |
| gfve_nn_uri | st(v) |
| } |  |
| } |  |
| gfve_matrix_element_precision_factor | ue(v) |
| gfve_num_matrices_minus1 | ue(v) |
| for(i=0; i <= gfve_num_matrices_minus1; i++){ |  |
| gfve_matrix_height_minus1[ i ] | ue(v) |
| gfve_matrix_width_minus1[ i ] | ue(v) |
| for( j = 0; j <= gfve_matrix_height_minus1[ i ]; j++ ) |  |
| for( k = 0; k <= gfve_matrix_width_minus1[ i ]; k++ ) { |  |
| gfve_matrix_element[ i ][ j ][ k ] | ue(v) |
| if( !gfve_matrix_element[ i ][ j ][ k ]) |  |
| gfve_matrix_element_sign_flag[ i ][ j ][ k ] | u(1) |
| } |  |
| } |  |
| gfve_pupil_coordinate_present_idx | u(2) |
| if (gfve_pupil_coordinate_present_idx != 0){ |  |
| gfve_pupil_pred_flag | u(1) |
| if(gfve_gfv_cnt==0) |  |
| gfve_pupil_update_parameter_flag | u(1) |
| if(gfve_pupil_update_parameter_flag ¦¦ !gfve_pupil_pred_flag) |  |
| gfve_pupil_coordinate_precision_factor_minus1 | ue(v) |
| if(gfve_pupil_pred_flag){ |  |
| if(gfve_pupil_coordinate_present_idx==1¦¦gfve_pupil_coordinate_present_idx == 3) { |  |
| gfve_pupil_left_eye_dx_coordinate_abs | ue(v) |
| if(gfve_pupil_left_eye_dx_coordinate_abs) |  |
| gfve_pupil_left_eye_dx_coordinate_sign_flag | u(1) |
| gfve_pupil_left_eye_dy_coordinate_abs | ue(v) |
| if(gfve_pupil_left_eye_dy_coordinate_abs) |  |
| gfve_pupil_left_eye_dy_coordinate_sign_flag | u(1) |
| } |  |
| if(gfve_pupil_coordinate_present _idx == 2){ |  |
| gfve_pupil_right_eye_dx_coordinate_abs | ue(v) |
| if(gfve_pupil_right_eye_dx_coordinate_abs) |  |
| gfve_pupil_right_eye_dx_coordinate_sign_flag | u(1) |
| gfve_pupil_right_eye_dy_coordinate_abs | ue(v) |
| if(gfve_pupil_right_eye_dy_coordinate_abs) |  |
| gfve_pupil_right_eye_dy_coordinate_sign_flag | u(1) |
| } |  |
| if(gfve_pupil_coordinate_present_idx == 3){ |  |
| gfve_pupil_pre_residual_right_eye_dx_coordinate_abs | ue(v) |
| if(gfve_pupil_pre_residual_right_eye_dx_coordinate_abs) |  |
| gfve_pupil_pre_residual_right_eye_dx_coordinate_sign_flag | u(1) |
| gfve_pupil_pre_residual_right_eye_dy_coordinate_abs | ue(v) |
| if(gfve_pupil_pre_residual_right_eye_dy_coordinate_abs) |  |
| gfve_pupil_pre_residual_right_eye_dy_coordinate_sign_flag | u(1) |
| } |  |
| }else{ |  |
| if(gfve_pupil_coordinate_present_idx==1¦¦gfve_pupil_coordinate_present_idx == 3) { |  |
| gfve_pupil_left_eye_x_coordinate_abs | ue(v) |
| if(gfve_pupil_left_eye_x_coordinate_abs) |  |
| gfve_pupil_left_eye_x_coordinate_sign_flag | u(1) |
| gfve_pupil_left_eye_y_coordinate_abs | ue(v) |
| if(gfve_pupil_left_eye_y_coordinate_abs) |  |
| gfve_pupil_left_eye_y_coordinate_sign_flag | u(1) |
| } |  |
| if(gfve_pupil_coordinate_present_idx==2¦¦gfve_pupil_coordinate_present_idx == 3) { |  |
| gfve_pupil_right_eye_x_coordinate_abs | ue(v) |
| if(gfve_pupil_right_eye_x_coordinate_abs) |  |
| gfve_pupil_right_eye_x_coordinate_sign_flag | u(1) |
| gfve_pupil_right_eye_y_coordinate_abs | ue(v) |
| if(gfve_pupil_right_eye_y_coordinate_abs) |  |
| gfve_pupil_right_eye_y_coordinate_sign_flag | u(1) |
| } |  |
| } |  |
| } |  |
| if( gfve_nn_present_flag ) |  |
| if( gfve_nn_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfve_nn_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| gfve_nn_payload_byte[ i ] | b(8) |
| } |  |

Use of this SEI message requires the definition of the following variables:

- The pupil center position in the base picture, denoted as BaseLeftPupilCoordinateX, BaseLeftPupilCoordinateY, BaseRightPupilCoordinateX and BaseRightPupilCoordinateY.

gfve_pupil_coordinate_present_idx indicates which pupil position information is transmitted. When equal to 0, it means that the pupil position information does not exist; when equal to 1, it means that it contains the left eye pupil position information; when equal to 2, it means that it contains the right eye pupil position information; when equal to 3, it means that it contains both left eye and right eye pupil position information. If gfve_pupil_ coordinate_present _idx does not exist, it is inferred to be equal to 0.

gfve_pupil_pred_flag equal to 1 indicates that the syntax elements gfve_pupil_left_eye_dx_coordinate_abs，and gfve_pupil_left_eye_dy_coordinate_abs are present, and the syntax elements gfve_pupil_left_eye_dx_coordinate_ sign_flag，and gfve_pupil_left_eye_dy_coordinate_sign_flag may be present , when gfve_pupil_coordinate_present_ idx is equal to 1; the syntax elements gfve_pupil_right_eye_dx_coordinate_abs，and gfve_pupil_right_eye_dy_ coordinate_abs are present, and the syntax elements gfve_pupil_right_eye_dx_coordinate_sign_flag，and gfve_pupil_right_eye_dy_coordinate_sign_flag may be present , when gfve_pupil_coordinate_present_idx is equal to 2; the syntax elements gfve_pupil_left_eye_dx_coordinate_abs, gfve_pupil_left_eye_dy_coordinate_abs, gfve_pupil_pre_residual_right_eye_dx_coordinate_abs，and gfve_pupil_pre_residual_right_ eye_dy_coordinate_abs are present, and the syntax elements gfve_pupil_left_eye_dx_coordinate_sign_flag， gfve_pupil_left_eye_dy_ coordinate_sign_flag, gfve_pupil_pre_residual_right_eye_dx_coordinate_sign_flag，and gfve_pupil_pre_residual_ right_eye_dy_coordinate_sign_flag may be present , when gfve_pupil_coordinate_present_idx is equal to 3. gfve_pupil_pred_flag equal to 0 indicates that the syntax elements gfve_pupil_left_eye_dx_coordinate_abs，and gfve_pupil_left_eye_dy_coordinate_abs are present, and the syntax elements gfve_pupil_left_eye_dx_coordinate_ sign_flag，and gfve_pupil_left_eye_dy_coordinate_sign_flag may be present , when gfve_pupil_coordinate_present_ idx is equal to 1 or 3; the syntax elements gfve_pupil_right_eye_dx_coordinate_abs，and gfve_pupil_right_eye_dy_ coordinate_abs are present, and the syntax elements gfve_pupil_right_eye_dx_coordinate_sign_flag，and gfve_pupil_right_eye_dy_coordinate_sign_flag may be present , when gfve_pupil_coordinate_present_idx is equal to 2 or 3.

gfve_pupil_update_parameter_flag equal to 1 indicates that the syntax element gfve_pupil_coordinate_precision_factor_minus1 is present. gfve_pupil_update_parameter_flag equal to 0 indicates that the syntax element gfve_pupil_coordinate_precision_factor_minus1 is not present. If gfve_pupil_update_parameter_flag dose not exist, it inferred to be equal to 0.

gfve_pupil_coordinate_precision_factor_minus1 plus 1 indicates the length, in bits, of syntax elements gfve_pupil_left_eye_dx_coordinate_abs， gfve_pupil_left_eye_dy_coordinate_abs， gfve_pupil_ right_eye_dx_ coordinate_abs，gfve_pupil_right_eye_dy_coordinate_abs， gfve_pupil_pre_ residual_right_eye_dx_coordinate _abs， gfve_pupil_pre_residual_right_eye_dy_coordinate_abs， gfve_pupil_left_eye_x_coordinate_abs， gfve_ pupil_left_eye_y_coordinate_abs， gfve_pupil_right_eye_x_coordinate_abs，and gfve_pupil_right_eye_y_ coordinate_abs. The value of gfv_coordinate_precision_factor_minus1 shall be in the range of 0 to 31, inclusive. When gfve_pupil_update_parameter_flag is equal to 0，gfve_pupil_pred_flag is equal to 1, the value of gfve_coordinate_precision_factor_minus1 is inferred to be equal to the gfve_coordinate_precision_factor_minus1 of the previous GFVE SEI message in decoding order with the same gfve_id as the current GFVE SEI message and gfve_pupil_update_parameter_flag equal to 1.

gfve_pupil_left_eye_dx_coordinate_abs indicates the absolute difference value of the normalized value of the x-axis coordinate of the left eye.

gfve_pupil_left_eye_dx_coordinate_sign_flag specifies the sign of the difference value of the x-axis coordinate of the left eye. When gfve_pupil_left_eye_dx_coordinate_sign_flag is not present, it is inferred to be equal to 0.

gfve_pupil_left_eye_dy_coordinate_abs indicates the absolute difference value of the normalized value of the y-axis coordinate of the left eye.

gfve_pupil_left_eye_dy_coordinate_sign_flag specifies the sign of the difference value of the y-axis coordinate of the left eye. When gfve_pupil_left_eye_dy_coordinate_sign_flag is not present, it is inferred to be equal to 0.

When gfve_pupil_coordinate_present_idx is equal to 1 or 3 and gfve_pupil_pred_flag is equal to 1, the variables LeftPupilCoordinateDeltaX and LeftPupilCoordinateDeltaY indicating the delta x-axis coordinate and delta y-axis coordinate of the left pupil center position, respectively, are derived as follows:

LeftPupilCoordinateDeltaX = ( 1 − 2 * gfve_pupil_left_eye_dx_coordinate_sign_flag ) * 
		gfve_pupil_left_eye_dx_coordinate_abs ÷ ( 1 << ( gfve_coordinate_precision_factor_minus1 + 1 ) ) 
LeftPupilCoordinateDeltaY = ( 1 − 2 * gfve_pupil_left_eye_dy_coordinate_sign_flag ) * 
		gfve_pupil_left_eye_dy_coordinate_abs ÷ ( 1 << ( gfve_coordinate_precision_factor_minus1 + 1 ) )

gfve_pupil_right_eye_dx_coordinate_abs indicates the absolute difference value of the normalized value of the x-axis coordinate of the right eye.

gfve_pupil_right_eye_dx_coordinate_sign_flag specifies the sign of the difference value of the x-axis coordinate of the right eye. When gfve_pupil_right_eye_dx_coordinate_sign_flag is not present, it is inferred to be equal to 0.

gfve_pupil_right_eye_dy_coordinate_abs indicates the absolute difference value of the normalized value of the y-axis coordinate of the right eye.

gfve_pupil_right_eye_dy_coordinate_sign_flag specifies the sign of the difference value of the y-axis coordinate of the right eye. When gfve_pupil_right_eye_dy_coordinate_sign_flag is not present, it is inferred to be equal to 0.

When gfve_pupil_coordinate_present_idx is equal to 2 and gfve_pupil_pred_flag is equal to 1, the variables RightPupilCoordinateDeltaX and RightPupilCoordinateDeltaY indicating the delta x-axis coordinate and delta y-axis coordinate of the right pupil center position, respectively, are derived as follows:

RightPupilCoordinateDeltaX = ( 1 − 2 * gfve_pupil_right_eye_dx_coordinate_sign_flag ) * 
		gfve_pupil_right_eye_dx_coordinate_abs ÷ ( 1 << ( gfve_coordinate_precision_factor_minus1 + 1 ) ) 
RightPupilCoordinateDeltaY = ( 1 − 2 * gfve_pupil_right_eye_dy_coordinate_sign_flag ) * 
		gfve_pupil_right_eye_dy_coordinate_abs ÷ ( 1 << ( gfve_coordinate_precision_factor_minus1 + 1 ) )

gfve_pupil_pre_residual_right_eye_dx_coordinate_abs indicates the absolute value of the residual in predicting the x-axis coordinate difference of the right eye using the x-axis coordinate difference of the left eye.

gfve_pupil_pre_residual_right_eye_dx_coordinate_sign_flag specifies the sign of the residual value of the x-axis coordinate in predicting the x-axis coordinate difference of the right eye using the x-axis coordinate difference of the left eye. When gfve_pupil_pre_residual_right_eye_dx_coordinate_sign_flag is not present, it is inferred to be equal to 0.

gfve_pupil_pre_residual_right_eye_dy_coordinate_abs indicates the absolute value of the residual in predicting the y-axis coordinate difference of the right eye using the y-axis coordinate difference of the left eye.

gfve_pupil_pre_residual_right_eye_dy_coordinate_sign_flag specifies the sign of the residual value of the y-axis coordinate in predicting the y-axis coordinate difference of the right eye using the y-axis coordinate difference of the left eye. When gfve_pupil_pre_residual_right_eye_dy_coordinate_sign_flag is not present, it is inferred to be equal to 0.

When gfve_pupil_coordinate_present_idx is equal to 3 and gfve_pupil_pred_flag is equal to 1, the variables PreResidualRightPupilCoordinateDeltaX and PreResidualRightPupilCoordinateDeltaY indicating the residual in predicting the delta x-axis and delta y-axis coordinate of the right eye using the delta x-axis and delta y-axis coordinate of the left eye, respectively, are derived as follows:

PreResidualRightPupilCoordinateDeltaX = ( 1 − 2 * gfve_pupil_pre_residual_right_eye_dx_coordinate_sign_flag ) * 
		gfve_pupil_pre_residual_right_eye_dx_coordinate_abs ÷ ( 1 << ( gfve_coordinate_precision_factor_minus1 + 
		1 ) ) 
PreResidualRightPupilCoordinateDeltaY = ( 1 − 2 * gfve_pupil_right_eye_dy_coordinate_sign_flag ) * 
		gfve_pupil_pre_residual_right_eye_dy_coordinate_abs ÷ ( 1 << ( gfve_coordinate_precision_factor_minus1+ 
		1 ) )

gfve_pupil_left_eye_x_coordinate_abs indicates the absolute value of the normalized value of the x-axis coordinate of the left eye.

gfve_pupil_left_eye_x_coordinate_sign_flag specifies the sign of the value of the x-axis coordinate of the left eye. When gfve_pupil_left_eye_x_coordinate_sign_flag is not present, it is inferred to be equal to 0.

gfve_pupil_left_eye_y_coordinate_abs indicates the absolute value of the normalized value of the y-axis coordinate of the left eye.

gfve_pupil_left_eye_y_coordinate_sign_flag specifies the sign of the value of the y-axis coordinate of the left eye. When gfve_pupil_left_eye_y_coordinate_sign_flag is not present, it is inferred to be equal to 0.

When gfve_pupil_coordinate_present_idx is equal to 1 or 3 and gfve_pupil_pred_flag is equal to 0, the variables LeftPupilCoordinateX and LeftPupilCoordinateY indicating the x-axis coordinate and y-axis coordinate of the left pupil center position, respectively, are derived as follows:

LeftPupilCoordinateX = ( 1 − 2 * gfve_pupil_left_eye_x_coordinate_sign_flag ) * 
		gfve_pupil_left_eye_x_coordinate_abs ÷ ( 1 << ( gfve_coordinate_precision_factor_minus1 + 1 ) ) 
LeftPupilCoordinateY = ( 1 − 2 * gfve_pupil_left_eye_y_coordinate_sign_flag ) * 
		gfve_pupil_left_eye_y_coordinate_abs ÷ ( 1 << ( gfve_coordinate_precision_factor_minus1 + 1 ) )

gfve_pupil_right_eye_x_coordinate_abs indicates the absolute value of the normalized value of the x-axis coordinate of the right eye.

gfve_pupil_right_eye_x_coordinate_sign_flag specifies the sign of the value of the x-axis coordinate of the right eye. When gfve_pupil_right_eye_x_coordinate_sign_flag is not present, it is inferred to be equal to 0.

gfve_pupil_right_eye_y_coordinate_abs indicates the absolute value of the normalized value of the y-axis coordinate of the right eye.

gfve_pupil_right_eye_y_coordinate_sign_flag specifies the sign of the value of the y-axis coordinate of the right eye. When gfve_pupil_right_eye_y_coordinate_sign_flag is not present, it is inferred to be equal to 0.

When gfve_pupil_coordinate_present_idx is equal to 2 or 3 and gfve_pupil_pred_flag is equal to 0, the variables RightPupilCoordinateX and RightPupilCoordinateY indicating the x-axis coordinate and y-axis coordinate of the right pupil center position, respectively, are derived as follows:

RightPupilCoordinateX = ( 1 − 2 * gfve_pupil_right_eye_x_coordinate_sign_flag ) * 
		gfve_pupil_right_eye_x_coordinate_abs ÷ ( 1 << ( gfve_coordinate_precision_factor_minus1 + 1 ) ) 
RightPupilCoordinateY = ( 1 − 2 * gfve_pupil_right_eye_y_coordinate_sign_flag ) * 
		gfve_pupil_right_eye_y_coordinate_abs ÷ ( 1 << ( gfve_coordinate_precision_factor_minus1 + 1 ) )

When gfve_pupil_pred_flag is equal to 1, the variables LeftPupilCoordinateX, LeftPupilCoordinateY RightPupilCoordinateX and RightPupilCoordinateY are derived as follows:

if (gfve_gfv_cnt == 0){ 
	if (gfve_pupil_coordinate_present_idx == 1 || gfve_pupil_coordinate_present_idx == 3){ 
		LeftPupilCoordinateX = LeftPupilCoordinateDeltaX + BaseRightPupilCoordinateX
		LeftPupilCoordinateY = LeftPupilCoordinateDeltaY + BaseRightPupilCoordinateY
	}
	if(gfve_pupil_coordinate_present_idx == 2){ 
		RightPupilCoordinateX = RightPupilCoordinateDeltaX + BaseLeftPupilCoordinateX
		RightPupilCoordinateY = RightPupilCoordinateDeltaY + BaseLeftPupilCoordinateY
	}
	if(gfve_pupil_coordinate_present_idx == 3){ 
		RightPupilCoordinateX = PrevResidualRightPupilCoordinateDeltaX + LeftPupilCoordinateDeltaX +
				 BaseRightPupilCoordinateX
		RightPupilCoordinateY = PrevResidualRightPupilCoordinateDeltaY + LeftPupilCoordinateDeltaY +
				 BaseRightPupilCoordinateX
	}
} else {
	if (gfve_pupil_coordinate_present_idx == 1 || gfve_pupil_coordinate_present_idx == 3){ 
		LeftPupilCoordinateX = LeftPupilCoordinateDeltaX + PrevLeftPupilCoordinateX
		LeftPupilCoordinateY = LeftPupilCoordinateDeltaY + PrevLeftPupilCoordinateY
	}
	if(gfve_pupil_coordinate_present_idx == 2){ 
		RightPupilCoordinateX = RightPupilCoordinateDeltaX + PrevRightPupilCoordinateX
		RightPupilCoordinateY = RightPupilCoordinateDeltaY + PrevRightPupilCoordinateX
	}
	if(gfve_pupil_coordinate_present_idx == 3){ 
		RightPupilCoordinateX = PrevResidualRightPupilCoordinateDeltaX + LeftPupilCoordinateDeltaX+
				PrevRightPupilCoordinateX
		RightPupilCoordinateY = PrevResidualRightPupilCoordinateDeltaY + LeftPupilCoordinateDeltaY+
				PrevRightPupilCoordinateY
	}
}

The following applies for derivation of the variables PrevLeftPupilCoordinateX, PrevLeftPupilCoordinateY, PrevRightPupilCoordinateX, PrevRightPupilCoordinateY:

PrevLeftPupilCoordinateX = LeftPupilCoordinateX
PrevLeftPupilCoordinateY = LeftPupilCoordinateY
PrevRightPupilCoordinateX = RightPupilCoordinateX
PrevRightPupilCoordinateY = RightPupilCoordinateY

- Example of usage

In this part we demonstrate an example of usage of the pupil information as part of the SEI message. This methodology consists in applying a post-processing method on the images generated by the current GFVC architectures. We rely on the gfvc_v1-AhG16_GFVC_Software_v3 (https://vcgit.hhi.fraunhofer.de/jvet-ahg-gfvc/gfvc_v1 ) and implement the relevant syntax and semantics in it to achieve pupil redirection. We note that that our proposal primarily focuses on transmitting pupil information as part of the SEI message. How this information is utilized at the receiver remains to be explored in future developments. The application could involve post-processing methods as the one proposed here for illustration purposes or a direct integration of this information into the generative network.

Figure 2 - Pupil Information Processing Block Diagram

The block diagram of the post-processing approch used in our experiment is shown in Figure 2. First, a Landmark Detection model is used to obtain the pupil position as well as the corners of the eyes for further processing. In our showcase, the landmark detection module uses Mediapipe. The Pupil Position Coordinates Computation converts the pupil position into the above mentioned coordinate system. As mentioned in [5], the use of landmarks may result in jitter, to avoid such proble, we add a smoothing filter in the architecture. In our showcase, a one-euro filter is used [6] for demonstration. Considering that the pupil's position rarely undergoes significant movement, we explored an adaptive mechanism to further reduce the additional bit rate increase. As shown in Figure 3, by adjusting the gfve_pupil_coordinate_present_idx to 0, 1, 2, or 3, we can selectively skip some of the pupil information, achieving such reduction. The corresponding non-adaptive (or initial) mode refers to transmitting information for both pupils in each frame, which is indicated by gfve_pupil_coordinate_present_idx=3. The Adaptive Analysis model analyzes the input pupil coordinates to determine the gfve_pupil_coordinate_present_idx for the current frame, deciding whether to skip left, righ or both pupil information(s). At the decoder side, the pupil coordinates from the base picture are also extracted for decoding purpose. Additionally, in the Pupil Redirection Processing model, landmarks of the reconstructed driving pictures are also extracted to serve as inputs to the neural network in charge or redirecting the pupils. In our demonstration, we use a simplified version of the FOMM network as redirecting model.

Figure 3 - Adaptive Module Diagram

- Experiments

  - Showcase

In this section, examples are provided to demonstrate the effectiveness of pupil redirection for GFVC. Additional visual examples will be presented during the presentation.

Figure 4 shows the redirection of the pupil positions using adaptive and non-adaptive methods for FV2V considering two different QP values (22 and 37). As shown in Figure 5, for QP values larger than 42 (e.g., 52), generated images by the initial FV2V GFVC model are too degraded to be used in practice. When dealing with such high QP values, it affects the pupil position detection mechanism.

As illustrated in Figure 4, the proposed demonstration using pupil position SEI message to redirect pupils at the receiver allows to achieve more realism and accurate position of the pupils.

Figure 4 - Corrected pupil position by non-adaptive and adaptive methods

Figure 5 - Illustration of the influence of other QP values on the generated video

  - Bitrate consideration

In terms of bitrate increase, we provide in Table 1 some results obtained based on the FV2V method. QF refers to the quantization factor, which is the bit quantization depth indicated by gfve_pupil_coordinate_precision_factor _minus1 plus 1. The table only shows results for QP = 22, as different QP values have only a slight effect on the Increasing Bitrate. From the table, it can be seen that when the adaptive usage mechanism is employed, there is a bitrate reduction of about 1/3 to 1/2 of the Increasing Bitrate. It should be noted that we only use a simple adaptive criterion (when the encoded data for one eye is entirely 0 (x and y values), the data for that eye is not transmitted at all, so we believe that adopting such adaptive usage criterion has great potential for further development in the transmission of pupil information.

Table 1 - The increased number of bits under different QF with adaptive and non-adaptive usage

| Dataset | QP | Original Bitrate (bps) | QF | Adaptive | Increasing Bitrate (bps) |
| --- | --- | --- | --- | --- | --- |
| CFVQA | 22 | 15856.53 | 3 | OFF | 210.88 |
| CFVQA | 22 | 15856.53 | 3 | ON | 86.72 |
| CFVQA | 22 | 15856.53 | 4 | OFF | 242.0267 |
| CFVQA | 22 | 15856.53 | 4 | ON | 124.5867 |
| CFVQA | 22 | 15856.53 | 5 | OFF | 293.5467 |
| CFVQA | 22 | 15856.53 | 5 | ON | 195.7333 |
| VOXCELEB | 22 | 11632.85 | 3 | OFF | 214.72 |
| VOXCELEB | 22 | 11632.85 | 3 | ON | 90.88 |
| VOXCELEB | 22 | 11632.85 | 4 | OFF | 245.7067 |
| VOXCELEB | 22 | 11632.85 | 4 | ON | 135.8933 |
| VOXCELEB | 22 | 11632.85 | 5 | OFF | 296.6933 |
| VOXCELEB | 22 | 11632.85 | 5 | ON | 205.6 |

- Conclusion

This contribution revises the pupil information transmission scheme proposed in JVET-AH0138[1]. Following recommendations from the previous meeting, we integrated our syntax elements into GFVE and introduced new elements to predict the right eye using data from the left eye for bitrate reduction. We also provide an example of usage of the pupil position, which is implemented and built upon the current gfvc_v1-AhG16_GFVC_Software_v3. In this implementation, we added an adaptive mechanism that, as demonstrated by experimental results, significantly reduces the bitrate of the initial approach while maintaining its quality gain. It is proposed to consider the inclusion of the pupil position information as part of the GFVE SEI message. Furthermore, it is proposed that technical solutions using pupil position SEI message for GFVC be further studied in AHG16.

- References

- A.Trioux, Y.Yao, F.Ma, F.Yang, F.Xing, Z.Wang. “AHG9/AHG16: Pupil position SEI message for Generative Face Video” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no.JVET-AH0138, April 2024.

- F.Ma, A.Trioux, Y.Gao, Y.Yao, F.Yang, F.Xing, Z.Wang. “AHG9/AHG16: Update on Pupil position SEI message for Generative Face Video” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no.JVET-AI0137, July 2024.

- Y.-K.Wang., K.Yang, Y.Li, Y.Xu. “AHG9: On GFV facial parameters signalling,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AI0191, July 2024.

- S. McCarthy, J. Chen, S. Deshpande, M. Hannuksela, Hendry. “Technologies under consideration for future extensions of VSEI (version 3)” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AG2032, January 2024.

- A.Siarohin, S.Lathuilière, S.Tulyakov, et al. First order motion model for image animation[J]. Advances in Neural Information Processing Systems, 2019, 32.

- G.Casiez, N.Roussel, & D.Vogel. "1€ filter: a simple speed-based low-pass filter for noisy input in interactive systems." Proceedings of the SIGCHI Conference on Human Factors in Computing Systems. 2012.

- Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Hisense Visual Tech. Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).