| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0137-v3 |
| --- | --- |

| Title: | AHG15: Scripts for verification of auxiliary information for gaming content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Johannes Sauer Timofey Solovyev | Email: | johannes.sauer@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

As part of the investigation of gaming content coding in AhG15 a variety of auxiliary information has been collected. This includes auxiliary meta data (e.g. camera parameters) and auxiliary video data (e.g. depth maps and motion maps). The format of this data is described in JVET-AI2027 and it has been made available on the JVET server. In order to facilitate further study of the auxiliary data this contribution provides stand-alone python scripts that can be used to verify the correctness of the auxiliary information. It is proposed to add those scripts to the software repositories of AhG15 and to maintain their functionality if the format of the auxiliary information should be modified.

# Introduction

## Usage of motion maps

The auxiliary information includes 32-bits floating point motion maps, where the resolution is the same as that of the video, i.e. motion information for each pixel is provided. Further, this motion information is provided relative to the previous frame. Hence, the displacement of the current pixel pc relative to the pixel in the previous frame pp, expressed in pixel is:

mvEngine=pc-pp

The motion maps can be used to warp the content of the previous frame to the current frame. A script to do this is provided along with this contribution. An example usage is as follows:

get_frame_by_motion_vectors.py --input_yuv SunTemple3_SDR_1920x1080_60fps_10bit_420.yuv --first_exr_file SunTemple_motion_depth_frame0.exr --output_yuv SunTemple3_SDR_1920x1080_60fps_10bit_420_exrmv.yuv --type huawei

## Usage of depth and camera information

The auxiliary information includes 32-bits floating point depth maps, where the resolution is the same as that of the video, i.e. depth information for each pixel is provided. Further, camera parameters (intrinsic and extrinsic) for each frame of the video are provided. Combined, this information can be used to warp the content of another frame to the current frame. A script to do this is provided along with this contribution. An example usage is as follows:

get_frame_by_reprojection.py --input_yuv Darktree_SDR_1920x1080_60fps_10bit_420.yuv --first_exr_file DarkTree_motion_depth_frame0.exr --cam_params cameraParam.txt --output_yuv Darktree_SDR_1920x1080_60fps_10bit_420_reproj.yuv --type interdigital

## Examples of warped frames

Figure 1 and Figure 2 show examples of the output of the script described above. The format of auxiliary information for sequences from Interdigital and Huawei is slightly different and should be aligned further, such that no “type” needs to be specified when using the auxiliary information. Figure 1 shows an example for the sequence (and auxiliary information) ARPG2 provided by Huawei, while Figure 2 shows an example for the sequence (and auxiliary information) Darktree provided by Interdigital. These images have also been included as separate files in this contribution.

| Original | Warped using auxiliary motion information | Warped using auxiliary depth and camera |
| --- | --- | --- |
|  | Difference to original | Difference to original |

Figure 1: Warped frame for the sequence ARPG2. Left: original frame 9. Middle: Frame 8 warped to frame 9 using per-pixel motion information. Right: Frame 8 warped to frame 9 by re-projection using depth and camera information.

| Original | Warped using auxiliary motion information | Warped using auxiliary depth and camera |
| --- | --- | --- |
|  | Difference to original | Difference to original |

Figure 2: Warped frame for the sequence Darktree. Left: original frame 7. Middle: Frame 6 warped to frame 7 using per-pixel motion information. Right: Frame 6 warped to frame 7 by re-projection using depth and camera information.

# Patent rights declaration(s)

Huawei Technologies Co. does not have any current or pending patent rights relating to the technology described in this contribution.