| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0138-v3 |
| --- | --- |

| Title: | EE2-5.1: Adaptive Mantissa Precision for ALF Coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vladislav Shchukin, Per Wennersten, Jacob Ström | Email: | vladislav.shchukin@ericsson.com per.wennersten@ericsson.com jacob.strom@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

This contribution proposes to restrict allowed values of ALF coefficients to the elements of set {m⋅2b}, where m∈{-1, 0, 1} or m∈{-3, -2, -1, 0, 1, 2, 3}, and b is a non-negative integer. The coefficients are derived using a simulated annealing approach and signaled into APS using Huffman coding.

The proposed algorithm is implemented on the top of ECM-14.0. According to simulation results under common test conditions (CTC), BD-rate PSNR impact, encoder/decoder averaged run time and averaged peak memory consumption for Test 5.1e is reported in the table below.

|  | EE2-5.1e over ECM-14.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| All Intra | -0.03% | -0.01% | 0.01% | 100.2% | 99.9% | 100.3% | 100.0% |
| Random Access | -0.05% | -0.14% | -0.09% | 100.5% | 100.7% | 100.2% | 100.3% |
| Low Delay B | -0.07% | -0.23% | -0.64% | 100.2% | 100.9% | 100.2% | 100.0% |

The proposal claims to replace the 9bit x 12bit multiplication that is needed for ALF with internal bit depth 10 by a 3bit x 12bit multiplication and a bit shift, and it is argued that this reduces complexity for hardware implementations.

# Introduction

Adaptive loop filtering (ALF) is a filter designed to reduce quantization errors of the output pictures as well as of the reference pictures used for inter-prediction [1]. ALF is included into Versatile Video Coding (VVC), also known as H.266, developed by the Joint Video Experts Team (JVET). ALF is also included into ECM-14.0 [2] as one of the stages in loop filtering as showed in Figure 1. Current proposal presents results of EE2-5.1 exploration experiments based on recent contribution JVET-AI0081 [5]. The following five tests were conducted:

| # | Test |
| --- | --- |
| 5.1a | Restriction of ALF coefficient values to form {m⋅2b} + Huffman-based coding |
| 5.1b | Restriction of ALF coefficient values to form {m⋅2b} + modified Golomb-Rice coding |
| 5.1c | Simulated annealing to derive ALF coefficients (encoder-only) |
| 5.1d | Test 5.1c + {m⋅2b} coefficient values restriction (encoder-only) |
| 5.1e | Test 5.1a + Test 5.1c |

# Background

Let R(x, y) be an output sample of the decoding stage preceding ALF, i.e. sample at position (x, y) outputted by combination of SAO, CC-SAO and BIF (see Figure 1), the ALF output R(x, y) is defined as

Rx, y=Rx, y+ i=0ncifix,y,    ( SEQ Equation \* ARABIC 1)

where ci is an i-th coefficient and fi(x,y) is an i-th tap for position (x, y). For luma samples, ECM-14.0 [2] utilizes ALF with the filtering shape depicted in Figure 2 with 40 taps, where the first 20 taps (taps 0-19) are samples in the neighboring pixels (spatial taps) and the other taps have more complex nature and include taps based on the output of the fixed filters, taps based on the input for deblocking filter (DBF), taps based on the reconstructed residuals.

DBF

SAO / CC-SAO

BIF

ALF / CC-ALF

Input samples

Output samples

Figure 1. In-loop filtering in ECM-14.0

Figure 2. ALF shape in ECM-14.0

The coefficients ci are derived by the encoder. The derivation used in ECM-14.0 can be split into several parts. Firstly, the optimal real-valued coefficients are obtained by solving the Wiener-Hopf equations to minimize the mean square error (MSE) between the original picture and the filtered picture. Secondly, the coefficients are quantized, namely, multiplied by a constant 2BALF-1, rounded to integers and clipped into the interval [-2BALF-1+1, 2BALF-1-1]. The parameter BALF is called the precision. Finally, an optimization procedure based on coordinate search is launched: at each iteration, each coefficient is tried to be increased or decreased by 1, the best (by MSE) change of a coefficient is accepted, and the iterations continue as long as there is an improvement of MSE. The actual filtering is done by formula (1) in integers and right bit shifting is utilized to properly scale the ALF offset i=0ncifix,y.

Quantized counterparts of coefficients ci are signaled in a special header called adaptation parameter set (APS). In ECM-14.0, the coefficients are split into two groups (0-33 and 34-39) and different Golomb-Rice codes for different groups are used to convert the coefficients into the APS bitstream. For each group, four parameters of Golomb-Rice coding are iterated, and the best (by total occupied bit volume) choice is signaled in APS.

In ECM-14.0, the precision parameter BALF is fixed to be 8 for the chroma components but is adaptive to luma component.  Namely, the values 6, 7, 8 and 9 are tried, and the best (by RD-cost) choice is signaled in the APS.

# Set of Allowed Coefficient Values

Let BALF be ALF coefficients precision and denote the mantissa precision by BM. In the solution proposed in this contribution, the quantized ALF coefficients are limited to the elements of the set CBALF,BM={m⋅2b}, where m and b are integers, m∈[-2BM+1, 2BM-1] and b∈[0, BALF-1], moreover, m⋅2b belongs to the interval [-2BALF-1, 2BALF-1]. For example, C6,1={-32,-16,-8,-4,-2,-1,0,1,2,4,8,16,32}, C7,2={-64, -48,-32,-24,-16,-12,-8,-6,-4,-3,-2,-1,0,1,2,3,4,6,8,12,16,24,32,48,64}. This restriction is similar to that proposed in JVET-Q0167 test 1 [4].

For each configuration, the range of allowed mantissa precisions is hardcoded. For the chroma components, mantissa precision BM=1 is utilized, i.e., only pure powers of two are allowed, but for the luma component, the mantissa precision is configured as BM∈{1,2}. For luma, the encoder signals one bit in the APS to indicate which mantissa precision is used, BM=1 or BM=2. The choice of BM value for luma is based on RD-cost.

The purpose of such a limitation of the coefficient values is to decrease the complexity of ALF when implemented in hardware. Similar ideas were presented to JVET, for example, in [3-4], however, resulting in small coding loses. In this contribution, like in [5] and opposite to [3-4], bit-rate savings are provided.

This restriction on coefficient values is applied in tests 5.1a-b, d-e.

# Signaling of Coefficients

The family of prefix-free binary codes is constructed to cover the full range of acceptable configurations defined by the following tuple: (luma/chroma, BALF, BM, coefficients group). Coefficients are split into the groups in the same way as it is done in ECM-14.0: for example, for luma component, coefficients 0-33 belong to group 0 and coefficients 34-39 belong to group 1.

The constructed binary codes are based on Huffman coding. A Huffman code is completely defined by the alphabet of symbols (e.g., allowed ALF coefficient values) and by the probabilities or, equivalently, frequencies of the alphabet symbols. For example, in the proposed algorithm, for tuple (luma, 6, 1, 0), the alphabet is C6,1=-32,-16,-8,-4,-2,-1,0,1,2,4,8,16,32, and the Huffman code is built from frequencies { 3,4,44,451,1625,1738,4212,1739,2151,861,153,10,2}, where the frequency of -32 is 3, of -16 is 4, of -8 is 44, of -4 is 451 and so on. The frequencies are collected by compression of BVI-DVC sequences using ECM-based encoder. In total, 18 Huffman codes (16 for luma and 2 for chroma) are included in the proposal.

Huffman-based signaling is used in tests 5.1a and 5.1e.

In tests 5.1c and 5.1d the signaling method of ECM-14.0 (based in Golomb-Rice coding) is preserved.

Modified Golomb-Rice coding is used in test 5.1b. In this test, the signaling method utilizes the knowledge of tuple (luma/chroma, BALF, BM, coefficients group) as follows: the absolute value of a coefficient is replaced by the corresponding index in the alphabet subarray of positive elements in increasing order. For example, for (luma, 6, 1, 0) coefficient values -32,-16,-8,-4,-2,-1,0,1,2,4,8,16,32 are mapped to -6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6, correspondingly. Afterwards, Golomb-Rice coding method from ECM-14.0 is applied to the mapped counterparts.

# Coefficient Derivation

To improve the search of the ALF coefficients, especially in the case when ECM-14.0 is combined with the coefficient values limitation proposed in Section 3, a simulated annealing-based search is introduced. A practical implementation of ALF, such as the implementation in ECM-14.0, works with quantized coefficients and is moreover limited to a certain range of values. This is why solving the Wiener-Hopf equations to find the optimal real-valued coefficients and simply quantizing them to the nearest integer does not result in optimal quantized coefficients. There is a variety of discrete optimization methods to approximate the optimal coefficients. In ECM-14.0, a deterministic coordinate descent approach is used, however, probabilistic methods such as simulated annealing can often demonstrate an advantage in searching for a solution in reasonable time.

Let C=(c0,c1,…,cn) denote the filter coefficients and F(C) is an objective function. We will use a simulated annealing approach to minimize F(C), where C is the subject for optimization. It starts from an initial solution C0  and continues for a number of iterations. At the i-th iteration, a random change to the input solution Ci is applied, resulting in a temporary solution Ci'. In the proposal, it is done in the following way: 3 random coefficients ck1,ck2, ck3 are chosen, and each of them is randomly increased or decreased by 1, i.e. ckj'=ckj+rand({-1,1}). Then, the objective function Fi(C) is calculated and one needs to decide whether to accept this random change or not. In the proposal, the acceptance probability is calculated as follows:

Pi,Ci,Ci'=11+exp-(i+1)2⋅FiCi-FiCi'.

Then, a random generator is used to make a decision:

Ci+1=Ci' with probability Pi,Ci,Ci',Ci, otherwise.

Note that at the last few iterations (i→∞), only solutions which decrease the value of objective function Fi(Ci) are accepted, but at the first few iterations (in particular, i=0), solutions that increase the value of the objective function might be accepted.

In the current optimization procedure in ECM-14.0, the objective function for the deterministic coordinate descent is equal to the MSE between the original picture and the filtered picture. In the proposed solution, FiC=MSE(C)+λi⋅Rate(C), where MSE(C) is the same MSE between the original picture and the filtered picture, Rate(C) is the number of bits required to be signaled in APS, and λi is defined as follows:

λi=λ⋅(21+110i+1-1),

i.e. at the last few iterations (i→∞), the objective function tends to RD-cost, but at the first few iterations, the objective function is closer to MSE. If the number of iterations exceeds a preconfigured limit imax or if there are no newly accepted solutions for a while (after 8⋅n+1 iterations in a row), the simulated annealing procedure is stopped, and the last solution is taken as output of the optimization procedure. The process of the simulated annealing is depicted in Figure 3.

The simulated annealing process is launched several times with different initial solutions. First, the existing coordinate descent optimization method from ECM-14.0 is run, and the first simulated annealing run is using that as a starting point. Then simulated annealing is run 39 more times, where each run takes the output of the previous run as its starting point.

A random number generator is extensively utilized in the simulated annealing approach. There is a variety of pseudo-random generators that can be used, for example, Xorshift. To provide an encoder reproducibility, we use the same hardcoded pseudo-random number generator with a fixed seed for each simulated annealing launch.

Simulated annealing is applied in tests 5.1c-e.

Initial solution: C0. nskip=0, i=0

i+=1. i-th iteration gets input Ci

C0

Ci' is a random change of Ci

rand0,1<Pi,Ci,Ci'

Accept: Ci+1=Ci', nskip=0

Yes

Skip: Ci+1=Ci, nskip+=1

No

nskip≥nskipmax or i≥imax

Output solution: Ci+1

Yes

No

Figure 3. Simulated annealing algorithm

# ALF Complexity Reduction for Hardware Implementations

For practical purposes, hardware implementation of multiplication of N1-bit integer and N2-bit integer can in general be assumed to take ON1N2 logical gates. However, if the values of N2-bit integers are limited to pure powers of two, then multiplication becomes left bit shifting which can be implemented using the barrel shifter and resulting in complexity of O(N1log2N2) logical gates.

In our particular case, we are replacing a 9bit×12bit multiplication by a 3bit×12bit multiplication plus a shift. Exactly how much complexity is saved in a hardware implementation by doing this may depend on many things, such as what process is used etc. However, we believe that it should still be clear that the proposed method should be significantly smaller than a 9bit×12bit multiplication.

For software implementations, there is likely no complexity reduction for the multiplication itself, given that several 9bit×12bit multiplications can often already be executed in a single clock cycle.

# Simulation Results

The proposal is implemented on top of ECM-14.0 and evaluated under the CTC. In the tables below, all reported runtimes (both EncT and DecT) are reliable. To estimate EncT/DecT in similar conditions, the anchor and the test are simulated on the same machine in parallel, and the total number of launched simulations on each machine is at most the number of physical cores on this machine. To avoid irrelevant costs during DecT estimation, the following settings of decoder are switched off: checking of MD5 sums, writing of YUV-output. DecT is averaged over 5 launches. Simulation environment: AMD EPYC 9354 + 1500GB RAM or similar; GCC 13.2.0; OS: RHEL 8; AVX2.

The following tables show the simulation results for all five EE2 tests 5.1a-e.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EE2-5.1a: ALF coefficient values in form {m⋅2b} + Huffman-based coding |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |
| Class A1 | -0.03% |  | 0.00% |  |  | 0.06% |  | 101.2% |  | 100.7% |  | 100.8% |  | 100.0% |
| Class A2 | 0.01% |  | 0.01% |  |  | 0.11% |  | 100.6% |  | 100.2% |  | 99.5% |  | 100.0% |
| Class B | 0.00% |  | -0.06% |  |  | -0.01% |  | 100.7% |  | 99.7% |  | 100.2% |  | 100.0% |
| Class C | -0.02% |  | 0.08% |  |  | 0.13% |  | 100.4% |  | 99.5% |  | 100.4% |  | 100.0% |
| Class E | -0.02% |  | 0.12% |  |  | 0.05% |  | 100.9% |  | 100.4% |  | 99.2% |  | 100.0% |
| Overall | -0.01% |  | 0.02% |  |  | 0.06% |  | 100.7% |  | 100.0% |  | 100.0% |  | 100.0% |
| Class D | -0.04% |  | -0.21% |  |  | -0.41% |  | 100.1% |  | 100.7% |  | 100.5% |  | 100.0% |
| Class F | -0.05% |  | -0.14% |  |  | -0.17% |  | 100.3% |  | 99.6% |  | 100.3% |  | 100.1% |
| Class TGM | -0.04% |  | -0.05% |  |  | -0.02% |  | 99.9% |  | 100.1% |  | 102.6% |  | 99.8% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1a: ALF coefficient values in form {m⋅2b} + Huffman-based coding |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |
| Class A1 | -0.03% |  | -0.24% |  |  | -0.18% |  | 100.4% |  | 100.9% |  | 100.2% |  | 100.4% |
| Class A2 | 0.04% |  | 0.09% |  |  | -0.01% |  | 100.4% |  | 100.8% |  | 100.1% |  | 100.3% |
| Class B | -0.04% |  | -0.16% |  |  | -0.10% |  | 100.6% |  | 100.6% |  | 100.1% |  | 100.2% |
| Class C | 0.03% |  | -0.16% |  |  | 0.12% |  | 100.5% |  | 100.6% |  | 100.2% |  | 100.4% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% |  | -0.12% |  |  | -0.04% |  | 100.5% |  | 100.7% |  | 100.2% |  | 100.3% |
| Class D | 0.06% |  | -0.24% |  |  | -0.05% |  | 100.9% |  | 100.9% |  | 100.6% |  | 100.0% |
| Class F | 0.01% |  | -0.09% |  |  | -0.08% |  | 100.5% |  | 100.6% |  | 100.3% |  | 100.1% |
| Class TGM | -0.11% |  | -0.02% |  |  | -0.03% |  | 100.5% |  | 101.7% |  | 100.2% |  | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1a: ALF coefficient values in form {m⋅2b} + Huffman-based coding |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.05% | 0.03% |  |  | -0.40% |  | 100.2% |  | 101.3% |  | 100.2% |  | 100.6% |  |
| Class C | 0.03% | -0.33% |  |  | -0.32% |  | 99.9% |  | 101.5% |  | 100.4% |  | 99.9% |  |
| Class E | 0.03% | -0.26% |  |  | -0.70% |  | 100.2% |  | 100.5% |  | 99.7% |  | 100.0% |  |
| Overall | 0.00% | -0.17% |  |  | -0.45% |  | 100.1% |  | 101.2% |  | 100.1% |  | 100.2% |  |
| Class D | 0.12% | 0.33% |  |  | -0.14% |  | 100.5% |  | 100.4% |  | 101.1% |  | 100.0% |  |
| Class F | -0.15% | -0.03% |  |  | 0.94% |  | 100.5% |  | 101.7% |  | 101.5% |  | 100.4% |  |
| Class TGM | -0.09% | -0.05% |  |  | -0.01% |  | 100.5% |  | 96.7% |  | 100.8% |  | 100.5% |  |
|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1b: ALF coefficient values in form {m⋅2b} + modified Golomb-Rice coding |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | 0.00% |  | 0.02% |  |  | 0.11% |  | 100.3% |  | 100.3% |  | 102.5% |  | 100.1% |  |
| Class A2 | 0.04% |  | 0.04% |  |  | 0.17% |  | 100.6% |  | 100.1% |  | 97.2% |  | 99.9% |  |
| Class B | 0.03% |  | 0.00% |  |  | 0.07% |  | 100.7% |  | 100.0% |  | 100.2% |  | 100.0% |  |
| Class C | 0.02% |  | 0.17% |  |  | 0.22% |  | 100.3% |  | 99.7% |  | 100.4% |  | 100.0% |  |
| Class E | 0.06% |  | 0.13% |  |  | 0.09% |  | 100.4% |  | 100.9% |  | 99.2% |  | 100.0% |  |
| Overall | 0.03% |  | 0.07% |  |  | 0.13% |  | 100.5% |  | 100.2% |  | 99.9% |  | 100.0% |  |
| Class D | 0.01% |  | -0.05% |  |  | -0.10% |  | 100.5% |  | 99.8% |  | 100.5% |  | 100.0% |  |
| Class F | 0.02% |  | 0.02% |  |  | -0.01% |  | 100.5% |  | 100.0% |  | 101.0% |  | 100.0% |  |
| Class TGM | -0.01% |  | -0.03% |  |  | 0.01% |  | 100.0% |  | 100.2% |  | 100.2% |  | 100.0% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1b: ALF coefficient values in form {m⋅2b} + modified Golomb-Rice coding |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | 0.05% |  | -0.25% |  |  | -0.10% |  | 100.5% |  | 100.3% |  | 100.2% |  | 99.7% |  |
| Class A2 | 0.07% |  | 0.27% |  |  | 0.11% |  | 100.4% |  | 100.6% |  | 100.2% |  | 100.4% |  |
| Class B | 0.04% |  | -0.12% |  |  | -0.03% |  | 100.6% |  | 100.0% |  | 100.2% |  | 99.9% |  |
| Class C | 0.08% |  | -0.04% |  |  | 0.23% |  | 100.7% |  | 100.0% |  | 100.6% |  | 100.1% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.06% |  | -0.05% |  |  | 0.05% |  | 100.6% |  | 100.2% |  | 100.3% |  | 100.0% |  |
| Class D | 0.11% |  | -0.21% |  |  | 0.05% |  | 100.5% |  | 100.0% |  | 100.5% |  | 100.0% |  |
| Class F | 0.08% |  | 0.08% |  |  | 0.01% |  | 100.7% |  | 100.5% |  | 100.2% |  | 99.9% |  |
| Class TGM | -0.06% |  | 0.02% |  |  | -0.03% |  | 100.6% |  | 102.0% |  | 99.6% |  | 100.0% |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1b: ALF coefficient values in form {m⋅2b} + modified Golomb-Rice coding |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  |  | DecVmPeak |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.06% | 0.18% |  |  | -0.17% |  | 100.3% |  | 100.5% |  | 100.2% |  |  | 100.1% |  |
| Class C | 0.19% | 0.12% |  |  | 0.01% |  | 100.4% |  | 100.6% |  | 100.4% |  |  | 99.6% |  |
| Class E | 0.11% | -0.39% |  |  | -0.70% |  | 100.4% |  | 99.3% |  | 99.9% |  |  | 99.7% |  |
| Overall | 0.12% | 0.02% |  |  | -0.24% |  | 100.4% |  | 100.2% |  | 100.2% |  |  | 99.8% |  |
| Class D | 0.26% | 0.96% |  |  | 1.00% |  | 100.7% |  | 99.9% |  | 100.7% |  |  | 100.0% |  |
| Class F | 0.11% | -0.76% |  |  | -0.27% |  | 100.9% |  | 102.1% |  | 101.5% |  |  | 100.1% |  |
| Class TGM | -0.04% | 0.11% |  |  | 0.05% |  | 100.3% |  | 99.7% |  | 101.4% |  |  | 100.7% |  |
|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1c: Simulated annealing to derive ALF coefficients (encoder-only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | -0.01% |  | -0.01% |  |  | -0.02% |  | 100.1% |  | 100.3% |  | 100.1% |  | 100.0% |  |
| Class A2 | -0.01% |  | -0.05% |  |  | -0.03% |  | 100.3% |  | 100.2% |  | 97.9% |  | 100.0% |  |
| Class B | -0.01% |  | -0.02% |  |  | -0.02% |  | 100.3% |  | 99.8% |  | 102.0% |  | 100.0% |  |
| Class C | -0.01% |  | 0.00% |  |  | 0.00% |  | 99.9% |  | 99.8% |  | 100.0% |  | 100.0% |  |
| Class E | -0.02% |  | 0.00% |  |  | 0.02% |  | 100.4% |  | 100.4% |  | 98.9% |  | 100.0% |  |
| Overall | -0.01% |  | -0.01% |  |  | -0.01% |  | 100.2% |  | 100.1% |  | 100.0% |  | 100.0% |  |
| Class D | -0.02% |  | 0.00% |  |  | -0.01% |  | 99.6% |  | 98.9% |  | 100.2% |  | 100.0% |  |
| Class F | -0.02% |  | 0.00% |  |  | -0.04% |  | 100.3% |  | 99.6% |  | 99.2% |  | 100.0% |  |
| Class TGM | -0.02% |  | -0.01% |  |  | -0.01% |  | 100.4% |  | 100.2% |  | 100.0% |  | 99.9% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1c: Simulated annealing to derive ALF coefficients (encoder-only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | 0.00% |  | -0.23% |  |  | -0.02% |  | 100.5% |  | 100.4% |  | 99.9% |  | 99.8% |  |
| Class A2 | 0.00% |  | 0.03% |  |  | 0.05% |  | 100.2% |  | 100.4% |  | 100.0% |  | 100.2% |  |
| Class B | -0.04% |  | -0.07% |  |  | 0.00% |  | 100.7% |  | 100.6% |  | 100.0% |  | 99.9% |  |
| Class C | -0.01% |  | -0.10% |  |  | 0.11% |  | 100.9% |  | 100.4% |  | 100.4% |  | 100.1% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% |  | -0.09% |  |  | 0.04% |  | 100.6% |  | 100.5% |  | 100.1% |  | 100.0% |  |
| Class D | -0.06% |  | -0.09% |  |  | -0.18% |  | 100.6% |  | 100.3% |  | 100.2% |  | 100.0% |  |
| Class F | 0.02% |  | -0.03% |  |  | 0.12% |  | 100.7% |  | 100.9% |  | 100.1% |  | 100.0% |  |
| Class TGM | -0.03% |  | -0.01% |  |  | -0.08% |  | 100.8% |  | 101.5% |  | 99.5% |  | 99.8% |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1c: Simulated annealing to derive ALF coefficients (encoder-only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  |  | DecVmPeak |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | 0.22% |  |  | -0.11% |  | 100.4% |  | 100.5% |  | 100.0% |  |  | 100.2% |  |
| Class C | 0.05% | 0.11% |  |  | -0.05% |  | 100.9% |  | 100.2% |  | 100.0% |  |  | 99.7% |  |
| Class E | 0.02% | 0.04% |  |  | -0.39% |  | 101.2% |  | 101.2% |  | 98.8% |  |  | 99.6% |  |
| Overall | 0.03% | 0.13% |  |  | -0.16% |  | 100.8% |  | 100.5% |  | 99.7% |  |  | 99.9% |  |
| Class D | 0.04% | -0.37% |  |  | -0.09% |  | 100.4% |  | 99.2% |  | 100.2% |  |  | 100.0% |  |
| Class F | -0.05% | -1.11% |  |  | -0.87% |  | 101.0% |  | 101.8% |  | 100.6% |  |  | 100.4% |  |
| Class TGM | -0.07% | -0.14% |  |  | -0.13% |  | 100.4% |  | 97.5% |  | 100.3% |  |  | 100.5% |  |
|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1d: EE2-5.1c + {m⋅2b} coefficient values restriction (encoder-only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | 0.00% |  | 0.03% |  |  | 0.11% |  | 100.7% |  | 100.0% |  | 101.7% |  | 100.0% |  |
| Class A2 | 0.02% |  | 0.05% |  |  | 0.12% |  | 101.0% |  | 100.2% |  | 98.4% |  | 99.9% |  |
| Class B | 0.03% |  | 0.00% |  |  | 0.03% |  | 100.8% |  | 100.2% |  | 102.2% |  | 100.0% |  |
| Class C | 0.03% |  | 0.16% |  |  | 0.18% |  | 100.5% |  | 99.7% |  | 100.4% |  | 100.0% |  |
| Class E | 0.07% |  | 0.15% |  |  | 0.07% |  | 100.5% |  | 100.6% |  | 99.2% |  | 100.0% |  |
| Overall | 0.03% |  | 0.07% |  |  | 0.10% |  | 100.7% |  | 100.1% |  | 100.6% |  | 100.0% |  |
| Class D | 0.01% |  | 0.07% |  |  | 0.08% |  | 99.8% |  | 99.4% |  | 100.6% |  | 100.0% |  |
| Class F | 0.02% |  | 0.07% |  |  | 0.14% |  | 100.6% |  | 99.6% |  | 102.2% |  | 100.0% |  |
| Class TGM | -0.03% |  | -0.06% |  |  | -0.02% |  | 100.7% |  | 99.7% |  | 101.4% |  | 99.9% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1d: EE2-5.1c + {m⋅2b} coefficient values restriction (encoder-only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | 0.02% |  | -0.03% |  |  | -0.02% |  | 100.9% |  | 100.1% |  | 100.0% |  | 100.4% |  |
| Class A2 | 0.07% |  | 0.10% |  |  | 0.07% |  | 100.7% |  | 99.7% |  | 99.9% |  | 100.0% |  |
| Class B | 0.02% |  | 0.10% |  |  | 0.02% |  | 101.0% |  | 100.2% |  | 100.1% |  | 100.0% |  |
| Class C | 0.08% |  | 0.03% |  |  | 0.12% |  | 101.2% |  | 99.9% |  | 100.2% |  | 100.0% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.05% |  | 0.05% |  |  | 0.05% |  | 101.0% |  | 100.0% |  | 100.1% |  | 100.1% |  |
| Class D | 0.13% |  | 0.04% |  |  | 0.03% |  | 100.5% |  | 100.1% |  | 100.7% |  | 100.0% |  |
| Class F | 0.10% |  | 0.01% |  |  | 0.12% |  | 101.0% |  | 100.1% |  | 100.2% |  | 99.9% |  |
| Class TGM | -0.06% |  | -0.03% |  |  | -0.09% |  | 101.1% |  | 101.3% |  | 99.9% |  | 100.3% |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1d: EE2-5.1c + {m⋅2b} coefficient values restriction (encoder-only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  |  | U | V |  | EncT |  | DecT |  | EncVmPeak |  |  | DecVmPeak |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.08% |  |  | 0.01% | -0.35% |  | 100.5% |  | 99.8% |  | 100.2% |  |  | 100.0% |  |
| Class C | 0.13% |  |  | 0.01% | -0.04% |  | 100.8% |  | 100.2% |  | 100.4% |  |  | 99.7% |  |
| Class E | -0.02% |  |  | 0.61% | -0.49% |  | 100.9% |  | 99.9% |  | 99.8% |  |  | 99.7% |  |
| Overall | 0.07% |  |  | 0.16% | -0.28% |  | 100.7% |  | 100.0% |  | 100.2% |  |  | 99.8% |  |
| Class D | 0.18% |  |  | 0.40% | 0.54% |  | 100.1% |  | 100.0% |  | 101.1% |  |  | 100.0% |  |
| Class F | 0.03% |  |  | 0.06% | 0.23% |  | 101.4% |  | 99.9% |  | 101.4% |  |  | 100.2% |  |
| Class TGM | -0.08% |  |  | 0.09% | 0.04% |  | 100.8% |  | 96.2% |  | 100.1% |  |  | 100.5% |  |
|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1e: EE2-5.1a + EE2-5.1c |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | -0.04% |  | -0.05% |  |  | 0.02% |  | 99.9% |  | 100.4% |  | 101.7% |  | 100.1% |  |
| Class A2 | -0.02% |  | -0.03% |  |  | 0.05% |  | 100.0% |  | 99.9% |  | 99.6% |  | 100.0% |  |
| Class B | -0.03% |  | -0.08% |  |  | -0.07% |  | 100.5% |  | 100.0% |  | 101.2% |  | 100.1% |  |
| Class C | -0.03% |  | 0.04% |  |  | 0.08% |  | 100.1% |  | 99.7% |  | 100.4% |  | 100.0% |  |
| Class E | -0.06% |  | 0.13% |  |  | 0.00% |  | 100.1% |  | 99.8% |  | 98.1% |  | 99.9% |  |
| Overall | -0.03% |  | -0.01% |  |  | 0.01% |  | 100.2% |  | 99.9% |  | 100.3% |  | 100.0% |  |
| Class D | -0.07% |  | -0.25% |  |  | -0.45% |  | 99.7% |  | 100.4% |  | 100.7% |  | 100.0% |  |
| Class F | -0.09% |  | -0.16% |  |  | -0.10% |  | 100.1% |  | 99.9% |  | 100.6% |  | 99.9% |  |
| Class TGM | -0.08% |  | -0.09% |  |  | -0.05% |  | 100.1% |  | 99.9% |  | 102.7% |  | 99.8% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1e: EE2-5.1a + EE2-5.1c |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | -0.05% |  | -0.37% |  |  | -0.23% |  | 100.4% |  | 101.0% |  | 100.1% |  | 100.3% |  |
| Class A2 | -0.07% |  | 0.08% |  |  | -0.15% |  | 100.4% |  | 100.1% |  | 100.0% |  | 100.3% |  |
| Class B | -0.09% |  | -0.24% |  |  | -0.07% |  | 100.5% |  | 101.0% |  | 100.1% |  | 100.3% |  |
| Class C | 0.00% |  | 0.01% |  |  | 0.04% |  | 100.8% |  | 100.6% |  | 100.6% |  | 100.5% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.05% |  | -0.14% |  |  | -0.09% |  | 100.5% |  | 100.7% |  | 100.2% |  | 100.3% |  |
| Class D | -0.02% |  | -0.14% |  |  | -0.16% |  | 100.7% |  | 100.6% |  | 100.8% |  | 100.1% |  |
| Class F | -0.05% |  | -0.32% |  |  | -0.30% |  | 100.8% |  | 100.6% |  | 100.2% |  | 100.0% |  |
| Class TGM | 0.07% |  | 0.07% |  |  | -0.08% |  | 100.7% |  | 101.6% |  | 100.3% |  | 99.9% |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EE2-5.1e: EE2-5.1a + EE2-5.1c |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.09% |  | -0.36% |  |  | -0.33% |  | 100.2% |  | 101.3% |  | 100.2% |  | 100.4% |  |
| Class C | 0.09% |  | -0.10% |  |  | -0.50% |  | 100.0% |  | 100.6% |  | 100.4% |  | 99.8% |  |
| Class E | -0.25% |  | -0.20% |  |  | -1.32% |  | 100.6% |  | 100.8% |  | 99.8% |  | 99.7% |  |
| Overall | -0.07% |  | -0.23% |  |  | -0.64% |  | 100.2% |  | 100.9% |  | 100.2% |  | 100.0% |  |
| Class D | 0.03% |  | 0.51% |  |  | -0.13% |  | 100.3% |  | 100.6% |  | 100.8% |  | 100.1% |  |
| Class F | -0.15% |  | -0.51% |  |  | 0.67% |  | 101.0% |  | 100.9% |  | 101.0% |  | 100.3% |  |
| Class TGM | 0.01% |  | 0.00% |  |  | -0.06% |  | 101.3% |  | 94.7% |  | 100.4% |  | 100.3% |  |

# Conclusion

Test 5.1e combines the ALF coefficient values restriction, Huffman-based signaling and simulated annealing approach for coefficient derivation. It provides not only hardware complexity reduction, but also BD-rate improvements. We recommend test 5.1e for adoption into the next version of ECM.

Comparing test 5.1a against test 5.1b one can conclude that Huffman-based coding is more effective than Golomb-Rice-based coding when the coefficient values restriction is introduced.

Comparing test 5.1c over ECM-14.0 and test 5.1e over test 5.1a, one can conclude that the impact of simulated annealing is higher when the coefficient values restriction is introduced.

Comparing test 5.1e and test 5.1d, one can conclude that APS syntax change is essential when the coefficient values restriction is introduced.

# References

- C. -Y. Tsai et al., "Adaptive Loop Filtering for Video Coding," in IEEE Journal of Selected Topics in Signal Processing, vol. 7, no. 6, pp. 934-945, Dec. 2013, doi: 10.1109/JSTSP.2013.2271974.

- "Algorithm description of Enhanced Compression Model 14 (ECM 14)," M. Coban, R.-L. Liao,K. Naser, J. Ström, L. Zhang, JVET-AI2025, 35th Meeting, Sapporo, JP, 12–19 July 2024.

- "CE2: Alf with Multiplication Replaced by Bit-Shifting (Test 2.5.1)," S. Esenlik, B. Wang, H. Gao, A. M. Kotra, J. Chen (Huawei), JVET-L0162, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- "Non-CE5: Multiplication simplification for ALF and CC-ALF," J. Ström, Z. Zhang, K. Andersson (Ericsson), JVET-Q0167, 17th Meeting: Brussels, BE, 7–17 January 2020.

- “AHG12: Adaptive Mantissa Precision for ALF Coefficients,” V. Shchukin, P. Wennersten, J. Ström (Ericsson), JVET-AI0081, 35th Meeting, Sapporo, JP, 12–19 July 2024.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).