| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0142-v1 |
| --- | --- |

| Title: | AHG9: On potential conflicts between NNPFs and other post-processings |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Claire-Hélène Demarty, Edouard François, Olivier Le Meur, Franck Aumont | Tel: Email: | 1stname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

Several Neural Network post filter (NNPF) purposes defined in the NNPFC SEI message relate to changes in colors, luminance or bit depth of input images. These transformations when applied one after another may conflict with each other.

They may also conflict with already existing metadata that may be present in the bitstream also targeting some color, brightness, contrast, etc. transformations of images.

This contribution proposes to add a recommendation in the NNPFC SEI message specification to give guidance of use of the NNPFC SEI messages, to avoid such conflict.

# Introduction

In the list of already existing NNPF purposes defined in the NNPFC SEI message, some correspond to transformation of the input decoded images in terms of color, luminance or bit depth.

On the other hand, some metadata, such as CRI/CTI SEI messages or external metadata (e.g., via user data unregistered SEI message) related for example to tone mapping operations may also exist in the bitstream.

In the case that both some NNPFC SEI messages related to the above transformations and other such metadata can be found in the bitstream, several problems arise:

- These metadata and consequently their related processes may conflict with each other, or (at least partly) overlap;

- In the hypothesis that the devices apply more than one of the processes associated with these metadata, the resulting images might not be compliant with the expected outputs.

No guidance exists in the NNPFC SEI message specification to solve or avoid these problems. This contribution proposes to add a recommendation helping the devices to choose between potentially conflicting post-processing metadata.

# Proposal

NNPFC SEI messages have 6 different purposes: general visual quality improvement, chroma upsampling, resolution resampling (increasing or decreasing the width or height), picture rate upsampling, bit depth upsampling, colourization, temporal extrapolation.

Among these purposes, general visual quality improvement, colourization and bit depth upsampling are processes that directly act on the values of colours, luminance, brightness, contrast, etc. of the input image, and that may be in conflict or overlap.

This issue also exists between NNPFC SEI messages with such purposes, and other post-processing SEI messages that impact the image colours, luminance, bit depth.

As it is not possible to specify normative exclusivity rules in the application of post-processes based on SEI messages, it is proposed to add a recommendation in the specification regarding the use of these three NNPF purposes, and of possible alternate SEI messages related to operations involving potentially a change in color, brightness, contrast, bit depth of the processed images.

The following changes indicated in yellow are proposed in clause 8.28.2.2.

      - Neural-network post-filter characteristics SEI message semantics

…

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20, where ( nnpfc_purpose & bitMask ) not equal to 0 indicates that the NNPF has the purpose associated with the bitMask value in Table 20. When nnpfc_purpose is greater than 0 and ( nnpfc_purpose & bitMask ) is equal to 0, the purpose associated with the bitMask value is not applicable to the NNPF. When nnpfc_pupose is equal to 0, the NNPF may be used as determined by the application and as specified by the nnpfc_application_purpose_tag_uri.

All NNPFC SEI messages with a particular value of nnpfc_id within a CLVS shall have the same value of nnpfc_purpose.

The value of nnpfc_purpose shall be in the range of 0 to 255, inclusive, in bitstreams conforming to this version of this Specification. Values of 256 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 256 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| bitMask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x40 | Temporal extrapolation (i.e., generating one or more future pictures) |
| 0x80 | Spatial extrapolation (i.e., generating content outside of the spatial area of the input pictures), possibly also with removal (i.e. remove partial content from the input pictures) |

The variables ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, ColourizationFlag, and TemporalExtrapolationFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, colourization, and temporal extrapolation, respectively, are derived as follows:

ChromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
ResolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
PictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0
BitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
ColourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
TemporalExtrapolationFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : 0
SpatialExtrapolationFlag = ( ( nnpfc_purpose & 0x80 ) > 0 ) ? 1 : 0

NOTE 2– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value or any one of a set of values including that value.

NOTE 3 – When more than one NNPFC SEI message with different values of nnpfc_purpose are present or activated for the same picture, the corresponding processes indicated by the NNPFC SEI messages could conflict or overlap. Similarly, when another SEI message specified in this Specification is present and an NNPFC SEI message with a related purpose is present or activated, the corresponding processes indicated by the other SEI message and the NNPFC SEI message could conflict or overlap. Processing of multiple SEI messages with related purposes is not specified by this Specification and may be determined by the application.

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).