| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0147-v1 |
| --- | --- |

| Title: | AHG9: On NNPF for brightness adaptation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Claire-Hélène Demarty, Edouard François, Olivier Le Meur, Franck Aumont | Tel: Email: | 1stname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to add a new purpose, called Brightness Adaptation to the list of neural-network post-filter characteristics (NNPFC) SEI message purposes.

Assuming that the energy consumption of display devices is correlated to the amount of emitted light [1], the NNPF Brightness Adaptation purpose can be used to adapt the energy consumption of display devices.

Different testing scenarios with collection of real power measures illustrate that the use of an NNPF for Brightness Adaptation is complementary to TV built-in display energy-saving modules.

To avoid potential conflicts with other SEI messages or external messages related to other brightness adaptation operations such as Tone Mapping-related ones, some recommendations are proposed to be added in the specification related to the NNPFC SEI message.

# Introduction

This contribution is a follow-up of contributions JVET-AH016 (Neural network post-filter for recovering images from energy-aware images) and JVET-AI0062 (Neural network post-filter for tone mapping operations).

During the 35th JVET meeting and discussions related to the former proposals, several concerns were raised:

- How to handle the application of the proposed SEI in the case of the existence in the bitstream of other metadata related to tone mapping operations (e.g., CRI/CTI SEI messages, external metadata related to tone mapping processes);

- What do TV makers think about the proposal;

- How the application of the proposed SEI interacts with the processes implemented in the TV or rendering devices.

To answer these concerns, the proposal first includes some experiments that show that the proposed new NNPF for brightness adaptation brings additional energy gains and is complementary with already built-in TV energy-saving modules.

To solve potential conflict issues between existing metadata and the proposed NNPF, it is further proposed to add some usage recommendation in the specification for the current metadata.

At this point, it is important to note that such conflicts already exist between different types of metadata related to Tone Mapping operations, or between some existing NNPFs e.g., for Bit Depth upsampling and these metadata, independently of the proposal on the new NNPF purpose for Brightness Adaptation.

Reducing the amount of emitted light of screens results in reduction of their energy consumption [1]. Moreover, screens are the most consuming part of devices, with 50% of the consumed energy in the video chain for TVs, for example. While reducing the global energy consumption of electronic devices, reducing the energy consumption of their screens is therefore an interesting target.

On the other hand, several motivations support the need for reducing the energy consumption of electronic devices, among which economic, battery-related or regulation-driven motivations. One may refer to the annex for a more detailed view of these motivations.

# Experiments: comparison with built-in TV energy-saving modules

## Usage workflow

Figure 1 illustrates the global workflow of use of the proposed NNPF for Brightness Adaptation. As shown in this figure, it operates before any built-in energy-savings modules existing in a TV display. This implies that the TV modules will always be capable of adapting their processing for energy savings to the content delivered by the NNPF, as they do for any other content.

Figure 1: Workflow of use of the proposed NNPF for brightness adaptation

## Experiments protocol

All the below tests were conducted on an LG42C3 TV screen which contains a built-in energy-saving mode accessible via the Energy Saving Step option, which adapts the screen brightness to the ambient light. This module is denoted ambient-light module in the following. This module was either turned OFF or set in AUTO mode in the experiments.

The TV is located in a dark room with no window, for which the light can be turned ON/OFF, resulting in a constant ambient light.

3 sequences were used for testing, namely: RushFieldCuts_8bit, Mitsubishi_Traffic and RedKayak_8bit.

5 different target brightness reduction ratios [None, Low, Middle-low, Middle-high, High] were considered. These ratios correspond to: “no reduction” (0%) and 4 reduction values equally spread in the range [10%, 60%]. From these 5 brightness reduction ratios, 5 different versions of each sequence were constructed using the DeepPVR energy-aware algorithm proposed in [2].

For all sequences, their real power consumption was measured using a ZES Zimmer 661 power meter.

To study how both the proposed NNPF for brightness adaptation and built-in TV energy-saving modules interact while applied on content, different scenarios were set in place:

|  | TV ambient light module | Light in the room | NNPF processing |
| --- | --- | --- | --- |
| Scenario #0 (Base) | OFF | Does not matter | No |
| Scenario #1 | AUTO | ON | No |
| Scenario #2 | AUTO | OFF | No |
| Scenario #3 | OFF | Does not matter | Yes |
| Scenario #4 | AUTO | ON | Yes |
| Scenario #5 | AUTO | OFF | Yes |

Table 1: Description of tested scenarios

## Quantitative results

Table 2, Table 3 and Table 4 report respectively the measured power of the TV screen for each of the above scenarios and for the 3 tested sequences in terms of power gain compared to scenario #0, which represents the base scenario, i.e., no energy savings processing is applied by the NNPF, nor by the TV screen.

| Brightness reduction ratio | None (original) | Low | Middle-low | Middle-high | High |
| --- | --- | --- | --- | --- | --- |
| Scenario #0 | 0% | - | - | - | - |
| Scenario #1 | 6.9% | - | - | - | - |
| Scenario #2 | 15.2% | - | - | - | - |
| Scenario #3 | 0% | 3.1% | 4.8% | 9% | 12.9% |
| Scenario #4 | 6.9% | 9.3% | 10.8% | 13.5% | 16.1% |
| Scenario #5 | 15.2% | 16.3% | 17% | 18.4% | 19.9% |

Table 2: Power gains (in %) for RushFieldCuts_8bit

| Brightness reduction ratio | None (original) | Low | Middle-low | Middle-high | High |
| --- | --- | --- | --- | --- | --- |
| Scenario #0 | 0% | - | - | - | - |
| Scenario #1 | 6.1% | - | - | - | - |
| Scenario #2 | 13.6% | - | - | - | - |
| Scenario #3 | 0% | 5.8% | 7.7% | 11% | 14% |
| Scenario #4 | 6.1% | 11% | 12.1% | 14.4% | 16.3% |
| Scenario #5 | 13.6% | 16.3% | 16.8% | 18% | 19.1% |

Table 3: Power gains (in %) for Mitsubishi_Traffic

| Brightness reduction ratio | None (original) | Low | Middle-low | Middle-high | High |
| --- | --- | --- | --- | --- | --- |
| Scenario #0 | 0% | - | - | - | - |
| Scenario #1 | 9.4% | - | - | - | - |
| Scenario #2 | 19.6% | - | - | - | - |
| Scenario #3 | 0% | 3.4% | 6% | 11.7% | 17.2% |
| Scenario #4 | 9.4% | 11.5% | 13.5% | 17.6% | 21.5% |
| Scenario #5 | 19.6% | 21.1% | 22.1% | 24.1% | 26% |

Table 4: Power gains (in %) for RedKayak_8bit

As can be seen from these tables, several conclusions can be drawn:

- Using only deepPVR (scenario 3) results to comparable results in terms of gain with the TV built-in module (scenarios 1 and 2).

- Processing the input sequences with deepPVR before letting the built-in TV module to reduce the consumption, further increases the energy savings (scenarios 4 and 5).

- Results suggest that both the application of the NNPF and the TV built-in modules are complementary and lead to additional energy gains.

## Qualitative results

Additional example qualitative results are provided on sequences A_Traffic and B_BQTerrace, for the 5 reduction ratios [None, Low, Middle-Low, Middle-High, High].

Note: these are results as output by the method in [2]. No TV built-in module is considered here.

| None |  |  |
| --- | --- | --- |
| Low |  |  |
| Middle-Low |  |  |
| Middle-High |  |  |
| High |  |  |

# Proposal: option #1

A new NNPF purpose dedicated to Tone Mapping as proposed in JVET-AI0062 is added. The syntax and semantics remain unchanged and are recalled below (changes are highlighted in yellow compared to the existing specification).

A recommendation is added to the text to ease the choice at the end-device side, in case other metadata also related to Tone Mapping operations (either through some CRI, CTI SEI messages or through some external user metadata) are provided. This new text is highlighted in blue in the following.

A sub-purpose related to Brightness Adaptation is added to the new Tone Mapping purpose, to indicate that Tone Mapping operation will use brightness adaptation.

The proposed modification and the associated semantics are detailed in Table 5.

| Bitmask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x40 | Temporal extrapolation (i.e., generating one or more future pictures) |
| 0x80 | Spatial extrapolation (i.e., generating content outside of the spatial area of the input pictures) |
| 0x100 | Tone mapping (i.e., modification of the color values, in terms of contrast, color, saturation, luminance, etc.). |

Table 5: Modified list of purposes.

The value of nnpfc_purpose shall be in the range of 0 to 511, inclusive, in bitstreams conforming to this edition of this document. Values of 512 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document.

The SEI message syntax is modified as followed (highlighted in yellow):

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( nnpfc_num_input_pics_minus1 > 0 ) { |  |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_filtering_flag[ i ] | u(1) |
| nnpfc_absent_input_pic_zero_flag | u(1) |
| } |  |
| if( ChromaUpsamplingFlag ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ColourizationFlag ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ResolutionResamplingFlag ) { |  |
| nnpfc_pic_width_num_minus1 | ue(v) |
| nnpfc_pic_width_denom_minus1 | ue(v) |
| nnpfc_pic_height_num_minus1 | ue(v) |
| nnpfc_pic_height_denom_minus1 | ue(v) |
| } |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| if( SpatialExtrapolationFlag ) { |  |
| nnpfc_spatial_extrapolation_left_offset | ue(v) |
| nnpfc_spatial_extrapolation_right_offset | ue(v) |
| nnpfc_spatial_extrapolation_top_offset | ue(v) |
| nnpfc_spatial_extrapolation_bottom_offset | ue(v) |
| } |  |
| if( ToneMappingFlag ) { |  |
| nnpfc_tone_mapping_subpurpose | u(8) |
| if( TM_BrightnessAdaptationFlag ){ |  |
| nnpfc_ratio_value | u(8) |
| nnpfc_ratio_sign_flag | u(1) |
| nnpfc_display_model | u(4) |
| } |  |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| if( nnpfc_inp_order_idc != 1 ) |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_inp_order_idc > 0 ) |  |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| if( nnpfc_out_order_idc != 1 ) |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_out_order_idc != 0 ) |  |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_matrix_coeffs | u(8) |
| nnpfc_full_range_flag | u(1) |
| } |  |
| } |  |
| if( nnpfc_out_order_idc > 0 ) |  |
| nnpfc_chroma_loc_info_present_flag | u(1) |
| if( nnpfc_chroma_loc_info_present_flag ) |  |
| nnpfc_chroma_sample_loc_type_frame | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| if( nnpfc_inp_order_idc != 1 ) |  |
| nnpfc_luma_padding_val | ue(v) |
| if( nnpfc_inp_order_idc != 0 ) { |  |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| } |  |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| nnpfc_num_metadata_extension_bits | ue(v) |
| if( nnpfc_num_metadata_extension_bits > 0 ) { |  |
| if( nnpfc_purpose = = 0 ) { |  |
| nnpfc_application_purpose_tag_uri_present_flag | u(1) |
| if( nnpfc_application_purpose_tag_uri_present_flag ) |  |
| nnpfc_application_purpose_tag_uri | st(v) |
| } |  |
| if( SpatialExtrapolationFlag ) |  |
| nnpfc_scan_type_idc | u(2) |
| nnpfc_reserved_metadata_extension | u(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

The variables ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, ColourizationFlag, TemporalExtrapolationFlag, SpatialExtrapolationFlag, and ToneMappingFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, colourization, temporal extrapolation, and tone mapping respectively, are derived as follows:

ChromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
ResolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
PictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(75)
BitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
ColourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
TemporalExtrapolationFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : 0
SpatialExtrapolationFlag = ( ( nnpfc_purpose & 0x80 ) > 0 ) ? 1 : 0

ToneMappingFlag = ( ( nnpfc_purpose & 0x100 ) > 0 ) ? 1 : 0

nnpfc_tone_mapping_subpurpose indicates the sub-purpose of the tone mapping operation as specified in Table 6, where (nnpfc_tone_mapping_subpurpose & bitMask) not equal to 0 indicates the NNPF of purpose Tone Mapping has the sub-purpose associated with the bitMask value in Table 6.

The value of the nnpfc_tone_mapping_subpurpose shall be 1, in bitstreams conforming to this version of this Specification. Values of 2 to 255, inclusive, for nnpfc_tone_mapping_subpurpose are reserved for future use.

| Bitmask | Interpretation |
| --- | --- |
| 0x01 | Brightness adaptation |

Table 6: Semantic of nnpfc_tone_mapping_subpurpose

The variable TM_BrightnessAdaptationFlag specifying whether nnpfc_tone_mapping_subpurpose indicates the sub-purpose of the Tone Mapping NNPF to include brightness adaptation, is derived as follows:

TM_BrightnessAdaptationFlag = ( ( nnpfc_tone_mapping_subpurpose & 0x01 ) > 0 ) ? 1 : 0

nnpfc_ratio_value, when ToneMappingFlag is equal to 1 and TM_BrightnessAdaptationFlag is equal to 1, specifies the expected percentage of brightness adaptation, through brightness decrease or increase (accordingly with the value of the nnpfc_ratio_sign_flag field), that the Tone Mapping post-filter should achieve. The percentage of brightness adaptation is to be considered relatively to the average brightness of the content before applying the NNPF Tone Mapping. The percentage ratio is computed using the following equation:

ratio= nnpfc_ratio_value100

The expected brightness after application of the NNPF Tone Mapping may be derived as follows:

If nnpfc_ratio_sign_flag == 1:

brightnessnew=(1-ratio)*brightnessold

If nnpfc_ratio_sign_flag == 0:

brightnessnew=(1+ratio)*brightnessold

nnpfc_ratio_sign_flag specifies if the value of nnpfc_ratio_value should be considered as negative or positive. If nnpfc_ratio_sign_flag equals 0, the use of the NNPF results in an increase of content brightness. If nnpfc_ratio_sign_flag equals 1, the use of the NNPF results in a decrease in terms of content brightness.

nnpfc_display_model is a bit field mask which indicates the recommended display models on which the tone mapping neural network post filter for brightness adaptation may be used.

| Bit number | Display model |
| --- | --- |
| 0 | Transmissive pixel |
| 1 | Emissive pixel |
| 2..3 | Reserved for future types |

Table 7: Interpretation of the bits of nnpfc_display_model.

For example, nnpfc_display_type_field=112 means that the NNPF can be used for both “Transmissive pixel” and “Emissive pixel” display models.

When other metadata related to Tone Mapping operations or any operation modifying the brightness, contrast, luminance, colours or bit depth of an image are present in the bitstream together with the NNPFC SEI message with purpose corresponding to Tone Mapping, it is recommended that only one of these operations is considered.

# Proposal: option #2

A new NNPF purpose solely dedicated to Brightness Adaptation is added. Changes in the current specification are highlighted in yellow.

A recommendation is added to the text to ease the choice at the device side, in case other metadata also related to Tone Mapping operations (either through some CRI, CTI SEI messages or through some external metadata) are provided. This new text is highlighted in blue in the following.

The proposed modification and the associated semantics are detailed in Table 8.

| Bitmask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x40 | Temporal extrapolation (i.e., generating one or more future pictures) |
| 0x80 | Spatial extrapolation (i.e., generating content outside of the spatial area of the input pictures) |
| 0x100 | Brightness adaptation |

Table 8: Modified list of purposes.

The value of nnpfc_purpose shall be in the range of 0 to 511, inclusive, in bitstreams conforming to this edition of this document. Values of 512 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document.

The SEI message syntax is modified as followed (highlighted in yellow):

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( nnpfc_num_input_pics_minus1 > 0 ) { |  |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_filtering_flag[ i ] | u(1) |
| nnpfc_absent_input_pic_zero_flag | u(1) |
| } |  |
| if( ChromaUpsamplingFlag ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ColourizationFlag ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ResolutionResamplingFlag ) { |  |
| nnpfc_pic_width_num_minus1 | ue(v) |
| nnpfc_pic_width_denom_minus1 | ue(v) |
| nnpfc_pic_height_num_minus1 | ue(v) |
| nnpfc_pic_height_denom_minus1 | ue(v) |
| } |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| if( SpatialExtrapolationFlag ) { |  |
| nnpfc_spatial_extrapolation_left_offset | ue(v) |
| nnpfc_spatial_extrapolation_right_offset | ue(v) |
| nnpfc_spatial_extrapolation_top_offset | ue(v) |
| nnpfc_spatial_extrapolation_bottom_offset | ue(v) |
| } |  |
| if( BrightnessAdaptationFlag ) { |  |
| nnpfc_ratio_value | u(8) |
| nnpfc_ratio_sign_flag | u(1) |
| nnpfc_display_model | u(4) |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| if( nnpfc_inp_order_idc != 1 ) |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_inp_order_idc > 0 ) |  |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| if( nnpfc_out_order_idc != 1 ) |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| if( nnpfc_out_order_idc != 0 ) |  |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_matrix_coeffs | u(8) |
| nnpfc_full_range_flag | u(1) |
| } |  |
| } |  |
| if( nnpfc_out_order_idc > 0 ) |  |
| nnpfc_chroma_loc_info_present_flag | u(1) |
| if( nnpfc_chroma_loc_info_present_flag ) |  |
| nnpfc_chroma_sample_loc_type_frame | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| if( nnpfc_inp_order_idc != 1 ) |  |
| nnpfc_luma_padding_val | ue(v) |
| if( nnpfc_inp_order_idc != 0 ) { |  |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| } |  |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| nnpfc_num_metadata_extension_bits | ue(v) |
| if( nnpfc_num_metadata_extension_bits > 0 ) { |  |
| if( nnpfc_purpose = = 0 ) { |  |
| nnpfc_application_purpose_tag_uri_present_flag | u(1) |
| if( nnpfc_application_purpose_tag_uri_present_flag ) |  |
| nnpfc_application_purpose_tag_uri | st(v) |
| } |  |
| if( SpatialExtrapolationFlag ) |  |
| nnpfc_scan_type_idc | u(2) |
| nnpfc_reserved_metadata_extension | u(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

The variables ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, ColourizationFlag, TemporalExtrapolationFlag, SpatialExtrapolationFlag, and BrightnessAdaptationFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, colourization, temporal extrapolation, and tone mapping respectively, are derived as follows:

ChromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
ResolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
PictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(75)
BitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
ColourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
TemporalExtrapolationFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : 0
SpatialExtrapolationFlag = ( ( nnpfc_purpose & 0x80 ) > 0 ) ? 1 : 0

BrightnessAdaptationFlag = ( ( nnpfc_purpose & 0x100 ) > 0 ) ? 1 : 0

nnpfc_ratio_value, when BrightnessAdaptationFlag is equal to 1, specifies the expected percentage of brightness adaptation, through brightness decrease or increase (accordingly with the value of the nnpfc_ratio_sign_flag field), that the Brightness Adaptation post-filter should achieve. The percentage of brightness adaptation is to be considered relatively to the average brightness of the content before applying the NNPF Brightness Adaptation. The percentage ratio is computed using the following equation:

ratio= nnpfc_ratio_value100

The expected brightness after application of the NNPF Brightness Adaptation may be derived as follows:

If nnpfc_ratio_sign_flag == 1:

brightnessnew=(1-ratio)*brightnessold

If nnpfc_ratio_sign_flag == 0:

brightnessnew=(1+ratio)*brightnessold

nnpfc_ratio_sign_flag specifies if the value of nnpfc_ratio_value should be considered as negative or positive. If nnpfc_ratio_sign_flag equals 0, the use of the NNPF results in an increase of content brightness. If nnpfc_ratio_sign_flag equals 1, the use of the NNPF results in a decrease in terms of content brightness.

nnpfc_display_model is a bit field mask which indicates the recommended display models on which the brightness adaptation neural network post filter may be used.

| Bit number | Display model |
| --- | --- |
| 0 | Transmissive pixel |
| 1 | Emissive pixel |
| 2..3 | Reserved for future types |

Table 9: Interpretation of the bits of nnpfc_display_model.

For example, nnpfc_display_type_field=112 means that the NNPF can be used for both “Transmissive pixel” and “Emissive pixel” display models.

When other metadata related to Tone Mapping operations or any operation modifying the brightness, contrast, luminance, colours or bit depth of an image are present in the bitstream together with the NNPFC SEI message with purpose corresponding to Brightness Adaptation, it is recommended that only one of these operations is considered.

# References

[1] GreenHouse Gas Protocol. Corporate Value Chain (Scope 3) Accounting and Reporting Standard.https://ghgprotocol.org/sites/default/files/standards/Corporate-Value-Chain-Accounting-Reporing-Standard_041613_2.pdf

[2] Le Meur, Olivier, Claire-Hélène Demarty, and Laurent Blondé. "Deep-learning-based energy aware images." 2023 IEEE International Conference on Image Processing (ICIP). IEEE, 2023.

# Annex

Several motivations support the need for reducing the energy consumption of electronic devices as described in the below subsections.

## Economic reasons

Depending on the hours of usage, different energy prices may be applied. Reducing the consumption of electronic devices depending on the hours may result in money savings for the end user.

## Battery powered devices

Battery powered devices obviously benefit from any processing that reduces their energy consumption, resulting in an increased duration between successive battery loadings, or to the use of smaller batteries.

## Regulations

Reporting carbon emissions has already or will become a necessity for companies in the context of climate change. As a reminder, scopes are divided as follows and all three of them need to be reported in the regulations already existing or currently set in place.

- Scope 1 emissions are direct emissions that occur from sources owned or controlled by the company.

- Scope 2 emissions are indirect emissions from a company’s purchased electricity or other forms of energy.

- Scope 3 emissions are indirect emissions from upstream and downstream activities in a company’s value chain.

Scope 3 directly concerns the consumption of user’s electronic devices.

Different regulations are already applied all over the world: the CSRD, the CSDDD, the California SB 253, the ISSB standards, to force companies to report their carbon emissions in terms of scopes 1 to 3. The figure below summarizes where, to which extent and when these regulations apply.

Figure 2: Existing sustainability regulations (source)

Focusing on the CSRD, which went into effect in January 2023, its phases introduction of required reporting by company type is the following:

- 2024 (reporting in 2025): Large Public Interest Entities (PIEs) with more than 500 employees.

- 2025 (reporting in 2026): Other large companies, including those listed on the EU-regulated markets.

- 2026 (reporting in 2027): Small and medium-sized companies listed on EU-regulated market.

- 2028 (reporting 2029): Ultimate non-EU parent companies with substantial activity and presence in the EU.

As a conclusion, a large number of companies worldwide are already or will soon be concerned with reporting their carbon emissions, with the ultimate goal of reducing their emissions. All three scopes are concerned, especially scope 3 which corresponds to the use of end-user devices.

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).