| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0151-v2 |
| --- | --- |

| Title: | AHG9: Showcase for Digitally Signed Content SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Tobias Hinz Yago Sanchez de la Fuente Jonathan Pfaff Heiko Schwarz Detlev Marpe Thomas Wiegand | Email: | first.last@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution describes the showcase implementation of digitally signed content initialisation, digitally signed content selection and digitally signed content verification SEI messages, which were proposed in JVET-AI0127 and adopted into the JVET TuC. Based on the demonstrated functionality it is proposed to adopt the digitally signed content SEI messages into VSEI.

# Introduction

Digitally signed content initialisation, digitally signed content selection and digitally signed content verification SEI messages were proposed in JVET-AI0127 and adopted into TuC at the 35th JVET meeting.

An implementation was provided in MR 31 for TuC, which can be found at:

https://vcgit.hhi.fraunhofer.de/jvet-tuc/VVCSoftware_VTM/-/merge_requests/31

# Showcase

The provided reference implementation has the following functionality.

When digitally signed content SEI messages are enabled at the encoder:

- A digitally signed content initialisation SEI message is written into the bitstream at every random access point.

- A digitally signed content selection SEI message is written for every picture.

- SPS, PPS, APS, PH and slice NAL units are associated with a substream and a hash is calculated over the binary data of these NAL units.

- A digitally signed content verification SEI message is written at the end of each random access period containing a digital signature.

When digitally signed content SEI messages are present in the bitstream, the decoder:

- Parses the content of the SEI messages.

- When a digitally signed content initialization message is received, the indicated content provider certificate is checked against a list of certification authority (CA) certificates, which are stored in a local trust store and the result of the check is output.

- SPS, PPS, APS, PH and slice NAL units are associated with the indicated substream and a hash is calculated over the binary data of these NAL units in each substream.

- When the digitally signed content verification SEI message is received for a substream, the signature for that substream is verified and the result of the verification is output.

- When NAL units associated to a specific substream are received after the digitally signed content verification SEI message and before a new digitally signed content initialization SEI message, a warning is output.

Hash and signature functionalities are implemented using the OpenSSL library version 3. If OpenSSL is not available during compilation (CMake), only support for parsing the SEI messages is compiled into the decoder. Since cryptography is not in the scope of JVET, it’s best practice to rely on a well-tested implementation. Optional usage of the library allows users, who do not need the digitally signed content SEI, to compile and run the software without additional burden.

And example configuration file is provided for the encoder, as well as example cryptographic keys and certificates for an example content provider and an example certification authority. Note that these example keys should never be used in production environments.

Limitations:

- The example encoder currently uses only one substream. Multiple substreams are supported at the decoder.

- The decoder currently only supports “file://” URIs, where the content provider certificate is expected to be stored in a key store that is available to the decoder.

- Signing and verification are directly integrated into encoder and decoder. The functionality could be implemented separately after encoding or before decoding with very limited syntax parsing/writing functionality.

The following is an example encoder command line call using the example configuration file:

./bin/EncoderApp -c ./cfg/encoder_randomaccess_vtm.cfg -c ./cfg/per-sequence/RaceHorses.cfg -c ./cfg/sei_vui/digitally_signed_content.cfg -ipp <sequence_directory> --SEIDecodedPictureHash=1

The following image shows the output when content signing is active:

The following is an example decoder command line call using the bitstream written by the encoder:

./bin/DecoderApp -b str.bin --TrustStoreDir=./keystore/ca --KeyStoreDir=./keystore/pub/

The following image shows the output for a valid CA signed content provider key and a valid content signature:

The following image shows the output for an invalid content signature:

# Showcase #2

The above-described signature verification functionality was implemented into a pre-filter for VVdeC.

The new module is placed between the file reader and the decoder. It scans the NAL units for SEI NAL units and parses these, if Digitally Signed Content SEIs are found. The verification code is initialized with these SEIs and following NAL units are passed into it. When the signature is checked, the status is passed through the VVdeC decoder. At the output a screen overlay is rendered in the left upper corner according to the verification status. In this example we use a green lock, if the content is successfully verified and a red lock, if the verification fails.

# Proposal

Based on the demonstrated functionality and feasibility of the digitally signed content SEI messages, we propose to adopt these SEI messages into the current draft of VSEI.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).