| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0161 |
| --- | --- |

| Title: | EE2-3.3: OBMC extension with intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Donghyun Kim Jinho Lee Jongho Kim Woong Lim Sung-Chang Lim Jin Soo Choi 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5369 kimddng@etri.re.kr jinosoul@etri.re.kr pooney@etri.re.kr woong.lim@etri.re.kr sclim@etri.re.kr jschoi@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution reports the EE2 test 3.3 results of the overlapped block motion compensation (OBMC) extension with intra prediction, as proposed in JVET-AI0154 at the previous meeting. In this OBMC extension, in addition to existing OBMC process, top and left boundary pixels of the current block are blended using intra prediction block generated with the intra prediction mode derived by applying decoder-side intra mode derivation (DIMD) on the neighboring reconstructed samples. In test 3.3a, OBMC extension with intra prediction is tested. In test 3.3b, DIMD with 2x2 edge operator is applied instead of the 3x3 edge operator to derive intra prediction mode from the neighbouring reconstructed samples. In test 3.3c, instead of using DIMD-derived mode, intra prediction mode of neighbouring block is used to generate intra subblock. In test 3.3d, in addition to test 3.3a, more neighbouring blocks are checked for available motion information to generate inter subblock. The EE2 test 3.3 results on top of ECM-14.0 are reported as follows:

Test 3.3a: OBMC extension with intra prediction

- RA: {-0.05%, -0.13%, -0.04%, EncT 99.9%, DecT 100.3%};

- LDB: {-0.08%, 0.13%, -0.06%; EncT 100.7%, DecT 100.8%}.

Test 3.3b: Test 3.3a + DIMD with 2x2 edge operator

- RA: {-0.05%, -0.08%, 0.03%, EncT 100.4%, DecT 100.4%};

- LDB: {-0.04%, 0.10%, -0.49%; EncT 100.3%, DecT 100.5%}.

Test 3.3c: Test 3.3a + using intra prediction mode of neighbouring block

- RA: {x%, x%, x%, EncT x%, DecT x%};

- LDB: {-0.02%, 0.07%, -0.17%; EncT 100.0%, DecT x%}.

Test 3.3d: Test 3.3a + more neighbouring blocks are checked for available motion information to generate inter subblock

- RA: {x%, x%, x%, EncT x%, DecT x%};

- LDB: {x%, x%, x%; EncT x%, DecT x%}.

Introduction

In OBMC of ECM, top and left boundary pixels of the current block are only blended with inter prediction block generated using motion information of neighbouring block. However, top and left boundary pixels adjacent to intra block remain unrefined due to the absence of motion information of neighbouring intra block. Consequently, discontinuities at these boundary pixels may still be present, which can result in large residual signals at the boundary. To address this issue, the OBMC extension was proposed in JVET-AI0154 [1]. In the proposed OBMC extension, in addition to the existing OBMC process, the top and left boundary pixels adjacent to intra block are blended with intra prediction subblock generated using the intra prediction mode derived by applying DIMD on the neighbouring reconstructed samples. Figure 1 illustrates an example of the OBMC extension with intra prediction, showing how boundary pixels adjacent to intra blocks are processed.

Figure 1. An example of the OBMC extension with intra prediction

Furthermore, the blending of top and left boundary pixels adjacent to intra blocks is performed only when the intra prediction mode derived from DIMD falls within the range defined according to the availability of neighboring reconstructed samples. This condition limits the usage of padded reference samples for intra prediction.

Test description

Test 3.3a: OBMC extension with intra prediction

In test 3.3a, OBMC is extended to perform the refinement of the top and left boundary pixels adjacent to intra block. These pixels are blended using an intra prediction block generated with the intra prediction mode derived by applying DIMD on the neighbouring reconstructed samples.

Test 3.3b: Test 3.3a + DIMD with 2x2 edge operator

In test 3.3b, the 3x3 edge operator used in test 3.3a is modified to the 2x2 edge operator proposed in JVET-AI0140 [2] to derive the intra prediction mode in DIMD. The horizontal and vertical filters of the 2x2 edge operator are defined as follows:

Fx=-1-111, Fy=1-11-1

Test 3.3c: Test 3.3a + using intra prediction mode of neighbouring block

In test 3.3c, instead of deriving intra prediction mode with DIMD as in test 3.3a, intra prediction mode of the neighbouring block is used to generate the intra prediction block.

Test 3.3d: Test 3.3a + more neighbouring blocks are checked for available motion information to generate inter subblock

On top of test 3.3a, the search positions for available motion information are expanded as follows:

- For subblocks on the top boundary: In addition to the above block, the blocks to its left and right are also checked.

- For subblocks on the left boundary: In addition to the left block, the blocks above and below it are also checked.

Experimental results

The proposed method was implemented on top of ECM-14.0 [3] and tested under JVET ECM CTC [4]. The EE2 test results of tests 3.3a to 3.3d for random access (RA) and low-delay B (LDB) are summarized in Tables 1 to 4, respectively.

Table 1. Experimental results of test 3.3a

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.16% | -0.11% | 100.8% | 100.7% | 100.4% | 100.3% |
| Class A2 | -0.07% | -0.05% | -0.06% | 100.6% | 99.4% | 100.4% | 100.3% |
| Class B | -0.07% | -0.15% | -0.07% | 100.3% | 100.4% | 100.1% | 100.0% |
| Class C | -0.06% | -0.14% | 0.06% | 98.3% | 100.7% | 100.0% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.13% | -0.04% | 99.9% | 100.3% | 100.2% | 100.2% |
| Class D | -0.05% | -0.03% | 0.04% | 99.8% | 101.0% | 100.3% | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | 0.05% | -0.02% | 100.8% | 100.7% | 100.0% | 99.9% |
| Class C | -0.05% | 0.15% | 0.06% | 100.4% | 101.2% | 100.0% | 100.1% |
| Class E | -0.18% | 0.24% | -0.28% | 100.7% | 100.4% | 101.3% | 100.0% |
| Overall | -0.08% | 0.13% | -0.06% | 100.7% | 100.8% | 100.3% | 100.0% |
| Class D | -0.04% | 0.46% | 0.86% | 100.3% | 100.5% | 99.9% | 100.2% |

Table 2. Experimental results of test 3.3b

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.18% | -0.23% | 100.9% | 100.9% | 100.1% | 99.9% |
| Class A2 | 0.00% | -0.03% | -0.03% | 100.4% | 99.7% | 99.9% | 100.0% |
| Class B | -0.08% | -0.07% | 0.09% | 100.0% | 100.3% | 100.0% | 99.9% |
| Class C | -0.06% | -0.06% | 0.19% | 100.4% | 100.8% | 100.0% | 100.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.08% | 0.03% | 100.4% | 100.4% | 100.0% | 100.0% |
| Class D | -0.05% | -0.32% | -0.07% | 99.7% | 100.6% | 100.3% | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.03% | 0.03% | -0.36% | 100.9% | 100.6% | 100.0% | 100.4% |
| Class C | -0.05% | 0.15% | -0.07% | 99.5% | 100.6% | 100.0% | 100.1% |
| Class E | -0.05% | 0.15% | -1.28% | 100.3% | 100.2% | 101.2% | 100.2% |
| Overall | -0.04% | 0.10% | -0.49% | 100.3% | 100.5% | 100.3% | 100.3% |
| Class D | 0.05% | 0.01% | 0.52% | 98.5% | 100.5% | 100.1% | 100.3% |

Table 3. Experimental results of test 3.3c

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.27% | -0.26% | 100.7% | #NUM! | 100.0% | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0.05% | -0.10% | -0.02% | 100.0% | #NUM! | 100.0% | #NUM! |
| Class C | -0.05% | -0.03% | 0.08% | 100.8% | #NUM! | 100.0% | #NUM! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.06% | -0.16% | -0.03% | 100.4% | #NUM! | 100.0% | #NUM! |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.04% | -0.03% | 100.7% | #NUM! | 100.2% | #NUM! |
| Class C | 0.05% | 0.19% | -0.02% | 99.8% | #NUM! | 100.0% | #NUM! |
| Class E | -0.11% | -0.05% | -0.62% | 99.0% | #NUM! | 101.2% | #NUM! |
| Overall | -0.02% | 0.07% | -0.17% | 100.0% | #NUM! | 100.4% | #NUM! |
| Class D | -0.02% | 0.81% | 0.16% | 99.4% | #NUM! | 99.7% | #NUM! |

Table 4. Experimental results of test 3.3d

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0.04% | -0.09% | -0.16% | 100.3% | #NUM! | 99.9% | #NUM! |
| Class C | -0.05% | 0.01% | 0.18% | 100.6% | #NUM! | 100.4% | #NUM! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.01% | -0.09% | -0.18% | 100.1% | #NUM! | 100.1% | #NUM! |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | 0.05% | -0.04% | 0.10% | 99.4% | #NUM! | 100.0% | #NUM! |
| Class E | -0.03% | 0.46% | -0.97% | 99.5% | #NUM! | 100.9% | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | 0.05% | -0.06% | 0.01% | 100.4% | #NUM! | 99.7% | #NUM! |

Conclusions

This contribution reports the test results of the EE2 test 3.3, the OBMC extension which enables the refinement of top and left boundary pixels adjacent to intra blocks. The experimental results show that the proposed method achieves coding gains under RA and LDB cases with negligible complexity impact. Therefore, it is recommended to adopt EE2 test 3.3a into the next version of ECM.

References

- D. Kim, J. Lee, J. Kim, W. Lim, S.-C. Lim and J. S. Choi, “Non-EE2: OBMC extension with intra prediction”, JVET-AI0154, Jul. 2024.

- A. Filippov, V. Rufitskiy, J. Konieczny, H. Qin, T. Dong and Z. Xu, “Non-EE2: On DIMD edge operators”, JVET-AI0140, Jul. 2024.

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AI2017, Jul. 2024.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).