| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0186-v1 |
| --- | --- |

| Title: | EE2-5.3: Extended Usage of Fixed-Filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Yang Wang1, Zhipin Deng1, Na Zhang1, Lei Zhao1, Mehdi Salehifar2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-5.3. It is proposed to apply fixed-filters in Adaptive Loop Filter (ALF) after Deblocking Filter (DBF) to perform a refinement process at this stage. The output of the proposed method will be further used as input of the following tools, such as BF/SAO/CCSAO.

On top of ECM-14.0, simulation results of the proposed method are reported as below:

AI: -0.08%, -0.03%, -0.04%, 100.4%, 100.7%.

RA: -0.06%, -0.01%, -0.04%, 100.3%, 100.9%.

LB: -0.10%, 0.17%, -0.41%, 100.4%, 101.3%.

Introduction

In ECM-14.0, the fixed-filters of ALF are designed with classification and offline-trained coefficients that are already stored at both encoder and decoder. The fixed-filters of ALF take the reconstruction samples before DBF and the reconstruction samples before ALF as the input to produce the filtered results, which can be either directly used as the output of the ALF process or as the input of the online-filters of ALF.

Proposed Method

In EE2-5.3, the fixed-filters defined in ALF are reused after the DBF process, as shown in Figure 1.

Figure 1: The Proposed Method in Loop-Filter Progress.

The proposed method reuses the fixed-filters of ALF and takes the input samples before DBF and the output samples after DBF as input to produce the filtered results at this stage. The classification and filtering logic of the fixed-filters are directly reused. The ALF process is kept unchanged, including both offline-filtering and online-filtering. A flag is signaled in the slice header to achieve on/off control.

Simulation Results

The proposed method was implemented on top of the ECM-14.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets.

Table 1 Performance of the EE2-5.3

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | 0.00% | -0.03% | 100.2% | 100.3% | 100.4% | 101.5% |
| Class A2 | -0.07% | -0.02% | -0.02% | 100.7% | 99.8% | 98.6% | 101.5% |
| Class B | -0.06% | -0.06% | -0.07% | 100.5% | 101.0% | 102.0% | 100.7% |
| Class C | -0.07% | -0.02% | -0.04% | 100.3% | 100.8% | 99.6% | 100.2% |
| Class E | -0.14% | -0.06% | -0.04% | 100.2% | 101.4% | 99.1% | 100.5% |
| Overall | -0.08% | -0.03% | -0.04% | 100.4% | 100.7% | 100.1% | 100.8% |
| Class D | 0.00% | 0.04% | 0.05% | 99.9% | 101.4% | 100.1% | 100.1% |
| Class F | -0.16% | -0.01% | 0.07% | 99.2% | 100.2% | 101.9% | 100.6% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.15% | -0.17% | 100.0% | 101.0% | 100.0% | 101.0% |
| Class A2 | -0.04% | 0.04% | -0.06% | 100.0% | 100.7% | 100.4% | 101.2% |
| Class B | -0.06% | -0.08% | -0.09% | 101.0% | 101.4% | 100.0% | 100.5% |
| Class C | -0.08% | 0.14% | 0.13% | 99.9% | 100.2% | 100.2% | 100.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.06% | -0.01% | -0.04% | 100.3% | 100.9% | 100.1% | 100.7% |
| Class D | -0.04% | 0.17% | 0.01% | 99.3% | 99.7% | 100.1% | 100.1% |
| Class F | 0.00% | -0.15% | 0.10% | 100.7% | 100.1% | 100.1% | 100.3% |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | 0.14% | -0.13% | 100.7% | 101.8% | 100.2% | 100.9% |
| Class C | -0.05% | 0.25% | -0.10% | 99.9% | 100.6% | 100.1% | 100.2% |
| Class E | -0.22% | 0.12% | -1.29% | 100.5% | 101.5% | 99.2% | 100.3% |
| Overall | -0.10% | 0.17% | -0.41% | 100.4% | 101.3% | 99.9% | 100.6% |
| Class D | 0.02% | 0.95% | 0.40% | 99.8% | 100.2% | 99.8% | 100.2% |
| Class F | -0.05% | -0.38% | 0.40% | 99.1% | 101.4% | 100.6% | 100.5% |

Conclusions

In this contribution, the test results of EE2-5.3 are reported. The fixed-filters of ALF are reused after DBF to refine the reconstruction quality at this stage. The proposed method can achieve promising coding performance with negligible coding time changes. It is recommended to include the proposed method into the next version of ECM.

Reference

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AI2017, JP, July 2024.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).