| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ0192 |
| --- | --- |

| Title: | AHG15: Low delay configuration for gaming content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Utida S. Puri I. Marzuki C. Bonnineau | Tel: Email: | Renan.Utida@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

One of the mandates of AhG15 is the investigation and proposal of test conditions for gaming applications. This contribution proposes a new low delay configurations aiming at encoding time reduction for the worst cases of the gaming sequences. Currently, out of the nine sequences in classes G1 and G2 (SDR content), four have slower encoding time for QP27 than for QP22, under LDB, and seven under LDP. The worst cases reductions are as follows:

LDB: Worst case (QP27) reduces from 22 to 13 days, making new worst case (QP22) of 17 days.

LDP: Worst case (QP27) reduces from 15 to 9 days, making new worst case (QP22) of 11 days.

For both cases, the average encoding time reduction for QP27 is around 30%.

# Introduction

During the evaluations under AhG15, it has been proposed [1] and decided to use the same per-class configurations used in CTC. However, for low delay configurations, the encoding time of QP27 is undesirably slower than that for QP22 on the same sequence.

On previous occasions [2,3], the encoding time reduction was achieved on the worst-case scenario on low delay tests by changing the Max MTT Depth by Tid for larger Tid values, for QP22. Currently, classB_lowdelay.cfg contains the following parameter setting:

MaxMTTHierarchyDepthByTidOverrideByQP: 22 322222

This contribution proposes to apply the same usage of MaxMTTHierarchyDepthByTidOverrideByQP done for QP22 also for QP27, i.e., thus using a new configuration file for tests of gaming content under low delay, i.e., by setting the two following parameters:

MaxMTTHierarchyDepthByTidOverrideByQP: 22 322222

MaxMTTHierarchyDepthByTidOverrideByQP: 27 322222

# Results

The results for the proposed configuration are provided below. The tests were executed on ECM-14.0, under the AhG15 gaming CTC [4]. The figures highlight the EncT results for QP27 that are larger than their QP22 counterparts. With the proposed new configuration, all EncT for QP27 are lower than their QP22 counterparts. The largest reductions are 8.49 days and 6.25 for LDB and LDP, respectively. For both cases, the average encoding time reduction for QP27 is around 30%. Figures 1 and 2 show the results for LDB and LDP, respectively.

Figure 1: Comparative results for LDB.

Figure 2: Comparative results for LDP.

Next, the average overall results are shown in Figure 3. The overall encoding loss is balanced by significant encoding time reduction, as expected.

Figure 3: Overall results.

# Conclusion

This contribution proposes a new configuration file classG_lowdelay.cfg for testing gaming content under low delay. The same configuration parameter MaxMTTHierarchyDepthByTidOverrideByQP used for QP22 should also be used for QP27 to considerably reduce encoding time for worst-case scenario.

It is proposed to adopt this contribution in the CTC for gaming applications, substituting all uses of  classB_lowdelay.cfg for classG_lowdelay.cfg.

# References

- JVET-AH0220, “AHG15: Configurations for gaming sequences”, 	R. Utida, S. Puri, I. Marzuki, C. Bonnineau, April 2024

- JVET-AE0163, “Ahg 12: modified CTC proposal for low-delay configurations”, S. Puri, C. Bonnineu, F. Le Leannec, T. Poirier, InterDigital, July 2023

- JVET-AH0180, “Modified CTC for encoding time reduction under low-delay config”, 	R. Utida, S. Puri, I. Marzuki, C. Bonnineau, April 2024

- JVET-AI2027, “Draft CTC for gaming applications”, 	J. Sauer, R. Chernyak, S. Puri, S. Thiebaud, July 2024

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).